Equipment Communication Specifications HSMS Type Ver 1.6 for LG Display ToE Counter Specification

Ver 1.0

- Revision History -

| Version | Date | Person | Contents |
| --- | --- | --- | --- |
| 1.0 | 2021.07.28 | 박효근 | Version 1.0 for LG Display ToE |

목차

1	장비구성	5

2	전기적 인터페이스 및 통신 규약	5

2.1	Frame Type	5

2.2	Connector	5

2.3	Ethernet Media	5

2.4	Signal Pins	5

2.5	HSMS Message Format	6

2.5.1	Message Format	6

2.6	Message Header	6

2.7	The method of communication	7

2.7.1	통신연결	7

2.7.2	Parameter 설정	7

2.7.3	Transaction	7

2.7.4	Procedures Description	8

3	SECS - II	9

3.1	Abort Transaction (S*FO)	9

3.2	Conversation Timeout	9

3.3	Interleaved Requests Interaction	9

4	Interface	10

5	Communication Protocol	10

6	지원되는 Steam Function	11

7	Message 구조	13

7.1	Standard Message Structure	13

7.2	Exception Message Structure	57

8	ECS 통신시 상태, 모드 시점	77

8.1	Alarm 발생 및 해제 시점	77

8.2	장비상태	77

8.2.1	Glass 상태 보고	77

8.2.2	장비 상태 보고	77

8.2.3	시간 동기화(S2F17)	77

8.2.4	Loopback Test(S2F25)	77

8.2.5	Conversation Timeout(S9F13)	77

9	Transaction Sequence	78

9.1	Glass Loading	78

9.1	Glass Exchange	79

9.1	Glass Unloading	80

10	Alarm List	81

11	Recipe	88

12	Scenario	89

12.1	Control State and Equipment Operation Mode Change	89

12.1.1	Are You There Request	89

12.1.2	Offline → Online Remote	90

12.1.3	Offline → Online Local	91

12.1.4	Online Remote → Online Local	92

12.1.5	Online Local → Online Remote	92

12.1.6	Online(Remote or Local) → Offline	93

12.1.7	Equipment Operation Mode Change (Under same Control State)	93

12.1.8	ECS Restart	94

12.2	Loop Back Diagnostics	95

12.2.1	Equipment initiates Loop Back Test (ECS  EQP)	95

12.2.2	ECS initiates Loop Back Test (ECS  EQP)	95

12.3	Glass Transfer Sequence	96

12.3.1	Glass Move State ‘LR’ Request(Load Request)	96

12.3.2	Glass Move State ‘UR’ Request - Only Unload Stage Type	98

12.3.3	Glass Move State ‘ER’ Request	99

12.3.4	Glass Move State ‘CR’ Request(Cancel Request)	104

12.3.5	Load Glass Data Change Case	106

12.3.6	Glass Transfer In Online Local Control State	119

12.4	LOT Control Sequence	121

12.4.1	LOT End Data Send To Equipment	121

12.4.2	Local LOT Status Change Request on Online Local Control State	122

12.5	Status Report	123

12.5.1	Equipment Status Change Report	123

12.5.2	All Equipment Status Report	123

12.5.3	All Equipment Status Request	123

12.5.4	OCR( or VCR ) Status Report	123

12.5.5	Ionizer Status Report	123

12.6	Recipe Management	124

12.6.1	Recipe ID List Request	124

12.6.2	Current Recipe ID Group List Request	124

12.6.3	Current Recipe ID List Request by Group	124

12.6.4	Recipe ID Confirm Request	124

12.6.5	Recipe Body Request	124

12.6.6	Recipe ID Confirm & Sampling Data Send	125

12.7	Date and Time Data Send	126

12.7.1	Date and Time Request	126

12.7.2	Date and Time Set	126

12.8	Alarm Occurrence and Clear	127

12.8.1	Heavy Alarm	127

12.8.2	Light Alarm	127

12.9	Scrap Occurrence and Report	128

12.10	All Glass Position Data Report	128

12.11	Cycle Stop Status Report	129

12.12	Rework Request	130

12.13	Dummy Lot Request	131

12.14	Glass Data Request	131

12.15	Chemical Change	131

12.16	Normal Operation	132

12.17	Lot Abort Scenario	137

12.17.1	The case where glass would be loaded is exist	137

12.17.2	The case where glass would be loaded is not exist	139

12.18	Error Messages	140

12.18.1	Message fault due to unrecognized Device ID	140

12.18.2	Message fault due to unrecognized Stream Type	140

12.18.3	Message fault due to unrecognized Function Type	140

12.18.4	Message fault due to Illegal Data Format	140

12.18.5	Message fault due to Transaction Timer Timeout	140

12.18.6	Message fault due to data too long	141

12.18.7	Message fault due to Conversation Timeout	141

12.19	Equipment Terminal Service	141

13	Exception Scenario	142

13.1	Inform processing glass count change scenario	142

13.2	Monitoring Lot Status Send	142

13.3	Equipment’s Part Data Report	142

13.4	Special Case of Glass Judge Report	143

13.5	Chamber Status Change Report	143

13.6	Recipe Management Sequence	144

13.6.1	Equipment Recipe List Request	144

13.6.2	Equipment Lower Recipe List Request	144

13.6.3	Equipment Recipe Parameter Request	144

13.6.4	Equipment Recipe Parameter Change Send	144

13.6.5	Equipment Recipe Change Report	144

13.6.6	Equipment Operation Reference Data Send	145

13.6.7	Equipment Operation Reference Data Apply Report	145

13.6.8	Current Recipe ID List Request	145

13.6.9	Equipment Constant Recipe Request	145

13.6.10	Equipment Constant Recipe Parameter Request	145

13.6.11	Equipment Constant Recipe Parameter Change Command	146

13.6.12	Equipment Constant Recipe Parameter Change Report	146

13.7	Buffer Lot Status Send	146

13.8	Remained Glass Count Request	146

13.9	Glass Call Request	147

13.10	Carrier Transfer Mode Change Report	147

13.10.1	장비의 온라인 상태로 변경	148

13.11	PD Down Request	149

13.12	LOT End Glass Injection Send	150

13.12.1	In case the normal sequence	150

13.12.2	In case the abnormal sequence	150

13.13	Multi Recipe Information Send	151

13.14	Port State and Cassette Map Data Send Scenario	151

13.15	Batch Job (Glass Injection) End Data Send	152

13.16	Injecting Glass Data Request	152

13.17	Port State Data Report	153

13.18	Port Status Change Data Report	153

13.19	Lot State Change Report	154

13.20	Mask Data Report	154

13.21	All Mask Position Data Report	155

13.22	Unload Process Report	156

13.22.1	Unload Start & End	156

13.22.2	Unload Start & End (EQP Restart After Unload Start) 1/2	157

13.22.3	Unload Start & End (EQP Restart After Unload Start) 2/2	158

13.22.4	Unload Start & End (ECS Restart After Unload Start) 1/2	159

13.22.5	Unload Start & End (ECS Restart After Unload Start) 2/2	160

장비구성

장비는 1개의 Stage와 1개의 Sub unit으로 구성되어 있다.

|  | NAME | Note |
| --- | --- | --- |
| EQP ID | PXCEPI0102 |  |
| UNITID | 102 |  |
| SUBUNITID | 100 |  |

전기적 인터페이스 및 통신 규약

Frame Type

IEEE 802.3u - Fast Ethernet (100Mbps)

Connector

RJ-45 (8-pin)

Ethernet Media

100 Base TX (Twisted Pair)

Signal Pins

Following table shows each pin of Connector functions(Pin 1,2,3,6 used only.)

| Pin | Signal | Description |
| --- | --- | --- |
| 1 | Transmit + | Transmit Positive Signal |
| 2 | Transmit - | Transmit Negative Signal |
| 3 | Receive + | Receive Positive Signal |
| 4 | Not used | - |
| 5 | Not used | - |
| 6 | Receive - | Receive Negative Signal |
| 7 | Not used | - |
| 8 | Not used | - |

[Process]	                           [ECS]

1----------------------------------------------------------1

2----------------------------------------------------------2

3----------------------------------------------------------3

6----------------------------------------------------------6

HSMS Message Format

Message Format

Maximum Message Length is 2 Mbytes.

| Number of Bytes | Description |
| --- | --- |
| 4 Bytes | Message Length. MSB First. |
| 10 Bytes | Message Header |
| 0 – n Bytes | Message Text |

Message Header

Following table shows the message header structure,

and P type always uses "0"(SECS-II Encoding).

| Bytes | Description |
| --- | --- |
| 0~1 | Session ID (Device ID). |
| 2 | Header Byte 2 |
| 3 | Header Byte 3 |
| 4 | P type |
| 5 | S type |
| 6~9 | System Bytes |

Following table shows the selectable value in S type.

Deselect and Reject Procedure is not used.

| Value | Description |
| --- | --- |
| 0 | Data Message |
| 1 | Select.req |
| 2 | Select.rsp |
| 5 | Linktest.req |
| 6 | Linktest.rsp |
| 9 | Separate.req |

The method of communication

통신연결

TCP/IP (Transmission Control Protocol/Internet Protocol)

Parameter 설정

HSMS 구현은 반드시 아래의 parameters에 대한 시간정보 설정에 대비해야 한다. Protocol Parameter의 범위 및 해결방안은 최소한 아래 표와 같이 나타나야 한다. 각각의 설정치는 사용자에 의해 장비에서 수시로 변경 가능하도록 한다.

단 connect mode는 항상 장비가 ACTIVE 이고, ECS가 PASSIVE이다.

| Symbol | Parameter Name | Typical Value | Value Range |
| --- | --- | --- | --- |
| DEVID | Device ID(Session ID) | 0 | 0 ~ 32767 |
| T3 | Reply Timeout | 90 sec | 1 ~ 120 sec |
| T5 | Separation Timeout | 10 sec | 1 ~ 240 sec |
| T6 | Transaction Timeout | 5 sec | 1 ~ 240 sec |
| T7 | NOT SELECTED Timeout | 10 sec | 1 ~ 240 sec |
| T8 | Intercharacter Timeout | 5 sec | 1 ~ 120 sec |
| CT | Conversation Timeout | 180 sec | 0 ~ 600 sec |
| Connect Mode | Connect Mode (Active/Passive) | - | - |
| Local Entity | Local Entity IP Address & Port Number | - | - |
| Remote Entity | Remote Entity IP Address & Port Number | - | - |

Transaction

Transaction allowed in states.

| Transaction | Allowed in State | Who initiates Transaction? |
| --- | --- | --- |
| Select | HSMS Not Selected | Active Entity |
| Linktest | HSMS Selected | Either Entity |
| Separate | HSMS Selected | Either Entity |

Procedures Description

2.2.4.1	“Select”(선택 절차)

선택 절차는 컨트롤 상태에서 선택 요청(Select req) 메시지를 사용하는 TCP/IP 연결상의 HSMS통신을 설정 할 때 사용된다. 선택 절차는 반드시 “NOT SELECTED” 상태에서만 가능하다. 성공적으로 0 선택 상태로 전환되지 못한 선택 절차는 반드시 TCP/IP 통신을 닫고, “Not CONNECTED” 상태로 전환되어야 한다.

2.2.4.2	“Linktest”(연결테스트 절차)

링크 테스트는 TCP/IP와 HSMS 통신의 동작상의 완결성을 결정짓는데 사용된다. “CONNECTED” 에서 이것의 사용은 언제라도 허용된다. 링크 테스트는 반드시 장비 연결 테스트에 적용된다.

- SELECTED’상태에서 장비 (Active)는 ECS (Passive)로 반드시 주기적으로 ‘Linktest req’를 보내야 한다.

- ‘SELECTED’상태, 그리고 (Active)로부터 120초 동안 아무런 메시지를 받지 못했을 때, ECS (Passive)는 장비 (Active)로 반드시 ‘Linktest req’를 보내야 한다.

2.2.4.3	“Separate”(분리 절차)

분리 절차는 TCP/IP 통신이 끊어지기 전에 갑자기 HSMS 통신이 끊어졌을 때 사용된다. HSMS는 연결 상태가 ‘SELECTED’ 상태에서 분리를 사용했을 때 요구한다. 실제 응답을 보내지 않고, 로컬 상태인지의 여부와는 상관없이 통신의 중단을 요구한다.

- 상태가 ‘SELECTED’에서 ‘NOT SELECTED’로 변경이 될 때, EQP는 반드시 ‘Separate.req’를 ECS로 보낸다.

SECS - II

Abort Transaction (S*FO)

Function 0은 모든 Stream에 존재하며 각각의 Stream에서 같은 특별한 의미를 가진다. Function 0 Message는 Transaction Timeout이 발생하는 것을 기다리지 않기 위해 Transaction을 종료한다. Function 0 은 Transaction Error 나 다른 몇 가지의 원인으로 인해 해석기가 기대되는 응답을 보낼 수 없을 때 그것을 대신하여 보내 짐. 해석기가 Transaction을 종료하기 위해 Function 0을 보내는 것은 요구가 아니다.

Conversation Timeout

CT (Conversation Timeout)는 ECS로부터 적정한 시간 내에 기대하는 응답을 받지 못했을 때 발생하는 일종의 timeout 메시지이다.

Interleaved Requests Interaction

비 동시적인 메시지의 상호작용은 작업 중 동시에 하나이상의 요청을 포함할 수도 있다. 예를 들어, ECS가 장비로부터 여러 가지의 동작 (응답 메시지)를 요구한다. 장비는 그 행동을 수행하고, 독립적으로 결과를 보내준다. ECS가 두 개(혹은 그 이상)의 Primary 메시지를 순서대로 보낼 때, 장비는 반드시 모든 Primary 메시지에 대해 Secondary 메시지를 보내야 한다. ECS는 응답 메시지를 결정되지 않은 순서대로 받는다. 평행적인 요구에 대한 이 메커니즘은 분산처리 수행과 스레드 안전 프로그래밍의 복잡한 요구가 아닌 비 동시적인 메시지의 자연스러운 사용이다.

Interface

Communication Protocol

지원되는 Steam Function

| S | F (Primary) | F (Secondary) | Primary Direction | Name of Function |
| --- | --- | --- | --- | --- |
| * | 0 | - | ECS ↔ EQP | Abort Transaction |
| 1 | 1 | 2 | ECS ↔ EQP | Are You There Request |
| 1 | 61 | 62 | ECS ← EQP | All Equipment Status Report |
| 1 | 63 | 64 | ECS → EQP | All Equipment Status Request |
| 1 | 67 | 68 | ECS ← EQP | All Glass Position Data Report |
| 1 | 75 | 76 | ECS ← EQP | Ionizer Status Report |
| 2 | 17 | 18 | ECS ← EQP | Date and Time Request |
| 2 | 25 | 26 | ECS ↔ EQP | Loopback Diagnostic Request |
| 2 | 31 | 32 | ECS → EQP | Date and Time Data Send |
| 2 | 61 | 62 | ECS ← EQP | Control State Change Request |
| 2 | 63 | 64 | ECS ← EQP | Equipment Operation Mode Report |
| 2 | 65 | 66 | ECS ← EQP | Local LOT Status Change Request |
| 2 | 67 | 68 | ECS ← EQP | Cycle Stop Status Report |
| 2 | 69 | 70 | ECS → EQP | Unload Glass Data Request |
| 2 | 75 | 76 | ECS ← EQP | Chemical Change Status Report |
| 2 | 79 | 80 | ECS ← EQP | Glass Data Request |
| 2 | 87 | 88 | ECS → EQP | Load Glass Data Send |
| 2 | 95 | 96 | ECS ← EQP | Chemical Change Enable Request |
| 2 | 97 | 98 | ECS → EQP | Chemical Change Enable Send |
| 5 | 1 | 2 | ECS ← EQP | Alarm Report |
| 6 | 61 | 62 | ECS ← EQP | Load Glass Data Request |
| 6 | 63 | 64 | ECS ← EQP | Unload Glass Data Report |
| 6 | 65 | 66 | ECS → EQP | LOT End Data Send |
| 6 | 67 | 68 | ECS ← EQP | Glass Move Request |
| 6 | 69 | 70 | ECS ← EQP | Cassette Map Request |
| 6 | 71 | 72 | ECS ← EQP | Sampling Slot Request |
| 6 | 73 | 74 | ECS ← EQP | Glass Actual Processing Data Report |
| 6 | 83 | 84 | ECS ← EQP | LOT Actual Processing Data Report |
| 6 | 85 | 86 | ECS ← EQP | Dummy Lot Request |
| 6 | 89 | 90 | ECS ← EQP | Rework Request |
| 6 | 91 | 92 | ECS ← EQP | Equipment Status Change Report |
| 6 | 93 | 94 | ECS ← EQP | Scrap Data Report |
| 6 | 95 | 96 | ECS ← EQP | OCR( VCR ) Status Report |
| 7 | 19 | 20 | ECS → EQP | Current EPPD Request |
| 7 | 61 | 62 | ECS → EQP | Recipe ID Confirm Request |
| 7 | 63 | 64 | ECS ← EQP | Sampling Data Report |
| 7 | 67 | 68 | ECS → EQP | Current Recipe Group List Request |
| 7 | 69 | 70 | ECS → EQP | Current Recipe ID List Request by Group |
| 7 | 77 | 78 | ECS → EQP | Recipe Body Request |
| 9 | 1 | - | ECS ← EQP | Unrecognized Device ID |
| 9 | 3 | - | ECS ← EQP | Unrecognized Stream Type |
| 9 | 5 | - | ECS ← EQP | Unrecognized Function Type |
| 9 | 7 | - | ECS ← EQP | Illegal Data |
| 9 | 9 | - | ECS ← EQP | Transaction Timer Timeout |
| 9 | 11 | - | ECS ← EQP | Data Too Long |
| 9 | 13 | - | ECS ← EQP | Conversation Timeout |
| 10 | 3 | 4 | ECS → EQP | Terminal Display, Single |

Message 구조

Standard Message Structure

| Sn,F0 | Abort transaction | ECS↔EQP |
| --- | --- | --- |
| Description | Function 0 exists in all streams and has the same special meaning in each stream. The function 0 message closes a transaction, so that the originator will not have to wait for a transaction timeout to proceed. Function 0 is sent in lieu of the expected secondary message when the interpreter cannot, because of a transmission error or some other reasons, respond with the expected reply. It is not a requirement that the interpreter send function 0 to close a transaction |  |
| Structure |  |  |
| Notice |  |  |
| Exception |  |  |

| S1,F1 | Are You There Request | ECS↔EQP |
| --- | --- | --- |
| Description | If the equipment turn to online mode, Function 0 response to this message means the communication is inoperative. In the equipment, a function 0 is equivalent to a timeout on the receive timer after issuing S1,F1 to ECS |  |
| Structure | Header Only |  |
| Notice |  |  |
| Exception |  |  |

| S1,F2 | On Line Data | ECS↔EQP |
| --- | --- | --- |
| Description | Data signifying that the equipment is alive. |  |
| Structure | <L,2> <A[8],PXCEPI >MDLN PXCEPI Fixed <A[6],V100 >SoftRev V100 fixed |  |
| Notice | <MDLN> : Equipment Model Type <SOFTREV> : Software revision code |  |
| Exception | The ECS sends a zero-length list to the equipment |  |

| S1,F61 | All Equipment Status Report | ECSEQP | reply |
| --- | --- | --- | --- |
| Description | When equipment is initialized or control state is changed to Online, the equipment reports this message |  |  |
| Structure | <L,2> <L,1> <L,4> <U2,102>UNITID 102 fixed <A[1], >EQPSTATUS <A[10], >ALID <A[120], … >ALTX <L,3> IONIZER number is fixed to 3 <L,2> <U1,1>IONIZERID1 ID fixed 1 <A[1], >IONIZERSTATUS1 <L,2> <U1,2>IONIZERID2 ID fixed 2 <A[1], >IONIZERSTATUS2 <L,2> <U1,3>IONIZERID3 ID fixed 3 <A[1], >IONIZERSTATUS3 |  |  |
| Notice | <UNITID>Unit ID <EQPSTATUS> Equipment Status R : RUN E : Experiment V : Development C : Model Change M : Monitoring I : IDLE D : DOWN P : Process Down <ALID> Alarm Identification If <EQPSTATUS> is ‘R’ or ‘I’, then <ALID> is fixed by ‘0’ <ALTX> Alarm Message <IONIZERID> Ionizer ID <IONIZERSTATUS> Ionizer Status R : RUN D : DOWN |  |  |
| Exception |  |  |  |

| S1,F62 | All Equipment Status Report Acknowledge | ECSEQP |  |
| --- | --- | --- | --- |
| Description | Acknowledge |  |  |
| Structure | <L,1> <U2,102>UNITID 102 fixed <U1,>ACKC1 |  |  |
| Notice | <UNITID> Unit ID <ACKC1> 0 = Accepted 1 = Not Accepted |  |  |
| Exception |  |  |  |

| S1,F63 | All Equipment Status Report | ECSEQP | reply |
| --- | --- | --- | --- |
| Description | ECS requests the status of all units of the Equipment. |  |  |
| Structure | Header Only |  |  |
| Notice |  |  |  |
| Exception |  |  |  |

| S1,F64 | All Equipment Status Data | ECSEQP | reply |
| --- | --- | --- | --- |
| Description | When equipment is initialized or control state is changed to Online, the equipment reports this message |  |  |
| Structure | <L,1> <L,4> <U2,102>UNITID 102 fixed <A[1], >EQPSTATUS <A[10], >ALID <A[120], … >ALTX <L,3> IONIZER number is fixed to 3 <L,2> <U1,1>IONIZERID1 ID fixed 1 <A[1], >IONIZERSTATUS1 <L,2> <U1,2>IONIZERID2 ID fixed 2 <A[1], >'IONIZERSTATUS2 <L,2> <U1,3>IONIZERID3 ID fixed 3 <A[1], >'IONIZERSTATUS3 |  |  |
| Notice | <UNITID>Unit ID <EQPSTATUS> Equipment Status R : RUN E : Experiment V : Development C : Model Change M : Monitoring I : IDLE D : DOWN P : Process Down <ALID> Alarm Identification If <EQPSTATUS> is ‘R’ or ‘I’, then <ALID> is fixed by ‘0’ <ALTX> Alarm Message <IONIZERID> Ionizer ID <IONIZERSTATUS> Ionizer Status R : RUN D : DOWN |  |  |
| Exception |  |  |  |

| S1,F67 | All Glass Position Data Report | ECSEQP | reply |
| --- | --- | --- | --- |
| Description | This message is used for tracking of glasses which moved in among sub units |  |  |
| Structure | <L,2> <L,3> <U2,102>UNITID 102 fixed <U2,100>SUNITID 100 fixed <L,n> if Glass Exist n = 2, Glass Not Exist n = 0 <U2, >GLSCODE1 <U2, >GLSCODE2 |  |  |
| Notice | <UNITID> Unit ID <SUNITID> Sub Unit ID <GLSCODE> Unique Glass Code defined by ECS. Equipment must keep this code, while glass is in equipment. |  |  |
| Exception |  |  |  |

| S1,F68 | All Glass Position Data Report Acknowledge | ECSEQP |  |
| --- | --- | --- | --- |
| Description | Acknowledge |  |  |
| Structure | <L,2> <U2,102>UNITID 102 fixed <U1, >ACKC1 |  |  |
| Notice | <UNITID> Unit ID <ACKC1> 0 = Accepted 1 = Not Accepted |  |  |
| Exception |  |  |  |

| S1,F75 | Ionizer Status Report | ECSEQP | reply |
| --- | --- | --- | --- |
| 8Description | Acknowledge |  |  |
| Structure | <L,2> <U2,102>UNITID 102 fixed <L,3> IONIZER number is fixed to 3 <L,2> <U1,1>IONIZERID1 ID fixed 1 <A[1], >IONIZERSTATUS1 <L,2> <U1,2>IONIZERID2 ID fixed 2 <A[1], >'IONIZERSTATUS2 <L,2> <U1,3>IONIZERID3 ID fixed 3 <A[1], >'IONIZERSTATUS3 |  |  |
| Notice | <UNITID>Unit ID <IONIZERID> Ionizer ID <IONIZERSTATUS> Ionizer Status R : RUN D : DOWN |  |  |
| Exception |  |  |  |

| S1,F76 | All Glass Position Data Report Acknowledge | ECSEQP |  |
| --- | --- | --- | --- |
| Description | Acknowledge |  |  |
| Structure | <L,2> <U2,102>UNITID 102 fixed <U1, >ACKC1 |  |  |
| Notice | <UNITID> Unit ID <ACKC1> 0 = Accepted 1 = Not Accepted |  |  |
| Exception |  |  |  |

| S2,F17 | Date and Time Request | ECSEQP | reply |
| --- | --- | --- | --- |
| Description | This message is Useful, when ECS checks the equipment time base or the equipment to synchronize with ECS time base. Normal flow에 의거하여 Send |  |  |
| Structure | <U2,102>UNITID 102 fixed |  |  |
| Notice | <UNITID> Unit ID |  |  |
| Exception |  |  |  |

| S2,F18 | Date and Time Data | ECSEQP |  |
| --- | --- | --- | --- |
| Description | Actual time data. |  |  |
| Structure | <A[14], >TIME |  |  |
| Notice | <TIME> : Current Time “YYYYMMDDhhmmss” |  |  |
| Exception |  |  |  |

| S2,F25 | Loopback Diagnostic Request | ECS↔EQP | reply |
| --- | --- | --- | --- |
| Description | This is the diagnostic message for checkout of protocol and communication circuits. When the binary string is sent, the same data is echoed back. CIM에서 Button을 눌러 Send 가능하도록 구현 |  |  |
| Structure | <U10, >ABS |  |  |
| Notice | <ABS> Any binary string |  |  |
| Exception |  |  |  |

| S2,F26 | Loopback Diagnostic Data | ECS↔EQP |  |
| --- | --- | --- | --- |
| Description | The echoed binary string |  |  |
| Structure | <U10, >ABS |  |  |
| Notice | <ABS> Any binary string |  |  |
| Exception |  |  |  |

| S2,F31 | Date and Time Data Send | ECSEQP | reply |
| --- | --- | --- | --- |
| Description | Actual time data Send EQP에서 Time value 수신 후 PC의 Time set 가능 하도록 구현 |  |  |
| Structure | <L,2> <U2,102> UNITID 102 fixed <A[14], >TIME |  |  |
| Notice | <UNITID> Unit ID <TIME>: Current Time “YYYYMMDDhhmmss” |  |  |
| Exception |  |  |  |

| S2,F32 | Date and Time Acknowledge | ECSEQP |  |
| --- | --- | --- | --- |
| Description | Acknowledge |  |  |
| Structure | <L,2> <U2,102> UNITID 102 fixed <U1, >ACKC2 |  |  |
| Notice | <UNITID> Unit ID <ACKC2> 0 = Accepted 1 = Not Accepted |  |  |
| Exception |  |  |  |

| S2,F61 | Control State Change Request | ECSEQP | reply |
| --- | --- | --- | --- |
| Description | This message is used when requesting the control state change. |  |  |
| Structure | <L,2> <U2,102>UNITID 102 fixed <A[1], >RCS |  |  |
| Notice | <UNITID> Unit ID <RCS> Remote Control State 0 = Online-Remote 1 = Online-Local 2 = Offline |  |  |
| Exception |  |  |  |

| S2,F62 | Control State Change Request Acknowledge | ECSEQP |  |
| --- | --- | --- | --- |
| Description | Reply for the control state change of the equipment |  |  |
| Structure | <L,2> <U2,102>UNITID 102 fixed <U1, >ACKC2A |  |  |
| Notice | <UNITID> Unit ID <ACKC2A> Acknowledge code, 1 byte 0 = Mode Change Accept 1 = Mode Change not Allowed 2 = Equipment Already Changed Mode |  |  |
| Exception |  |  |  |

| S2,F63 | Equipment Operation Mode Report | ECSEQP | reply |
| --- | --- | --- | --- |
| Description | Change operation mode of the equipment |  |  |
| Structure | <L,2> <U2,102>UNITID 102 fixed <A[1], 1>OPMODE 1 fixed |  |  |
| Notice | <UNITID> Unit ID <OPMODE> Operation Mode Code The following OPMODE values define the case of operate to Online(Remote or Local) 1 = Normal / Load & Process / Auto / Remote / Operation / Automatic Mode 2 = Sequence Transfer / Online Local / Local / Semi-Auto Mode 3 = Cleanout / Recovery Mode 4 = Test / Atmospheric Transfer Mode 5 = Transfer / Vacuum Transfer Mode 6 = Particle Check / Pass Mode 7 = Reverse Mode 8 = Repair Mode 9 = Manual A = Restoration OPMODE는 현재 1로 고정하였음. UI를 통해서 수정 가능함. |  |  |
| Exception | The operation mode to operate Offline dose not define OPMODE (ex. Manual / Maintenance / Offline / Local / etc…) |  |  |

| S2,F64 | Equipment Operation Mode Report Acknowledge | ECSEQP |  |
| --- | --- | --- | --- |
| Description | Reply for Change operation mode of the equipment |  |  |
| Structure | <L,2> <U2,102>UNITID 102 fixed <U1, >ACKC2A |  |  |
| Notice | <UNITID> Unit ID <ACKC2A> Acknowledge code, 1 byte 0 = Mode Change Accept 1 = Mode Change not Allowed 2 = Equipment Already Changed Mode 3 - 63 Reversed |  |  |
| Exception |  |  |  |

| S2,F65 | Local LOT Status Change Request | ECSEQP | reply |
| --- | --- | --- | --- |
| Description | This message is used when sending Lot End Data under Online Local Control state |  |  |
| Structure | <L,4> <U2,102>UNITID 102 fixed <U1, >PORTID <A[16], >LOTID <A[1], >ENDFLAG |  |  |
| Notice | <UNITID> Unit ID <PORTID> Port ID <LOTID> LOT ID <ENDFLAG> End Flag N : Normal End P : Operator End |  |  |
| Exception |  |  |  |

| S2,F66 | Local LOT Status Change Request Acknowledge | ECSEQP |  |
| --- | --- | --- | --- |
| Description | Acknowledge |  |  |
| Structure | <L,2> <U2,102>UNITID 102 fixed <U1, >ACKC2 |  |  |
| Notice | <UNITID> Unit ID <ACKC2> 0 = Accepted 1 = Not Accepted |  |  |
| Exception |  |  |  |

| S2,F67 | Cycle Stop Status Report | ECSEQP | reply |
| --- | --- | --- | --- |
| Description | Change cycle stop status of the equipment |  |  |
| Structure | <L,2> <U2,102>UNITID 102 fixed <U1, >CSSTATUS |  |  |
| Notice | <UNITID> Unit ID <CSSTATUS> 0 = Cycle Stop Release 1 = Cycle Stop Set |  |  |
| Exception |  |  |  |

| S2,F68 | Cycle Stop Status Report Acknowledge | ECSEQP |  |
| --- | --- | --- | --- |
| Description | Reply for Change cycle stop status of the equipment |  |  |
| Structure | <L,2> <U2,102>UNITID 102 fixed <U1, >ACKC2 |  |  |
| Notice | <UNITID> Unit ID <ACKC2> 0 = Accepted 1 = Not Accepted 2 – 63 Reserved |  |  |
| Exception |  |  |  |

| S2,F69 | Unload Glass Data Request | ECSEQP | reply |
| --- | --- | --- | --- |
| Description | This message is used when ECS Request Unload Glass Data from the Equipment |  |  |
| Structure | <L,3> <U2,102>UNITID 102 fixed <U1, >STAGEID <A[80], … >HOSTMSG |  |  |
| Notice | <UNITID> Unit ID <STAGEID> Stage ID <HOSTMSG> Host message in Korean and English |  |  |
| Exception |  |  |  |

| S2,F70 | Unload Glass Data Request Acknowledge | ECSEQP |  |
| --- | --- | --- | --- |
| Description | Acknowledge |  |  |
| Structure | <L,2> <U2,102>UNITID 102 fixed <U1, >ACKC2 |  |  |
| Notice | <UNITID> Unit ID <ACKC2> 0 = Accepted 1 = Not Accepted 2 = Unload Glass Not Exist 3 – 63 Reserved |  |  |
| Exception |  |  |  |

| S2,F79 | Glass Data Request | ECSEQP | reply |
| --- | --- | --- | --- |
| Description | The Equipment only can use this control sequence in its manual operation. It can be use when equipment lost ‘Glass Data’, but equipment still have Glass ID or Glass code |  |  |
| Structure | <L,4> <U2,102>UNITID 102 fixed <A[16], >GLSID <U2, >GLSCODE <A[1], >REQOPTION |  |  |
| Notice | <UNITID> Unit ID <GLSID> Glass ID <REQOPTION> Request Option C : Glass Code I : Glass ID A : Glass Code and Glass ID |  |  |
| Exception |  |  |  |

| S2,F80 | Glass Data | ECSEQP | reply |
| --- | --- | --- | --- |
| Description | The Equipment (manual operation) sends S2F79 and then ECS sends ‘Glass Load Data’ as reply. ‘Glass Load Data’ is data about injected glass when glass inject to equipment |  |  |
| Structure | <L,3> <U2,102>UNITID 102 fixed <U1, >ACKC2 <L,1> <L,21> GLSSTATUS = 0, 0 / GLSSTATUS = 1, 21 <A[16], >LOTID <A[14], >CSTID <A[3], >SLOTID <A[24], >RECIPEID <A[8], >'PRCID <A[1], >GLSODR <A[16], >GLSID <A[2], >GLSJUDGE <A[2000], … >PNLIF <A[2000], … >SUBMDLIF <U1, >'PORTID <U2, >GLSCODE <U1, >REINPUT <A[5], >GLSTHICK <A[14], >PARTNUM <A[12], >PRODTYPE <A[12], >ATTRIBUTE <A[1], >GLSTYPE <A[32], >KEYID <L,n> n = number of LSD <L,2> <A[24], >LSDITEM1 <A[40], … >LSDVALUE1 …. <L,2> <A[24], >LSDITEMn <A[40], … >LSDVALUEn <L,m> m = number of GSD <L,2> <A[24], >GSDITEM1 <A[40], … >GSDVALUE1 …. <L,2> <A[24], >GSDITEMm <A[40], … >GSDVALUEm |  |  |
| Notice | <UNITID> Unit ID <ACKC2> 0 = Accepted 1 = Not Accepted (Unknown GLSID or GLSCODE) 2 – 63 Reserved <LOTID> LOT ID <CSTID> Cassette ID <SLOTID> Slot ID <RECIPEID> Recipe ID <PRCID> Processing Code <GLSODR> Glass Order 0 = The first glass 1 = All glasses except the first glass and the last glass 2 = The Last glass( In this case, this value may not be delivered for Scrap or reason of LOT Abort ) 3 = the only glass, when there is just one in a LOT. <GLSID> Glass ID <GLSJUDGE> Glass Grade Code <PNLIF> Panel Judge Information <SUBMDLIF> Sub Panel Information <PORTID> Port ID <GLSCODE> Glass ID Code. <REINPUT> Glass Re-input Flag 0 = Not Re-input 1 = Re-input <GLSTHICK> Glass’s Thickness <PARTNUM> Part Number <PRODTYPE> Type of Producing <ATTRIBUTE> LOT Attribute <GLSTYPE> Glass Type N = Normal Glass D = Dummy Glass <KEYID> Glass Key ID <LSDITEM> LOT Specific Data Item defined by LG Display <LSDVALUE> LOT Specific Data Value defined by LG Display <GSDITEM> Glass Specific Data Item defined by LG Display <GSDVALUE> Glass Specific Data Value defined by LG Display |  |  |
| Exception |  |  |  |

| S2,F87 | Load Glass Data Send | ECSEQP | reply |
| --- | --- | --- | --- |
| Description | This is the Glass data that would be loaded into the equipment. |  |  |
| Structure | <L,4> <U2,102>UNITID 102 fixed <U1, >STAGEID <A[1], >GLSSTATUS <L,2> <L,21>A CUT GLASS GLSSTATUS = 0, 0 / GLSSTATUS = 1, 21 <A[16], >LOTID <A[14], >CSTID <A[3], >SLOTID <A[24], >RECIPEID <A[8],PRCID >PRCID <A[1], >GLSODR <A[16], >GLSID <A[2], >GLSJUDGE <A[2000], … >PNLIF <A[2000], … >SUBMDLIF <U1, >PORTID <U2, >GLSCODE <U1, >REINPUT <A[5], >GLSTHICK <A[14], >PARTNUM <A[12], >PRODTYPE <A[12], >ATTRIBUTE <A[1], >GLSTYPE <A[32], >KEYID <L,n> n = number of LSD <L,2> <A[24], >LSDITEM1 <A[40], … >LSDVALUE1 …. <L,2> <A[24], >LSDITEMn <A[40], … >LSDVALUEn <L,m> m = number of GSD <L,2> <A[24], >GSDITEM1 <A[40], … >GSDVALUE1 …. <L,2> <A[24], >GSDITEMm <A[40], … >GSDVALUEm <L,21>B CUT GLASS <A[16], >LOTID <A[14], >CSTID <A[3], >SLOTID <A[24], >RECIPEID <A[8], >PRCID <A[1], >GLSODR <A[16], >GLSID <A[2], >GLSJUDGE <A[2000], … >PNLIF <A[2000], … >SUBMDLIF <U1, >PORTID <U2, >GLSCODE <U1, >REINPUT <A[5], >GLSTHICK <A[14], >PARTNUM <A[12], >PRODTYPE <A[12], >ATTRIBUTE <A[1], >GLSTYPE <A[32], >KEYID <L,n> n = number of LSD <L,2> <A[24], >LSDITEM1 <A[40], … >LSDVALUE1 …. <L,2> <A[24], >LSDITEMn <A[40], … >LSDVALUEn <L,m> m = number of GSD <L,2> <A[24], >GSDITEM1 <A[40], … >GSDVALUE1 …. <L,2> <A[24], >GSDITEMm <A[40], … >GSDVALUEm |  |  |
| Notice | <UNITID> Unit ID <STAGEID> Stage ID <GLSSTATUS> Status of the glass that would be loaded 0 = Not Exist 1 = Exist <LOTID> LOT ID <CSTID> Cassette ID <SLOTID> Slot ID <RECIPEID> Recipe ID <PRCID> Processing Code <GLSODR> Glass Order 0 = The first glass 1 = All glasses except the first glass and the last glass 2 = The Last glass( In this case, this value may not be delivered for Scrap or reason of LOT Abort ) 3 = the only glass, when there is just one in a LOT. <GLSID> Glass ID <GLSJUDGE> Glass Grade Code <PNLIF> Panel Judge Information <SUBMDLIF> Sub Panel Information <PORTID> Port ID <GLSCODE> Glass ID Code. <REINPUT> Glass Re-input Flag 0 = Not Re-input 1 = Re-input <GLSTHICK> Glass’s Thickness <PARTNUM> Part Number <PRODTYPE> Type of Producing <ATTRIBUTE> LOT Attribute <GLSTYPE> Glass Type N = Normal Glass D = Dummy Glass <KEYID>Glass Key ID <LSDITEM> LOT Specific Data Item defined by LG Display <LSDVALUE> LOT Specific Data Value defined by LG Display <GSDITEM> Glass Specific Data Item defined by LG Display <GSDVALUE> Glass Specific Data Value defined by LG Display <--GSD Item/Value Define-> <CUT_TYPE> Glass Cut Type A : A Cut DA : Dummy A Cut B : B Cut DB : Dummy B Cut <PAIRINFO> A,B Cut Glass Pair Information Y : Paired N : Unpaired <PAIR_GLASS_CODE> Matching Pair Glass Code Glass Code <ROUTE_CHAMBER> Chamber Route Information Chamber Route Information |  |  |
| Exception |  |  |  |

| S2,F88 | Load Glass Data Send Acknowledge | ECSEQP |  |
| --- | --- | --- | --- |
| Description | Acknowledge |  |  |
| Structure | <L,3> <U2,102>UNITID 102 fixed <U1, >STAGEID <U1, >ACKC2 |  |  |
| Notice | <UNITID> Unit ID <STAGEID> Stage ID <ACKC2> 0 = Accept 1 = Process Condition is not ready 2 = The recipe is not exist in equipment’s recipe list 3 = Glass data is not accepted 4 = Process is not ready to load this glass 5 = A, B Cut Recipe Mismatching 6~63 = Reserved |  |  |
| Exception |  |  |  |

| S5,F1 | Alarm Report | ECSEQP | reply |
| --- | --- | --- | --- |
| Description | This message reports the alarm condition changes inside EQP or of the previous step. One message will be issued when the alarm is set and the other will be issued when cleared. Irrecoverable errors and attention flags may not have the clear message corresponding to them |  |  |
| Structure | <L,4> <U2,102>UNITID 102 fixed <U1, >ALCD <A[10], >ALID <A[120], … >ALTX |  |  |
| Notice | <UNITID> Unit ID <ALCD>Alarm Code bit8 = 1 Alarm set bit 8 = 0 Alarm Reset bit 7~1 is Alarm category (0~127) 0 = Not Used 1 = Personal safety 2 = Equipment safety 3 = Parameter control warning 4 = Parameter control error 5 = Irrecoverable error 6 = Equipment status warning 7 = Attention flags 8 = Data integrity <ALID> Alarm Identification <ALTX>Alarm Message |  |  |
| Exception |  |  |  |

| S5,F2 | Alarm Report Acknowledge | ECSEQP |  |
| --- | --- | --- | --- |
| Description | Acknowledge or Error |  |  |
| Structure | <L,2> <U2,102>UNITID 102 fixed <U1,0>ACKC5 |  |  |
| Notice | <ACKC5> Acknowledge code, 1 byte 0 = Accepted (Fixed) |  |  |
| Exception |  |  |  |

| S6,F61 | Load Glass Data Request | ECSEQP | reply |
| --- | --- | --- | --- |
| Description | This message is used in Load Glass Data Request |  |  |
| Structure | <L,2> <U2,102>UNITID 102 fixed <U1, >STAGEID |  |  |
| Notice | <UNITID> Unit ID <STAGEID> Stage ID |  |  |
| Exception |  |  |  |

| S6,F62 | Load Glass Data Request Acknowledge | ECSEQP |  |
| --- | --- | --- | --- |
| Description | Acknowledge |  |  |
| Structure | <L,3> <U2,102>UNITID 102 fixed <U1, >STAGEID <U1, >ACKC6 |  |  |
| Notice | <UNITID> Unit ID <STAGEID> Stage ID <ACKC2> 0 = Accept 1 = Not Accept |  |  |
| Exception |  |  |  |

| S6,F63 | Unload Glass Data Report | ECSEQP | reply |
| --- | --- | --- | --- |
| Description | This message is used in Unload Glass Data Report |  |  |
| Structure | <L,3> <U2,102>UNITID 102 fixed <U1, >STAGEID <L,2> <L,21>A CUT GLASS <A[1], >GLSODR <A[16], >LOTID <A[14], >CSTID <A[3], >SLOTID <A[16], >GLSID <A[2000], … >PNLIF <A[2000], … >SUBMDLIF <A[8], >PRCID <U1, >DICODE <U1, >PORTID <A[24], >RECIPEID <A[2], >GLSJUDGE <A[14], >PARTNUM <A[12], >PRODTYPE <A[12], >ATTRIBUTE <A[1], >GLSTYPE <A[1], >GLSPROC <U2, >GLSCODE <U2, >DEST <A[32], >KEYID <L,n> n = number of GSD <L,2> <A[24], >GSDITEM1 <A[40], … >GSDVALUE1 …. <L,2> <A[24], >GSDITEMn <A[40], … >GSDVALUEn <L,21>B CUT GLASS <A[1], >GLSODR <A[16], >LOTID <A[14], >CSTID <A[3], >SLOTID <A[16], >GLSID <A[2000], … >PNLIF <A[2000], … >SUBMDLIF <A[8], >PRCID <U1, >DICODE <U1, >PORTID <A[24], >RECIPEID <A[2], >GLSJUDGE <A[14], >PARTNUM <A[12], >PRODTYPE <A[12], >ATTRIBUTE <A[1], >GLSTYPE <A[1], >GLSPROC <U2, >GLSCODE <U2, >DEST <A[32], >KEYID <L,m> m = number of GSD <L,2> <A[24], >GSDITEM1 <A[40], … >GSDVALUE1 …. <L,2> <A[24], >GSDITEMm <A[40], … >GSDVALUEm |  |  |
| Notice | <UNITID> Unit ID <STAGEID> Stage ID <GLSODR> Glass Order 0 = The first glass 1 = All glasses except the first glass and the last glass 2 = The Last glass 3 = the only glass, when there is just one in a LOT. <LOTID> LOT ID <CSTID> Cassette ID <SLOTID> Slot ID <GLSID> Glass ID <PNLIF> Panel Information <SUBMDLIF> Sub Panel Information <PRCID> Processing Code <DICODE> Different Code 0 : All GLS IDs are completely matching with GLS IDs from ECS. 1 : Any GLS ID is not matching with GLS ID from ECS 2 : When each ID of glass is not checked by VCR or OCR in the equipment has VCR or OCR <PORTID> Port ID <RECIPE ID> Recipe ID <GLSJUDGE> Glass Grade Code <PARTNUM> Part Number <PRODTYPE> Type of Producing <ATTRIBUTE> LOT Attribute <GLSTYPE> Glass Type N = Normal Glass D = Dummy Glass <GLSPROC> Glass Process Status 0 : Unprocessed 1 : Processing 2 : Processed 3 : Rejected <GLSCODE> Glass ID Code <DEST> Glass Next Destination 0 = ECS determine destination of glass. >0 = Equipment determine destination of glass. The value of <DEST> will be decided later through conversation <KEYID> Glass Key ID <GSDITEM> Glass Specific Data Item defined by LG Display <GSDVALUE> Glass Specific Data Value defined by LG Display <--GSD Item/Value Define-> <CUT_TYPE> Glass Cut Type A : A Cut DA : Dummy A Cut B : B Cut DB : Dummy B Cut <PAIRINFO> A,B Cut Glass Pair Information Y : Paired N : Unpaired <PAIR_GLASS_CODE> Matching Pair Glass Code Glass Code <ROUTE_CHAMBER> Chamber Route Information Chamber Route Information |  |  |
| Exception | When <GSDITEM>, <GSDVALUE> is not necessary, n is 0. |  |  |

| S6,F64 | Unload Glass Data Report Acknowledge | ECSEQP |  |
| --- | --- | --- | --- |
| Description | Acknowledge |  |  |
| Structure | <L,2> <U2,102>UNITID 102 fixed <U1, >ACKC6 |  |  |
| Notice | <UNITID> Unit ID <ACKC6> 0 = Accepted >0 = Error, not accepted 1 – 63 Reserved |  |  |
| Exception |  |  |  |

| S6,F65 | LOT End Data Send | ECSEQP | reply |
| --- | --- | --- | --- |
| Description | This message is used in Lot End Data Send |  |  |
| Structure | <L,9> <U2,102>UNITID 102 fixed <U1,1>PORTID <A[16], >LOTID <A[14], >CSTID <A[24], >RECIPEID <A[8], >PRCID <A[1], >ENDFLAG <A[1], >MULTIFLAG <L,n> n = number of slot <L,10> <A[1], >GLSODR <A[3], >SLOTID <A[16], >GLSID <A[24], >RECIPEID <U1, >GLSEXIST1 <U1, >GLSSELECTED <U2,102>PROCUNITID <A[2], >GLSJUDGE <A[2000], … >PNLIF <A[2000], … >SUBMDLIF ….. <L,10> <A[1], >GLSODR <A[3], >SLOTID <A[16], >GLSID <A[24], >RECIPEID <U1, >GLSEXIST1 <U1, >GLSSELECTED <U2,102>PROCUNITID <A[2], >GLSJUDGE <A[2000], … >PNLIF <A[2000], … >SUBMDLIF |  |  |
| Notice | <UNITID> Unit ID <PORTID> Port ID <LOTID> LOT ID <CSTID> Cassette ID <RECIPE ID> Recipe ID <PRCID> Processing Code <ENDFLAG> End Flag N : Normal End P : Operator End E : Last processed unit R : Last processed unit for rework <MULTIFLAG> T : True(Multi Recipe ID) F : False(Single Recipe ID) <GLSODR> Glass Order 0 = The first glass 1 = All glasses except the first glass and the last glass 2 = The Last glass 3 = the only glass, when there is just one in a LOT <SLOTID> Slot ID <GLSID> Glass ID <GLSEXIST> Glass Existence 0 = Not exist 1 = Exist <GLSSELECTED> Glass Selection 0 = Not selected 1 = selected <PROCUNITID> Unit ID to process Glass <GLSJUDGE> Glass Grade Code <PNLIF> Panel Information <SUBMDLIF> Sub Panel Information |  |  |
| Exception | If n=0, this means ‘Lot Judge Report’ facility is not used. If n≠0, this means ‘Lot Judge Report’ facility is used. |  |  |

| S6,F66 | LOT End Data Send Acknowledge | ECSEQP |  |
| --- | --- | --- | --- |
| Description | Acknowledge |  |  |
| Structure | <L,4> <U2,102>UNITID 102 fixed <U1, >PORTID <A[16], >LOTID <U1, >ACKC6 |  |  |
| Notice | <UNITID> Unit ID <PORTID> Port ID <LOTID> Lot ID <ACKC6> Acknowledge code, 1 byte 0 = Accepted 1 = Wait Lot Actual Processing Data 2 = Lot End without Lot Actual Processing Data >3 = Error , not accepted 1 – 63 Reserved |  |  |
| Exception |  |  |  |

| S6,F67 | Glass Move Request | ECSEQP | reply |
| --- | --- | --- | --- |
| Description | This message is used in Load Request, Unload Request from Equipment. |  |  |
| Structure | <L,7> <A[2], >GLSMVSTATUS <U2,102>UNITID 102 fixed <U1, >STAGEID <U2, >LDGLSCODE_A <U2, >LDGLSCODE_B <U2, >UDGLSCODE_A <U2, >UDGLSCODE_B |  |  |
| Notice | <GLSMVSTATUS> Glass Move Status LW : Glass Load Wait Request UW : Glass Unload Wait Request LR : Glass Load Request UR : Glass Unload Request ER : Glass Exchange Request CR : Cancel Request <UNITID> Unit ID <STAGEID> Stage ID <LDGLSCODE_A> A Cut Load Glass Code <LDGLSCODE_B> B Cut Load Glass Code <UDGLSCODE_A> A Cut Unload Glass Code <UDGLSCODE_B> B Cut Unload Glass Code GLSCODE는 보고 할 data가 없을시 0으로 보고 (ex. <U2, 0>LDGLSCODE_A) |  |  |
| Exception |  |  |  |

| S6,F68 | Glass Move Request Acknowledge | ECSEQP |  |
| --- | --- | --- | --- |
| Description | Acknowledge |  |  |
| Structure | <L,2> <U2,102>UNITID 102 fixed <U1, >ACKC6 |  |  |
| Notice | <UNITID>Unit ID <ACKC6> 0 = Accepted 1 = Not Accepted |  |  |
| Exception |  |  |  |

| S6,F69 | Cassette Map Request | ECSEQP | reply |
| --- | --- | --- | --- |
| Description | The Equipment requests Cassette Map(Online Local Control state only) |  |  |
| Structure | <L,1> <U2,102>UNITID 102 fixed |  |  |
| Notice | <UNITID>Unit ID |  |  |
| Exception |  |  |  |

| S6,F70 | Cassette Map Request Acknowledge | ECSEQP |  |
| --- | --- | --- | --- |
| Description | Cassette Map Data Send |  |  |
| Structure | <L,2> <U2,102>UNITID 102 fixed <L,n> n = Cassette port number <L,5> <A[16], >LOTID1 <A[14], >CSTID1 <U1, >PORTID1 <A[40], … >MAP1 <A[1], >CSTREADY1 <L,5> <A[16], >LOTID2 <A[14], >CSTID2 <U1, >PORTID2 <A[40], … >MAP2 <A[1], >CSTREADY2 …. <L,n> <A[16], >LOTIDn <A[14], >CSTIDn <U1, >PORTIDn <A[40], … >MAPn <A[1], >CSTREADYn |  |  |
| Notice | <UNITID>Unit ID <LOTID>LOT ID <CSTID>Cassette ID <PORTID> Port ID <MAP> Cassette Mapping Information e.g. In case there are 2 glassed in Slot 1 and Slot 20. Then MAP = “1000000000000000000100000000000000000000” - : Glass Not Exist - : Glass Exist <CSTREADY> Cassette State 0 : Cassette Not Exist 1 : Cassette Exist (LC) |  |  |
| Exception |  |  |  |

| S6,F71 | Sampling Slot Request | ECSEQP | reply |
| --- | --- | --- | --- |
| Description | The Equipment requests the specific glass selected by the equipment from ECS on Online Local State |  |  |
| Structure | < L,3> <U2,102>UNITID 102 fixed <U1, >STAGEID <L,4> <A[16], >LOTID <A[14], >CSTID <U1, >PORTID <A[3], >SLOTID |  |  |
| Notice | <UNITID> Unit ID <STAGEID> Stage ID <LOTID> LOT ID <CSTID> Cassette ID <PORTID> Port ID <SLOTID> Slot ID |  |  |
| Exception |  |  |  |

| S6,F72 | Sampling Slot Request Acknowledge | ECSEQP |  |
| --- | --- | --- | --- |
| Description | Acknowledge or error |  |  |
| Structure | < L,2> <U2,102>UNITID 102 fixed <U1, >ACKC6A |  |  |
| Notice | <ACKC6A> Acknowledge code, 1byte 0 = Accept 1 = Glass not exist 2 = Cassette not exist 3 = Invalid LOT ID 4 = Busy, Try later 5 – 63 Reserved |  |  |
| Exception |  |  |  |

| S6,F73 | Glass Actual Processing Data Report | ECSEQP | reply |
| --- | --- | --- | --- |
| Description | The purpose of this message is for the equipment to send the glass processing data 보고 순서는 A → B 이다. |  |  |
| Structure | <L,8> <A[16], >LOTID <A[14], >CSTID <A[24], … >RECIPEID <A[8], >PRCID <A[3], >SLOTID <U2,102>UNITID 102 fixed <L,7> <A[16], >GLSID <A[2], >GLSJUDGE <A[2000], … >PNLIF <A[2000], … >SUBMDLIF <U2, >GLSCODE <A[32], >KEYID <L,1> n = number of GSD <L,2> <A[24], >GSDITEM1 <A[40], … >GSDVALUE1 … <L,2> <A[24], >GSDITEMn <A[40], … >GSDVALUEn <L,3> n = number of parameter <L,2> <A[14], >DVNAME11 <A[14], >DVVAL11 … <L,2> <A[14], >DVNAME1n <A[14], >DVVAL1n |  |  |
| Notice | <LOTID> LOT ID <CSTID> Cassette ID <RECIPE ID> Recipe ID <PRCID> Processing Code <SLOTID> Slot ID <UNITID>Unit ID <GLSID> Glass ID <GLSJUDGE> Glass Judge Code <PNLIF> Panel Information <SUBMDLIF>Sub Panel Information <GLSCODE> Glass Code <KEYID> Glass Key ID <GSDITEM> Glass Specific Data Item defined by LG Display <GSDVALUE> Glass Specific Data Value defined by LG Display <DVNAME> APD Data Name defined by LG Display <DVVALUE> APD Data Value defined by LG Display |  |  |
| Exception | When <GSDITEM>, <GSDVALUE> is not necessary, p is 0 |  |  |

| S6,F74 | Glass Actual Processing Data Report Acknowledge | ECSEQP |  |
| --- | --- | --- | --- |
| Description | Acknowledge |  |  |
| Structure | <L,2> <U2,102>UNITID 102 fixed <U1, >ACKC6 |  |  |
| Notice | <UNITID> Unit ID <ACKC6> 0 = Accepted 1 = Not Accepted 2 ~ 63 Reserved |  |  |
| Exception |  |  |  |

| S6,F83 | LOT Actual Processing Data Report | ECSEQP | reply |
| --- | --- | --- | --- |
| Description | The purpose of this message is that the equipment sends the Lot Actual processing data. Only when ECS sends Lot End Data, the equipment sends this data |  |  |
| Structure | <L,9> <A[16], >LOTID <A[14], >CSTID <A[24], >RECIPEID <A[8], >PRCID <U2,102>UNITID 102 fixed <A[32], … >KEYID <A[2], >LOTJUDGE <L,n> n = number of LSD <L,2> <A[24], >LSDITEM1 <A[40], … >LSDVALUE1 … <L,2> <A[24], >LSDITEMn <A[40], … >LSDVALUEn <L,m> m = number of parameter <L,2> <A[14], >DVNAME11 <A[14], >DVVAL11 … <L,2> <A[14], >DVNAME1m <A[14], >DVVAL1m |  |  |
| Notice | <LOTID> LOT ID <CSTID> Cassette ID <RECIPE ID> Recipe ID <PRCID> Processing Code <UNITID>Unit ID <KEYID> Glass Key ID <LOTJUDGE>Lot Judge <LSDITEM> LOT Specific Data Item defined by LG Display <LSDVALUE> LOT Specific Data Value defined by LG Display <DVNAME> APD Data Name defined by LG Display <DVVALUE> APD Data Value defined by LG Display |  |  |
| Exception |  |  |  |

| S6,F84 | LOT Actual Processing Data Report Acknowledge | ECSEQP |  |
| --- | --- | --- | --- |
| Description | Acknowledge |  |  |
| Structure | <L,2> <U2,102>UNITID 102 fixed <U1, >ACKC6 |  |  |
| Notice | <UNITID> Unit ID <ACKC6> 0 = Accepted 1 = Not Accepted 2 ~ 63 Reserved |  |  |
| Exception |  |  |  |

| S6,F85 | Dummy Lot Request | ECSEQP | reply |
| --- | --- | --- | --- |
| Description | This message is used to request Dummy Lot. |  |  |
| Structure | <U2,102>UNITID 102 fixed |  |  |
| Notice | <UNITID> Unit ID |  |  |
| Exception | . 3 |  |  |

| S6,F86 | Dummy Lot Request Acknowledge | ECSEQP |  |
| --- | --- | --- | --- |
| Description | Acknowledge. |  |  |
| Structure | <L,2> <U2,102>UNITID 102 fixed <U1, >ACKC6 |  |  |
| Notice | <UNITID> Unit ID <ACKC6> 0 = Accepted 1 = Not Accepted 2 ~ 63 Reserved |  |  |
| Exception |  |  |  |

| S6,F89 | Rework Request | ECSEQP | reply |
| --- | --- | --- | --- |
| Description | This message is used to request rework |  |  |
| Structure | <L,4> <U2,102>UNITID 102 fixed <A[16], >LOTID <A[1], >RESERVECD <L,n> n = number of slot <L,2> <A[3], >SLOTID1 <A[1], >SELECTED1 …. <L,2> <A[3], >SLOTIDn <A[1], >SELECTEDn |  |  |
| Notice | <UNITID> Unit ID <LOTID> Lot ID <RESERVECD> Rework reservation code R : Reservation of rework S : Rework Start <SELECTED> Glass Selected Information. 0 : Not Selected 1 : Selected |  |  |
| Exception |  |  |  |

| S6,F90 | Rework Request Acknowledge | ECSEQP |  |
| --- | --- | --- | --- |
| Description | Acknowledge |  |  |
| Structure | <L,4> <U2,102>UNITID 102 fixed <A[16], >LOTID <A[1], >RESERVECD <L,n> n = number of slot <L,2> <A[3], >SLOTID1 <A[1], >SELECTED1 … <L,2> <A[3], >SLOTIDn <A[1], >SELECTEDn |  |  |
| Notice | <UNITID> Unit ID <LOTID> Lot ID <RESERVECD> Rework reservation code R : Reservation of rework S : Rework Start <SELECTED> Glass Selected Information. 0 : Not Selected 1 : Selected |  |  |
| Exception |  |  |  |

| S6,F91 | Equipment Status Change Report | ECSEQP | reply |
| --- | --- | --- | --- |
| Description | When the Equipment status is changed, the Equipment reports this message |  |  |
| Structure | <L,4> <U2,102>UNITID 102 fixed <A[1], >EQPSTATUS <A[10], >ALID <A[120], … >ALTX |  |  |
| Notice | <UNITID>Unit ID <EQPSTATUS> Equipment status R : RUN E : Experiment V : Development C : Model Change M : Monitoring I : IDLE D : DOWN P : Process Down <ALID> Alarm Identification If <EQPSTATUS> is ‘R’ or ‘I’, then <ALID> is fixed by ‘0’ <ALTX> Alarm Message |  |  |
| Exception |  |  |  |

| S6,F92 | Equipment Status Change Report Acknowledge | ECSEQP |  |
| --- | --- | --- | --- |
| Description | Acknowledge |  |  |
| Structure | <L,2> <U2,102>UNITID 102 fixed <U1, >ACKC6 |  |  |
| Notice | <UNITID> Unit ID <ACKC6> 0 = Accepted 1 = Not Accepted 2 ~ 63 Reserved |  |  |
| Exception |  |  |  |

| S6,F93 | Scrap Data Report | ECSEQP | reply |
| --- | --- | --- | --- |
| Description | When a glass or a panel is scrapped, the Equipment reports this message |  |  |
| Structure | <L,5> <A[16], >LOTID <A[16], >GLSID <U2,102>UNITID 102 fixed <A[1], >SCRPTYPE <A[4], >SCRPCODE |  |  |
| Notice | <LOTID> LOT ID <GLSID> Glass ID <UNITID> Unit ID <SCRPTYPE> Scrap Type P = Panel Scarp G = Glass Scrap <SCRPCODE>Scrap Code defined by LG Display |  |  |
| Exception |  |  |  |

| S6,F94 | Scrap Data Report Acknowledge | ECSEQP |  |
| --- | --- | --- | --- |
| Description | Acknowledge |  |  |
| Structure | <L,2> <U2,102>UNITID 102 fixed <U1, >ACKC6 |  |  |
| Notice | <UNITID> Unit ID <ACKC6> 0 = Accepted 1 = Not Accepted 2 ~ 63 Reserved |  |  |
| Exception |  |  |  |

| S7,F19 | Current EPPD Request | ECSEQP | reply |
| --- | --- | --- | --- |
| Description | This message is used to request the transmission of the current equipment process program directory. This is the list of all the RECIPE IDs of the process programs stored in the equipment |  |  |
| Structure | <U2,102>UNITID 102 fixed |  |  |
| Notice | <UNITID> Unit ID |  |  |
| Exception |  |  |  |

| S7,F20 | Current EPPD Request Acknowledge | ECSEQP |  |
| --- | --- | --- | --- |
| Description | This message is used to transmit the current EPPD. |  |  |
| Structure | <L,2> <U2,102>UNITID 102 fixed <L,n> n = Recipe List Count <A[24], >RECIPEID1 … <A[24], >RECIPEIDn |  |  |
| Notice | <UNITID> Unit ID <RECIPEID> Recipe ID |  |  |
| Exception |  |  |  |

| S7,F61 | Recipe ID Confirm Request | ECSEQP | reply |
| --- | --- | --- | --- |
| Description | This message is used to ask the equipment whether the recipe ID is or not in the list, which the equipment has |  |  |
| Structure | <L,14> <A[16], >'LOTID <A[14], >'CSTID <A[24], >'RECIPEID <A[8], >'PRCID <U1,1>'PORTID <U2,102>'UNITID 102 fixed <A[5], >'GLSTHICK <A[14], >'PARTNUM <A[12], >'PRODTYPE <A[12], >'ATTRIBUTE <A[1], >'MODECODE <A[1], >'MULTIFLAG <L,n> n = number of LSD <L,2> <A[24], >'LSDITEM <A[40], … >'LSDVALUE …. <L,2> <A[24], >'LSDITEMn <A[40], … >'LSDVALUEn <L,m> m = number of slot <L,10> <A[3], >'SLOTID1 <A[16], >'SLTLOTID1 <A[16], >'GLSID1 <A[24], >'SRECIPEID1 <U1, >'GLSEXIST1 <A[2], >'GLSJUDGE1 <A[14], >'PARTNUM1 <A[2000], … >'PNLIF1 <A[2000], … > 'SUBMDLIF1 <L,n> j = number of GSD <L,2> <A[24], >'GSDITEM1 <A[40], … > 'GSDVALUE1 … <L,2> <A[24], >'GSDITEMj <A[40], … > 'GSDVALUEj …. <L,10> <A[3], >'SLOTIDm <A[16], >'SLTLOTIDm <A[16], >'GLSIDm <A[24], >'SRECIPEIDm <U1, >'GLSEXISTm <A[2], >'GLSJUDGEm <A[14], >'PARTNUMm <A[2000], … >'PNLIFm <A[2000], … > 'SUBMDLIFm <L,k> k = number of GSD <L,2> <A[24], >'GSDITEM <A[40], … > 'GSDVALUE …. <L,2> <A[24], >'GSDITEMk <A[40], … > 'GSDVALUEk |  |  |
| Notice | <UNITID> Unit ID <LOTID> LOT ID <CSTID> Cassette ID <SLOTID> Slot ID <RECIPEID> Recipe ID <PRCID> Processing Code <SLTLOTID> Each Slot LOT ID <SRECIPEID> Each Slot Recipe ID <GLSEXIST>Glass Existence 0 = Not exist 1 = Exist <GLSID> Glass ID <GLSJUDGE> Glass Grade Code <PNLIF> Panel Information <SUBMDLIF> Sub Panel Information <PORTID> Port ID <GLSTHICK> Glass’s Thickness <PARTNUM> Part Number <PRODTYPE> Type of Producing <ATTRIBUTE> LOT Attribute <MODECODE> Host Online / Offline Code N : Recipe has downloaded by Host (Host Online Mode) F : Recipe has inputted by Operator (Host Offline Mode) <MULTIFLAG> Multi Flag T : True(Multi Lot or Multi Recipe) F : False(Single Lot and Single Recipe) <KEYID>Glass Key ID <LSDITEM> LOT Specific Data Item defined by LG Display <LSDVALUE> LOT Specific Data Value defined by LG Display <GSDITEM> Glass Specific Data Item defined by LG Display <GSDVALUE> Glass Specific Data Value defined by LG Display |  |  |
| Exception |  |  |  |

| S7,F62 | Recipe ID Confirm and Sampling Data Request Acknowledge | ECSEQP |  |
| --- | --- | --- | --- |
| Description | This message is used to transmit the confirm result of the recipe id and the sampling data of the equipment. |  |  |
| Structure | <L,7> <U2,102>UNITID 102 fixed <U1, >PORTID <A[24], >RECIPEID <A[16], >LOTID <A[1], >SAMPLINGFLAG <U1, >ACKC7M <L,n> n = number of slot <L,3> <A[3], >SLOTID1 <A[24], >SRECIPEID1 <U1, >ACKC7S1 … <L,3> <A[3], > SLOTIDn <A[24], > SRECIPEIDn <U1, > ACKC7Sn |  |  |
| Notice | <UNITID> Unit ID <PORTID> Port ID <RECIPEID> Recipe ID ( This is recipe for LOT ) <LOTID> Lot ID < SAMPLINGFLAG> Sampling Flag W : Wait equipment’s Sampling Data S : Lot start without equipment’s Sampling Data <ACKC7M> Acknowledge code, 1 byte 0 = Accept 1 = Not accept 2 = Process Condition is not ready 3 – 63 Reserved <SLOTID> Slot ID <SRECIPEID> Each Slot Recipe ID <ACKC7S> 0 = Accept 1 = Not accept 2 – 63 Reserved |  |  |
| Exception |  |  |  |

| S7,F63 | Sampling Data Report | ECSEQP | reply |
| --- | --- | --- | --- |
| Description | This message is used to report the sampling data of the equipment |  |  |
| Structure | <L,5> <U2,102>UNITID 102 fixed <U1, >PORTID <A[24], >RECIPEID <A[16], >LOTID <L,n> n = number of slot <L,4> <A[8], >SLOID1 <A[24], >SRECIPEID1 <U1, >ACKC7S1 <A[1], >SELECTED1 … <L,4> <A[8], > SLOIDn <A[24], > SRECIPEIDn <U1, > ACKC7Sn <A[1], > SELECTEDn |  |  |
| Notice | <UNITID> Unit ID <PORTID> Port ID <RECIPEID> Recipe ID ( This is recipe for LOT ) <LOTID> Lot ID <SELECTED> 0 : Not Selected 1 : Selected |  |  |
| Exception |  |  |  |

| S7,F64 | Sampling Data Report Acknowledge | ECSEQP |  |
| --- | --- | --- | --- |
| Description | Acknowledge |  |  |
| Structure | <L,4> <U2,102>UNITID 102 fixed <U1, >PORTID <A[16], >LOTID <U1, >ACKC7 |  |  |
| Notice | <UNITID> Unit ID <PORTID> Port ID <LOTID> Lot ID <ACKC2> 0 = Accepted >0 = Error, not accepted 1 – 63 Reserved |  |  |
| Exception |  |  |  |

| S7,F67 | Current Recipe Group List Request | ECSEQP | reply |
| --- | --- | --- | --- |
| Description | This message is used to request the transmission of the current equipment process program group. This is the list of all the Groups of the process programs stored in the equipment |  |  |
| Structure | <U2,102>'UNITID 102 fixed |  |  |
| Notice | <UNITID> Unit ID |  |  |
| Exception |  |  |  |

| S7,F68 | Current Recipe Group List | ECSEQP |  |
| --- | --- | --- | --- |
| Description | This message is used to transmit the current EPPG. |  |  |
| Structure | <L,2> <U2,102>UNITID 102 fixed <L,n> n = group list count <A[12], >GROUPID1 <A[12], > GROUPID2 … <A[12], > GROUPIDn |  |  |
| Notice | <UNITID> Unit ID <GROUPID> Recipe Group ID |  |  |
| Exception |  |  |  |

| S7,F69 | Current Recipe ID List Request by Group | ECSEQP | reply |
| --- | --- | --- | --- |
| Description | This message is used to request the transmission of the current equipment process program directory. This is the list of all RECIPE IDs of the specified Recipe Group IDs in the equipment. |  |  |
| Structure | <L,2> <U2,102>UNITID 102 fixed <L,n> n = group list count <A[12], >GROUPID1 <A[12], > GROUPID2 … <A[12], > GROUPIDn |  |  |
| Notice | <UNITID> Unit ID <GROUPID> Recipe Group ID |  |  |
| Exception | Equipment using Recipe Group needs This Message. ‘n=0’ means all Recipe Group. |  |  |

| S7,F70 | Current Recipe ID List Request by Group Acknowledge | ECSEQP |  |
| --- | --- | --- | --- |
| Description | This message is used to transmit all the RECIPE IDs of the specified Recipe Group IDs in the equipment |  |  |
| Structure | <L,2> <U2,102>UNITID 102 fixed <L,m> m = group ID count <L,2> <A[12], >GROUPID <L,n> n = recipe list count <A[24], >RECIPEID1 <A[24], >RECIPEID2 … <A[24], >RECIPEIDn <L,2> <A[12], >GROUPID <L,k> k = recipe list count <A[24], >RECIPEID1 <A[24], >RECIPEID2 … <A[24], >RECIPEIDk |  |  |
| Notice | <UNITID> Unit ID <GROUPID> Recipe Group ID |  |  |
| Exception | Equipment using Recipe Group needs This Message. ‘n=0’ means all Recipe Group. |  |  |

| S7,F77 | Recipe Body Request | ECSEQP | reply |
| --- | --- | --- | --- |
| Description | Recipe Body List Request |  |  |
| Structure | <L,6> <A[16], >'LOTID <A[14], >'CSTID <A[24], >'RECIPEID <A[8], >'PRCID <U1, >'PORTID <U2,102>'UNITID 102 fixed |  |  |
| Notice | <LOTID> LOT ID <CSTID> Cassette ID <RECIPEID> Recipe ID <PRCID> Processing Code <PORTID> Port ID <UNITID> Unit ID |  |  |
| Exception |  |  |  |

| S7,F78 | Recipe Body | ECSEQP |  |
| --- | --- | --- | --- |
| Description | This message reports the recipe parameter of the equipment |  |  |
| Structure | <L,8> <A[16], >LOTID <A[14], >CSTID <A[24], > RECIPEID <A[12], >RECIPEIDVER <A[8], >PRCID <U1, >PORTID <U2,102>UNITID 102 fixed <L,1> <L,6> <U2,102>EUNITID 102 fixed <A[24], >LOWERRECIPEID <A[12], >LOWERRECIPEVER <A[1], >LEVEL <A[1],E>RECIPEIDTYPE fixed E <L,n> n = number of step <L,2> <A[14],0 >STEPID1 <L,m> m = number of parameter <L,2> <A[24], >PARAMNAME1 <A[24], >PARAMVAL1 … <L,2> <A[24], > PARAMNAMEm <A[24], > PARAMVALm …. <L,2> <A[14],0 >STEPIDn <L,m> m = number of parameter <L,2> <A[24], >PARAMNAME1 <A[24], >PARAMVAL1 … <L,2> <A[24], > PARAMNAMEm <A[24], > PARAMVALm |  |  |
| Notice | <LOTID> LOT ID <CSTID> Cassette ID <PRCID> Processing Code <RECIPEID> Recipe ID <PORTID> Port ID <UNITID> Unit ID <EUNITID> Each Unit ID <LOWERRECIPEID> Lower Recipe ID <LOWERRECIPEIDVER> Lower Recipe ID Version <LEVEL> Level of RECIPEID 1 : Level 1 RECIPEID 2 : Level 2 RECIPEID 3 : Level 3 RECIPEID < RPARMNAME> Recipe Parameter Name < RPARMVAL> Recipe Parameter Value <RECIPEIDTYPE>Type of Recipe E : Equipment Recipe / M : Machine Recipe / U : Unit Recipe 1,2,3,.. : Special Recipe (Defined with LG Display) <STEP> Recipe Step 0 = Step Not Used, Another Number is Recipe Step |  |  |
| Exception |  |  |  |

| S9,F1 | Unrecognized Device ID | ECSEQP |  |
| --- | --- | --- | --- |
| Description | The device ID in the message block header did not correspond to any known device ID in the node detecting the error. |  |  |
| Structure | <MHEAD> |  |  |
| Notice |  |  |  |
| Exception |  |  |  |

| S9,F3 | Unrecognized Stream Type | ECSEQP |  |
| --- | --- | --- | --- |
| Description | The equipment does not recognize the stream type in the message block header |  |  |
| Structure | <MHEAD> |  |  |
| Notice |  |  |  |
| Exception |  |  |  |

| S9,F5 | Unrecognized Function Type | ECSEQP |  |
| --- | --- | --- | --- |
| Description | This message indicates that the function in the message ID is not recognized by the receiver |  |  |
| Structure | <MHEAD> |  |  |
| Notice |  |  |  |
| Exception |  |  |  |

| S9,F7 | Illegal Data | ECSEQP |  |
| --- | --- | --- | --- |
| Description | This message indicates that the stream and function were recognized, but the associated data format could not be interpreted |  |  |
| Structure | <MHEAD> |  |  |
| Notice |  |  |  |
| Exception |  |  |  |

| S9,F9 | Transaction Timer Timeout | ECSEQP |  |
| --- | --- | --- | --- |
| Description | This message indicates that a transaction(receive) timer has timed out and that the corresponding transaction has been aborted. It is up to the host to respond to this error in an appropriate manner to keep the system operational. |  |  |
| Structure | <MHEAD> |  |  |
| Notice |  |  |  |
| Exception |  |  |  |

| S9,F11 | Data Too Long | ECSEQP |  |
| --- | --- | --- | --- |
| Description | This message to the host indicates that the equipment has been sent more data than it can handle. |  |  |
| Structure | <MHEAD> |  |  |
| Notice |  |  |  |
| Exception |  |  |  |

| S9,F13 | Conversation Timeout | ECSEQP |  |
| --- | --- | --- | --- |
| Description | Data were expected but none was received within a reasonable length of time. Resources have been cleared. |  |  |
| Structure | <L,2> <A[6], >MEXP <A[14], >EDID |  |  |
| Notice | <MEXP> Message Expected The format is “SxxFyy” Ex) S07F83, S02F41 etc. <EDID> Expected Data ID MAX 14 Byte Case S7F83 : <EDID>= ‘<CSTID>’ Case S2F41 : <EDID>= ‘<CSTID>’ |  |  |
| Exception |  |  |  |

| S10,F3 | Terminal Display, Single | ECSEQP |  |
| --- | --- | --- | --- |
| Description | Data to be displayed |  |  |
| Structure | <L,2> <U2,0>TID <A[80], … >TEXT |  |  |
| Notice | <TID> Terminal Number Fixed as ‘0’ <TEXT> A single line of characters. |  |  |
| Exception |  |  |  |

| S10,F4 | Terminal Display, Single Acknowledge | ECSEQP |  |
| --- | --- | --- | --- |
| Description | Acknowledge or Error |  |  |
| Structure | <U1, >ACKC10 |  |  |
| Notice | <ACKC10> Acknowledge Code, 1 byte 0 = Accepted for display 1 = Message will not be displayed 2 = Terminal not available 3 – 63 Reserved |  |  |
| Exception |  |  |  |

Exception Message Structure

| S1,F69 | Change Glass Count Send | ECSEQP | reply |
| --- | --- | --- | --- |
| Description | This message is used when ECS informs engaged glass’ quantity data to the Equipment |  |  |
| Structure | <L,2> <U2,102>UNITID 102 fixed <U1, >GLSCNT |  |  |
| Notice | <GLSCNT> Total glasses count for processing <UNITID> Unit ID |  |  |
| Exception |  |  |  |

| S1,F70 | Change Glass Count Send Acknowledge | ECSEQP |  |
| --- | --- | --- | --- |
| Description | Acknowledge |  |  |
| Structure | <L,2> <U2,102>UNITID 102 fixed <U1, >ACKC1 |  |  |
| Notice | <UNITID> Unit ID <ACKC1> 0 = Accepted 1 = Not Accepted |  |  |
| Exception |  |  |  |

| S1,F71 | Change Glass Count Send Acknowledge | ECSEQP | reply |
| --- | --- | --- | --- |
| Description | The Equipment requests remained glass count from the lot. |  |  |
| Structure | <L,3> <U2,102>UNITID 102 fixed <U1, >PORTID <A[16], >LOTID |  |  |
| Notice | <UNITID> Unit ID <PORTID> Port ID <LOTID> Lot ID |  |  |
| Exception |  |  |  |

| S1,F72 | Remained Glass Count Send | ECSEQP |  |
| --- | --- | --- | --- |
| Description | Acknowledge |  |  |
| Structure | <L,4> <U2,102>UNITID 102 fixed <U1, >PORTID <A[16], >LOTID <U1, >REMAINGLSCNT |  |  |
| Notice | <UNITID> Unit ID <PORTID> Port ID <LOTID> Lot ID <REMAINGLSCNT> Remained Glass Count of Requested Lot |  |  |
| Exception |  |  |  |

| S1,F73 | Monitoring Lot Status Data Send | ECSEQP | reply |
| --- | --- | --- | --- |
| Description | This message sends monitoring lot status data. |  |  |
| Structure | <L,2> <U2,102>UNITID 102 fixed <A[1], >MLOTSTATUS |  |  |
| Notice | <UNITID> Unit ID < MLOTSTATUS> Monitoring Lot Status S = Start / E = End |  |  |
| Exception |  |  |  |

| S1,F74 | Monitoring Lot Status Data Send Acknowledge | ECSEQP |  |
| --- | --- | --- | --- |
| Description | Acknowledge |  |  |
| Structure | <L,2> <U2,102>UNITID 102 fixed <U1, >ACKC1 |  |  |
| Notice | <UNITID> Unit ID <ACKC1> 0 = Accepted 1 = Not Accepted |  |  |
| Exception |  |  |  |

| S1,F77 | Equipment’s Part Data Report | ECSEQP | reply |
| --- | --- | --- | --- |
| Description | This message is used for tracking of glasses which moved in among sub units |  |  |
| Structure | <L,1> sub UNIT count is fixed to 1 <L,3> <U2,102>UNITID 102 fixed <U1,100>SUNITID 100 fixed <L,n> n = glass count <L,2> <A[6], >CRRID1 <U2, >GLSCODE1 … <L,2> < A[6], >CRRIDn <U2, >GLSCODEn |  |  |
| Notice | <UNITID> Unit ID <SUNITID> Sub Unit ID <CRRID> Carrier ID <GLSCODE> Unique Glass Code defined by ECS. Equipment must keep this code, while glass is in equipment. |  |  |
| Exception | A one-to-one correspondence of UNITID and SUNITID depends on Equipment unit configuration |  |  |

| S1,F78 | Equipment’s Part Data Report Acknowledge | ECSEQP |  |
| --- | --- | --- | --- |
| Description | Acknowledge |  |  |
| Structure | <L,2> <U2,102>UNITID 102 fixed <U1, >ACKC1 |  |  |
| Notice | <UNITID> Unit ID <ACKC1> 0 = Accepted 1 = Not Accepted |  |  |
| Exception |  |  |  |

| S1,F79 | Buffer Lot Status Data Send | ECSEQP | reply |
| --- | --- | --- | --- |
| Description | This message sends Buffer lot status data. |  |  |
| Structure | <L,3> <U2,102>UNITID 102 fixed <A[1], >BLOTYPE <A[1], >BLOSTATUS |  |  |
| Notice | <UNITID> Unit ID <BLOTSTATUS> Buffer Lot Status S = Start E = End <BLOTTYPE> Buffer Lot Type L = Load to Buffer U = Unload from Buffer |  |  |
| Exception |  |  |  |

| S1,F80 | Buffer Lot Status Data Send Acknowledge | ECSEQP |  |
| --- | --- | --- | --- |
| Description | Acknowledge |  |  |
| Structure | <L,2> <U2,102>UNITID 102 fixed <U1, >ACKC1 |  |  |
| Notice | <UNITID> Unit ID <ACKC1> 0 = Accepted 1 = Not Accepted |  |  |
| Exception |  |  |  |

| S1,F81 | Port State Data Send | ECSEQP | reply |
| --- | --- | --- | --- |
| Description | This message is used to transfer Port State Data and Map Data |  |  |
| Structure | <L,7> <U2,102>UNITID 102 fixed <A[14], >CSTID <U1, >PORTID <A[2], >PORTSTATE <A[1], >LOTSTATE <A[40], … >CSTMAP <A[40], … >SELECTEDMAP |  |  |
| Notice | <UNITID> Controller ID <CSTID> CST ID <PORTID> Port ID <PORTSTATE> Port State LC : Cassette Load Complete UC : Cassette Unload Complete LR : Cassette Load Request UR : Cassette Unload Request <LOTSTATE> LOT State R : Ready W : Wait P : Processing E : End G : Going <CSTMAP> Cassette Mapping Information <SELECTEDMAP> Selected glass in the Cassette Map e.g. In case there are 2 glassed in Slot 1 and Slot 40. Then MAP = “1000000000000000000000000000000000000001” 0 : Glass Not Exist 1 : Glass Exist |  |  |
| Exception | In case of PORTSTATE is UC, MAP is space. |  |  |

| S1,F82 | Port State Data Send Acknowledge | ECSEQP |  |
| --- | --- | --- | --- |
| Description | Acknowledge |  |  |
| Structure | <L,2> <U2,102>UNITID 102 fixed <U1, >ACKC1 |  |  |
| Notice | <UNITID> Unit ID <ACKC1> 0 = Accepted 1 = Not Accepted |  |  |
| Exception |  |  |  |

| S1,F83 | Port State Data Report | ECSEQP | reply |
| --- | --- | --- | --- |
| Description | This message is used to transfer Port State Data and Map Data |  |  |
| Structure | <L,5> <U2,102>UNITID 102 fixed <A[14], >CSTID <U1, >PORTID <A[2], >PORTSTATE <A[1], >TRSMODE |  |  |
| Notice | <UNITID> Controller ID <CSTID> CST ID <PORTID> Port ID <PORTSTATE> Port State LC : Cassette Load Complete UC : Cassette Unload Complete LR : Cassette Load Request UR : Cassette Unload Request <TRSMODE> Transfer Mode E : PIO Enable D : PIO Disable |  |  |
| Exception |  |  |  |

| S1,F84 | Port State Data Report Acknowledge | ECSEQP |  |
| --- | --- | --- | --- |
| Description | Acknowledge |  |  |
| Structure | <L,2> <U2,102>UNITID 102 fixed <U1, >ACKC1 |  |  |
| Notice | <UNITID> Unit ID <ACKC1> 0 = Accepted 1 = Not Accepted |  |  |
| Exception |  |  |  |

| S1,F85 | Port Status Change Data Report | ECSEQP | reply |
| --- | --- | --- | --- |
| Description | This message is used when the Status (RUN, IDLE, DOWN) of Port in Equipment is changed. |  |  |
| Structure | <L,2> <U2,102>UNITID 102 fixed <L,n> n = number of Port <L,2> <U1, >PORTID1 <A[1], >PORTSTATUS1 ….. <L,2> <U1, >PORTIDn <A[1], >PORTSTATUSn |  |  |
| Notice | <UNITID> Controller ID <PORTID> Port ID <PORTSTATUS> Port Status R : RUN I : IDLE D : DOWN |  |  |
| Exception |  |  |  |

| S1,F86 | Port Status Change Data Report Acknowledge | ECSEQP |  |
| --- | --- | --- | --- |
| Description | Acknowledge |  |  |
| Structure | <L,2> <U2,102>UNITID 102 fixed <U1, >ACKC1 |  |  |
| Notice | <UNITID> Unit ID <ACKC1> 0 = Accepted 1 = Not Accepted |  |  |
| Exception |  |  |  |

| S1,F89 | Mask Data Report | ECSEQP | reply |
| --- | --- | --- | --- |
| Description | This message is used after the RFID of Mask reading in EQP |  |  |
| Structure | <L,4> <U2,102>UNITID 102 fixed <A[14], >MASKID <U1, >PORTID <L,n> n = number of MaskSheet <A[20], … >MASKSHEETID1 … < A[20], … >MASKSHEETIDn |  |  |
| Notice | <UNITID> Controller ID <MASKID> MASK ID <PORTID> Port ID <MASKSHEETID> MaskSheet ID |  |  |
| Exception |  |  |  |

| S1,F90 | Mask Data Report Acknowledge | ECSEQP |  |
| --- | --- | --- | --- |
| Description | Acknowledge |  |  |
| Structure | <L,2> <U2,102>UNITID 102 fixed <U1, >ACKC1 |  |  |
| Notice | <UNITID> Unit ID <ACKC1> 0 = Accepted 1 = Not Accepted |  |  |
| Exception |  |  |  |

| S1,F91 | All Mask Position Data Report | ECSEQP | reply |
| --- | --- | --- | --- |
| Description | This message is used for tracking of glasses which moved in among sub units |  |  |
| Structure | <L,1> SubUnit count is fixed to 1 <L,4> <U2,102>UNITID 102 fixed <U1,100>SUINTID 100 fixed <A[14], >MASKID <L,n> n = number of masksheet count <A[20], … >MASKSHEETID1 … <A[20], … >MASKSHEETIDn |  |  |
| Notice | <UNITID> Unit ID <SUNITID> Sub Unit ID <MASKID> Mask ID <MASKSHEETID>Masksheet ID |  |  |
| Exception | A one-to-one correspondence of UNITID and SUNITID depends on Equipment unit configuration |  |  |

| S1,F92 | All Mask Position Data Report Acknowledge | ECSEQP |  |
| --- | --- | --- | --- |
| Description | Acknowledge |  |  |
| Structure | <U1, >ACKC1 |  |  |
| Notice | <ACKC1> 0 = Accepted 1 = Not Accepted |  |  |
| Exception |  |  |  |

| S2,F71 | Carrier Transfer Mode Change Report | ECSEQP | reply |
| --- | --- | --- | --- |
| Description | This message is reported by equipment, when carrier transfer mode is changed |  |  |
| Structure | <L,2> <U2,102>UNITID 102 fixed <A[1], >CTMODE |  |  |
| Notice | <UNITID> Unit ID <CTMODE> Carrier Transfer Mode The following CTMODE values are defined. 1 = Normal Transferring 2 = Batch Transferring |  |  |
| Exception |  |  |  |

| S2,F72 | Carrier Transfer Mode Change Report Acknowledge | ECSEQP |  |
| --- | --- | --- | --- |
| Description | Acknowledge |  |  |
| Structure | <L,2> <U2,102>UNITID 102 fixed <U1, >ACKC2 |  |  |
| Notice | <UNITID> Unit ID <ACKC2> 0 = Accepted 1 = Not Accepted 2 = Unload Glass Not Exist 3 – 63 Reserved |  |  |
| Exception |  |  |  |

| S2,F77 | Glass Call Request | ECSEQP | reply |
| --- | --- | --- | --- |
| Description | This message is used when operator wants to inject glass into the process. |  |  |
| Structure | <L,3> <U2,102>UNITID 102 fixed <U1, >STAGEID <U2, >TARGETID |  |  |
| Notice | <UNITID> Unit ID <STAGEID>Stage ID <TARGETID> Target Unit ID |  |  |
| Exception |  |  |  |

| S2,F78 | Glass Call Request Acknowledge | ECSEQP |  |
| --- | --- | --- | --- |
| Description | Acknowledge |  |  |
| Structure | <L,2> <U2,102>UNITID 102 fixed <U1, >ACKC2 |  |  |
| Notice | <UNITID> Unit ID <ACKC2> 0 = Accepted 1 = Not Accepted 2 = Unload Glass Not Exist 3 – 63 Reserved |  |  |
| Exception |  |  |  |

| S2,F99 | Equipment PD Down Request | ECSEQP | reply |
| --- | --- | --- | --- |
| Description | This message is used when requesting the PD Down state change. |  |  |
| Structure | <L,2> <A[6], >RCMD <A[80], … >RESUME |  |  |
| Notice | <UNITID>Unit ID <RCMD>Remote command code or string – PAUSE / RESUME <HOSTMSG> Host message in Korean and English |  |  |
| Exception |  |  |  |

| S2,F100 | Equipment PD Down Request Acknowledge | ECSEQP |  |
| --- | --- | --- | --- |
| Description | Acknowledge |  |  |
| Structure | <L,2> <U2,102>UNITID 102 fixed <U1, >ACKC6 |  |  |
| Notice | <UNITID> Unit ID <ACKC6>s 0 = Accepted 1 = Not Accepted 2 = Already Release 3~ 63 Reserved |  |  |
| Exception |  |  |  |

| S7,F81 | Multi Recipe Information Send | ECS  EQP | reply |
| --- | --- | --- | --- |
| Description | This message is used to send each slot recipe ID to EQP. |  |  |
| Structure | <L,5> <U2,102>UNITID 102 Fixed <U1, >PORTID <A[16],LOTID >LOTID <U2, >LOTNUMBER .<L,n> n = The Number of slot . <L,4> <A[3], >SLOTID <B[1], >SELECTED <A[24], >RECIPEID <B[1], >SAMPLECODE … <L,4> <A[3], >SLOTIDn <B[1], >SELECTEDn <A[24], >RECIPEIDn <B[1], >SAMPLECODEn |  |  |
| Notice | <UNITID> Controller ID <PORTID> Port ID <LOTID> Lot ID <LOTNUMBER> Lot Unique Number <SLOTID> Slot ID <SELECTED> Glass Injection Flag 0 : Not Injection 1 : Injection <RECIPEID> Recipe ID <SAMPLECODE> Glass Inspection flag 0 : Not Inspection 1 : Inspection |  |  |

| S7,F82 | Multi Recipe Information Send Acknowledge | ECSEQP |  |
| --- | --- | --- | --- |
| Description | Acknowledge |  |  |
| Structure | <L,2> <U2,102>UNITID 102 Fixed <U1, >ACKC7 |  |  |
| Notice | <UNITID> Controller ID <ACKC7> Acknowledge code, 1byte 0 = Accepted 1 = Not Accepted 2 ~ 63 Reserved |  |  |
| Exception |  |  |  |

| S15,F71 | Recipe List Report Request | ECSEQP | reply |
| --- | --- | --- | --- |
| Description | This message is used when ECS requests recipe list of the equipment |  |  |
| Structure | <L,4> <U2,102>UNITID 102 fixed <A[1],1>LEVEL <A[1],E>RECIPEIDTYPE fixed E <L,n> n = number of LSD <L,2> <A[24], >ESDITEM1 <A[40], … > ESDVALUE1 …. <L,2> <A[24], >ESDITEMn <A[40], …>ESDVALUEn |  |  |
| Notice | <UNITID> Unit ID <LEVEL> Level of RECIPEID 1 : Level 1 RECIPEID 2 : Level 2 RECIPEID 3 : Level 3 RECIPEID <RECIPEIDTYPE>Type of Recipe E : Equipment Recipe / M : Machine Recipe / U : Unit Recipe 1,2,3,.. : Special Recipe (Defined with LG Display) <ESDITEM> EQP Specific Data Item defined by LG Display <ESDVALUE> EQP Specific Data Value defined by LG Display |  |  |
| Exception |  |  |  |

| S15,F72 | Equipment Recipe List Report | ECS EQP |  |
| --- | --- | --- | --- |
| Description | Acknowledge |  |  |
| Structure | <L,2> <L,n> n = number of Recipe ID <L,5> <U2,102>UNITID 102 fixed <A[24], >'RECIPEID <A[1], >LEVEL <A[1],E>RECIPEIDTYPE fixed E <L,1> number of version fixed to 1 <L,2> <A[12], >RECIPEIDVER1 <A[1], >RECIPEIDSTATUS1 <L,5> <U2,102>UNITID 102 fixed <A[24], >RECIPEIDn <A[1], >LEVELn <A[1],E>RECIPEIDTYPEn fixed E <L,1> <L,2> <A[12], >'RECIPEIDVERn <A[1], >RECIPEIDSTATUSn <L,m> m = number of ESD <L,2> <A[24], >ESDITEM1 <A[40], … >ESDVALUE1 … <L,2> <A[24], >ESDITEMm <A[40], … >ESDVALUEm |  |  |
| Notice | <UNITID> Unit ID <RECIPEIDTYPE>Type of Recipe E : Equipment Recipe / M : Machine Recipe / U : Unit Recipe 1,2,3,.. : Special Recipe (Defined with LG Display) <RECIPEID> Recipe ID <LEVEL> Level of Recipe ID <RECIPEIDVER> Recipe ID Version <RECIPEIDSTATUS> Recipe ID Status R : Running (using) I : Idle (Editable) D : Disable (Locked) <ESDITEM> EQP Specific Data Item defined by LG Display <ESDVALUE> EQP Specific Data Value defined by LG Display |  |  |
| Exception |  |  |  |

| S15,F73 | Lower Recipe List Report Request | ECSEQP | reply |
| --- | --- | --- | --- |
| Description | This message is used when ECS requests lower recipe list of the equipment |  |  |
| Structure | <L,2> <L,5> <U2,102>'UNITID 102 fixed <A[24], >'RECIPEID <A[1], >LEVEL <A[1],E>RECIPEIDTYPE fixed E <A[12], >'RECIPEIDVER <L,n> n = number of ESD <L,2> <A[24], >'ESDITEM <A[40], … >'ESDVALUE … <L,2> <A[24], >'ESDITEMn <A[40], … > 'ESDVALUEn |  |  |
| Notice | <UNITID> Unit ID <RECIPEID> Recipe ID <LEVEL> Level of Recipe ID <RECIPEIDVER> Recipe ID Version <RECIPEIDTYPE>Type of Recipe E : Equipment Recipe / M : Machine Recipe / U : Unit Recipe 1,2,3,.. : Special Recipe (Defined with LG Display) <ESDITEM> EQP Specific Data Item defined by LG. Display <ESDVALUE> EQP Specific Data Value defined by LG. Display |  |  |
| Exception |  |  |  |

| S15,F74 | Lower Recipe List Report | ECS EQP |  |
| --- | --- | --- | --- |
| Description | Acknowledge |  |  |
| Structure | <L,2> <L,n> n = number of Recipe ID <L,5> <U2,102>UNITID 102 fixed <A[24], … >RECIPEID1 <A[1], >LEVEL1 <A[1],E>RECIPEIDTYPE1 fixed E <L,1> number of version fixed to 1 <L,2> <A[12], >RECIPEIDVER1 <A[1], >RECIPEIDSTATUS1 … <L,5> <U2,102>UNITID 102 fixed <A[24], … >RECIPEIDn <A[1], >LEVELn <A[1],E>RECIPEIDTYPEn fixed E <L,1> number of version fixed to 1 <L,2> <A[12], >RECIPEIDVERn <A[1], >RECIPEIDSTATUSn <L,m> m = number of ESD <L,2> <A[24], >'ESDITEM1 <A[40], … >ESDVALUE1 … <L,2> <A[24], >ESDITEMm <A[40], … >ESDVALUEm |  |  |
| Notice | <UNITID> Unit ID <RECIPEID> Recipe ID <LEVEL> Level of Recipe ID <RECIPEIDVER> Recipe ID Version <RECIPEIDTYPE>Type of Recipe E : Equipment Recipe / M : Machine Recipe / U : Unit Recipe 1,2,3,.. : Special Recipe (Defined with LG Display) <ESDITEM> LOT Specific Data Item defined by LG. Display <ESDVALUE> LOT Specific Data Value defined by LG. Display |  |  |
| Exception |  |  |  |

| S15,F75 | Recipe Parameter Report Request | ECSEQP | reply |
| --- | --- | --- | --- |
| Description | This message is used when ECS requests recipe parameter of the equipment |  |  |
| Structure | <L,6> <U2,102>UNITID 102 fixed <A[24], >'RECIPEID <A[1], >LEVEL <A[1],E>RECIPEIDTYPE1 fixed E <A[12], >'RECIPEIDVER <L,m> m = number of ESD <L,2> <A[24], >'ESDITEM1 <A[40], … >ESDVALUE1 … <L,2> <A[24], >ESDITEMm <A[40], … >ESDVALUEm |  |  |
| Notice | <UNITID> Unit ID <RECIPEID> Recipe ID <LEVEL> Level of Recipe ID <RECIPEIDVER> Recipe ID Version <RECIPEIDTYPE>Type of Recipe E : Equipment Recipe / M : Machine Recipe / U : Unit Recipe 1,2,3,.. : Special Recipe (Defined with LG Display) <ESDITEM> EQP Specific Data Item defined by LG. Display <ESDVALUE> EQP Specific Data Value defined by LG. Display |  |  |
| Exception |  |  |  |

| S15,F76 | Recipe Parameter Report | ECS EQP |  |
| --- | --- | --- | --- |
| Description | This message reports the Recipe Parameter of the equipment |  |  |
| Structure | <L,2> <L,6> <U2,102>UNITID 102 fixed <A[24], >'RECIPEID <A[1], >LEVEL <A[1],E>RECIPEIDTYPE1 fixed E <A[12], >'RECIPEIDVER <L,n> n = number of Step <L,2> <A[14], >'STEP <L,m> m = number of Parameter <L,2> <A[24], >'RPARMNAME1 <A[24], >'RPARMVAL1 … <L,2> <A[24], >'RPARMNAMEm <A[24], >'RPARMVALm … <L,2> <A[14], >'STEPn <L,m> m = number of Parameter <L,2> <A[24], >'RPARMNAME1 <A[24], >'RPARMVAL1 … <L,2> <A[24], >'RPARMNAMEm <A[24], >'RPARMVALm <L,s> s = number of ESD <L,2> <A[24], >'ESDITEM <A[40], … >ESDVALUE … <L,2> <A[24], >'ESDITEMs <A[40], … >ESDVALUEs |  |  |
| Notice | <UNITID> Unit ID <RECIPEID> Recipe ID <LEVEL> Level of Recipe ID <RECIPEIDVER> Recipe ID Version <RECIPEIDTYPE>Type of Recipe E : Equipment Recipe / M : Machine Recipe / U : Unit Recipe 1,2,3,.. : Special Recipe (Defined with LG Display) <RPARMNAME> Recipe Parameter Name <RPARMVAL> Recipe Parameter Value <ESDITEM> EQP Specific Data Item defined by LG. Display <ESDVALUE> EQP Specific Data Value defined by LG. Display <STEP> Recipe Step 0 = Step Not Used, Another Number is Recipe Step |  |  |
| Exception |  |  |  |

| S15,F77 | Recipe Parameter Change Send | ECSEQP | reply |
| --- | --- | --- | --- |
| Description | This message commands Recipe Parameter change to the equipment |  |  |
| Structure | <L,8> <U2,102>UNITID 102 fixed <A[1], >RECIPEIDCMD1 <A[24], >RECIPEID <A[1], >LEVEL <A[12], >'RECIPEIDVER <A[1],E>RECIPEIDTYPE1 fixed E <L,3> <L,2> <L,n> n = number of Step <L,2> <A[14],0 >'STEP <L,m> m = number of Parameter <L,2> <A[24], >'RPARMNAME1 <A[24], >'RPARMVAL1 … <L,2> <A[24], >'RPARMNAMEm <A[24], >'RPARMVALm … <L,2> <A[14],0 >'STEPn <L,m> m = number of Parameter <L,2> <A[24], >'RPARMNAME1 <A[24], >'RPARMVAL1 … <L,2> <A[24], >'RPARMNAMEm <A[24], >'RPARMVALm <L,s> s = number of ESD <L,2> <A[24], >'ESDITEM <A[40], … >ESDVALUE … <L,2> <A[24], >'ESDITEMs <A[40], … >ESDVALUEs |  |  |
| Notice | <UNITID> Unit ID <RECIPEIDCMD> RECIPEID Command B : Built C : change D : Delete R : Rename <RECIPEID> Recipe ID <LEVEL> Level of RECIPEID <RECIPEIDVER> RECIPEID Version <RECIPEIDTYPE>Type of Recipe E : Equipment Recipe / M : Machine Recipe / U : Unit Recipe 1,2,3,.. : Special Recipe (Defined with LG Display) < RPARMNAME> Recipe Parameter Name < RPARMVAL> Recipe Parameter Value <ESDITEM> EQP Specific Data Item defined by LG. Display <ESDVALUE> EQP Specific Data Value defined by LG. Display <STEP> Recipe Step 0 = Step Not Used, Another Number is Recipe Step |  |  |
| Exception |  |  |  |

| S15,F78 | Recipe Change Command Acknowledge | ECS EQP |  |
| --- | --- | --- | --- |
| Description | Recipe Parameter change command acknowledge |  |  |
| Structure | <L,6> <U2,102>UNITID 102 fixed <A[24], >RECIPEID <A[1], >LEVEL <A[12], >RECIPEIDVER <A[1],E>RECIPEIDTYPE1 fixed E <U1, >REACK |  |  |
| Notice | <UNITID> Unit ID <LEVEL> Level of RECIPEID <RECIPEIDVER> RECIPEID Version <RECIPEIDTYPE>Type of Recipe E : Equipment Recipe / M : Machine Recipe / U : Unit Recipe 1,2,3,.. : Special Recipe (Defined with LG Display) <REACK> 0 : Accepted 1 : Permission not granted 2 : RECIPEID using 3 : Equipment not responding 4 : Version not found (Roll back) 5 : RECIPEID not found 6 : Length error 7 : Parameter range error 8 : Same name of recipes exist |  |  |
| Exception |  |  |  |

| S15,F79 | Recipe Parameter Change Report | ECS EQP | reply |
| --- | --- | --- | --- |
| Description | This message reports Recipe Parameter change from equipment |  |  |
| Structure | <L,8> <U2,102>UNITID 102 fixed <A[1], >RECIPEIDCMD <A[24], >'RECIPEID <A[1], >LEVEL <A[12], >RECIPEIDVER <A[1],E>RECIPEIDTYPE1 fixed E <L,n> n = number of Step <L,2> <A[14], >'STEP <L,m> m = number of Parameter <L,2> <A[24], >'RPARMNAME1 <A[24], >'RPARMVAL1 … <L,2> <A[24], >'RPARMNAMEm <A[24], >'RPARMVALm … <L,2> <A[14], >'STEPn <L,m> m = number of Parameter <L,2> <A[24], >'RPARMNAME1 <A[24], >'RPARMVAL1 … <L,2> <A[24], >'RPARMNAMEm <A[24], >'RPARMVALm |  |  |
| Notice | <UNITID> Unit ID <RECIPEIDCMD> RECIPEID Command B : Built C : change D : Delete R : Rename <RECIPEID> Recipe ID <LEVEL> Level of RECIPEID <RECIPEIDVER> RECIPEID Version <RECIPEIDTYPE>Type of Recipe E : Equipment Recipe / M : Machine Recipe / U : Unit Recipe 1,2,3,.. : Special Recipe (Defined with LG Display) < RPARMNAME> Recipe Parameter Name < RPARMVAL> Recipe Parameter Value <ESDITEM> EQP Specific Data Item defined by LG. Display <ESDVALUE> EQP Specific Data Value defined by LG. Display <STEP> Recipe Step 0 = Step Not Used, Another Number is Recipe Step |  |  |
| Exception |  |  |  |

| S15,F80 | Recipe Change Report Acknowledge | ECS  EQP |  |
| --- | --- | --- | --- |
| Description | Recipe Parameter change acknowledge |  |  |
| Structure | <L,6> <U2,102>UNITID 102 fixed <A[24], >RECIPEID <A[1], >LEVEL <A[12], >RECIPEIDVER <A[1],E>RECIPEIDTYPE1 fixed E <U1, >REACK |  |  |
| Notice | <UNITID> Unit ID <LEVEL> Level of RECIPEID <RECIPEIDVER> RECIPEID Version <RECIPEIDTYPE>Type of Recipe E : Equipment Recipe / M : Machine Recipe / U : Unit Recipe 1,2,3,.. : Special Recipe (Defined with LG Display) <REACK> 0 : Accepted 1 : Permission not granted 2 : RECIPEID using 3 : Equipment not responding 4 : Version not found (Roll back) 5 : RECIPEID not found 6 : Length error 7 : Parameter range error 8 : Same name of recipes exist |  |  |
| Exception |  |  |  |

ECS 통신시 상태, 모드 시점

Alarm 발생 및 해제 시점

알람은 발생 즉시 보고되며, Main Program의 Alarm Clear 버튼을 이용해서 해지한다.

여러 개의 알람이 동시에 발생하는 경우 대표 알람 한 개만 보고한다.

장비상태

| EQP STATUS | Reason |
| --- | --- |
| Idle | LOT END / Glass not exist in EQP |
| Run | After Glass In |
| Down |  |

Glass 상태 보고

All Glass Position Data Report (S1F67)를 보고하는 시점은 아래와 같다.

- Offline에서 Online으로 상태 변경 시.

- Glass의 Loading, Exchange, Unloading 완료 후(Robot End=>On)

장비 상태 보고

Equipment Status Change Report(S6F91)를 보고하는 시점은 아래와 같다.

- Alarm 발생 / 해지 시

- Glass Load 완료 시(R 보고)

- Glass Unload 완료 시(I 보고)

시간 동기화(S2F17)

- Offline => Online 전환 시

- 사용자 요청 시(UI에 버튼 구성)

Loopback Test(S2F25)

- 사용자 요청 시(UI에 버튼 구성)

Conversation Timeout(S9F13)

다음의 경우에 Conversation Timeout을 발생시킨다

- Glass Data Request(S6F61) 후 Glass Data(S2F87)가 전송되지 않을 때.

Transaction Sequence

Glass Loading

Offline → Online으로 전환 시 All Glass Position Data Report (S1F67) 전송 후

장비가 LR, UR 상태라면 전송함. Online 상태에서 장비가 LR, UR 상태가 되면 전송.

  - Glass Exchange

  - Glass Unloading

Alarm List

| No | Name | Description | Alarm Code | Alarm State | Alarm Enabled |
| --- | --- | --- | --- | --- | --- |
| 1 | LOADING (ROBOT) ALARM |  | 001 | Heavy Alarm |  |
| 2 | UNLOADING (ROBOT) ALARM |  | 002 | Heavy Alarm |  |
| 3 | LOADING (EQP) ALARM |  | 003 | Heavy Alarm |  |
| 4 | UNLOADING (EQP) ALARM |  | 004 | Heavy Alarm |  |
| 5 | EXCANGE (ROBOT) ALARM |  | 005 | Heavy Alarm |  |
| 6 | EXCANGE (EQP) ALARM |  | 006 | Heavy Alarm |  |
| 7 | ROBOT DOOR OPEN ALARM |  | 007 | Heavy Alarm |  |
| 8 | ROBOT EMO ALARM |  | 008 | Heavy Alarm |  |
| 9 | ERR_SPARE9 |  | 009 | Heavy Alarm |  |
| 10 | SCAN START TIME OVER ALARM |  | 010 | Heavy Alarm |  |
| 11 | SCAN END TIMVE OVER ALARM |  | 011 | Heavy Alarm |  |
| 12 | GRAB START TIME OVER ALARM |  | 012 | Heavy Alarm |  |
| 13 | GRAB END TIME OVER ALARM |  | 013 | Heavy Alarm |  |
| 14 | CONSOLE PROGRAM 상태 이상 ALARM |  | 014 | Heavy Alarm |  |
| 15 | CIM PROGRAM 상태 이상 ALARM |  | 015 | Heavy Alarm |  |
| 16 | CONSOLE PARAMETER SETTING ALARM |  | 016 | Heavy Alarm |  |
| 17 | CONTROL PARAMETER SETTING ALARM |  | 017 | Heavy Alarm |  |
| 18 | UINT ALIGN CAMERA 상태 이상 ALARM |  | 018 | Heavy Alarm |  |
| 19 | REVIW CAMERA 상태 이상 ALARM |  | 019 | Heavy Alarm |  |
| 20 | SCAN CAMERA 상태 이상 ALARM |  | 020 | Heavy Alarm |  |
| 21 | UINT ALIGN CAMERA 조명 상태 이상 ALARM |  | 021 | Heavy Alarm |  |
| 22 | REVIW CAMERA 조명 상태 이상 ALARM |  | 022 | Heavy Alarm |  |
| 23 | SCAN CAMERA 조명 상태 이상 ALARM |  | 023 | Heavy Alarm |  |
| 24 | AUTORUN STOP ALARM |  | 024 | Heavy Alarm |  |
| 25 | ERR_SPARE25 |  | 025 | Heavy Alarm |  |
| 26 | ERR_SPARE26 |  | 026 | Heavy Alarm |  |
| 27 | ERR_SPARE27 |  | 027 | Heavy Alarm |  |
| 28 | ERR_SPARE28 |  | 028 | Heavy Alarm |  |
| 29 | ERR_SPARE29 |  | 029 | Heavy Alarm |  |
| 30 | ERR_SPARE30 |  | 030 | Heavy Alarm |  |
| 31 | ERR_SPARE31 |  | 031 | Heavy Alarm |  |
| 32 | ERR_SPARE32 |  | 032 | Heavy Alarm |  |
| 33 | ERR_SPARE33 |  | 033 | Heavy Alarm |  |
| 34 | ERR_SPARE34 |  | 034 | Heavy Alarm |  |
| 35 | ERR_SPARE35 |  | 035 | Heavy Alarm |  |
| 36 | ERR_SPARE36 |  | 036 | Heavy Alarm |  |
| 37 | ERR_SPARE37 |  | 037 | Heavy Alarm |  |
| 38 | ERR_SPARE38 |  | 038 | Heavy Alarm |  |
| 39 | ERR_SPARE39 |  | 039 | Heavy Alarm |  |
| 40 | ERR_SPARE40 |  | 040 | Heavy Alarm |  |
| 41 | ERR_SPARE41 |  | 041 | Heavy Alarm |  |
| 42 | ERR_SPARE42 |  | 042 | Heavy Alarm |  |
| 43 | ERR_SPARE43 |  | 043 | Heavy Alarm |  |
| 44 | ERR_SPARE44 |  | 044 | Heavy Alarm |  |
| 45 | ERR_SPARE45 |  | 045 | Heavy Alarm |  |
| 46 | ERR_SPARE46 |  | 046 | Heavy Alarm |  |
| 47 | ERR_SPARE47 |  | 047 | Heavy Alarm |  |
| 48 | ERR_SPARE48 |  | 048 | Heavy Alarm |  |
| 49 | ERR_SPARE49 |  | 049 | Heavy Alarm |  |
| 50 | ELECTRIC PANEL DOOR SW OPEN ALARM |  | 050 | Heavy Alarm |  |
| 51 | ELECTRICITY BOX HIGH TEMP ALARM |  | 051 | Heavy Alarm |  |
| 52 | DOOR 1 LOCK SWITCH (FRONT TOP) ALARM |  | 052 | Heavy Alarm |  |
| 53 | DOOR 2 LOCK SWITCH (FRONT LEFT TOP) ALARM |  | 053 | Heavy Alarm |  |
| 54 | DOOR 3 LOCK SWITCH (FRONT CENTER TOP) ALARM |  | 054 | Heavy Alarm |  |
| 55 | DOOR 4 LOCK SWITCH (FRONT RIGHT TOP) ALARM |  | 055 | Heavy Alarm |  |
| 56 | DOOR 5 LOCK SWITCH (REAR RIGHT TOP) ALARM |  | 056 | Heavy Alarm |  |
| 57 | DOOR 6 LOCK SWITCH (REAR CENTER TOP) ALARM |  | 057 | Heavy Alarm |  |
| 58 | DOOR 7 LOCK SWITCH (REAR LEFT TOP) ALARM |  | 058 | Heavy Alarm |  |
| 59 | DOOR 8 LOCK SWITCH (FRONT LEFT BOTTOM) ALARM |  | 059 | Heavy Alarm |  |
| 60 | DOOR 9 LOCK SWITCH (FRONT CENTER BOTTOM) ALARM |  | 060 | Heavy Alarm |  |
| 61 | DOOR 10 LOCK SWITCH (FRONT RIGHT BOTTOM) ALARM |  | 061 | Heavy Alarm |  |
| 62 | DOOR 11 LOCK SWITCH (REAR RIGHT BOTTOM) ALARM |  | 062 | Heavy Alarm |  |
| 63 | DOOR 12 LOCK SWITCH (REAR CENTER BOTTOM) ALARM |  | 063 | Heavy Alarm |  |
| 64 | 비상탈출 스위치1 ALARM |  | 064 | Heavy Alarm |  |
| 65 | 비상탈출 스위치2 ALARM |  | 065 | Heavy Alarm |  |
| 66 | EMS SW 1 ALARM |  | 066 | Heavy Alarm |  |
| 67 | EMS SW 2 ALARM |  | 067 | Heavy Alarm |  |
| 68 | EMS SW 3 ALARM |  | 068 | Heavy Alarm |  |
| 69 | EMS SW 4 ALARM |  | 069 | Heavy Alarm |  |
| 70 | EMS SW 5 ALARM |  | 070 | Heavy Alarm |  |
| 71 | TEACHING PANDENT EMO BYPASS ON ALARM |  | 071 | Heavy Alarm |  |
| 72 | ELEC FAN ALARM 1 |  | 072 | Heavy Alarm |  |
| 73 | ELEC FAN ALARM 2 |  | 073 | Heavy Alarm |  |
| 74 | ELEC FAN ALARM 3 |  | 074 | Heavy Alarm |  |
| 75 | ELEC FAN ALARM 4 |  | 075 | Heavy Alarm |  |
| 76 | ELEC FAN ALARM 5 |  | 076 | Heavy Alarm |  |
| 77 | ELEC FAN ALARM 6 |  | 077 | Heavy Alarm |  |
| 78 | ELEC FAN ALARM 7 |  | 078 | Heavy Alarm |  |
| 79 | ELEC FAN ALARM 8 |  | 079 | Heavy Alarm |  |
| 80 | KEY RELEASE DOOR 1 ALARM |  | 080 | Heavy Alarm |  |
| 81 | KEY RELEASE DOOR 2 ALARM |  | 081 | Heavy Alarm |  |
| 82 | KEY RELEASE DOOR 3 ALARM |  | 082 | Heavy Alarm |  |
| 83 | KEY RELEASE DOOR 4 ALARM |  | 083 | Heavy Alarm |  |
| 84 | KEY RELEASE DOOR 5 ALARM |  | 084 | Heavy Alarm |  |
| 85 | KEY RELEASE DOOR 6 ALARM |  | 085 | Heavy Alarm |  |
| 86 | KEY RELEASE DOOR 7 ALARM |  | 086 | Heavy Alarm |  |
| 87 | GLASS1 CLAMP ALARM |  | 087 | Heavy Alarm |  |
| 88 | GLASS2 CLAMP ALARM |  | 088 | Heavy Alarm |  |
| 89 | GLASS1 DETECT ALARM |  | 089 | Heavy Alarm |  |
| 90 | GLASS2 DEFECT ALARM |  | 090 | Heavy Alarm |  |
| 91 | EFU ALARM |  | 091 | Heavy Alarm |  |
| 92 | ISOLATOR 공급1 (FRAME) ALARM |  | 092 | Heavy Alarm |  |
| 93 | ISOLATOR 공급2 (FRAME) ALARM |  | 093 | Heavy Alarm |  |
| 94 | ISOLATOR 공급1 (COVER) ALARM |  | 094 | Heavy Alarm |  |
| 95 | ISOLATOR 공급2 (COVER) ALARM |  | 095 | Heavy Alarm |  |
| 96 | CDA1 압력계 ALARM |  | 096 | Heavy Alarm |  |
| 97 | CDA2 압력계 ALARM |  | 097 | Heavy Alarm |  |
| 98 | CDA3 압력계 ALARM |  | 098 | Heavy Alarm |  |
| 99 | CDA4 압력계 ALARM |  | 099 | Heavy Alarm |  |
| 100 | CDA5 압력계 ALARM |  | 100 | Heavy Alarm |  |
| 101 | CDA6 압력계 ALARM |  | 101 | Heavy Alarm |  |
| 102 | CDA7 압력계 ALARM |  | 102 | Heavy Alarm |  |
| 103 | VAC1 압력계 ALARM |  | 103 | Heavy Alarm |  |
| 104 | VAC2 압력계 ALARM |  | 104 | Heavy Alarm |  |
| 105 | ERR_SPARE105 |  | 105 | Heavy Alarm |  |
| 106 | ERR_SPARE106 |  | 106 | Heavy Alarm |  |
| 107 | ERR_SPARE107 |  | 107 | Heavy Alarm |  |
| 108 | ERR_SPARE108 |  | 108 | Heavy Alarm |  |
| 109 | ERR_SPARE109 |  | 109 | Heavy Alarm |  |
| 110 | CONTACT X4-AXIS 위치 이상 ALARM |  | 110 | Heavy Alarm |  |
| 111 | CONTACT X5-AXIS 위치 이상 ALARM |  | 111 | Heavy Alarm |  |
| 112 | CAMERA Y-AXIS 위치 이상 ALARM |  | 112 | Heavy Alarm |  |
| 113 | CAMERA X2-AXIS 위치 이상 ALARM |  | 113 | Heavy Alarm |  |
| 114 | CAMERA X3-AXIS 위치 이상 ALARM |  | 114 | Heavy Alarm |  |
| 115 | REVIEW Y3-AXIS 위치 이상 ALARM |  | 115 | Heavy Alarm |  |
| 116 | REVIEW Y4-AXIS 위치 이상 ALARM |  | 116 | Heavy Alarm |  |
| 117 | WORKTABLE X1-AXIS 위치 이상 ALARM |  | 117 | Heavy Alarm |  |
| 118 | CONTACT Y2-AXIS 위치 이상 ALARM |  | 118 | Heavy Alarm |  |
| 119 | CONTACT Z-AXIS 위치 이상 ALARM |  | 119 | Heavy Alarm |  |
| 120 | STAGE U-AXIS 위치 이상 ALARM |  | 120 | Heavy Alarm |  |
| 121 | STAGE V-AXIS 위치 이상 ALARM |  | 121 | Heavy Alarm |  |
| 122 | STAGE W-AXIS 위치 이상 ALARM |  | 122 | Heavy Alarm |  |
| 123 | STAGE T-AXIS 위치 이상 ALARM |  | 123 | Heavy Alarm |  |
| 124 | CAMER TURN-AXIS 위치 이상 ALARM |  | 124 | Heavy Alarm |  |
| 125 | CONTACT X4-AXIS LIMIT 감지 ALARM |  | 125 | Heavy Alarm |  |
| 126 | CONTACT X5-AXIS LIMIT 감지 ALARM |  | 126 | Heavy Alarm |  |
| 127 | CAMERA Y-AXIS LIMIT 감지 ALARM |  | 127 | Heavy Alarm |  |
| 128 | CAMERA X2-AXIS LIMIT 감지 ALARM |  | 128 | Heavy Alarm |  |
| 129 | CAMERA X3-AXIS LIMIT 감지 ALARM |  | 129 | Heavy Alarm |  |
| 130 | REVIEW Y3-AXIS LIMIT 감지 ALARM |  | 130 | Heavy Alarm |  |
| 131 | REVIEW Y4-AXIS LIMIT 감지 ALARM |  | 131 | Heavy Alarm |  |
| 132 | WORKTABLE X1-AXIS LIMIT 감지 ALARM |  | 132 | Heavy Alarm |  |
| 133 | CONTACT Y2-AXIS LIMIT 감지 ALARM |  | 133 | Heavy Alarm |  |
| 134 | CONTACT Z-AXIS LIMIT 감지 ALARM |  | 134 | Heavy Alarm |  |
| 135 | STAGE U-AXIS LIMIT 감지 ALARM |  | 135 | Heavy Alarm |  |
| 136 | STAGE V-AXIS LIMIT 감지 ALARM |  | 136 | Heavy Alarm |  |
| 137 | STAGE W-AXIS LIMIT 감지 ALARM |  | 137 | Heavy Alarm |  |
| 138 | STAGE T-AXIS LIMIT 감지 ALARM |  | 138 | Heavy Alarm |  |
| 139 | CAMER TURN-AXIS LIMIT 감지 ALARM |  | 139 | Heavy Alarm |  |
| 140 | CONTACT X4-AXIS 축 상태 점검 ALARM |  | 140 | Heavy Alarm |  |
| 141 | CONTACT X5-AXIS 축 상태 점검 ALARM |  | 141 | Heavy Alarm |  |
| 142 | CAMERA Y-AXIS 축 상태 점검 ALARM |  | 142 | Heavy Alarm |  |
| 143 | CAMERA X2-AXIS 축 상태 점검 ALARM |  | 143 | Heavy Alarm |  |
| 144 | CAMERA X3-AXIS 축 상태 점검 ALARM |  | 144 | Heavy Alarm |  |
| 145 | REVIEW Y3-AXIS 축 상태 점검 ALARM |  | 145 | Heavy Alarm |  |
| 146 | REVIEW Y4-AXIS 축 상태 점검 ALARM |  | 146 | Heavy Alarm |  |
| 147 | WORKTABLE X1-AXIS 축 상태 점검 ALARM |  | 147 | Heavy Alarm |  |
| 148 | CONTACT Y2-AXIS 축 상태 점검 ALARM |  | 148 | Heavy Alarm |  |
| 149 | CONTACT Z-AXIS 축 상태 점검 ALARM |  | 149 | Heavy Alarm |  |
| 150 | STAGE U-AXIS 축 상태 점검 ALARM |  | 150 | Heavy Alarm |  |
| 151 | STAGE V-AXIS 축 상태 점검 ALARM |  | 151 | Heavy Alarm |  |
| 152 | STAGE W-AXIS 축 상태 점검 ALARM |  | 152 | Heavy Alarm |  |
| 153 | STAGE T-AXIS 축 상태 점검 ALARM |  | 153 | Heavy Alarm |  |
| 154 | CAMER TURN-AXIS 축 상태 점검 ALARM |  | 154 | Heavy Alarm |  |
| 155 | ERR_SPARE155 |  | 155 | Heavy Alarm |  |
| 156 | ERR_SPARE156 |  | 156 | Heavy Alarm |  |
| 157 | ERR_SPARE157 |  | 157 | Heavy Alarm |  |
| 158 | ERR_SPARE158 |  | 158 | Heavy Alarm |  |
| 159 | ERR_SPARE159 |  | 159 | Heavy Alarm |  |
| 160 | ERR_SPARE160 |  | 160 | Heavy Alarm |  |
| 161 | ERR_SPARE161 |  | 161 | Heavy Alarm |  |
| 162 | ERR_SPARE162 |  | 162 | Heavy Alarm |  |
| 163 | ERR_SPARE163 |  | 163 | Heavy Alarm |  |
| 164 | ERR_SPARE164 |  | 164 | Heavy Alarm |  |
| 165 | ERR_SPARE165 |  | 165 | Heavy Alarm |  |
| 166 | ERR_SPARE166 |  | 166 | Heavy Alarm |  |
| 167 | ERR_SPARE167 |  | 167 | Heavy Alarm |  |
| 168 | ERR_SPARE168 |  | 168 | Heavy Alarm |  |
| 169 | ERR_SPARE169 |  | 169 | Heavy Alarm |  |
| 170 | ERR_SPARE170 |  | 170 | Heavy Alarm |  |
| 171 | ERR_SPARE171 |  | 171 | Heavy Alarm |  |
| 172 | ERR_SPARE172 |  | 172 | Heavy Alarm |  |
| 173 | ERR_SPARE173 |  | 173 | Heavy Alarm |  |
| 174 | ERR_SPARE174 |  | 174 | Heavy Alarm |  |
| 175 | ERR_SPARE175 |  | 175 | Heavy Alarm |  |
| 176 | ERR_SPARE176 |  | 176 | Heavy Alarm |  |
| 177 | ERR_SPARE177 |  | 177 | Heavy Alarm |  |
| 178 | ERR_SPARE178 |  | 178 | Heavy Alarm |  |
| 179 | ERR_SPARE179 |  | 179 | Heavy Alarm |  |
| 180 | ERR_SPARE180 |  | 180 | Heavy Alarm |  |
| 181 | ERR_SPARE181 |  | 181 | Heavy Alarm |  |
| 182 | ERR_SPARE182 |  | 182 | Heavy Alarm |  |
| 183 | ERR_SPARE183 |  | 183 | Heavy Alarm |  |
| 184 | ERR_SPARE184 |  | 184 | Heavy Alarm |  |
| 185 | ERR_SPARE185 |  | 185 | Heavy Alarm |  |
| 186 | ERR_SPARE186 |  | 186 | Heavy Alarm |  |
| 187 | ERR_SPARE187 |  | 187 | Heavy Alarm |  |
| 188 | ERR_SPARE188 |  | 188 | Heavy Alarm |  |
| 189 | ERR_SPARE189 |  | 189 | Heavy Alarm |  |
| 190 | ERR_SPARE190 |  | 190 | Heavy Alarm |  |
| 191 | ERR_SPARE191 |  | 191 | Heavy Alarm |  |
| 192 | ERR_SPARE192 |  | 192 | Heavy Alarm |  |
| 193 | ERR_SPARE193 |  | 193 | Heavy Alarm |  |
| 194 | ERR_SPARE194 |  | 194 | Heavy Alarm |  |
| 195 | ERR_SPARE195 |  | 195 | Heavy Alarm |  |
| 196 | ERR_SPARE196 |  | 196 | Heavy Alarm |  |
| 197 | ERR_SPARE197 |  | 197 | Heavy Alarm |  |
| 198 | ERR_SPARE198 |  | 198 | Heavy Alarm |  |
| 199 | ERR_SPARE199 |  | 199 | Heavy Alarm |  |
| 200 | ERR_SPARE200 |  | 200 | Heavy Alarm |  |

Recipe

Scenario

Control State and Equipment Operation Mode Change

Are You There Request

장비의 운영이 시작되고 초기화 될 때, 장비는 ECS와 연결한 후 ECS로 ‘Are You There Request’ 메시지를 보내야 한다

Offline → Online Remote

- Offline 에서 Online Remote 로 상태 변경은 아래의 Scenario를 따른다

- 만약 응답이 없거나 통신 에러가 발생하면, Offline 상태를 계속 유지된다.

Offline → Online Local

- Offline 에서 Online Local로 상태 변경은 아래의 Scenario를 따른다.

- 만약 응답이 없거나 통신 에러가 발생하면, Offline 상태를 계속 유지된다.

Online Remote → Online Local

- Online Remote에서 Online Local로 상태 변경은 아래의 Scenario를 따른다.

Online Local → Online Remote

- Online Local 에서 Online Remote 로 상태 변경은 아래의 Scenario를 따른다.

Online(Remote or Local) → Offline

- Online Remote 에서 Offline으로 상태 변경은 아래의 Scenario를 따른다.

- Online 에서 Offline으로 전환 시, 응답이 없거나 통신 에러가 발생하더라도 off-line으로 전환된다.

Equipment Operation Mode Change (Under same Control State)

- 같은 통제 상태 하에서 운영모드의 변경은 다음 Scenario를 따른다. 이 경우, 장비는 통제 상태를 보내지 않는다.

- 만약 응답이 없거나 통신 에러가 발생하면, 현재의 모드를 유지한다.

ECS Restart

- ECS 재기동시, 장비와 ECS간의 통제 상태는 다음 Scenario에 의해 변경된다.

Loop Back Diagnostics

Equipment initiates Loop Back Test (ECS  EQP)

ECS initiates Loop Back Test (ECS  EQP)

Glass Transfer Sequence

Glass Move State ‘LR’ Request(Load Request)

‘Load Glass Data Request(S6F61)’는 장비에서 실제 Glass Load (‘LR’)가 발생하기

5 ~ 10초 이전에 보낼 수 있어야 한다.

장비의 특성에 따라 더 많은 시간이 요구될 때는 LG측과 협의가 필요하다.

      - Load Glass Data Accepted

      - Load Glass Data Not Accepted

Glass Move State ‘UR’ Request - Only Unload Stage Type

‘UW(S6F67)’는 장비에서 실제 Glass Unload (‘UR’)가 발생하기 5 ~ 10초 이전에 보낼 수 있어야 한다. 장비의 특성에 따라 더 많은 시간이 요구될 때는 LG측과 협의가 필요하다.

Glass Move State ‘ER’ Request

‘UW(S6F67)’는 장비에서 실제 Glass Exchange (‘ER’)가 발생하기 5 ~ 10 초 이전에 보낼 수 있어야 한다. 장비의 특성에 따라 더 많은 시간이 요구될 때는 LG측과 협의가 필요하다.

11.3.3.1    Load Glass Data Accepted

11.3.3.2	Load Glass Data Not Accepted – Glass Unloading Sequence after data not accept error release

11.3.3.3		Load Glass Data Not Accepted – Glass Exchange Sequence after data not accept error release

11.3.3.4	Load Glass Not Exist

Glass Move State ‘CR’ Request(Cancel Request)

- 세부 사항은 LG CNS, LG Display와의 협의가 필요하다

11.3.4.1		Case of Unload Glass Not Exist

11.3.4.2		Case of Unload Glass Exist

Load Glass Data Change Case

-	장비로 투입될 Glass의 정보가 변경되었을 때, ECS는 장비로 이 순서를 보낸다.

-	Glass data change message는 언제라도 보내질 수 있다.

-	장비는 ECS가 전송하는 S2F87 수신 후 Glass 존재 여부에 따라 S6F67 송신 여부를 판단한다.

-	세부 사항은 LG CNS, LG Display 와의 협의가 필요하다.

11.3.5.1		Change form Not Exist to Exist

1) Unload Glass Not Exist

2) Unload Glass Exist

- Case A : Glass Data Change Send (S2F87) before ‘UW’

2) Unload Glass Exist

- Case B : Glass Data Change Send ( S2F87 ) after ‘UW’

2) Unload Glass Exist

- Case C : Glass Data Change Send (S2F87) after ‘UR’

11.3.5.2		Change form Exist to Not Exist

1) Unload Glass Not Exist

2) Unload Glass Exist

- Case A : Glass Data Change Send ( S2F87 ) before ‘UW’

2) Unload Glass Exist

- Case B : Glass Data Change Send ( S2F87 ) after ‘UW’

2) Unload Glass Exist

- Case C : Glass Data Change Send ( S2F87 ) after ‘ER’

11.3.5.3	Change form Exist to Exist

1) Unload Glass Not Exist

2) Unload Glass Exist

Case A : Glass Data Change Send ( S2F87 ) before ‘UW’

2) Unload Glass Exist

- Case B : Glass Data Change Send ( S2F87 ) after ‘UW’

2) Unload Glass Exist

- Case C : Glass Data Change Send (S2F87 ) after ‘ER’

11.3.5.4		Unload Glass Data Request

-	이 메시지는 ECS가 ‘unload glass data report’를 받지 못했을 때 사용된다.

-	세부적인 사용에 대해서는 LG CNS, LG Display와의 협의가 필요하다

Glass Transfer In Online Local Control State

- 다음 Scenario는 Online Local 시 첫 번째 Glass를 장비에 투입할 때를 보여준다.

LOT Control Sequence

LOT End Data Send To Equipment

Local LOT Status Change Request on Online Local Control State

- Online local state 하에서 Lot end process 는 아래의 순서를 따른다

Status Report

Equipment Status Change Report

- Unit의 상태가 변할때마다 수행한다.

All Equipment Status Report

- ECS가 재시작하거나 Offline에서 Online으로 전환되는 경우에 모든 장비 상태를 보고한다(Remote or Local).

All Equipment Status Request

- 모든 장비 상태를 요청 시 수행한다.

OCR( or VCR ) Status Report

Ionizer Status Report

- Ionizer의 상태를 보고한다.

Recipe Management

Recipe ID List Request

Current Recipe ID Group List Request

Current Recipe ID List Request by Group

- 특정 Group에 속하는 Recipe ID List를 요청할 시 사용한다.

Recipe ID Confirm Request

Recipe Body Request

Recipe ID Confirm & Sampling Data Send

- 이 Scenario는 각 Slot별 모든 Recipes를 즉시 확인할 때 사용되고, 이 메시지는 Lot Recipe (cassette recipe) 인 하나의 Recipe에만 사용될 수 있다.

- 만약 EQP가 샘플링과 < Recipe ID Confirm >의 사용이 가능하다.면, 장비는 Sampling Data Report (S7F63) 를 반드시 ECS로 보내야 한다.

11.6.6.1		Recipe Accept (Single LOT)

11.6.6.2		Recipe Accept(Multi LOT)

11.6.6.3		Recipe Accept(Multi LOT)

Date and Time Data Send

Date and Time Request

Date and Time Set

Alarm Occurrence and Clear

Heavy Alarm

Light Alarm

Scrap Occurrence and Report

- 일반적으로 Scrap 보고는 장비에서 Glass 파손이 확인되는 즉시 ECS로 보고 해야 한다.

- 부득이한 경우는 Lot End 이전에는 보고 해야 한다.

All Glass Position Data Report

- Online으로 전환되는 경우

- 각 Sub unit간 glass가 이동하는 경우

Cycle Stop Status Report

Rework Request

- Operator가 Rework flag code를 설정하면 장비는 ECS에게 rework 요청을 한다.

Dummy Lot Request

- 만약 장비가 Dummy Lot을 사용해야 한다면 Dummy Lot이 필요한 시점에 ECS에게 요청한다.

Glass Data Request

- Operator가 Glass ID 또는 Glass Code로 Glass Data를 재 요청 시 사용한다.

- 세부적인 사용에 대해서는 LG CNS, LG Display의 협의가 필요하다.

Chemical Change

Normal Operation

- In case of 1 lot sequence (from lot start to end)

Glass Actual Processing Data

Report

Glass Actual Processing Data

Report

S6F74

S6F73

S6F74

S6F73

Lot Abort Scenario

The case where glass would be loaded is exist

The case where glass would be loaded is not exist

Error Messages

Message fault due to unrecognized Device ID

Message fault due to unrecognized Stream Type

Message fault due to unrecognized Function Type

Message fault due to Illegal Data Format

Message fault due to Transaction Timer Timeout

Message fault due to data too long

Message fault due to Conversation Timeout

Equipment Terminal Service

Exception Scenario

Inform processing glass count change scenario

장비로 투입될 Glass 수량이 변경되었을 경우 ECS는 해당 메시지를 장비로 송신한다.

- Cassette Loading 시

- Offline에서 Online 전환 시

- Cassette가 port에 로딩되고 작업준비가 된 후, 해당 Cassette가 cancel 되었을 시

Monitoring Lot Status Send

- 모니터링 Lot의 시작과 종료를 장비로 전달한다.

- 상세 사용은 LG CNS와 LG Display와 협의한다.

Equipment’s Part Data Report

- 온라인 전환 시 사용

- Glass 또는 장비의 Part가 Unit 또는 Sub Unit간에 이동이 있을 경우 장비는 보고한다.

Special Case of Glass Judge Report

Chamber Status Change Report

Recipe Management Sequence

Equipment Recipe List Request

- ECS가 장비의 recipe list를 조회 시 사용한다.

Equipment Lower Recipe List Request

- ECS가 장비의 하위 recipe list를 조회 시 사용한다.

Equipment Recipe Parameter Request

- 장비의 Recipe Parameter를 조회 시 사용한다.

Equipment Recipe Parameter Change Send

- 장비의 Recipe Parameter를 변경할 경우 사용한다.

Equipment Recipe Change Report

- 장비의 Recipe Parameter를 변경할 경우 사용한다.

Equipment Operation Reference Data Send

- 장비로 Operation Reference Data를 전송할 경우 사용한다.

Equipment Operation Reference Data Apply Report

- 장비가 operation reference data를 적용하면 장비는 이 메시지를 전송한다.

Current Recipe ID List Request

- 현재 장비 process program directory의 전송을 요청 시

- 장비에 등록된 process program의 모든 Recipe ID 와 Recipe ID Version 목록이다.

Equipment Constant Recipe Request

Equipment Constant Recipe Parameter Request

Equipment Constant Recipe Parameter Change Command

Equipment Constant Recipe Parameter Change Report

Buffer Lot Status Send

- Buffer Lot 상태를 전달할 경우 사용한다.

- 상세 사용은 LG CNS와 LG Display와 협의한다.

Remained Glass Count Request

- 장비가 해당 Glass가 속한 Lot의 투입 대기 Glass의 수량을 요청할 경우 사용한다.

- 상세 사용은 LG CNS와 LG Display와 협의한다.

Glass Call Request

- 수동으로 장비에 glass를 투입할 때 사용한다.

- 만약 장비에서 이를 사용하려면 추후 ECS와 상세 협의가 필요하다.

Carrier Transfer Mode Change Report

장비의 Carrier 반송 모드가 변경 보고 용도로 사용한다.

- 본 메시지는 장비의 Carrier 반송 모드가 변경이 되었을 경우 장비에서 보고한다.

- 장비의 보고에 대한 ECS 응답이 없는 경우 현재 상태를 유지한다.

- 상세 사용 및 용도는 LG CNS 및 LG Display 와 협의한다.

장비의 온라인 상태로 변경

( ECS Restart 포함 )중에 Carrier 반송 모드 변경 보고

- 장비는 Online 변경 중 Operation Mode 변경 보고 이후에 Carrier 반송 모드 상태를 보고한다.

PD Down Request

- 장비에 Pause를 요청할 때 사용한다.

- PD Request 시나리오는 Machine/Unit 단에서 Glass Hold 가 가능한 장비에 한해서 적용한다.

LOT End Glass Injection Send

- This Message is used when the last glass of LOT was injected in Equipment

In case the normal sequence

In case the abnormal sequence

Multi Recipe Information Send

- This message is used when equipment requires recipe and sampling codes for each slots

- If equipment want to use this sequence, it needs detailed conference later with ECS.

Port State and Cassette Map Data Send Scenario

- This message is used when equipment requires the map data for specific port.

- ECS informs the map data to equipment when Port state is changed

Batch Job (Glass Injection) End Data Send

- This message is used when the last glass of Batch Job is injected in Equipment.

- If equipment want to use this sequence, it needs detailed conference later with ECS.

Injecting Glass Data Request

- This message is used when the glass is injecting in Equipment.

- If equipment want to use this sequence, it needs detailed conference later with ECS.

Port State Data Report

- This message is used when the State (LR, LC, UR, UC) of Port in Equipment is changed.

- If equipment want to use this sequence, it needs detailed conference later with ECS.

Port Status Change Data Report

- This message is used when the Status (RUN, IDLE, DOWN) of Port in Equipment is changed.

- If equipment want to use this sequence, it needs detailed conference later with ECS.

Lot State Change Report

- This message is used when the Lot State (R, W, P, E) of port in Equipment is changed.

- If equipment want to use this sequence, it needs detailed conference later with ECS.

Mask Data Report

- This message is used after the ID of Mask reading in EQP .

- If equipment want to use this sequence, it needs detailed conference later with ECS.

All Mask Position Data Report

- In case of changing to Online

- In case of mask moving between each sub unit.

Unload Process Report

- Equipment performs this scenario when unit’s status is changed.

Unload Start & End

Unload Start & End (EQP Restart After Unload Start) 1/2

Unload Start & End (EQP Restart After Unload Start) 2/2

Unload Start & End (ECS Restart After Unload Start) 1/2

Unload Start & End (ECS Restart After Unload Start) 2/2