ToE ECS-Equipment Communication

Copyright  2011 LG CNS Co., Ltd., All rights reserved	0/219

Equipment Communication Specifications HSMS Type Ver 1.6

for LG Display ToE

한글 버전

# - Revision History -

| Version | Date | Person | Contents |
| --- | --- | --- | --- |
| 1.0 | 2017.06.22 | Lee Sung Woo | Version 1.0 for LG Display ToE |
| 1.1 | 2018.02.08 | Kim Si Won | Version 1.1 for LG Display ToE |
| 1.2 | 2018.02.26 | Kim Si Won | Version 1.2 for LG Display ToE |
| 1.3 | 2018.06.14 | Cha Jae Gook | Version 1.3 for LG Display ToE |
| 1.4 | 2018.06.19 | Lee Dae Ki | Version 1.4 for LG Display ToE |
| 1.5 | 2019.02.13 | Cha Jae Gook | Version 1.5 for LG Display ToE |
| 1.6 | 2019.05.09 | Cha Jae Gook | Version 1.6 for LG Display ToE |

- Revision List -

아래 변경 목록은 단순참고용으로 상세내역은 이 문서의 본문을 참조해야 한다.

- From Ver AP4 1.4 to Ver ToE 1.0

| No | Part or Section | Ver AP4 1.4 Page | Ver ToE 1.0 Page | Contents & Description |
| --- | --- | --- | --- | --- |
| 1 | 1.11 PD Down Request | p147 p181 | p147 p180 | Equipment PD Down Request S2,F95  S2,F99 Changed (7.15 Chemical Change, Stream/Function Overlap) |
| 2 | 19. Lot State Change Report | p152 p168 | p152 p181 | S1,F87  S6,F81 Changed (CA Specification Standard) |
| 3 | 6.9 Equipment Constant Recipe Request | p143 p184 | p143 p183 | S7,F81  S7,F79 Changed (CA Specification Standard) |
| 4 | 1.13 Multi Recipe Information Send | p149 p183 | p149 p186 | S7,F79  S7,F81 Stream/Function Changed (CA Specification Standard) |
| 5 | S7,F78 : Recipe Body S7,F80 : Equipment Constant Recipe S15,F76 : Recipe Parameter Report S15,F77 : Recipe Parameter Change Send S15,F79 : Recipe Parameter Change Report S15,F96 : Equipment Constant Recipe Parameter Send S15,F97 : Equipment Constant Recipe Change Command S15,F99 : Equipment Constant Recipe Change Report | - | p128 p185 p195 p196 p199 p208 p209 p212 | - A[14] <RPARMNAME>  A[24] <RPARMNAME> Changed - A[14] <RPARMVAL>  A[24] <RPARMVAL> Changed |

- From Ver ToE 1.0 to Ver ToE 1.1

| No | Part or Section | Ver 1.0 Page | Ver 1.1 Page | Contents & Description |
| --- | --- | --- | --- | --- |
| 1 | Load Glass Data Send (S2,F87) | 97-98 | 97-99 | Message Format Modify |
| 2 | Unload Glass Data Report (S6,F63) | 103- 104 | 104- 106 |  |
| 3 | Glass Move Request (S6,F67) | 107 | 109 |  |

| No | Part or Section | Ver 1.1 Page | Ver 1.2 Page | Contents & Description |
| --- | --- | --- | --- | --- |
| 1 | 7. Scenario | 30-35, 53,68, 69,71, 72,73, 149 | 30-35, 53,68, 69,71, 72,73, 149 | S6,F73 Sequence Changed (ToE Specification Standard) |

From Ver ToE 1.1 to Ver ToE 1.2

# - Revision List -

아래 변경 목록은 단순참고용으로 상세내역은 이 문서의 본문을 참조해야 한다.

- From Ver ToE 1.2 to Ver ToE 1.3

| No | Part or Section | Ver 1.2 Page | Ver 1.3 Page | Contents & Description |
| --- | --- | --- | --- | --- |
| 1 | Load Glass Data Send (S2,F87) | 97-99 | 98-100 | Message Format Modify |
| 2 | Unload Glass Data Report (S6,F63) | 104-106 | 105-107 |  |

| No | Part or Section | Ver 1.3 Page | Ver 1.4 Page | Contents & Description |
| --- | --- | --- | --- | --- |
| 1 | Load Glass Data Send (S2,F87) | 98-100 | 98-100 | GSD Item Add |
| 2 | Unload Glass Data Report (S6,F63) | 105-107 | 105-107 |  |
| 3 | Unload Glass Data Request (S2,F69) | 93 | 93 | 2. B[2] <STAGEID>  2. B[1] <STAGEID> Changed |

From Ver ToE 1.3 to Ver ToE 1.4

- From Ver ToE 1.4 to Ver ToE 1.5

| No | Part or Section | Ver 1.4 Page | Ver 1.5 Page | Contents & Description |
| --- | --- | --- | --- | --- |
| 1 | Load Glass Data Send (S2,F87) |  | 100 | GSD Item(Cut_Type) Change |
| 2 | Unload Glass Data Report (S6,F63) |  | 107 |  |

- From Ver ToE 1.5 to Ver ToE 1.6

| No | Part or Section | Ver 1.5 Page | Ver 1.6 Page | Contents & Description |
| --- | --- | --- | --- | --- |
| 1 | Message Format Change S2 F80, S2 F87, S6 F63, S6 F65, S6 F73, S7 F61, S2 F82, S2 F84, S2 F93 |  | 96,98,99, 105,108, 113,123, 124,176, 178,182 | <PNLIF> A[400]→A[2000] <SUBMDLIF> A[400]→A[2000] <SUBPNLIF> A[400]→A[2000] |

# - Explanation -

- 이 문서는 ECS에 적용된 Equipment Control System을 LG Display에 공급한 업체에 대한 장비 통신 사양서이다. 이 문서는 공통 장비 제어 통신을 다루기 위해 배포되었다.

- 이 사양서는 LG Display의 비즈니스 절차에 따라 배포된다.

- LG Display의 승인 없이, 이 명세서를 회사 밖으로 가지고 나가거나 배포/공유하는 경우, 해당 업체는 LG Display의 내부 방침에 따른 처벌과 보상을 해야 할 뿐만 아니라 ‘불공정 거래 방지법’과 ‘기업 비밀 보장법’에 의거 민/형사상 의 책임을 져야 한다.

- LG Display와 LG CNS는 이 명세서의 공도 저작권을 가지며 LG Display와 LG CNS의 사 전 합의 없이 이 문서를 복제하거나 수정, 배포하는 것은 금지되어 있다.

- This document is the equipment communication specification for the maker who supplies ‘Equipment Control System provides ECS to LG Display. This document is distributed for the purpose of dealing with common equipment control communication.

- This specification is distributed according to business procedure of LG Display.

- In case of draining away this specification outside the company, or distributing/sharing without permission of LG Display, the maker will be charged with not only the punishment

and compensation for damages according to internal provision of LG Display, but also any civil/criminal liability under the provision of ‘Law of Unfair Competition Prevention’ and ‘Law of Trade Secret Protection’.

- LG Display and LG CNS have the joint-ownership of the copyright of this specification, and it is prohibited to copy, modify or distribute this document without prior consent of LG Display and LG CNS.

# - Index -

Chapter Ⅰ. ECS-Equipment HSMS Standard Specification For LG Display

- System Format	8

- HSMS(High-speed SECS Message Services)	9

- SECS - II	12

- Software Specifications	16

- Mode Definition	20

- Required Items of the Counter Specification Document	21

- Scenario

  - Control State and Equipment Operation Mode Change	22

  - Loop Back Diagnostics	28

  - Glass Transfer Sequence	29

  - Lot Control Sequence	53

  - Status Report	55

  - Recipe Management	57

  - Date and Time Data Send	60

  - Alarm Occurrence and Clear	61

  - Scrap Occurrence and Report	62

  - All Glass Position Data Report	62

  - Cycle Stop Status Report	63

  - Rework Request	64

  - Dummy Lot Request	65

  - Glass Data Request	65

  - Chemical Change	66

  - Normal Operation	67

  - Lot Abort Scenario	72

  - Error Messages	75

  - Equipment Terminal Service	77

- Standard Message Structure	78

Chapter Ⅱ. ECS-Equipment HSMS Exception Specification For LG Display

- Scenario

  - Inform processing glass count change scenario	141

  - Monitoring Lot Status Send	141

  - Equipment’s Part Data Report	141

  - Special Case of Glass Judge Report	142

  - Chamber Status Change Report	143

  - Recipe Management Sequence	144

  - Buffer Lot Status Send	147

  - Remained Glass Count Request	147

  - Glass Call Request	148

  - Carrier Transfer Mode Change Report	149

  - PD Down Request	150

  - LOT End Glass Injection Send	151

  - Multi Recipe Information Send	152

  - Port State and Cassette Map Data Send Scenario	152

  - Batch Job (Glass Injection) End Data Send	153

  - Injecting Glass Data Request	153

  - Port State Data Report	154

  - Port Status Change Data Report	154

  - Lot State Change Report	155

  - Mask Data Report	155

  - All Mask Position Data Report	156

  - Unload Process Report	157

- Exception Message Structure	162

- Attached Page	217

ChapterⅠ. ECS-Equipment HSMS Standard Specification

For LG Display

1.1 System Configuration

# System Format

본 문서는 장비와 ECS (Equipment Control System)간의 통신 사양에 적용된다. 이 사양은 하나의

Loader와 여러 종의 장비간 제어 시스템을 정의한다. 장비는 하나 또는 그 이상 존재할 수 있다.

<Active>

IP : 192.168.7.2

100 BASE-T

ECS

EQP#1

<Active>

IP : 192.168.7.3

HUB

<Passive>

EQP#2

100 BASE-T

IP : 192.168.7.1

<HSMS>

EQP#n

<Active>

IP : 192.168.7.n+1

100 BASE-T

<MELSECNET/G> or <CC-Link>

…

DIO(D-Sub 37 Pin)

& EMO Cable

Loader

ECS 는 장비가 요청하는 Glass ID, LOT ID, CST ID, Recipe ID, Slot ID 등과 같은 data들을 주고 받 는다. ECS와 장비간 통신은 HSMS Protocol에 의해 이루어진다.

주의) ECS와 장비가 로컬 네트워크로 연결되어 있는 경우, 다른 시스템과 절대 연결되어서는 안 된다.

- HSMS (High-speed SECS Message Services)

더 많은 정보를 얻고자 하는 사람은 "SEMI E37 HIGH-SPEED SECS MESSAGE SERVICES (HSMS)

GENERIC SERVICES" 를 참고하기 바랍니다.

### Physical Interface

    - Frame Type

IEEE 802.3u – Fast Ethernet (100Mbps)

    - Connector RJ-45 (8-pin)

    - Ethernet Media 100BaseTX (Twisted Pair)

    - Signal Pins

Following table shows each pin of Connector functions .(Pin 1,2,3,6 used only.)

| Pin | Signal | Description |
| --- | --- | --- |
| 1 | Transmit + | Transmit Positive Signal |
| 2 | Transmit - | Transmit Negative Signal |
| 3 | Receive + | Receive Positive Signal |
| 4 | Not used | - |
| 5 | Not used | - |
| 6 | Receive - | Receive Negative Signal |
| 7 | Not used | - |
| 8 | Not used | - |

[Process]	[ECS]

1	1

2	2

3	3

6	6

### HSMS Message Format

    - Message Format

Maximum Message Length is 2 Mbytes.

| Number of Bytes | Description |
| --- | --- |
| 4 Bytes | Message Length. MSB First. |
| 10 Bytes | Message Header |
| 0 – n Bytes | Message Text |

    - Message Header

Following table shows the message header structure, and P type always uses "0"(SECS-II Encoding).

| Bytes | Description |
| --- | --- |
| 0~1 | Session ID (Device ID). |
| 2 | Header Byte 2 |
| 3 | Header Byte 3 |
| 4 | P type |
| 5 | S type |
| 6~9 | System Bytes |

Following table shows the selectable value in S type. Deselect and Reject Procedure is not used.

| Value | Description |
| --- | --- |
| 0 | Data Message |
| 1 | Select.req |
| 2 | Select.rsp |
| 5 | Linktest.req |
| 6 | Linktest.rsp |
| 9 | Separate.req |

### The method of communication

    - 통신연결

TCP/IP (Transmission Control Protocol/Internet Protocol)

    - Parameter 설정

HSMS 구현은 반드시 아래의 parameters에 대한 시간정보 설정에 대비해야 한다. Protocol Parameter의 범위 및 해결방안은 최소한 아래 표와 같이 나타나야 한다. 각각의 설정치는 사용자에 의해 장비에서 수시로 변경 가능하도록 한다. 단 connect mode는 항상 장비가 ACTIVE 이고, ECS가 PASSIVE이다.

| Symbol | Parameter Name | Typical Value | Value Range |
| --- | --- | --- | --- |
| DEVID | Device ID(Session ID) | 0 | 0 ~ 32767 |
| T3 | Reply Timeout | 90 sec | 1 ~ 120 sec |
| T5 | Separation Timeout | 10 sec | 1 ~ 240 sec |
| T6 | Transaction Timeout | 5 sec | 1 ~ 240 sec |
| T7 | NOT SELECTED Timeout | 10 sec | 1 ~ 240 sec |
| T8 | Intercharacter Timeout | 5 sec | 1 ~ 120 sec |
| CT | Conversation Timeout | 180 sec | 0 ~ 600 sec |
| Connect Mode | Connect Mode (Active/Passive) | - | - |
| Local Entity | Local Entity IP Address & Port Number | - | - |
| Remote Entity | Remote Entity IP Address & Port Number | - | - |

    - Transaction

Transaction allowed in states.

| Transaction | Allowed in State | Who initiates Transaction? |
| --- | --- | --- |
| Select | HSMS Not Selected | Active Entity |
| Linktest | HSMS Selected | Either Entity |
| Separate | HSMS Selected | Either Entity |

    - Procedures Description

      - “Select”(선택 절차)

선택 절차는 컨트롤 상태에서 선택 요청(Select.req) 메시지를 사용하는 TCP/IP 연결상의 HSMS통신을 설정 할 때 사용된다. 선택 절차는 반드시 “NOT SELECTED” 상태에서만 가능하다. 성공적으로 0 선택 상태로 전환되지 못한 선택 절차는 반드시 TCP/IP 통신을 닫고, “Not CONNECTED” 상태로 전환되어야 한다.

      - “Linktest”(연결테스트 절차)

링크 테스트는 TCP/IP와 HSMS 통신의 동작상의 완결성을 결정짓는데 사용된다. “CONNECTED” 에서 이것의 사용은 언제라도 허용된다. 링크 테스트는 반드시 장비 연결 테스트에 적용된다.

        - ‘SELECTED’상태에서 장비 (Active)는 ECS (Passive)로 반드시 주기적으로 ‘Linktest.req’를 보내야 한다.

        - ‘SELECTED’상태, 그리고 (Active)로부터 120초 동안 아무런 메시지를 받지 못했을 때, ECS (Passive)는 장비 ( Active )로 반드시 ‘Linktest.req’를 보내야 한다.

      - “Separate”(분리 절차)

분리 절차는 TCP/IP 통신이 끊어지기 전에 갑자기 HSMS 통신이 끊어졌을 때 사용된다. HSMS는 연결 상태가 ‘SELECTED’ 상태에서 분리를 사용했을 때 요구한다. 실제 응답이 보내지지 않고, 로컬 상태인지의 여부와는 상관없이 통신의 중단을 요구한다.

        - 상태가 ‘SELECTED’에서 ‘NOT SELECTED’로 변경이 될 때, EQP는 반드시 ‘Separate.req’를 ECS로

보낸다.

# SECS - II

더 많은 정보를 원할 시에는 "SEMI E5-95 SEMI EQUIPMENT COMMUNICATIONS STANDARD 2

MESSAGE CONTENT (SECS-II)" 혹은 이후 버전을 참고하시기 바랍니다.

### Abort Transaction (S*FO)

Function 0은 모든 Stream 에 존재하며 각각의 Stream에서 같은 특별한 의미를 가진다. Function 0 Message는 Transaction Timeout이 발생하는 것을 기다리지 않기 위해 Transaction을 종료한다. Function 0 은 Transaction Error 나 다른 몇 가지의 원인으로 인해 해석기가 기대되는 응답을 보낼 수 없을 때 그것을 대신하여 보내진다. 해석기가 Transaction을 종료하기 위해 Function 0을 보내는 것은 요구가 아니다.

### Conversation Timeout

CT (Conversation Timeout) 는 ECS로부터 적정한 시간 내에 기대하는 응답을 받지 못했을 때 발생하는 일종의

timeout 메시지이다.

### Interleaved Requests Interaction

비 동시적인 메시지의 상호작용은 작업 중 동시에 하나이상의 요청을 포함할 수도 있다. 예를 들어, ECS가 장비로부터 여러 가지의 동작 (응답 메시지)를 요구한다. 장비는 그 행동을 수행하고, 독립적으로 결과를 보내준다. ECS가 두 개(혹은 그 이상) 의 Primary 메시지를 순서대로 보낼 때, 장비는 반드시 모든 Primary 메시지에 대해 Secondary 메시지를 보내야 한다. ECS는 응답 메시지를 결정되지 않은 순서대로 받는다. 평행적인 요구에 대한 이 메커니즘은 분산처리 수행과 스레드 안전 프로그래밍의 복잡한 요구가 아닌

비 동시적인 메시지의 자연스러운 사용이다.

### A List of Stream/Function

    - A Standard Stream/Function

| S | F (Primary) | F (Secondary) | Primary Direction | Name of Function |
| --- | --- | --- | --- | --- |
| * | 0 | - | ECS 🠜🠞 EQP | Abort Transaction |
| 1 | 1 | 2 | ECS 🠜🠞 EQP | Are You There Request |
| 1 | 61 | 62 | ECS 🢤 EQP | All Equipment Status Report |
| 1 | 63 | 64 | ECS 🢧 EQP | All Equipment Status Request |
| 1 | 67 | 68 | ECS 🢤 EQP | All Glass Position Data Report |
| 1 | 75 | 76 | ECS 🢤 EQP | Ionizer Status Report |
| 2 | 17 | 18 | ECS 🢤 EQP | Date and Time Request |
| 2 | 25 | 26 | ECS 🠜🠞 EQP | Loopback Diagnostic Request |
| 2 | 31 | 32 | ECS 🢧 EQP | Date and Time Data Send |
| 2 | 61 | 62 | ECS 🢤 EQP | Control State Change Request |
| 2 | 63 | 64 | ECS 🢤 EQP | Equipment Operation Mode Report |
| 2 | 65 | 66 | ECS 🢤 EQP | Local LOT Status Change Request |
| 2 | 67 | 68 | ECS 🢤 EQP | Cycle Stop Status Report |
| 2 | 69 | 70 | ECS 🢧 EQP | Unload Glass Data Request |
| 2 | 75 | 76 | ECS 🢤 EQP | Chemical Change Status Report |
| 2 | 79 | 80 | ECS 🢤 EQP | Glass Data Request |
| 2 | 87 | 88 | ECS 🢧 EQP | Load Glass Data Send |
| 2 | 95 | 96 | ECS 🢤 EQP | Chemical Change Enable Request |
| 2 | 97 | 98 | ECS 🢧 EQP | Chemical Change Enable Send |
| 5 | 1 | 2 | ECS 🢤 EQP | Alarm Report |
| 6 | 61 | 62 | ECS 🢤 EQP | Load Glass Data Request |
| 6 | 63 | 64 | ECS 🢤 EQP | Unload Glass Data Report |
| 6 | 65 | 66 | ECS 🢧 EQP | LOT End Data Send |
| 6 | 67 | 68 | ECS 🢤 EQP | Glass Move Request |
| 6 | 69 | 70 | ECS 🢤 EQP | Cassette Map Request |
| 6 | 71 | 72 | ECS 🢤 EQP | Sampling Slot Request |
| 6 | 73 | 74 | ECS 🢤 EQP | Glass Actual Processing Data Report |
| 6 | 83 | 84 | ECS 🢤 EQP | LOT Actual Processing Data Report |
| 6 | 85 | 86 | ECS 🢤 EQP | Dummy Lot Request |
| 6 | 89 | 90 | ECS 🢤 EQP | Rework Request |
| 6 | 91 | 92 | ECS 🢤 EQP | Equipment Status Change Report |
| 6 | 93 | 94 | ECS 🢤 EQP | Scrap Data Report |

| 6 | 95 | 96 | ECS 🢤 EQP | OCR( VCR ) Status Report |
| --- | --- | --- | --- | --- |
| 7 | 19 | 20 | ECS 🢧 EQP | Current EPPD Request |
| 7 | 61 | 62 | ECS 🢧 EQP | Recipe ID Confirm Request |
| 7 | 63 | 64 | ECS 🢤 EQP | Sampling Data Report |
| 7 | 67 | 68 | ECS 🢧 EQP | Current Recipe Group List Request |
| 7 | 69 | 70 | ECS 🢧 EQP | Current Recipe ID List Request by Group |
| 7 | 77 | 78 | ECS 🢧 EQP | Recipe Body Request |
| 9 | 1 | - | ECS 🢤 EQP | Unrecognized Device ID |
| 9 | 3 | - | ECS 🢤 EQP | Unrecognized Stream Type |
| 9 | 5 | - | ECS 🢤 EQP | Unrecognized Function Type |
| 9 | 7 | - | ECS 🢤 EQP | Illegal Data |
| 9 | 9 | - | ECS 🢤 EQP | Transaction Timer Timeout |
| 9 | 11 | - | ECS 🢤 EQP | Data Too Long |
| 9 | 13 | - | ECS 🢤 EQP | Conversation Timeout |
| 10 | 3 | 4 | ECS 🢧 EQP | Terminal Display, Single |

Note)	EQP : Equipment,

ECS : Equipment Control System S/F : Stream/Function

    - A Exception Stream/Function

| S | F (Primary) | F (Secondary) | Primary Direction | Name of Function |
| --- | --- | --- | --- | --- |
| 1 | 69 | 70 | ECS 🢧 EQP | Change Glass Count Send |
| 1 | 71 | 72 | ECS 🢤 EQP | Remained Glass Count Request |
| 1 | 73 | 74 | ECS 🢧 EQP | Monitoring Lot Status Data Send |
| 1 | 77 | 78 | ECS 🢤 EQP | Equipment’s Part Data Report |
| 1 | 79 | 80 | ECS 🢧 EQP | Buffer Lot Status Data Send |
| 1 | 81 | 82 | ECS 🢧 EQP | Port State Data Send |
| 1 | 83 | 84 | ECS 🢤 EQP | Port State Data Report |
| 1 | 85 | 86 | ECS 🢤 EQP | Port Status Change Data Report |
| 1 | 87 | 88 | ECS 🢤 EQP | Lot State Change Report |
| 1 | 89 | 90 | ECS 🢤 EQP | Mask Data Report |
| 1 | 91 | 92 | ECS 🢤 EQP | All Mask Position Data Report |
| 2 | 71 | 72 | ECS 🢤 EQP | Carrier Transfer Mode Change Report |
| 2 | 77 | 78 | ECS 🢤 EQP | Glass Call Request |
| 2 | 81 | 82 | ECS 🢤 EQP | Lost Glass Data Request |
| 2 | 83 | 84 | ECS 🢤 EQP | Injecting Glass Data Request |
| 2 | 85 | 86 | ECS 🢤 EQP | Unload Process Report |
| 2 | 91 | 92 | ECS 🢧 EQP | Batch Job (Glass Injection) End Data Send |
| 2 | 93 | 94 | ECS 🢤 EQP | Various Data Report |
| 2 | 99 | 100 | ECS 🢧 EQP | Equipment PD Down Request |
| 6 | 97 | 98 | ECS 🢤 EQP | Chamber Status Change Report |
| 7 | 79 | 80 | ECS 🢧 EQP | Equipment Constant Recipe Request |
| 7 | 81 | 82 | ECS 🢧 EQP | Multi Recipe Information Send |
| 15 | 65 | 66 | ECS 🢧 EQP | Current Recipe ID List Request |
| 15 | 71 | 72 | ECS 🢧 EQP | Recipe List Report Request |
| 15 | 73 | 74 | ECS 🢧 EQP | Lower Recipe List Report Request |
| 15 | 75 | 76 | ECS 🢧 EQP | Recipe Parameter Report Request |
| 15 | 77 | 78 | ECS 🢧 EQP | Recipe Parameter Change Send |
| 15 | 79 | 80 | ECS 🢤 EQP | Recipe Parameter Change Report |
| 15 | 81 | 82 | ECS 🢧 EQP | Operation Reference Data Send |
| 15 | 83 | 84 | ECS 🢤 EQP | Equipment Operation Reference Data Apply Report |
| 15 | 95 | 96 | ECS 🢧 EQP | Equipment Constant Recipe Parameter Request |
| 15 | 97 | 98 | ECS 🢧 EQP | Equipment Constant Recipe Change Command |
| 15 | 99 | 100 | ECS 🢤 EQP | Equipment Constant Recipe Change Report |

Note)	EQP : Equipment,

ECS : Equipment Control System S/F : Stream/Function

# Software Specifications

### Codes and ID

    - CST ID (Cassette ID)

CST ID는 개개의 Cassette를 나타내는 고유한 이름이다.

#### LOT ID

1 - LOT 은 Cassette에 로딩된 Glass (Panel)에 대한 논리적인 ID로서, 특별한 경우를 제외하고 1-Cassette 와 동등하다.

    - Recipe ID (PPID)

Recipe ID는 장비가 공정을 진행하기 위해 작성하는 Process Program에 대한 ID로서, 대소문자 구별 없이 사용한다.

#### GLS ID (PNL ID)

이것은 각각의 Glass (Panel)에 대한 유일한 이름으로서, ECS로부터 다운로드 받거나 OCR (VCR)이 장착된 장비는 Glass (Panel)로부터 OCR (혹은 VCR)을 통해 읽어 들인다.

    - 장비는 CST ID, LOT ID, GLS ID, PNL ID,RECIPEID 같은 문자열을 확인한다. 만약 ID가 공백이나 제어 불가능한 문자나 정확하지 않는 ID길이를 포함할 시 Error처리하고, 재입력을 요청한다.

#### SLOT ID

SLOT ID 는 Cassette에 Glass (Panel)가 들어가는 고유한 위치를 나타낸다. SLOT ID의 발번 규칙은 아래 그림과 같이 정면에서 보았을 때 상단에서 하단으로 1번에서부터 ‘1’씩 증가한다.

1

2

.	1	.

2	.

.

.

n-1 n

#### PORT ID

PORT ID 는 각각의 Port를 구별하는 고유한 번호이다.

n-1 n

    - Unit ID (for EQP ↔ ECS)

장비의 개별 Unit에 매겨지는 고유한 번호이다. Unit ID는 LGD(or ECS) 에 의해 할당되고, 추후 상세항목에

대한 협의가 필요하다.

아래 표는 단순히 참고사항입니다.

    - Sub Unit ID

장비의 각 부분과 세부 Unit에 사용되는 고유 번호이다. 이는 Sub Unit ID 와 같을 수도 있고, 다를 수도 있다. 이에 대한 추후 상세 협의가 필요하다.

    - Alarm Code (<ALCD>)

ALCD is divided Alarm Set/Cleared bit and Alarm Category bit. ALCD Bit

|  | 8 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 = Not Used |
|  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 = Personal Safety |
|  |  |  | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 2 = Equipment Safety |
|  |  |  | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 3 = Parameter control warning |
|  |  |  | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 4 = Parameter control error |
|  |  |  | 0 | 0 | 0 | 0 | 1 | 0 | 1 | 5 = Irrecoverable error |
|  |  |  | 0 | 0 | 0 | 0 | 1 | 1 | 0 | 6 = Equipment status warning |
|  |  |  | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 7 = Attention flag |
|  |  |  | 0 1 = | 0 Alar | 0 m Se | 1 t | 0 | 0 | 0 | 8 = Data integrity |

0 = Alarm reset

    - Different ID Code (<DICODE>)

      - : ECS에서 받은 Glass ID와 VCR 혹은 OCR을 통해 장비로부터 읽어 들인 Glass ID 가 정확하게 일치할 때

      - : ECS에서 받은 Glass ID와 VCR 혹은 OCR을 통해 장비로부터 읽어 들인 Glass ID 가 일치하지 않을 때

      - : 각각의 Glass ID가 장비에 있는 VCR or OCR에 의해 확인되지 않았을 때

    - Scrap Type Code (<SCRPTYPE>)

Glass 또는 Panel에 Scrap 이 발생함을 나타낸다.

- G : Glass에 Scrap 발생.	- P : Panel에 Scrap 발생.

    - Port Transfer State Code(<PORTSTATE>)

- LR : Load 요청	- LC : Load 완료

- UR : Unload 요청	- UC : Unload 완료

    - Judgment Information(<PNLIF>,<SUBMDLIF>,<GLSJUDGE>,<LOTJUDGE>)

장비에서의 각각의 판정의 사용과 설정치는 LG Display와 협의해야 한다.

<PNLIF> : Panel 판정 정보 . Panel에 대한 판정은 영문 한 글자로 표현되며, 이는 Good / NG / Repair / Rework 을 의미한다.

<SUBMDLIF> : Sub Panel 판정 정보 . Sub Panel에 대한 판정은 영문 한 글자로 표현되며, 이는 Good / NG / Repair

/ Rework을 의미한다.

<GLSJUDGE> : Glass Judge. 이는 한 글자 또는 두 글자의 영문으로 표현되며, 이는 Good / NG / Rework을 의미한다.

<LOTJUDGE> : LOT Judge. 이는 한 글자 또는 두 글자의 영문으로 표현되며, 이는 Good / NG / Rework / 1 Step Skip / 2 Step Skip / Hold를 의미한다.

* 장비는 반드시 최소한 16개 이상의 (<PNLIF>, <SUBPNLIF>, <GLSJUDGE>,<LOTJUDGE>) judgment에 대해 응답할 수 있어야 한다.

    - Glass Key Code (<KEYID>)

Glass 데이터에 대한 고유한 값이다. 이는 동일 LOT의 모든 Glass는 반드시 같은 값을 가진다.

- 형태 : <LOTID(14Byte)_YYYYMMDDhhmmss(14Byte)>

- 예 : <7EA001000000_20050301115623>

    - Glass Order (<GLSODR>)

- : 첫 번째 Glass

- : 첫 번째 Glass와 마지막 Glass를 제외한 모든 Glass

- : 마지막 glass ( 이 경우 , 이 값은 Scrap 또는 LOT Abort 에 대해서는 전달되지 않는다 )

- : LOT에 Glass가 한 장 밖에 없을 경우, 유일한 Glass를 나타낸다.

    - Cassette Port Status (<CSTREADY>) 0 : Port에 Cassette가 존재하지 않을 때. 1 : Port에 Cassette가 존재할 때.

    - Stage ID (<STAGEID>)

장비에서 Glass가 투입 / 취출되는 영역에 대한 고유한 번호를 나타낸다.

    - Naming rule of Actual Processing Data ( APD )

      - Numeric Data

        - Data가 숫자일지라도, 프로세스는 ASCII로 보고해야 한다.

        - 부호가 처음에 위치하고, 좌측에서 우측으로 채워나가고, 나머지는 공백으로 채운다. 만약 첫 번째 위치하는 부호가 공백일 때는 무효로 처리한다.

(예외 : “[]”는 문자 리스트의 나눔, “{}”는 생략가능, “ ”은 공백)

-고정소수점 : [{+}-][string]{[소수점l][string]}[space...]

-가변: [{+}-][string]{[소수점][string]}[E][{+}-][space...] 예) “849	”

#### “-3.14E-10	”

      - Character String Data

부호는 좌측에서 우측으로 채워나가고, 나머지는 공백으로 한다. 만약 첫번째 부호가 공백이면 무효이

다.

예) “ABCDEF	”

    - Part Number ( <PARTNUM> )

LOT 의 Model 종류를 말한다. 해당 Data 사용시 LG Display와 협의한다.

    - Prod Type ( <PRODTYPE> )

공정을 시작하려는 LOT의 속성을 말한다. 장비는 PRODTYPE 을 참조하여 공정을 변경하거나

Glass 의 장비 내 이동을 설정할 수 있다. 자세한 사항에 대하여는 LG Display와 협의한다

    - Attribute ( <ATTRIBUTE> )

공정을 시작하려는 LOT의 특이 사항 Data이다. 해당 LOT의 Model Type 또는 공정 관련한 특이 Data를

전송한다. 장비는 ATTRIBUTE 을 참조하여 공정을 변경하거나 Glass 의 장비 내 이동을 설정할 수 있다

    - Recipe Parameter Name ( < RPARMNAME> )

Recipe Parameter Name을 의미한다. Parameter Name은 RPARMVAL과 짝을 이루며, RPARMNAME은 PPAMRVAL의 Name이다. 항목의 선정은 LG Display와 협의하며, 꼭 영문 대문자와 ‘_’만을 사용한다

    - Recipe Parameter Value ( <RPARMVAL> )

RPARMNAME에 대한 Recipe Parameter Value 이다. List내에 배치되는 순서와 관계 없이 짝을 이루는

RPARMNAME에 해당하는 Value여야 한다.

### The conservation of ECS communication history

(Log - Interlock Communication & SECS Message )

장비는 적어도 최근 3일 간의 ECS 통신 데이터와 Interlock 통신 데이터를 저장할 능력을 가져야 한다. 유효기간이 만료되었을 시 Log는 자동으로 삭제된다.

### Change of HSMS communication parameter.

HSMS 통신 parameter 는 장비 내에서 사용자에 의해 언제라도 변경이 가능하다.

### Manual Correction of Glass Information

예기치 않게 ECS로부터 받은 Glass 정보가 사라졌을 때, 장비는 반드시 ECS로 수동으로 Glass 정보

(e.g. GLS ID, Original Port ID, Original Slot ID, 등 … ) 를 입력하여 보낼 수 있어야 한다.

### Injection Glasses of Multi LOTs

진행중인 장비에 복수 개의 LOT을 투입할 수 있어야 한다.하지만 Glass는 순차적으로 투입해야 한다. 장비가 한 LOT의 Glass를 처리하고 있을 때, 다른 LOT의 Glass를 장비에 투입할 수 있어야 한다. 모든 형태의 장비 (Chamber or 다른 형태) 가 가능해야 한다.

### Caution about Glass Order( <GLSODR> )

<GLSODR> = 2일 떄 , 이 값( = 2 )은 Glass Scrap 혹은 LOT Abort 때문에 전달되지 않을 수도 있다.

만약 장비에서 어떠한 GLSODR 정보를 사용하여 무언가 할 가능성이 있다면, 장비업체는 LG CNS와 협의해야 한다.

### Using of Glass Data Request ( S2F79, S2F80 )

이 stream function 은 수동사용 상황, 장비에서 Glass 정보를 상실했을 경우에 사용된다. 하지만 이때 장비는 ‘Glass ID or Glass Code’ (또는 에러 상황에서 작업자의 Glass ID 또는 Glass Code 입력 )를 가지고 있다. 만약 장비가 이 stream function을 사용하면, 수동 조작 하에서 S2F79를 요청함에 있어 사용자 접속환경을 제공한다.(예. 작업자가 GLSID또는 GLSCODE를 UI화면에서 입력하고, 작업자의 확인 버튼.. 등)

# Mode Definition

기본적으로 장비는 각각의 운영 모드로 변경하는 기능을 가지고 있다. 필요한 경우가 발생하면, ECS와 장비 업체간에 협의를 통해 ECS는 이를 할 수 있다.

| Communication State | Control state |  | Operation Mode | Description |
| --- | --- | --- | --- | --- |
| Communicating | Online | Remote | 장비의 특징적인 운영 모드 NOTE) It needs conference with ECS later. | 장비로의 Glass 투입은 ECS가 결정 한다 |
|  |  | Local |  | 장비는 Glass 가 장비로 투입될지를 결정 하고 ECS로부터 Glass 를 요청한다 |
|  | Offline | N/A |  | 이 상태에서 장비는 S2F25를 제외하고 ECS로부터 받는 모든 최초의 메시지에 대 해서 S*F0로 응답한다. 그리고 ECS는 장 비의 S1F1/ S2F25/ S2F61를 2차 메시지로 응답한다. 또한 장비는 S1F1/ S2F25/ S2F61를 제외한 어떤 1차 메시지를 보내서는 안 된다. ECS 는 S2F25를 제외한 어떤 메시지도 보낼 수 없다. 장비가 Offline 상태일 때, 장비는 반드시 ECS로 부터 받은 이전 진행상태의 데이터 를 가지고 있어야 한다. |
| Not communicating | N/A |  |  |  |

### Communicating

물리적으로 통신이 이루어진 상태

### Not communicating

물리적으로 통신이 이루어지지 않은 상태

### Online – Remote

HSMS 통신이 성립됨. 이 상태에서는 ECS Glass 처리에 대한 권한을 가진다.

즉, 장비가 Glass 투입을 요청하면, ECS는 적절한 시간에 Glass를 장비에 투입한다. Online – Remote 요청은 언제나 가능하다.

### Online – Local

HSMS 통신이 성립됨. 이 경우, 장비는 Glass 처리에 대한 권한을 가진다.

즉, 장비가 Glass의 진행에 대한 결정을 하고 , ECS로부터 Glass를 요청 받는다 . Online – Local 요청은 언제나 가능하다.

### Offline

통신이 물리적으로 연결된 상태. 하지만 ECS와 장비 양쪽모두 목적에 맞는 메시지를 보내지 못한다.

이 경우, ECS와 장비는 위 테이블에서 정의한 메시지를 제외한 모든 메시지에 대해 처리 실패를 수행한다. Offline 요청은 언제나 가능하다.

# Required Items of the Counter Specification Document

대응 사양서는 최소한 다음과 같은 사항을 포함하여야 한다.

### Control unit function을 포함한 장비 구성도

🖙 장비 내부 제어 구성도와 Subunit구성 및 구성도

  - 전기적 interface 특성과 통신규약 (communication protocol)

  - 지원되는 Stream Function 목록과 Message Block의 세부사항

  - Stream Function내의 각 Data(혹은 code)에 대한 기술

🖙 언제, 어떻게, 무엇을, 수서에 어떤 영향을 미치는가 등등..

### ECS와 통신시, 상태, 모드, 시점 등에 대한 세부적인 기술

🖙 Alarm 발생 / 해제 시점 기술

🖙 장비 상태에 대한 정의와 변경 시점

🖙 Glass loading / unloading에 대한 정의와 요청 시점

  - Transaction Sequence에 대한 세부적인 기술

다음 순서에 대한 설명을 위해 적절한 차트가 필요하다. Message Block 안의 각 단계의 코드 상태를 포함한다.

🖙 최초의 시작 순서

🖙 Glass 작업 순서

🖙 LOT End 진행 (Last Glass 진행) 순서

🖙 각 운영 모드의 순서

note) 각 Sequence 상세 묘사에는 Loader와 장비간의 하드웨어 인터페이스인 Interlock diagram을 포함한다.

### 장비와 각 unit별(ALID, ALCD, ALTX) Alarm ID 표

  - A.P.D / Recipe Body list

🖙 보고 data (Data에 대한 설명, 설정치, 이름…)

🖙 Data 단위 (℃, Å, ㎖ etc)

🖙 각 Data의 허용치(범위), 보고된 각각의 Data의 명확한 기준(약속) (Ex : -10↔100℃)

🖙 각 Data의 보고 형식 (Ex : 10E-3 or 10.000 etc)

🖙 Glass unit과 LOT unit의 A.P.D format이 같다 하더라도 이 둘은 구분할 수 있어야 한다.

  - RECIPE ID 세부사항(명명규칙, 최대 / 최소 등록 개수 etc)

  - 장비 통신 사용자 환경

  - 각 운영 모드의 설명

🖙 동의에 의해 수용된 운영 모드의 세부사항

🖙 만약 운영상에 제약이 있다면, 이는 기술되어야 한다.

# Scenario

이번 단락에서는 각 경우의 일반적인 순서를 보여준다. 이는 LG의 필요에 의해 더해지거나 변경 가능하다.

### Control State and Equipment Operation Mode Change

    - Are You There Request

- 장비의 운영이 시작되고 초기화 될 때, 장비는 ECS와 연결한 후 ECS로 ‘Are You There Request’ 메시지를 보내야 한다.

## ECS	EQP

Confirm connection to ECS

S1F1 S1F2

Are You There Request

    - Offline  Online Remote

- Offline 에서 Online-Remote 로 상태 변경은 아래의 Scenario를 따른다.

- 만약 응답이 없거나 통신 에러가 발생하면, Offline 상태를 계속 유지된다

## ECS	EQP

S2F61

S2F62

S2F63

S2F64

S2F17

S2F18

S1F61

S1F62

If OCR( or VCR ) is equipped in the Equipment

S1F67

S1F68

S6F95

S6F96

이후의 Sequence는 장비의 현재 상태에 따라 결정된다.

- 장비내의 Unloading Glass가 존재하지 않으면 ‘LR’ sequence (7.3.1)를

- 장비내의 Unloading Glass가 존재하는 경우 ‘UR’ Sequence(7.3.2) 또는 ‘ER’ sequence(7.3.3 )를 따른다.

Operation Mode Change Button Click

Online-Remote Change Button Click

Control State Change Request

<RCS> = ‘0’ ( Online – Remote )

Equipment Operation Mode Report

<OPMODE> = defined number

Date and Time Request

All Equipment Status Report

All Glass Position Data Report

OCR( or VCR ) Status Report

    - Offline  Online Local

      - Offline 에서 Online Local 로 상태 변경은 아래의 Scenario를 따른다.

      - 만약 응답이 없거나 통신 에러가 발생하면, Offline 상태를 계속 유지된다.

## ECS	EQP

Online-Local Change Button Click

S2F61

S2F62

S2F63

S2F64

S2F17

S2F18

S1F61

S1F62

S1F67

S1F68

If OCR( or VCR ) is equipped in the Equipment

S6F95

S6F96

Operation Mode Change Button Click

Control State Change Request

<RCS> = ‘1’ ( Online-Local )

Equipment Operation Mode Report

<OPMODE> = defined number

Date and Time Request

All Equipment Status Report

All Glass Position Data Report

OCR( or VCR ) Status Report

    - Online Remote  Online Local

- Online Remote 에서 Online Local 로 상태 변경은 아래의 Scenario를 따른다.

S2F61

S2F62

S2F63

S2F64

ECS	EQP

Online Local Change Button Click

Control State Change Request

<RCS> = ‘1’ ( Online-Local )

Operation Mode Change Button Click

Equipment Operation Mode Report

<OPMODE> = defined number

    - Online Local  Online Remote

      - Online Local 에서 Online Remote 로 상태 변경은 아래의 Scenario를 따른다.

## ECS	EQP

S2F61

S2F62

S2F63

S2F64

이후의 Sequence는 장비의 현재 상태에 따라 결정된다.

- 장비내의 Unloading Glass가 존재하지 않으면 ‘LR’ sequence (7.3.1)를

- 장비내의 Unloading Glass가 존재하는 경우 ‘UR’ Sequence(7.3.2) 또는 ‘ER’ sequence(7.3.3 )를 따른다.

Operation Mode Change Button Click

Online Remote Change Button Click

Control State Change Request

<RCS> = ‘0’ ( Online-Remote )

Equipment Operation Mode Report

<OPMODE> = defined number

    - Online(Remote or Local)  Offline

- Online Remote에서 Offline으로 상태 변경은 아래의 Scenario를 따른다.

- Online에서 Offline으로 전환 시, 응답이 없거나 통신에러가 발생하더라도 off-line으로 전환된다.

S2F61 S2F62

Offline Change Button Click

ECS	EQP

Control State Change Request

<RCS> = ‘2’ ( Offline )

    - Equipment Operation Mode Change (Under same Control State)

- 같은 통제 상태 하에서 운영모드의 변경은 다음 Scenario를 따른다. 이 경우, 장비는 통제 상태 보고를 보내지 않는다.

- 만약 응답이 없거나 통신 에러가 발생하면, 현재의 모드를 유지한다.

## ECS	EQP

Operation Mode Change Button Click

S2F63

S2F64

Equipment Operation Mode Report

<OPMODE> = defined number

이후의 Sequence는 장비의 현재 상태에 따라 결정된다.

- 장비내의 Unloading Glass가 존재하지 않으면 ‘LR’ sequence (7.3.1)를

- 장비내의 Unloading Glass가 존재하는 경우 ‘UR’ Sequence(7.3.2) 또는 ‘ER’ sequence(7.3.3 )를 따른다.

    - ECS Restart

- ECS 재기동시, 장비와 ECS간의 통제 상태는 다음 Scenario에 의해 변경된다.

Confirm connection to ECS

ECS	EQP

#### S1F1 S1F2

Are You There Request

If RCS = 2 (Offline) then

#### S2F61 S2F62

Control State Change Request (Report current control state )

<RCS> = ‘0’ ( Online – Remote )

<RCS> = ‘1’ ( Online – Local )

<RCS> = ‘2’ (Offline)

Equipment maintain Offline state.

RCS = 2	Current Control State ?

S2F63 S2F64

S2F17 S2F18

S1F61 S1F62

RCS = 0 or 1

Equipment Operation Mode Report

<OPMODE> = defined number

Date and Time Request

All Equipment Status Report

If OCR( VCR ) is equipped in the Equipment

#### S1F67 S1F68

S6F95 S6F96

All Glass Position Data Report

OCR( VCR ) Status Report

이후의 Sequence는 장비의 현재 상태에 따라 결정된다.

- 장비내의 Unloading Glass가 존재하지 않으면 ‘LR’ sequence (7.3.1)를

- 장비내의 Unloading Glass가 존재하는 경우 ‘UR’ Sequence(7.3.2) 또는 ‘ER’ sequence(7.3.3 )를 따른다.

### Loop Back Diagnostics

    - Equipment initiates Loop Back Test (ECS  EQP)

S2F25 S2F26

ECS	EQP

Loopback Diagnostic Request

    - ECS initiates Loop Back Test (ECS  EQP)

S2F25 S2F26

Loopback Diagnostic Request

## ECS	EQP

### Glass Transfer Sequence

    - Glass Move State ‘LR’ Request(Load Request)

‘Load Glass Data Request(S6F61)’는 장비에서 실제 Glass Load (‘LR’)가 발생하기 5 ~ 10초 이전에 보낼 수 있어야 한다.

장비의 특성에 따라 더 많은 시간이 요구 될 때는 LG측과 협의가 필요하다.

      - Load Glass Data Accepted

## ECS	EQP

S6F61

S6F62

Load Glass Data Send

<GLSSTATUS> = ‘1’(Exist)

S2F87

S2F88

If the equipment were able to receive the glass.

S6F67

S6F68

S6F91

S6F92

S1F67 S1F68

Loading Glass

Load Glass Data Request

<ACK> = ‘0’(Data Accepted)

Glass Move Request

#### <GLSMVSTATUS> = ‘LR’

If the equipment status is changed.

Equipment Status Change Report

All Glass Position Data Report

      - Load Glass Data Not Accepted

## ECS	EQP

#### S6F61 S6F62

Load Glass Data Request

Load Glass Data Send

<GLSSTATUS> = ‘1’(Exist)

If equipment status is

changed.

S2F87 S2F88

S5F1 S5F2

S6F91 S6F92

<ACK> ≠ ‘0’(Data Not Accepted) Alarm Set Report

Equipment Status Change Report

Operator Call, Error Release

If equipment status is changed.

#### S5F1 S5F2

S6F91 S6F92

Alarm Release Report

Equipment Status Change Report

#### S6F61 S6F62

Load Glass Data Request

Load Glass Data Send

<GLSSTATUS> = ‘1’(Exist)

If equipment were able to

receive the glass.

#### S2F87 S2F88

S6F67 S6F68

<ACK> = ‘0’(Data Accepted) Glass Move Request

#### <GLSMVSTATUS> = ‘LR’

Loading Glass

If equipment status is changed.

#### S6F91 S6F92

Equipment Status Change Report

#### S1F67 S1F68

All Glass Position Data Report

Copyright  2011 LG CNS Co., Ltd., All rights reserved	30/219

    - Glass Move State ‘UR’ Request(Unload Request) - Only Unload Stage Type

‘UW(S6F67)’는 장비에서 실제 Glass Unload (‘UR’)가 발생하기 5 ~ 10초 이전에 보낼 수 있어야 한다. 장비의 특성에 따라 더 많은 시간이 요구 될 때는 LG측과 협의가 필요하다.

S6F63

S6F64

It limits in the equipment

which uses the UW

S6F67

S6F68

If the Equipment were able to unload the glass.

S6F67

S6F68

If the Equipment status is changed.

S6F91

S6F92

S6F73 S6F74

S6F73 S6F74

S1F67

S1F68

장비는 Unloading Glass Count 만큼 각 Glass 별 S6F73 보고 실시함. 단, A/B Stick 보고 순서는 무관함.

Unloading Glass

ECS	EQP

Unload Glass Data Report

Glass Move Request

#### <GLSMVSTATUS> = ‘UW’

Glass Move Request

#### <GLSMVSTATUS> = ‘UR’

Equipment Status Change Report

Glass Actual Processing Data Report

Glass Actual Processing Data Report

All Glass Position Data Report

    - Glass Move State ‘ER’ Request

‘UW(S6F67)’는 장비에서 실제 Glass Exchange (‘ER’)가 발생하기 5 ~ 10 초 이전에 보낼 수 있어야 한다. 장비의 특성에 따라 더 많은 시간이 요구 될 때는 LG측과 협의가 필요하다.

      - Load Glass Data Accepted

S6F63

S6F64

S6F61

S6F62

Load Glass Data Send

<GLSSTATUS> = ‘1’(Exist)

S2F87

S2F88

It limits in the equipment which uses the UW

S6F67

S6F68

If the Equipment were able to exchange the glass.

S6F67

S6F68

장비는 Unloading Glass Count 만큼 각 Glass 별 S6F73 보고 실시함. 단, A/B Stick 보고 순서는 무관함.

S6F73 S6F74

S6F73 S6F74

S1F67

S1F68

Exchanging Glass

ECS	EQP

Unload Glass Data Report

Load Glass Data Request

<ACK> = ‘0’(Data Accepted)

Glass Move Request

#### <GLSMVSTATUS> = ‘UW’

Glass Move Request

#### <GLSMVSTATUS> = ‘ER’

Glass Actual Processing Data Report

Glass Actual Processing Data Report

All Glass Position Data Report

      - Load Glass Data Not Accepted – Glass Unloading Sequence after data not accept error release

## ECS	EQP

#### S6F63 S6F64

Unload Glass Data Report

#### S6F61 S6F62

Load Glass Data Request

Load Glass Data Send

<GLSSTATUS> = ‘1’(Exist)

#### S2F87 S2F88

<ACK> ≠ ‘0’(Data Not Accepted)

If equipment status is

changed.

#### S5F1 S5F2

S6F91 S6F92

Alarm Set Report

Equipment Status Change Report

Operator Call, Error Release

If equipment status is changed.

It limits in the equipment which uses the UW

If the equipment were able to unload the glass.

This message can be sent before “Error Release”.

S5F1 S5F2

S6F91 S6F92

S6F67 S6F68

Unloading Glass

S6F67 S6F68

Alarm Release Report

Equipment Status Change Report

Glass Move Request

#### <GLSMVSTATUS> = ‘UW’

Glass Move Request

#### <GLSMVSTATUS> = ‘UR’

장비는 Unloading Glass Count 만큼 각 Glass 별 S6F73 보고 실시함. 단, A/B Stick 보고 순서는 무관함.

S6F73 S6F74

S6F73 S6F74

#### S1F67 S1F68

Glass Actual Processing Data Report

Glass Actual Processing Data Report All Glass Position Data Report

Copyright  2011 LG CNS Co., Ltd., All rights reserved	33/219

      - Load Glass Data Not Accepted – Glass Exchange Sequence after data not accept error release

## ECS	EQP

#### S6F63 S6F64

Unload Glass Data Report

#### S6F61 S6F62

Load Glass Data Request

Load Glass Data Send

<GLSSTATUS> = ‘1’(Exist)

#### S2F87 S2F88

<ACK> ≠ ‘0’(Data Not Accepted)

If equipment status is changed.

#### S5F1 S5F2

S6F91 S6F92

Alarm Set Report

Equipment Status Change Report

Operator Call, Error Release

If equipment status is changed.

Load Glass Data Send

<GLSSTATUS> = ‘1’(Exist)

It limits in the equipment which uses the UW

S5F1 S5F2

S6F91 S6F92

S6F61 S6F62

S2F87

S2F88

S6F67 S6F68

Alarm Release Report

Equipment Status Change Report

Load Glass Data Request

<ACK> = ‘0’(Data Accepted)

Glass Move Request

#### <GLSMVSTATUS> = ‘UW’

If the equipment were able to exchange the glass.

This message can be sent before “Error Release”.

#### S6F67 S6F68

Next Page

Glass Move Request

<GLSMVSTATUS> = ‘ER’

## ECS	EQP

Exchanging Glass

장비는 Unloading Glass Count 만큼 각 Glass 별 S6F73 보고 실시함. 단, A/B Stick 보고 순서는 무관함.

S6F73 S6F74

S6F73 S6F74

S1F67

S1F68

Glass Actual Processing Data Report

Glass Actual Processing Data Report

All Glass Position Data Report

      - Load Glass Not Exist

## ECS	EQP

#### S6F63

S6F64

Unload Glass Data Report

#### S6F61 S6F62

Load Glass Data Request

Load Glass Data Send

<GLSSTATUS> = ‘0’(Not Exist)

It limits in the equipment which uses the ‘UW’

If the Equipment were able to unload the glass and there is no Glass Data Send (S2F87) message until ‘UR’ is possible, EQP must send ‘UR’.

S2F87 S2F88

S6F67 S6F68

S6F67 S6F68

<ACK> = ‘0’(Data Accepted) Glass Move Request

#### <GLSMVSTATUS> = ‘UW’

Glass Move Request

#### <GLSMVSTATUS> = ‘UR’

If equipment status is changed.

Unloading Glass

S6F91 S6F92

Equipment Status Change Report

장비는 Unloading Glass Count 만큼 각 Glass 별 S6F73 보고 실시함. 단, A/B Stick 보고 순서는 무관함.

#### S6F73 S6F74

S6F73 S6F74

#### S1F67 S1F68

●

●

●

Glass Actual Processing Data Report

Glass Actual Processing Data Report

All Glass Position Data Report

Load Glass is ready

Load Glass Data Send

<GLSSTATUS> = ‘1’(Exist)

#### S2F87 S2F88

    - Glass Move State ‘CR’ Request(Cancel Request)

      - 세부 사항은 LG CNS, LG Display와의 협의가 필요하다.

      - Case of Unload Glass Not Exist

## ECS	EQP

장비가 이전에 ECS로 보낸 GLSMVSTATUS 를 장비 측에서 수행하지 못하는 경우

장비가 ECS로 Glass Move Request 보낸 상태

S6F67 S6F68

S6F61 S6F62

Glass Move Request

#### <GLSMVSTATUS> = ‘CR’

Load Glass Data Request

이후 Sequence는 Glass Move State ‘LR’(7.3.1) Sequence 을 따른다.

      - Case of Unload Glass Exist

## ECS	EQP

S6F67

S6F68

S6F63

If Equipment is able to inject the next glass

S6F64

S6F61

S6F62

장비가 이전에 ECS로 보낸 GLSMVSTATUS 를 장비 측에서 수행하지 못하는 경우

이후는 장비의 현재 상태에 따라 ‘UR’(7.3.2) 또는 ‘ER’(7.3.3) sequence.을 따른다

장비가 ECS로 Glass Move Request 보낸 상태

Glass Move Request

#### <GLSMVSTATUS> = ‘CR’

Unload Glass Data Report

Load Glass Data Request

    - Load Glass Data Change Case

- 장비로 투입될 Glass의 정보가 변경되었을 때, ECS는 장비로 이 순서를 보낸다.

- Glass data change message는 언제라도 보내질 수 있다.

- 장비는 ECS가 전송하는 S2F87 수신 후 Glass 존재 여부에 따라 S6F67 송신 여부를 판단한다.

- 세부 사항은 LG CNS, LG Display와의 협의가 필요하다.

      - Change form Not Exist to Exist

        - Unload Glass Not Exist

## ECS	EQP

S6F61

S6F62

Load Glass Data Send

<GLSSTATUS> = ‘0’(Not Exist)

S2F87

S2F88

Load Glass Data Send

<GLSSTATUS> = ‘1’(Exist)

S2F87

S2F88

If the equipment were able to load the glass.

S6F67

S6F68

Load Glass is ready

Load Glass Data Request

<ACK> = ‘0’(Data Accepted)

<ACK> = ‘0’(Data Accepted)

Glass Move Request

#### <GLSMVSTATUS> = ‘LR’

        - Unload Glass Exist

          - Case ⓐ : Glass Data Change Send ( S2F87 ) before ‘UW’

## ECS	EQP

S6F63

S6F64

S6F61

S6F62

Load Glass Data Send

<GLSSTATUS> = ‘0’(Not Exist)

S2F87

S2F88

Load Glass Data Send

<GLSSTATUS> = ‘1’(Exist)

S2F87

S2F88

It limits in the equipment which uses the ‘UW’

S6F67

S6F68

S6F67

S6F68

Load Glass is ready

Unload Glass Data Report

Load Glass Data Request

<ACK> = ‘0’(Data Accepted)

<ACK> = ‘0’(Data Accepted)

Glass Move Request

#### <GLSMVSTATUS> = ‘UW’

Glass Move Request

#### <GLSMVSTATUS> =‘ER’

2) Unload Glass Exist

- Case ⓑ : Glass Data Change Send ( S2F87 ) after ‘UW’

## ECS	EQP

S6F63

S6F64

S6F61

S6F62

Load Glass Data Send

<GLSSTATUS> = ‘0’(Not Exist)

S2F87

S2F88

It limits in the equipment which uses the ‘UW’

S6F67

S6F68

Load Glass Data Send

<GLSSTATUS> = ‘1’(Exist)

S2F87

S2F88

S6F67

S6F68

Load Glass is ready

Unload Glass Data Report

Load Glass Data Request

<ACK> = ‘0’(Data Accepted)

Glass Move Request

#### <GLSMVSTATUS> = ‘UW’

<ACK> = ‘0’(Data Accepted)

Glass Move Request

#### <GLSMVSTATUS> =‘ER’

2) Unload Glass Exist

- Case ⓒ : Glass Data Change Send ( S2F87 ) after ‘UR’

## ECS	EQP

S6F63

S6F64

S6F61

S6F62

Load Glass Data Send

<GLSSTATUS> = ‘0’(Not Exist)

S2F87

S2F88

It limits in the equipment which uses the ‘UW’

S6F67

S6F68

S6F67

S6F68

This message can be sent before Glass Unload action (Interlock Start).

Load Glass Data Send

<GLSSTATUS> = ‘1’(Exist)

S2F87

S2F88

S6F67

S6F68

Load Glass is ready

Unload Glass Data Report

Load Glass Data Request

<ACK> = ‘0’(Data Accepted)

Glass Move Request

#### <GLSMVSTATUS> = ‘UW’

Glass Move Request

#### <GLSMVSTATUS> = ‘UR’

<ACK> = ‘0’(Data Accepted)

Glass Move Request

#### <GLSMVSTATUS> =‘ER’

      - Change form Exist to Not Exist

        - Unload Glass Not Exist

## ECS	EQP

S6F61

S6F62

Load Glass Data Send

<GLSSTATUS> = ‘1’(Exist)

S2F87

S2F88

S6F67

S6F68

Load Glass Data Send

<GLSSTATUS> = ‘0’(Not Exist)

S2F87

S2F88

Load Glass Data Send

<GLSSTATUS> = ‘1’(Exist)

S2F87

S2F88

If the equipment were able to load the glass.

S6F67

S6F68

Load Glass is ready

Glass Load Canceled

Load Glass Data Request

<ACK> = ‘0’(Data Accepted)

Glass Move Request

#### <GLSMVSTATUS> = ‘LR’

<ACK> = ‘0’(Data Accepted)

<ACK> = ‘0’(Data Accepted)

Glass Move Request

#### <GLSMVSTATUS> = ‘LR’

        - Unload Glass Exist

          - Case ⓐ : Glass Data Change Send ( S2F87 ) before ‘UW’

S6F63

S6F64

S6F61

S6F62

Load Glass Data Send

<GLSSTATUS> = ‘1’(Exist)

S2F87

S2F88

Load Glass Data Send

<GLSSTATUS> = ‘0’(Not Exist)

S2F87

S2F88

It limits in the equipment which uses the UW

S6F67

S6F68

S6F67

S6F68

Load Glass Canceled

ECS	EQP

Unload Glass Data Report

Load Glass Data Request

<ACK> = ‘0’(Data Accepted)

<ACK> = ‘0’(Data Accepted)

Glass Move Request

#### <GLSMVSTATUS> = ‘UW’

Glass Move Request

#### <GLSMVSTATUS> = ‘UR’

2) Unload Glass Exist

- Case ⓑ : Glass Data Change Send ( S2F87 ) after ‘UW’

S6F63

S6F64

S6F61

S6F62

Load Glass Data Send

<GLSSTATUS> = ‘1’(Exist)

S2F87

S2F88

It limits in the equipment which uses the UW

S6F67

S6F68

Load Glass Data Send

<GLSSTATUS> = ‘0’(Not Exist)

S2F87

S2F88

S6F67

S6F68

Load Glass Canceled

ECS	EQP

Unload Glass Data Report

Load Glass Data Request

<ACK> = ‘0’(Data Accepted)

Glass Move Request

#### <GLSMVSTATUS> = ‘UW’

<ACK> = ‘0’(Data Accepted)

Glass Move Request

#### <GLSMVSTATUS> = ‘UR’

2) Unload Glass Exist

- Case ⓒ : Glass Data Change Send ( S2F87 ) after ‘ER’

S6F63

S6F64

S6F61

S6F62

Load Glass Data Send

<GLSSTATUS> = ‘1’(Exist)

S2F87

S2F88

It limits in the equipment which uses the UW

S6F67

S6F68

S6F67

S6F68

This message can be sent before Glass Unload action (Interlock Start).

Load Glass Data Send

<GLSSTATUS> = ‘0’(Not Exist)

S2F87

S2F88

S6F67

S6F68

Load Glass Canceled

ECS	EQP

Unload Glass Data Report

Load Glass Data Request

<ACK> = ‘0’(Data Accepted)

Glass Move Request

#### <GLSMVSTATUS> = ‘UW’

Glass Move Request

#### <GLSMVSTATUS> = ‘ER’

<ACK> = ‘0’(Data Accepted)

Glass Move Request

#### <GLSMVSTATUS> = ‘UR’

      - Change form Exist to Exist

        - Unload Glass Not Exist

## ECS	EQP

S6F61

S6F62

Load Glass Data Send

<GLSSTATUS> = ‘1’(Exist)

S2F87

S2F88

If the equipment were able to load the glass.

S6F67

S6F68

Load Glass Data Send

<GLSSTATUS> = ‘1’(Exist)

S2F87

S2F88

If the equipment were able to load the glass.

S6F67

S6F68

Load Glass Changed

Load Glass Data Request

<ACK> = ‘0’(Data Accepted)

Glass Move Request

#### <GLSMVSTATUS> = ‘LR’

<ACK> = ‘0’(Data Accepted)

Glass Move Request

#### <GLSMVSTATUS> = ‘LR’

        - Unload Glass Exist

          - Case ⓐ : Glass Data Change Send ( S2F87 ) before ‘UW’

## ECS	EQP

S6F63

S6F64

S6F61

S6F62

Load Glass Data Send

<GLSSTATUS> = ‘1’(Exist)

S2F87

S2F88

Load Glass Data Send

<GLSSTATUS> = ‘1’(Exist)

S2F87

S2F88

It limits in the equipment which uses the UW

S6F67

S6F68

S6F67

S6F68

Load Glass Changed

Unload Glass Data Report

Load Glass Data Request

<ACK> = ‘0’(Data Accepted)

<ACK> = ‘0’(Data Accepted)

Glass Move Request

#### <GLSMVSTATUS> = ‘UW’

Glass Move Request

#### <GLSMVSTATUS> = ‘ER’

2) Unload Glass Exist

- Case ⓑ : Glass Data Change Send ( S2F87 ) after ‘UW’

## ECS	EQP

S6F63

S6F64

S6F61

S6F62

Load Glass Data Send

<GLSSTATUS> = ‘1’(Exist)

S2F87

S2F88

It limits in the equipment

which uses the UW

S6F67

S6F68

Load Glass Data Send

<GLSSTATUS> = ‘1’(Exist)

S2F87

S2F88

S6F67

S6F68

Load Glass Changed

Unload Glass Data Report

Load Glass Data Request

<ACK> = ‘0’(Data Accepted)

Glass Move Request

#### <GLSMVSTATUS> = ‘UW’

<ACK> = ‘0’(Data Accepted)

Glass Move Request

#### <GLSMVSTATUS> = ‘ER’

2) Unload Glass Exist

- Case ⓒ : Glass Data Change Send ( S2F87 ) after ‘ER’

## ECS	EQP

S6F63

S6F64

S6F61

S6F62

Load Glass Data Send

<GLSSTATUS> = ‘1’(Exist)

S2F87

S2F88

It limits in the equipment

which uses the UW

S6F67

S6F68

S6F67

S6F68

This message can be sent before Glass Exchange action (Interlock Start).

Load Glass Data Send

<GLSSTATUS> = ‘1’(Exist)

S2F87

S2F88

S6F67

S6F68

Load Glass Changed

Unload Glass Data Report

Load Glass Data Request

<ACK> = ‘0’(Data Accepted)

Glass Move Request

#### <GLSMVSTATUS> = ‘UW’

Glass Move Request

#### <GLSMVSTATUS> = ‘ER’

<ACK> = ‘0’(Data Accepted)

Glass Move Request

#### <GLSMVSTATUS> = ‘ER’

    - Unload Glass Data Request

- 이 메시지는 ECS가 ‘unload glass data report’를 받지 못했을 때 사용된다.

- 세부적인 사용에 대해서는 LG CNS, LG Display와의 협의가 필요하다

## ECS	EQP

S6F67

S6F68

Unload Glass Data Request

S2F69

S2F70

S6F63

S6F64

When ‘Unload Glass Data’ is Exist.

( S2F70’s Ack = 0 )

S6F61

S6F62

Load Glass Data Send

S2F87

S2F88

S6F67

S6F68

장비는 작업자가 해당 Glass에 대한 Glass Data를 ECS로 요구버튼을 선택하거나 Glass Data를 수정 하여 Unload Glass Data Report를 할 수 있는 버튼을 선택할 수 있는 팝업 창을 제공 해야 한다.

Case 1 : 장비가 Unload 될 Glass에 대한 Unload Glass Data Report를 송신하지 않은 상태

Case 2 : 장비가 잘 못된 Unload Glass Data를 보고 한 상태

Case3 : ECS가 장비가 보고한 Unload Glass Data를 잊어버린 상태

Glass Move Request

<GLSMVSTATUS> = ‘UR’ or ‘ER’

Unload Glass Data Report

Load Glass Data Request

<ACK> = ‘0’(Data Accepted)

Glass Move Request

<GLSMVSTATUS> = Defined

Command

    - Glass Transfer In Online Local Control State

- 다음 Scenario는 Online Local 시 첫 번째 Glass를 장비에 투입할 때를 보여준다.

## ECS	EQP

Online-Local Change Button Click

S2F61	Control State Change Request

#### S2F62

<RCS> = 1 ( Online Local )

Operation Mode Change Button Click

S2F63	Equipment Operation Mode Report

#### S2F64

<OPMODE> = defined number

If (Online Remote)

Previous Control State ?

If (Offline)

#### S2F17 S2F18

S1F61 S1F62

Date and Time Request

All Equipment Status Report

If OCR( or VCR ) is equipped

in the Equipment

#### S1F67

S1F68

All Glass Position Data Report

#### S6F95

S6F96

OCR( or VCR ) Status Report

## ECS	EQP

S6F69

S6F70

S6F71

S6F72

S2F87

S2F88

S6F67

S6F68

- Display Map information each port

- Select Slot for Operating

Cassette MAP Request Button Click

Cassette Map Request

Sampling Slot Request

Load Glass Data Send

<GLSSTATUS> = ‘1’(Exist)

<ACK> = ‘0’(Data Accepted)

Glass Move Request

<GLSMVSTATUS> = ‘LR’

### Lot Control Sequence

    - Lot End Data Send To Equipment

S6F65 S6F66 S6F83 S6F84

ECS	EQP

LOT End Data Send

LOT Actual Processing Data Report

    - Local LOT Status Change Request on Online Local Control State

- Online local state 하에서 Lot end process 는 아래의 순서를 따른다

## ECS	EQP

Repeating Glass Load /Unload

•

•

•

It limits in the equipment which uses the UW

S6F63 S6F64

S6F67 S6F68

S6F67

S6F68

Unload Glass Data Report

Glass Move Request

#### <GLSMVSTATUS> = ‘UW’

Glass Move Request

#### <GLSMVSTATUS> = ‘UR’

Unloading Glass

장비는 Unloading Glass Count 만큼 각 Glass 별 S6F73 보고 실시함. 단, A/B Stick 보고 순서는 무관함.

S6F73 S6F74

S6F73 S6F74

#### S1F67 S1F68

Glass Actual Processing Data Report

Glass Actual Processing Data Report

All Glass Position Data Report

Lot End Button Click by Operator

#### S2F65 S2F66

Local LOT Status Change Request

<ENDFLAG> = ‘P’ ( Operator End )

All glasses retuned to the original lot

LOT End Data Send

#### S6F65 S6F66

S6F83 S6F84

LOT Actual Processing Data

Report

### Status Report

    - Equipment Status Change Report

- unit의 상태가 변할때마다 수행한다.

## ECS	EQP

S6F91 S6F92

Equipment Status Change Report

<EQPSTATUS> = defined status

    - All Equipment Status Report

- ECS가 재시작하거나 Offline에서 Online으로 전환되는 경우에 모든 장비 상태를 보고한다.(Remote or Local).

## ECS	EQP

S1F61

S1F62

All Equipment Status Report

    - All Equipment Status Request

- 모든 장비 상태를 요청 시 수행한다.

## ECS	EQP

S1F63

S1F64

All Equipment Status Request

    - OCR( or VCR ) Status Report

- OCR( or VCR)이 Offline에서 Online으로 전환될 때 OCR( or VCR)의 상태를 ECS로 보고한다.

## ECS	EQP

S6F95 S6F96

OCR( or VCR ) Status Report

    - Ionizer Status Report

- Ionizer의 상태를 보고한다.

## ECS	EQP

S1F75 S1F76

Ionizer Status Report

### Recipe Management

    - Recipe ID List Request

## ECS

Current EPPD Request

#### S7F19 S7F20

## EQP

    - Current Recipe ID Group List Request

S7F67 S7F68

ECS	EQP

Current Recipe ID Group List Request

    - Current Recipe ID List Request by Group

- 특정 Group에 속하는 Recipe ID List를 요청할 시 사용한다.

## ECS

Current Recipe ID List Request by Group

#### S7F69 S7F70

EQP

    - Recipe ID Confirm Request

S7F61 S7F62

ECS	EQP

Recipe ID Confirm Request

    - Recipe Body Request

## ECS	EQP

S7F77 S7F78

Recipe Body Request

    - Recipe ID Confirm & Sampling Data Send

- 이 Scenario는 각 Slot별 모든 Recipes를 즉시 확인할 때 사용되고, 이 메시지는 Lot Recipe (cassette recipe) 인 하나의 Recipe에만 사용될 수 있다.

- 만약 EQP가 샘플링과 < Recipe ID Confirm >의 사용이 가능하다.면, 장비는 Sampling Data Report (S7F63) 를 반드시 ECS로 보내야 한다.

      - Recipe Accept (Single LOT)

| st | S7F61 S7F62 |  |
| --- | --- | --- |
|  | S7F63 S7F64 |  |

ECS	EQP

Recipe ID Confirm Reque

The equipments using ‘EQP Sampling’ follow their own individual Sequences.

      - Recipe Accept(Multi LOT)

## ECS	EQP

Sampling Data Report

CST Loading(Port #1)

Recipe ID Confirm Request

(Port #1)

#### S7F61

S7F62

ACKC7M = ‘0’(Accept)

CST Loading(Port #2)

Recipe ID Confirm Request

(Port #2)

#### S7F61 S7F62

ACKC7M = ‘0’(Accept)

The equipments using ‘EQP Sampling’ follow their own individual Sequences.

Sampling Data Report (Port#1)

S7F63 S7F64

S7F63 S7F64

Sampling Data Report

(Port#2)

      - Recipe Not Accept

## ECS	EQP

S7F61 S7F62

Recipe Change or LOT Cancel

CST Loading

Recipe ID Confirm Request

ACKC7M = ‘1’(Not Accept)

### Date and Time Data Send

    - Date and Time Request

## ECS	EQP

S2F17

S2F18

Date and Time Request

    - Date and Time Set

S2F31

S2F32

ECS	EQP

Date and Time Set

### Alarm Occurrence and Clear

    - Heavy Alarm

## ECS	EQP

S5F1

S5F2

S6F91

S6F92

The case where all alarm is canceled

S5F1

S5F2

S6F91

S6F92

- Call Operator (Buzzer & Signal)

- Clear alarm

Heavy Alarm Occurs

Alarm Report

<ALCD> Lower Byte bit 8 = 1

<ALID> Alarm ID is the cause of Equipment’s down

Equipment Status Change Report

#### <EQPSTATUS> = ‘D’ ( DOWN)

<ALID> Alarm ID is the cause of Equipment’s down

Alarm Report

<ALCD> Lower Byte bit 8 = 0

Equipment Status Change Report

<EQPSTATUS> = defined status

    - Light Alarm

## ECS	EQP

Light Alarm Occurs

S5F1

S5F2

S5F1

S5F2

Alarm Report

<ALCD> Lower Byte bit 8 = 1

- Call Operator (Buzzer & Signal)

- Clear alarm

Alarm Report

<ALCD> Lower Byte bit 8 = 0

### Scrap Occurrence and Report

- 일반적으로 Scrap 보고는 장비에서 Glass 파손이 확인되는 즉시 ECS로 보고 해야 한다.

- 부득이한 경우는 Lot End 이전에는 보고 해야 한다.

## ECS	EQP

- Glass Scrap Occurs

- Scrap Data input by Operator

S6F93 S6F94

Scrap Data Report

### All Glass Position Data Report

-Online으로 전환되는 경우

-각 Sub unit간 glass가 이동하는 경우

S1F67 S1F68

ECS	EQP

All Glass Position Data Report

### Cycle Stop Status Report

## ECS	EQP

S2F67

S2F68

S2F67

S2F68

S6F61

S6F62

S2F87

S2F88

이후의 Sequence는 장비의 현재 상태에 따라 결정된다.

- 장비내의 Unloading Glass가 존재하지 않으면 ‘LR’ sequence (7.3.1)를

- 장비내의 Unloading Glass가 존재하는 경우 ‘UR’ Sequence(7.3.2) 또는 ‘ER’ sequence(7.3.3 )를 따른다.

Cycle Stop Release

Cycle Stop Button Click

Cycle Stop Status Report

<CSSTATUS> = ‘1’(Set)

Cycle Stop Status Report

<CSSTATUS> = ‘0’(Release)

Load Glass Data Request

Load Glass Data Send

<GLSSTATUS> = ‘1’(Exist)

<ACK> = ‘0’(Data Accepted)

### Rework Request

-Operator가 Rework flag code를 설정하면 장비는 ECS에게 rework 요청을 한다.

## ECS	EQP

S6F89

S6F90

S6F65

S6F66

S6F89

S6F90

Rework Request ( Rework reserved )

#### <SELECTED> = ‘00000…000’

…

<RESERVECD> = R

LOT End Data Send

#### <ENDFLAG>=‘R’

setting whether rework or not by operator

Rework Request

<SELECTED> = ‘01011…011’

<RESERVECD> = S

ECS는 Rework 작업 확인 창을 보여주거나 또는 자동으로 Rework 진행

### Dummy Lot Request

-만약 장비가 Dummy Lot을 사용해야 한다면 Dummy Lot이 필요한 시점에 ECS에게 요청한다.

- 미리 Dummy Lot이 준비 되지 않은 경우

OP Call, Buzzer & Melody

- Dummy Lot 이 준비 되어 있는 경우

Dummy Lot Start

장비 측 판단에 따른 Dummy Lot 요청

시점

S6F85 S6F86

ECS	EQP

Dummy Lot Request

### Glass Data Request

- Operator가 Glass ID 또는 Glass Code로 Glass Data를 재 요청 시 사용한다.

- 세부적인 사용에 대해서는 LG CNS, LG Display의 협의가 필요하다.

## ECS	EQP

Operator keying Glass ID

S2F79

S2F80

Glass Data Request

### Chemical Change

- 다음은 chemical change 시나리오이다.

S2F95

S2F96

S2F97

S2F98

S2F67

S2F68

S2F75

S2F76

S2F75 S2F76

S2F67 S2F68

S6F61 S6F62

S2F87 S2F88

약액 교환 중

Chemical Change 시작 시점

투입 중인 Glass 가 존재 하는 경우 장비는 투입 되는 Glass의 투입 완료 후 Cycle Stop을 요구해야 한다.

Glass Processing

ECS	EQP

Chemical Change Enable Request

● ●

Chemical Change Enable Send

Cycle Stop Status Report

<CSSTATUS> = ‘1’(Set)

Chemical Change Status Report

<CHEMCH> = ‘1’ ( Start )

Chemical Change Status Report

<CHEMCH> = ‘0’ ( End )

Cycle Stop Status Report

<CSSTATUS> = ‘0’(Release)

Load Glass Data Request

Load Glass Data Send

<ACK> = ‘0’(Data Accepted)

### Normal Operation

- In case of 1 lot sequence (from lot start to end)

Online-Remote Change Button Click

S2F61

S2F62

S2F63

S2F64

S2F17

S2F18

S1F61

S1F62

S1F67

S1F68

If OCR( or VCR ) is equipped in the Equipment

S6F95

S6F96

S6F61

S6F62

Operation Mode Change Button Click

ECS	EQP

Control State Change Request

<RCS> = 0 ( Online – Remote )

Equipment Operation Mode Report

<OPMODE> = defined number

Date and Time Request

All Equipment Status Report

All Glass Position Data Report

OCR( or VCR ) Status Report

Load Glass Data Request

## ECS	EQP

CST Not Exist On The Port.

Load Glass Data Send

<GLSSTATUS> = ‘0’(Not Exist)

#### S2F87 S2F88

<ACK> = ‘0’(Data Accepted)

- Loading Cassette to Port

- Cassette Clamp, Mapping or CSTID Read

- Lot information confirm or Recipe ID selecting

- Start Button Click

Multi Process Program Load Inquire

#### S7F61

S7F62

Recipe Body Request

S7F63 S7F64

S7F77

S7F78

Sampling Data Report

Confirm this part depending

on Equipment Controller

Load Glass Data Send

<GLSSTATUS> = ‘1’(Exist)

#### S2F87 S2F88

<ACK> = ‘0’(Data Accepted)

If Equipment were able to

receive the glass,

#### <GLSMVSTATUS>= ‘LR’

#### S6F67 S6F68

Glass Move Request

<GLSMVSTATUS> = ‘LR’

## ECS	EQP

Glass Loading

#### S6F91 S6F92

Equipment Status Change Report

#### S1F67

S1F68

All Glass Position Data Report

If glass moves per Sub unit, this will be reported every time

Processing Glass

Load Glass Data Send

<GLSSTATUS> = ‘1’(Exist)

It limits in the equipment which uses the UW

…

S6F63 S6F64

S6F61 S6F62

S2F87 S2F88

S6F67 S6F68

Unload Glass Data Report

<GLSODR> = 0 ( First Glass ) Load Glass Data Request

<ACK> = ‘0’(Data Accepted) Glass Move Request

#### <GLSMVSTATUS> = ‘UW’

If the Equipment were able to receive the glass,

Start Glass Exchange

<GLSMVSTATUS> = ‘ER’ Unless <GLSMVSTATUS>

#### = ‘UR’

S6F67 S6F68

Glass Move Request

#### <GLSMVSTATUS> = ‘ER’

장비는 Unloading Glass Count 만큼 각 Glass 별 S6F73 보고 실시함. 단, A/B Stick 보고 순서는 무관함.

S6F73 S6F74

S6F73 S6F74

Glass Actual Processing Data Report

Glass Actual Processing Data Report

## ECS	EQP

#### S1F67 S1F68

All Glass Position Data Report

Complete Glass Exchange

Processing Glass

While intermediate Glasses (<GLSODR> = 1) are

Repeat Glass Exchange

…

controlling of LOT

#### S6F63 S6F64

Unload Glass Data Report

<GLSODR> = 2 ( Last Glass )

#### S6F61 S6F62

Load Glass Data Request

Load Glass Data Send

<GLSSTATUS> = ‘0’(Not Exist)

#### S2F87 S2F88

<ACK> = ‘0’(Data Accepted)

It limits in the equipment which uses the UW

#### S6F67 S6F68

Glass Move Request

#### <GLSMVSTATUS> = ‘UW’

S6F67 S6F68

Glass Move Request

#### <GLSMVSTATUS> = ‘UR’

Last Glass Unloading

#### S6F91 S6F92

Equipment Status Change Report

장비는 Unloading Glass Count 만큼 각 Glass 별 S6F73 보고 실시함. 단, A/B Stick 보고 순서는 무관함.

#### S6F73 S6F74

S6F73 S6F74

Glass Actual Processing Data Report

Glass Actual Processing Data Report

S1F67 S1F68 S6F65 S6F66 S6F83 S6F84

ECS	EQP

All Glass Position Data Report

LOT End Data Send

LOT Actual Processing Data Report

### Lot Abort Scenario

    - The case where glass would be loaded is exist

S6F67 S6F68

The Lot Aborted by operator

Start Glass Exchange

장비는 Unloading Glass Count 만큼 각 Glass 별 S6F73 보고 실시함. 단, A/B Stick 보고 순서는 무관함.

S6F73 S6F74 S6F73 S6F74 S1F67 S1F68

Complete Glass Exchange

Processing Glass

S6F63 S6F64 S6F61 S6F62 S2F87 S2F88 S6F67 S6F68 S6F67 S6F68

ECS	EQP

If the Equipment were able to receive the glass,

<GLSMVSTATUS>=‘ER’. Unless <GLSMVSTATUS>

= ‘UR’

Glass Move Request

#### <GLSMVSTATUS> = ‘ER’

Glass Actual Processing Data Report

Glass Actual Processing Data Report

All Glass Position Data Report

Unload Glass Data Report

Load Glass Data Request

Load Glass Data Send

<GLSSTATUS> = ‘0’(Not Exist)

<ACK> = ‘0’(Data Accepted)

It limits in the equipment which uses the UW

Glass Move Request

#### <GLSMVSTATUS> = ‘UW’

Glass Move Request

<GLSMVSTATUS> = ‘UR’

Glass Unloading

S6F91 S6F92

장비는 Unloading Glass Count 만큼 각 Glass 별 S6F73 보고 실시함. 단, A/B Stick 보고 순서는 무관함.

S6F73 S6F74 S6F73 S6F74 S1F67 S1F68 S6F65 S6F66 S6F83 S6F84

ECS	EQP

Equipment Status Change Report

Glass Actual Processing Data Report

Glass Actual Processing Data Report

All Glass Position Data Report

LOT End Data Send

LOT Actual Processing Data Report

    - The case where glass would be loaded is not exist

## ECS	EQP

If the Equipment were able to receive the glass,

#### <GLSMVSTATUS> = ‘ER’

Unless <GLSMVSTATUS>

#### = ‘UR’

S6F67 S6F68

Glass Move Request

#### <GLSMVSTATUS> = ‘ER’

The Lot Aborted by operator

Load Glass Data Send

<GLSSTATUS> = ‘0’(Not Exist)

#### S2F87 S2F88

<ACK> = ‘0’(Data Accepted)

#### S6F67 S6F68

GLS Move Request

#### <GLSMVSTATUS> = ‘UR’

Glass Unloading

S6F91 S6F92

Equipment Status Change Report

장비는 Unloading Glass Count 만큼 각 Glass 별 S6F73 보고 실시함. 단, A/B Stick 보고 순서는 무관함.

#### S6F73 S6F74

Glass Actual Processing Data Report

S6F73 S6F74

Glass Actual Processing Data Report

#### S1F67 S1F68

All Glass Position Data Report

#### S6F65

S6F66

LOT End Data Send

#### S6F83 S6F84

LOT Actual Processing Data Report

### Error Messages

    - Message fault due to unrecognized Device ID

## ECS	EQP

SxFy S9F1

Unrecognized Device ID

    - Message fault due to unrecognized Stream Type

## ECS	EQP

SxFy S9F3

Unrecognized Stream Type

    - Message fault due to unrecognized Function Type

## ECS	EQP

SxFy S9F5

Unrecognized Function Type

    - Message fault due to Illegal Data Format

## ECS	EQP

SxFy S9F7

Illegal Data Format

    - Message fault due to Transaction Timer Timeout

S9F9

ECS	EQP

Transaction Timer Timeout

    - Message fault due to data too long

## ECS	EQP

SxFy S9F11

Data Too Long

    - Message fault due to Conversation Timeout

## ECS	EQP

SxFy

SxFy+1

S9F13

장비는 이 시점에 ECS로부터 이전 transaction의 결과인 특정 메시지를 받을 준비가 된 상태이 다. 장비는 아직 해당 메시지를 받은 상태는 아니며 conversation timeout이 일어날 때까지 기다린 다.

Conversation Timeout

### Equipment Terminal Service

S10F3 S10F4

ECS	EQP

Terminal Display, Single

# Standard Message Structure

Stream n

Function m

Short mnemonic

Valid direction

ECS=Equipment Control System

EQP = Equipment

Reply option

# Sn,Fm

Name of Function(XXX)

#### ECSEQP ECSEQP ECS↔EQP

Reply(required) [Reply](optional) “blank”(never)

Description	The description of the action generated by the function.

Structure

Notice

Detailed structure shows lists and defined items. Lists are denoted by the number separated by a comma following a capital “L”. The individual elements in the list are numbered on separate lines. Nested lists are indented to emphasize the structure. The detailed form of the items is given in the define section at the beginning of the transaction. The symbols “<” and “>” are used to enclose each item in the structure data and imply that there is an item header. A detailed description of each data item as well as a list of the allowable data formats can be found in the Data Item Dictionary

A description of the data item, with the meanings of specific values.

Exception	Special cases in the structure that have a different meaning.

| Sn,F0 |  | Abort transaction | ECS↔EQP |  |
| --- | --- | --- | --- | --- |
| Description | Function 0 exists in all streams and has the same special meaning in each stream. The function 0 message closes a transaction, so that the originator will not have to wait for a transaction timeout to proceed. Function 0 is sent in lieu of the expected secondary message when the interpreter cannot, because of a transmission error or some other reasons, respond with the expected reply. It is not a requirement that the interpreter send function 0 to close a transaction. |  |  |  |
| Structure | Header Only |  |  |  |
| Notice |  |  |  |  |
| Exception |  |  |  |  |

| S1,F1 |  | Are You There Request | ECS↔EQP | reply |
| --- | --- | --- | --- | --- |
| Description | If the equipment turn to online mode, Function 0 response to this message means the communication is inoperative. In the equipment, a function 0 is equivalent to a timeout on the receive timer after issuing S1,F1 to ECS |  |  |  |
| Structure | Header Only |  |  |  |
| Notice |  |  |  |  |
| Exception |  |  |  |  |

| S1,F2 |  | On Line Data | ECS↔EQP |  |
| --- | --- | --- | --- | --- |
| Description | Data signifying that the equipment is alive. |  |  |  |
| Structure | L,2 - A[8] <MDLN> - A[6] <SOFTREV> |  |  |  |
| Notice | <MDLN> : Equipment Model Type <SOFTREV> : Software revision code |  |  |  |
| Exception | The ECS sends a zero-length list to the equipment |  |  |  |

| S1,F61 |  | All Equipment Status Report | ECSEQP | reply |
| --- | --- | --- | --- | --- |
| Description | When equipment is initialized or control state is changed to Online, the equipment reports this message |  |  |  |
| Structure | L,2 - L,n [ n = the number of Unit] 1. L,4 - U2[1] <UNITID1> - A[1] <EQPSTATUS1> 3. A[10] <ALID1> 4. A[120] <ALTX1> … n. L,4 - U2[1] <UNITIDn> - A[1] <EQPSTATUSn> - A[10] <ALIDn> 4. A[120] <ALTXn> - L,p [ p = the number of Ionizer] 1. L,2 - B[1] <IONIZERID1> - A[1] <IONIZERSTATUS1> … p. L,2 - B[1] <IONIZERIDp> - A[1] <IONIZERSTATUSp> |  |  |  |
| Notice | <UNITID>Unit ID <EQPSTATUS> Equipment Status R : RUN E : Experiment V : Development C : Model Change M : Monitoring I : IDLE D : DOWN P : Process Down <ALID> Alarm Identification If <EQPSTATUS> is ‘R’ or ‘I’, then <ALID> is fixed by ‘0’ <ALTX> Alarm Message <IONIZERID> Ionizer ID <IONIZERSTATUS> Ionizer Status R : RUN D : DOWN |  |  |  |
| Exception |  |  |  |  |

| S1,F62 |  | All Equipment Status Report Acknowledge | ECSEQP |  |
| --- | --- | --- | --- | --- |
| Description | Acknowledge |  |  |  |
| Structure | L,2 1. U2[1]<UNITID> 2. B[1]<ACKC1> |  |  |  |
| Notice | <UNITID> Unit ID <ACKC1> 0 = Accepted 1 = Not Accepted |  |  |  |
| Exception |  |  |  |  |

| S1,F63 |  | All Equipment Status Request | ECSEQP | reply |
| --- | --- | --- | --- | --- |
| Description | ECS requests the status of all units of the Equipment. |  |  |  |
| Structure | Header Only |  |  |  |
| Notice |  |  |  |  |
| Exception |  |  |  |  |

| S1,F64 |  | All Equipment Status Data | ECSEQP |  |
| --- | --- | --- | --- | --- |
| Description | The Equipment reports all EQP Status Data |  |  |  |
| Structure | L,2 - L,n [ n = the number of Unit] 1. L,4 - U2[1] <UNITID1> - A[1] <EQPSTATUS1> 3. A[10] <ALID1> 4. A[120] <ALTX1> … n. L,4 - U2[1] <UNITIDn> - A[1] <EQPSTATUSn> - A[10] <ALIDn> 4. A[120] <ALTXn> - L,p [ p = the number of Ionizer] 1. L,2 - B[1] <IONIZERID1> - A[1] <IONIZERSTATUS1> … p. L,2 - B[1] <IONIZERIDp> - A[1] <IONIZERSTATUSp> |  |  |  |
| Notice | <UNITID>Unit ID <EQPSTATUS> Equipment Status R : RUN E : Experiment V : Development C : Model Change M : Monitoring I : IDLE D : DOWN P : Process Down <ALID> Alarm Identification If <EQPSTATUS> is ‘R’ or ‘I’, then <ALID> is fixed by ‘0’ <ALTX> Alarm Message <IONIZERID> Ionizer ID <IONIZERSTATUS> Ionizer Status R : RUN D : DOWN |  |  |  |
| Exception |  |  |  |  |

| S1,F67 |  | All Glass Position Data Report | ECSEQP | reply |
| --- | --- | --- | --- | --- |
| Description | This message is used for tracking of glasses which moved in among sub units |  |  |  |
| Structure | L,n [n=SubUnit count] 1.L,3 1.U2[1] <UNITID1> 2.B[1] <SUNITID1> 3.L,m [ m = glass count ] 1.U2[1] <GLSCODE1> . . m.U2[1]<GLSCODEm> . . n.L,3 1.U2[1] <UNITIDn> 2.B[1] <SUNITIDn> 3.L,m [ m = glass count ] 1.U2[1] <GLSCODE1> . . m.U2[1]<GLSCODEm> |  |  |  |
| Notice | <UNITID> Unit ID <SUNITID> Sub Unit ID <GLSCODE> Unique Glass Code defined by ECS. Equipment must keep this code, while glass is in equipment. |  |  |  |
| Exception | A one-to-one correspondence of UNITID and SUNITID depends on Equipment unit configuration |  |  |  |

| S1,F68 |  | All Glass Position Data Report Acknowledge | ECSEQP |  |
| --- | --- | --- | --- | --- |
| Description | Acknowledge |  |  |  |
| Structure | L,2 1. U2[1]<UNITID> 2. B[1]<ACKC1> |  |  |  |
| Notice | <UNITID> Unit ID <ACKC1> 0 = Accepted 1 = Not Accepted |  |  |  |
| Exception |  |  |  |  |

| S1,F75 |  | Ionizer Status Report | ECSEQP | reply |
| --- | --- | --- | --- | --- |
| Description | This message is used in Ionizer Status Report . |  |  |  |
| Structure | L,2 - U2[1] <UNITID> - L,n [ n = the number of Ionizer ] 1. L,2 - B[1] <IONIZERID> - A[1] <IONIZERSTATUS1> … n. L,2 - B[1] <IONIZERIDn> - A[1] <IONIZERSTATUSn> |  |  |  |
| Notice | <UNITID>Unit ID <IONIZERID> Ionizer ID <IONIZERSTATUS> Ionizer Status R : RUN D : DOWN |  |  |  |
| Exception |  |  |  |  |

| S1,F76 |  | Ionizer Status Report Acknowledge | ECSEQP |  |
| --- | --- | --- | --- | --- |
| Description | Acknowledge |  |  |  |
| Structure | L,2 1. U2[1]<UNITID> 2. B[1]<ACKC1> |  |  |  |
| Notice | <UNITID> Unit ID <ACKC1> 0 = Accepted 1 = Not Accepted |  |  |  |
| Exception |  |  |  |  |

| S2,F17 |  | Date and Time Request | ECSEQP | reply |
| --- | --- | --- | --- | --- |
| Description | This message is Useful, when ECS checks the equipment time base or the equipment to synchronize with ECS time base. |  |  |  |
| Structure | U2[1]<UNITID> |  |  |  |
| Notice | <UNITID> Unit ID |  |  |  |
| Exception |  |  |  |  |

| S2,F18 |  | Date and Time Data | ECSEQP |  |
| --- | --- | --- | --- | --- |
| Description | Actual time data. |  |  |  |
| Structure | A[14] <TIME> |  |  |  |
| Notice | <TIME> : Current Time “YYYYMMDDhhmmss” |  |  |  |
| Exception |  |  |  |  |

| S2,F25 |  | Loopback Diagnostic Request | ECS↔EQP | reply |
| --- | --- | --- | --- | --- |
| Description | This is the diagnostic message for checkout of protocol and communication circuits. When the binary string is sent, the same data is echoed back. |  |  |  |
| Structure | B[10] <ABS> |  |  |  |
| Notice | <ABS> Any binary string |  |  |  |
| Exception |  |  |  |  |

| S2,F26 |  | Loopback Diagnostic Data | ECS↔EQP |  |
| --- | --- | --- | --- | --- |
| Description | The echoed binary string |  |  |  |
| Structure | B[10] <ABS> |  |  |  |
| Notice |  |  |  |  |
| Exception |  |  |  |  |

| S2,F31 |  | Date and Time Data Send | ECSEQP | reply |
| --- | --- | --- | --- | --- |
| Description | Actual time data Send. |  |  |  |
| Structure | L,2 1. U2[1] <UNITID> 2. A[14] <TIME> |  |  |  |
| Notice | <UNITID> Unit ID <TIME>: Current Time “YYYYMMDDhhmmss” |  |  |  |
| Exception |  |  |  |  |

| S2,F32 |  | Date and Time Acknowledge | ECSEQP |  |
| --- | --- | --- | --- | --- |
| Description | Acknowledge |  |  |  |
| Structure | L,2 1. U2[1]<UNITID> 2. B[1]<ACKC2> |  |  |  |
| Notice | <UNITID> Unit ID <ACKC2> 0 = Accepted 1 = Not Accepted |  |  |  |
| Exception |  |  |  |  |

| S2,F61 |  | Control State Change Request | ECSEQP | reply |
| --- | --- | --- | --- | --- |
| Description | This message is used when requesting the control state change. |  |  |  |
| Structure | L,2 1.U2[1]<UNITID> 2.A[1] <RCS> |  |  |  |
| Notice | <UNITID> Unit ID <RCS> Remote Control State 0 = Online-Remote 1 = Online-Local 2 = Offline |  |  |  |
| Exception |  |  |  |  |

| S2,F62 |  | Control State Change Request Acknowledge | ECSEQP |  |
| --- | --- | --- | --- | --- |
| Description | Reply for the control state change of the equipment |  |  |  |
| Structure | L2 1.U2[1]<UNITID> 2.B[1] <ACKC2A> |  |  |  |
| Notice | <UNITID> Unit ID < ACKC2A > Acknowledge code, 1 byte 0 = Mode Change Accept 1 = Mode Change not Allowed 2 = Equipment Already Changed Mode 3 - 63 Reversed |  |  |  |
| Exception |  |  |  |  |

| S2,F63 |  | Equipment Operation Mode Report | ECSEQP | reply |
| --- | --- | --- | --- | --- |
| Description | Change operation mode of the equipment |  |  |  |
| Structure | L,2 1.U2[1]<UNITID> 2.A[1] <OPMODE> |  |  |  |
| Notice | <UNITID> Unit ID < OPMODE > Operation Mode Code The following OPMODE values define the case of operate to Online(Remote or Local) 1 = Normal / Load & Process / Auto / Remote / Operation / Automatic Mode 2 = Sequence Transfer / Online Local / Local / Semi-Auto Mode 3 = Cleanout / Recovery Mode 4 = Test / Atmospheric Transfer Mode 5 = Transfer / Vacuum Transfer Mode 6 = Particle Check / Pass Mode 7 = Reverse Mode 8 = Repair Mode 9 = Manual A = Restoration |  |  |  |
| Exception | The operation mode to operate Offline dose not define OPMODE (ex. Manual / Maintenance / Offline / Local / etc…) |  |  |  |

| S2,F64 |  | Equipment Operation Mode Report Acknowledge | ECSEQP |  |
| --- | --- | --- | --- | --- |
| Description | Reply for Change operation mode of the equipment |  |  |  |
| Structure | L,2 1.U2[1]<UNITID> 2.B[1] <ACKC2A> |  |  |  |
| Notice | <UNITID> Unit ID < ACKC2A > Acknowledge code, 1 byte 0 = Mode Change Accept 1 = Mode Change not Allowed 2 = Equipment Already Changed Mode 3 - 63 Reversed |  |  |  |
| Exception |  |  |  |  |

| S2,F65 |  | Local LOT Status Change Request | ECSEQP | reply |
| --- | --- | --- | --- | --- |
| Description | This message is used when sending Lot End Data under Online Local Control state. |  |  |  |
| Structure | L,4 - U2[1]<UNITID> - B[1]<PORTID> - A[16] <LOTID> - A[1]<ENDFLAG> |  |  |  |
| Notice | <UNITID> Unit ID <PORTID> Port ID <LOTID> LOT ID <ENDFLAG> End Flag N : Normal End P : Operator End |  |  |  |
| Exception |  |  |  |  |

| S2,F66 |  | Local LOT Status Change Request Acknowledge | ECSEQP |  |
| --- | --- | --- | --- | --- |
| Description | Acknowledge |  |  |  |
| Structure | L,2 1. U2[1]<UNITID> 2. B[1]<ACKC2> |  |  |  |
| Notice | <UNITID> Unit ID <ACKC2> 0 = Accepted 1 = Not Accepted |  |  |  |
| Exception |  |  |  |  |

| S2,F67 |  | Cycle Stop Status Report | ECSEQP | reply |
| --- | --- | --- | --- | --- |
| Description | Change cycle stop status of the equipment |  |  |  |
| Structure | L,2 1.U2[1] <UNITID> 2.B[1]<CSSTATUS> |  |  |  |
| Notice | <UNITID> Unit ID <CSSTATUS> 0 = Cycle Stop Release 1 = Cycle Stop Set |  |  |  |
| Exception |  |  |  |  |

| S2,F68 |  | Cycle Stop Status Report Acknowledge | ECSEQP |  |
| --- | --- | --- | --- | --- |
| Description | Reply for Change cycle stop status of the equipment |  |  |  |
| Structure | L,2 1.U2[1] <UNITID> 2.B[1]<ACKC2> |  |  |  |
| Notice | <UNITID> Unit ID <ACKC2> 0 = Accepted 1 = Not Accepted 2 – 63 Reserved |  |  |  |
| Exception |  |  |  |  |

| S2,F69 |  | Unload Glass Data Request | ECSEQP | reply |
| --- | --- | --- | --- | --- |
| Description | This message is used when ECS Request Unload Glass Data from the Equipment |  |  |  |
| Structure | L,3 - U2[1] <UNITID> - B[1] <STAGEID> - A[80] <HOSTMSG> |  |  |  |
| Notice | <UNITID> Unit ID <STAGEID> Stage ID <HOSTMSG> Host message in Korean and English |  |  |  |
| Exception |  |  |  |  |

| S2,F70 |  | Unload Glass Data Request Acknowledge | ECSEQP |  |
| --- | --- | --- | --- | --- |
| Description | Acknowledge |  |  |  |
| Structure | L,2 1.U2[1] <UNITID> 2.B[1]<ACKC2> |  |  |  |
| Notice | <UNITID> Unit ID <ACKC2> 0 = Accepted 1 = Not Accepted 2 = Unload Glass Not Exist 3 – 63 Reserved |  |  |  |
| Exception |  |  |  |  |

| S2,F75 |  | Chemical Change Status Report | ECSEQP | reply |
| --- | --- | --- | --- | --- |
| Description | When process changes the chemical, process reports chemical change status report. |  |  |  |
| Structure | L,2 - U2[1] <UNITID> - A[1] <CHEMCH> |  |  |  |
| Notice | <UNITID> Unit ID <CHEMCH> Chemical Change Status 1 = Start of Chemical Change 0 = End of Chemical Change |  |  |  |
| Exception |  |  |  |  |

| S2,F76 |  | Chemical Change Status Report Acknowledge | ECSEQP |  |
| --- | --- | --- | --- | --- |
| Description | Acknowledge |  |  |  |
| Structure | L,2 1. U2[1]<UNITID> 2. B[1]<ACKC2> |  |  |  |
| Notice | <UNITID>Unit ID <ACKC2> 0 = Accepted >0 = Error, not accepted 1 – 63 Reserved |  |  |  |
| Exception |  |  |  |  |

| S2,F79 |  | Glass Data Request | ECSEQP | reply |
| --- | --- | --- | --- | --- |
| Description | The Equipment only can use this control sequence in its manual operation. It can be use when equipment lost ‘Glass Data’, but equipment still have Glass ID or Glass code. |  |  |  |
| Structure | L,4 - U2[1]<UNITID> - A[16] <GLSID> - U2[1] <GLSCODE> - A[1] <REQOPTION> |  |  |  |
| Notice | <UNITID> Unit ID <GLSID> Glass ID <REQOPTION> Request Option C : Glass Code I : Glass ID A : Glass Code and Glass ID |  |  |  |
| Exception |  |  |  |  |

| S2,F80 |  | Glass Data | ECSEQP |  |
| --- | --- | --- | --- | --- |
| Description | The Equipment (manual operation) sends S2F79 and then ECS sends ‘Glass Load Data’ as reply. ‘Glass Load Data’ is data about injected glass when glass inject to equipment. |  |  |  |
| Structure | L, 3 1. U2[1] <UNITID> 2. B[1]<ACKC2> - L,n [ n = 0( GLSSTATUS = 0 ) or 21 ( GLSSTATUS = 1 ) ] - A[16] <LOTID> - A[14] <CSTID> - A[3] <SLOTID> - A[24] <RECIPEID> - A[8] <PRCID> - A[1] <GLSODR> - A[16] <GLSID> - A[2] <GLSJUDGE> 9. A[2000] <PNLIF> - A[2000] <SUBMDLIF> - B[1] <PORTID> - U2[1] <GLSCODE> - B[1] <REINPUT> - A[5] <GLSTHICK> - A[14] <PARTNUM> - A[12] <PRODTYPE> - A[12] <ATTRIBUTE> - A[1] <GLSTYPE> - A[32]<KEYID> - L, p [ p = the number of LSD] 1. L,2 - A[24] <LSDITEM1> - A[40] <LSDVALUE1> : p. L,2 - A[24] <LSDITEMp> - A[40] <LSDCALUEp> - L,q [ q = the number of GSD] 1.L,2 - A[24] <GSDITEM11> - A[40] <GSDVALUE11> : q. L,2 - A[24] <GSDITEM1q> - A[40] <GSDVALUE1q> |  |  |  |

| Notice | <UNITID> Unit ID <ACKC2> 0 = Accepted 1 = Not Accepted (Unknown GLSID or GLSCODE) 2 – 63 Reserved <LOTID> LOT ID <CSTID> Cassette ID <SLOTID> Slot ID <RECIPEID> Recipe ID <PRCID> Processing Code <GLSODR> Glass Order 0 = The first glass 1 = All glasses except the first glass and the last glass 2 = The Last glass( In this case, this value may not be delivered for Scrap or reason of LOT Abort ) 3 = the only glass, when there is just one in a LOT. <GLSID> Glass ID <GLSJUDGE> Glass Grade Code <PNLIF> Panel Judge Information <SUBMDLIF> Sub Panel Information <PORTID> Port ID <GLSCODE> Glass ID Code. <REINPUT> Glass Re-input Flag 0 = Not Re-input 1 = Re-input <GLSTHICK> Glass’s Thickness <PARTNUM> Part Number <PRODTYPE> Type of Producing <ATTRIBUTE> LOT Attribute <GLSTYPE> Glass Type N = Normal Glass D = Dummy Glass <KEYID> Glass Key ID <LSDITEM> LOT Specific Data Item defined by LG Display <LSDVALUE> LOT Specific Data Value defined by LG Display <GSDITEM> Glass Specific Data Item defined by LG Display <GSDVALUE> Glass Specific Data Value defined by LG Display |
| --- | --- |
| Exception |  |

| S2,F87 |  | Load Glass Data Send | ECSEQP | reply |
| --- | --- | --- | --- | --- |
| Description | This is the Glass data that would be loaded into the equipment. |  |  |  |
| Structure | L, 4 |  |  |  |
|  | 1. U2[1] <UNITID> |  |  |  |
|  | 2. B[1] <STAGEID> |  |  |  |
|  | 3. A[1] <GLSSTATUS> |  |  |  |
|  | 4. L,n [ n = 0 ( GLSSTATUS = 0 ) or 2 ( GLSSTATUS = 1 )] |  |  |  |
|  | 1. L,21 [ ( A Cut Glass ) ] |  |  |  |
|  | 1. A[16] <LOTID> |  |  |  |
|  | 2. A[14] <CSTID> |  |  |  |
|  | 3. A[3] <SLOTID> |  |  |  |
|  | 4. A[24] <RECIPEID> |  |  |  |
|  | 5. A[8] <PRCID> |  |  |  |
|  | 6. A[1] <GLSODR> |  |  |  |
|  | 7. A[16] <GLSID> |  |  |  |
|  | 8. A[2] <GLSJUDGE> |  |  |  |
|  | 9. A[2000] <PNLIF> |  |  |  |
|  | 10. A[2000] <SUBMDLIF> |  |  |  |
|  | 11. B[1] <PORTID> |  |  |  |
|  | 12. U2[1] <GLSCODE> |  |  |  |
|  | 13. B[1] <REINPUT> |  |  |  |
|  | 14. A[5] <GLSTHICK> |  |  |  |
|  | 15. A[14] <PARTNUM> |  |  |  |
|  | 16. A[12] <PRODTYPE> |  |  |  |
|  | 17. A[12] <ATTRIBUTE> |  |  |  |
|  | 18. A[1] <GLSTYPE> |  |  |  |
|  | 19. A[32] <KEYID> |  |  |  |
|  | 20. L,q [ q = the number of LSD] |  |  |  |
|  | 1. L,2 |  |  |  |
|  | 1. A[24] <LSDITEM1> |  |  |  |
|  | 2. A[40] <LSDVALUE1> |  |  |  |
|  | : |  |  |  |
|  | q. L,2 |  |  |  |
|  | 1. A[24] <LSDITEMq> |  |  |  |
|  | 2. A[40] <LSDCALUEq> |  |  |  |
|  | 21. L,r [ r = the number of GSD] |  |  |  |
|  | 1.L,2 |  |  |  |
|  | 1. A[24] <GSDITEM11> |  |  |  |
|  | 2. A[40] <GSDVALUE11> |  |  |  |
|  | : |  |  |  |
|  | r. L,2 |  |  |  |
|  | 1. A[24] <GSDITEM1r> |  |  |  |
|  | 2. A[40] <GSDVALUE1r> |  |  |  |
|  | 2. L,21 [ ( B Cut Glass) ] |  |  |  |
|  | 1. A[16] <LOTID> |  |  |  |
|  | 2. A[14] <CSTID> |  |  |  |
|  | 3. A[3] <SLOTID> |  |  |  |
|  | 4. A[24] <RECIPEID> |  |  |  |
|  | 5. A[8] <PRCID> |  |  |  |
|  | 6. A[1] <GLSODR> |  |  |  |

| Structure | - A[16] <GLSID> - A[2] <GLSJUDGE> 9. A[2000] <PNLIF> - A[2000] <SUBMDLIF> - B[1] <PORTID> - U2[1] <GLSCODE> - B[1] <REINPUT> - A[5] <GLSTHICK> - A[14] <PARTNUM> - A[12] <PRODTYPE> - A[12] <ATTRIBUTE> - A[1] <GLSTYPE> - A[32] <KEYID> - L,t [ t = the number of LSD] 1. L,2 - A[24] <LSDITEM1> - A[40] <LSDVALUE1> : t. L,2 - A[24] <LSDITEMt> - A[40] <LSDCALUEt> - L,u [ u = the number of GSD] 1.L,2 - A[24] <GSDITEM11> - A[40] <GSDVALUE11> : u. L,2 - A[24] <GSDITEM1u> - A[40] <GSDVALUE1u> |
| --- | --- |
| Notice | <UNITID> Unit ID <STAGEID> Stage ID <GLSSTATUS> Status of the glass that would be loaded 0 = Not Exist 1 = Exist <LOTID> LOT ID <CSTID> Cassette ID <SLOTID> Slot ID <RECIPEID> Recipe ID <PRCID> Processing Code <GLSODR> Glass Order 0 = The first glass 1 = All glasses except the first glass and the last glass 2 = The Last glass ( In this case, this value may not be delivered for Scrap or reason of LOT Abort ) 3 = the only glass, when there is just one in a LOT. <GLSID> Glass ID <GLSJUDGE> Glass Grade Code <PNLIF> Panel Judge Information <SUBMDLIF> Sub Panel Information <PORTID> Port ID <GLSCODE> Glass ID Code. <REINPUT> Glass Re-input Flag 0 = Not Re-input 1 = Re-input <GLSTHICK> Glass’s Thickness <PARTNUM> Part Number |

| Notice | <PRODTYPE> Type of Producing <ATTRIBUTE> LOT Attribute <GLSTYPE> Glass Type N = Normal Glass D = Dummy Glass <KEYID>Glass Key ID <LSDITEM> LOT Specific Data Item defined by LG Display <LSDVALUE> LOT Specific Data Value defined by LG Display <GSDITEM> Glass Specific Data Item defined by LG Display <GSDVALUE> Glass Specific Data Value defined by LG Display <--GSD Item/Value Define-> <CUT_TYPE> Glass Cut Type A : A Cut DA : Dummy A Cut B : B Cut DB : Dummy B Cut <PAIRINFO> A,B Cut Glass Pair Information Y : Paired N : Unpaired <PAIR_GLASS_CODE> Matching Pair Glass Code Glass Code <ROUTE_CHAMBER> Chamber Route Information Chamber Route Information |
| --- | --- |
| Exception |  |

| S2,F88 |  | Load Glass Data Send Acknowledge | ECSEQP |  |
| --- | --- | --- | --- | --- |
| Description | Acknowledge |  |  |  |
| Structure | L,3 - U2[1] <UNITID> - B[1] <STAGEID> 3. B[1] <ACKC2> |  |  |  |
| Notice | <UNITID> Unit ID <STAGEID> Stage ID <ACKC2> 0 = Accept 1 = Process Condition is not ready 2 = The recipe is not exist in equipment’s recipe list 3 = Glass data is not accepted 4 = Process is not ready to load this glass 5 = A, B Cut Recipe Mismatching 6~63 = Reserved |  |  |  |
| Exception |  |  |  |  |

| S2,F95 |  | Chemical Change Enable Request | ECSEQP | reply |
| --- | --- | --- | --- | --- |
| Description | This message is used to request chemical change enable to ECS. |  |  |  |
| Structure | U2[1] <UNITID> |  |  |  |
| Notice | <UNITID> Unit ID |  |  |  |
| Exception |  |  |  |  |

| S2,F96 |  | Chemical Change Enable Request Acknowledge | ECSEQP |  |
| --- | --- | --- | --- | --- |
| Description | Acknowledge |  |  |  |
| Structure | L,2 1. U2[1]<UNITID> 2. B[1]<ACKC2> |  |  |  |
| Notice | <UNITID>Unit ID <ACKC2> 0 = Accepted 1 = Not Accepted 2 ~ 63 Reserved |  |  |  |
| Exception |  |  |  |  |

| S2,F97 |  | Chemical Change Enable Send | ECSEQP | reply |
| --- | --- | --- | --- | --- |
| Description | This message is used to send the chemical change enable to the equipment |  |  |  |
| Structure | L,2 - U2[1] <UNITID> - A[1] <PERMITCD> |  |  |  |
| Notice | <UNITID> Unit ID <PERMITCD> Chemical change permit code 0 : change enable 1 ~ 63 : Reserved |  |  |  |
| Exception |  |  |  |  |

| S2,F98 |  | Chemical Change Enable Send Acknowledge | ECSEQP |  |
| --- | --- | --- | --- | --- |
| Description | Acknowledge |  |  |  |
| Structure | L,2 1. U2[1]<UNITID> 2. B[1]<ACKC2> |  |  |  |
| Notice | <UNITID>Unit ID <ACKC2> 0 = Accepted 1 = Not Accepted 2 ~ 63 Reserved |  |  |  |
| Exception |  |  |  |  |

| S5,F1 |  | Alarm Report | ECSEQP | reply |
| --- | --- | --- | --- | --- |
| Description | This message reports the alarm condition changes inside EQP or of the previous step. One message will be issued when the alarm is set and the other will be issued when cleared. Irrecoverable errors and attention flags may not have the clear message corresponding to them. |  |  |  |
| Structure | L,4 - U2[1] <UNITID> - B[1] <ALCD> 3. A[10] <ALID> 4. A[120] <ALTX> |  |  |  |
| Notice | <UNITID> Unit ID <ALCD>Alarm Code bit 8 = 1 Alarm set bit 8 = 0 Alarm Reset bit 7~1 is Alarm category(0~127) 0 = Not Used 1 = Personal safety 2 = Equipment safety 3 = Parameter control warning 4 = Parameter control error 5 = Irrecoverable error 6 = Equipment status warning 7 = Attention flags 8 = Data integrity <ALID> Alarm Identification <ALTX>Alarm Message |  |  |  |
| Exception |  |  |  |  |

| S5,F2 |  | Alarm Report Acknowledge | ECSEQP |  |
| --- | --- | --- | --- | --- |
| Description | Acknowledge or error. |  |  |  |
| Structure | L,2 1. U2[1]<UNITID> 2. B[1] <ACKC5> |  |  |  |
| Notice | <ACKC5> Acknowledge code, 1 byte 0 = Accepted (Fixed) |  |  |  |
| Exception |  |  |  |  |

| S6,F61 |  | Load Glass Data Request | ECSEQP | reply |
| --- | --- | --- | --- | --- |
| Description | This message is used in Load Glass Data Request |  |  |  |
| Structure | L,2 - U2[1] <UNITID> - B[1] <STAGEID> |  |  |  |
| Notice | <UNITID> Unit ID <STAGEID> Stage ID |  |  |  |
| Exception |  |  |  |  |

| S6,F62 |  | Load Glass Data Request Acknowledge | ECSEQP |  |
| --- | --- | --- | --- | --- |
| Description | Acknowledge |  |  |  |
| Structure | L,3 - U2[1] <UNITID> - B[1] <STAGEID> 3. B[1]<ACKC6> |  |  |  |
| Notice | <UNITID> Unit ID <STAGEID> Stage ID <ACKC2> 0 = Accept 1 = Not Accept |  |  |  |
| Exception |  |  |  |  |

| S6,F63 |  | Unload Glass Data Report | ECSEQP | reply |
| --- | --- | --- | --- | --- |
| Description | This message is used in Unload Glass Data Report. |  |  |  |
| Structure | L, 3 - U2[1] <UNITID> - B[1] <STAGEID> 3. L,2 - L,21 [ ( A Cut Glass ) ] - A[1] <GLSODR> - A[16] <LOTID> - A[14] <CSTID> - A[3] <SLOTID> - A[16] <GLSID> 6. A[2000] <PNLIF> - A[2000] <SUBMDLIF> - A[8] <PRCID> - B[1] <DICODE> - B[1] <PORTID> - A[24] <RECIPEID> - A[2] <GLSJUDGE> - A[14] <PARTNUM> - A[12] <PRODTYPE> - A[12] <ATTRIBUTE> - A[1] <GLSTYPE> - A[1] <GLSPROC> - U2[1] <GLSCODE> 19. U2[1] <DEST> - A[32] <KEYID> - L,p [ n = the number of GSD] 1. L, 2 - A[24] <GSDITEM1> - A[40] <GSDVALUE1> : p. L, 2 - A[24] <GSDITEMp> - A[40] <GSDVALUEp> - L,21 [ ( B Cut Glass ) ] - A[1] <GLSODR> - A[16] <LOTID> - A[14] <CSTID> - A[3] <SLOTID> - A[16] <GLSID> 6. A[2000] <PNLIF> - A[2000] <SUBMDLIF> - A[8] <PRCID> - B[1] <DICODE> - B[1] <PORTID> - A[24] <RECIPEID> - A[2] <GLSJUDGE> - A[14] <PARTNUM> - A[12] <PRODTYPE> - A[12] <ATTRIBUTE> |  |  |  |

| Structure | - A[1] <GLSTYPE> - A[1] <GLSPROC> - U2[1] <GLSCODE> 19. U2[1] <DEST> - A[32] <KEYID> - L,r [ n = the number of GSD] 1. L, 2 - A[24] <GSDITEM1> - A[40] <GSDVALUE1> : r. L, 2 - A[24] <GSDITEMr> - A[40] <GSDVALUEr> |
| --- | --- |
| Notice | <UNITID> Unit ID <STAGEID> Stage ID <GLSODR> Glass Order 0 = The first glass 1 = All glasses except the first glass and the last glass 2 = The Last glass 3 = the only glass, when there is just one in a LOT. <LOTID> LOT ID <CSTID> Cassette ID <SLOTID> Slot ID <GLSID> Glass ID <PNLIF> Panel Information <SUBMDLIF> Sub Panel Information <PRCID> Processing Code <DICODE> Different Code 0 : All GLS IDs are completely matching with GLS IDs from ECS. 1 : Any GLS ID is not matching with GLS ID from ECS 2 : When each ID of glass is not checked by VCR or OCR in the equipment has VCR or OCR <PORTID> Port ID <RECIPE ID> Recipe ID <GLSJUDGE> Glass Grade Code <PARTNUM> Part Number <PRODTYPE> Type of Producing <ATTRIBUTE> LOT Attribute <GLSTYPE> Glass Type N = Normal Glass D = Dummy Glass <GLSPROC> Glass Process Status 0 : Unprocessed - : Processing - : Processed - : Rejected <GLSCODE> Glass ID Code |

| Notice | <DEST> Glass Next Destination 0 = ECS determine destination of glass. >0 = Equipment determine destination of glass. The value of <DEST> will be decided later through conversation <KEYID> Glass Key ID <GSDITEM> Glass Specific Data Item defined by LG Display <GSDVALUE> Glass Specific Data Value defined by LG Display <--GSD Item/Value Define-> <CUT_TYPE> Glass Cut Type A : A Cut DA : Dummy A Cut B : B Cut DB : Dummy B Cut <PAIRINFO> A,B Cut Glass Pair Information Y : Paired N : Unpaired <PAIR_GLASS_CODE> Matching Pair Glass Code Glass Code <ROUTE_CHAMBER> Chamber Route Information Chamber Route Information |
| --- | --- |
| Exception | When <GSDITEM>, <GSDVALUE> is not necessary, n is 0. |

| S6,F64 |  | Unload Glass Data Report Acknowledge | ECSEQP |  |
| --- | --- | --- | --- | --- |
| Description | Acknowledge. |  |  |  |
| Structure | L,2 1. U2[1]<UNITID> 2. B[1]<ACKC6> |  |  |  |
| Notice | <UNITID>Unit ID <ACKC6> 0 = Accepted >0 = Error, not accepted 1 – 63 Reserved |  |  |  |
| Exception |  |  |  |  |

| S6,F65 |  | LOT End Data Send | ECSEQP | reply |
| --- | --- | --- | --- | --- |
| Description | This message is used in Lot End Data Send |  |  |  |
| Structure | L,9 - U2[1]<UNITID> - B[1]<PORTID> - A[16] <LOTID> - A[14] <CSTID> - A[24] <RECIPEID> - A[8] <PRCID> - A[1]<ENDFLAG> - A[1] <MULTIFLAG> - L,n ( n = 0 or the number of slot ) 1. L,10 - A[1] <GLSODR1> - A[3] <SLOTID1> 3. A[16] <GLSID1> - A[24] <RECIPEID1> - B[1]<GLSEXIST1> - B[1]<GLSSELECTED1> - U2[1]<PROCUNITID1> - A[2]<GLSJUDGE1> 9. A[2000]<PNLIF1> 10. A[2000]<SUBMDLIF1> : n. L,10 - A[1] <GLSODRn> - A[3] <SLOTIDn> - A[16] <GLSIDn> - A[24] <RECIPEIDn> - B[1]<GLSEXISTn> - B[1]<GLSSELECTEDn> - U2[1]<PROCUNITIDn> - A[2]<GLSJUDGEn> 9. A[2000]<PNLIFn> 10. A[2000]<SUBMDLIFn> |  |  |  |
| Notice | <UNITID> Unit ID <PORTID> Port ID <LOTID> LOT ID <CSTID> Cassette ID <RECIPE ID> Recipe ID <PRCID> Processing Code <ENDFLAG> End Flag N : Normal End P : Operator End E : Last processed unit R : Last processed unit for rework |  |  |  |

| Notice | <MULTIFLAG> T : True(Multi Recipe ID) F : False(Single Recipe ID) <GLSODR> Glass Order 0 = The first glass 1 = All glasses except the first glass and the last glass 2 = The Last glass 3 = the only glass, when there is just one in a LOT <SLOTID> Slot ID <GLSID> Glass ID <GLSEXIST> Glass Existence 0 = Not exist 1 = Exist <GLSSELECTED> Glass Selection 0 = Not selected 1 = selected <PROCUNITID> Unit ID to process Glass <GLSJUDGE> Glass Grade Code <PNLIF> Panel Information <SUBMDLIF> Sub Panel Information |
| --- | --- |
| Exception | If n=0, this means ‘Lot Judge Report’ facility is not used. If n≠0, this means ‘Lot Judge Report’ facility is used. |

| S6,F66 |  | LOT End Data Send Acknowledge | ECSEQP |  |
| --- | --- | --- | --- | --- |
| Description | Acknowledge |  |  |  |
| Structure | L,4 - U2[1]<UNITID> - B[1] <PORTID> - A[16] <LOTID> 4. B[1] <ACKC6> |  |  |  |
| Notice | <UNITID> Unit ID <PORTID> Port ID <LOTID> Lot ID <ACKC6> Acknowledge code, 1 byte 0 = Accepted 1 = Wait Lot Actual Processing Data 2 = Lot End without Lot Actual Processing Data >3 = Error , not accepted 1 – 63 Reserved |  |  |  |
| Exception |  |  |  |  |

| S6,F67 |  | Glass Move Request | ECSEQP | reply |
| --- | --- | --- | --- | --- |
| Description | This message is used in Load Request, Unload Request from Equipment. |  |  |  |
| Structure | L,5 - A[2] <GLSMVSTATUS> - U2[1] <UNITID> - B[1] <STAGEID> - U2[1] <LDGLSCODE_A> - U2[1] <LDGLSCODE_B> - U2[1] <UDGLSCODE_A> - U2[1] <UDGLSCODE_B> |  |  |  |
| Notice | <GLSMVSTATUS> Glass Move Status LW : Glass Load Wait Request UW : Glass Unload Wait Request LR : Glass Load Request UR : Glass Unload Request ER : Glass Exchange Request CR : Cancel Request <UNITID> Unit ID <STAGEID> Stage ID <LDGLSCODE_A> A Cut Load Glass Code <LDGLSCODE_B> B Cut Load Glass Code <UDGLSCODE_A> A Cut Unload Glass Code <UDGLSCODE_B> B Cut Unload Glass Code |  |  |  |
| Exception |  |  |  |  |

| S6,F68 |  | Glass Move Request Acknowledge | ECSEQP |  |
| --- | --- | --- | --- | --- |
| Description | Acknowledge |  |  |  |
| Structure | L,2 1. U2[1]<UNITID> 2. B[1]<ACKC6> |  |  |  |
| Notice | <UNITID>Unit ID <ACKC6> 0 = Accepted 1 = Not Accepted |  |  |  |
| Exception |  |  |  |  |

| S6,F69 |  | Cassette Map Request | ECSEQP | reply |
| --- | --- | --- | --- | --- |
| Description | The Equipment requests Cassette Map(Online Local Control state only) |  |  |  |
| Structure | U2[1] <UNITID> |  |  |  |
| Notice | <UNITID> Unit ID |  |  |  |
| Exception |  |  |  |  |

| S6,F70 |  | Cassette Map Request Acknowledge | ECSEQP |  |
| --- | --- | --- | --- | --- |
| Description | Cassette Map Data Send |  |  |  |
| Structure | L,2 - U2[1]<UNITID> - L,n ( n = Cassette port number) 1.L,5 1. A[16] <LOTID1> 2. A[14]<CSTID1> - B[1] <PORTID1> 4. A[40] <MAP1> 5. A[1] <CSTREADY1> ……. n.L,5 - A[16] <LOTIDn> - A[14]<CSTIDn> - B[1] <PORTIDn> 4. A[40] <MAPn> 5. A[1] < CSTREADYn> |  |  |  |
| Notice | <UNITID>Unit ID <LOTID>LOT ID <CSTID>Cassette ID <PORTID> Port ID <MAP> Cassette Mapping Information e.g. In case there are 2 glassed in Slot 1 and Slot 20. Then MAP = “1000000000000000000100000000000000000000” - : Glass Not Exist 1 : Glass Exist <CSTREADY> Cassette State 0 : Cassette Not Exist - : Cassette Exist (LC) |  |  |  |
| Exception |  |  |  |  |

| S6,F71 |  | Sampling Slot Request | ECSEQP | reply |
| --- | --- | --- | --- | --- |
| Description | The Equipment requests the specific glass selected by the equipment from ECS on Online Local State. |  |  |  |
| Structure | L,3 - U2[1]<UNITID> 2.B[1]<STAGEID> 3.L,4 - A[16] <LOTID> - A[14]<CSTID> - B[1] <PORTID> - A[3] <SLOTID> |  |  |  |
| Notice | <UNITID> Unit ID <STAGEID> Stage ID <LOTID> LOT ID <CSTID> Cassette ID <PORTID> Port ID <SLOTID> Slot ID |  |  |  |
| Exception |  |  |  |  |

| S6,F72 |  | Sampling Slot Request Acknowledge | ECSEQP |  |
| --- | --- | --- | --- | --- |
| Description | Acknowledge or error |  |  |  |
| Structure | L,2 - U2[1]<UNITID> - B[1] <ACKC6A> |  |  |  |
| Notice | <ACKC6A> Acknowledge code, 1byte 0 = Accept 1 = Glass not exist 2 = Cassette not exist 3 = Invalid LOT ID 4 = Busy, Try later 5 – 63 Reserved |  |  |  |
| Exception |  |  |  |  |

| S6,F73 |  | Glass Actual Processing Data Report | ECSEQP | reply |
| --- | --- | --- | --- | --- |
| Description | The purpose of this message is for the equipment to send the glass processing data. |  |  |  |
| Structure | L,8 |  |  |  |
|  | 1. A[16] <LOTID> |  |  |  |
|  | 2. A[14] <CSTID> |  |  |  |
|  | 3. A[24] <RECIPEID> |  |  |  |
|  | 4. A[8] <PRCID> |  |  |  |
|  | 5. A[3] <SLOTID> |  |  |  |
|  | 6. U2[1] <UNITID> |  |  |  |
|  | 7. L,7 |  |  |  |
|  | 1. A[16] <GLSID> |  |  |  |
|  | 2. A[2] <GLSJUDGE> |  |  |  |
|  | 3. A[2000] <PNLIF> |  |  |  |
|  | 4. A[2000] <SUBMDLIF> |  |  |  |
|  | 5. U2[1]<GLSCODE> |  |  |  |
|  | 6. A[32]<KEYID> |  |  |  |
|  | 7. L,p [ p = the number of GSD] |  |  |  |
|  | 1. L, 2 |  |  |  |
|  | 1. A[24] <GSDITEM1> |  |  |  |
|  | 2. A[40] <GSDVALUE1> |  |  |  |
|  | : |  |  |  |
|  | p. L, 2 |  |  |  |
|  | 1. A[24] <GSDITEMp> |  |  |  |
|  | 2. A[40] <GSDVALUEp> |  |  |  |
|  | 8. L, q [q = number of parameter] |  |  |  |
|  | 1. L, 2 |  |  |  |
|  | 1. A[14] <DVNAME11> |  |  |  |
|  | 2. A[14] <DVVAL11> |  |  |  |
|  | : |  |  |  |
|  | q. L, 2 |  |  |  |
|  | 1. A[14] <DVNAME1q> |  |  |  |
|  | 2. A[14] <DVVAL1q> |  |  |  |
| Notice | <LOTID> LOT ID <CSTID> Cassette ID <RECIPE ID> Recipe ID <PRCID> Processing Code <SLOTID> Slot ID <UNITID>Unit ID <GLSID> Glass ID <GLSJUDGE> Glass Judge Code <PNLIF> Panel Information <SUBMDLIF>Sub Panel Information <GLSCODE> Glass Code <KEYID> Glass Key ID |  |  |  |

| Notice | <GSDITEM> Glass Specific Data Item defined by LG Display <GSDVALUE> Glass Specific Data Value defined by LG Display <DVNAME> APD Data Name defined by LG Display <DVVALUE> APD Data Value defined by LG Display |
| --- | --- |
| Exception | When <GSDITEM>, <GSDVALUE> is not necessary, p is 0. |

| S6,F74 |  | Glass Actual Processing Data Report Acknowledge | ECSEQP |  |
| --- | --- | --- | --- | --- |
| Description | Acknowledge |  |  |  |
| Structure | L,2 1. U2[1]<UNITID> 2. B[1]<ACKC6> |  |  |  |
| Notice | <UNITID>Unit ID <ACKC6> 0 = Accepted 1 = Not Accepted 2 ~ 63 Reserved |  |  |  |
| Exception |  |  |  |  |

| S6,F83 |  | LOT Actual Processing Data Report | ECSEQP | reply |
| --- | --- | --- | --- | --- |
| Description | The purpose of this message is that the equipment sends the Lot Actual processing data. Only when ECS sends Lot End Data, the equipment sends this data |  |  |  |
| Structure | L,9 - A[16] <LOTID> - A[14] <CSTID> - A[24] <RECIPE ID> - A[8] <PRCID> - U2[1] <UNITID> - A[32] <KEYID> - A[2] <LOTJUDGE> - L,p [ p = the number of LSD ] 1. L,2 - A[24] <LSDITEM1> - A[40] <LSDVALUE1> : p. L, 2 - A[24] <LSDITEMp> - A[40] <LSDVALUEp> 9. L, q [q = number of parameter] 1. L, 2 1. A[14] <DVNAME11> 2. A[14] <DVVAL11> : q. L, 2 - A[14] <DVNAME1q> - A[14] <DVVAL1q> |  |  |  |
| Notice | <LOTID> LOT ID <CSTID> Cassette ID <RECIPE ID> Recipe ID <PRCID> Processing Code <UNITID>Unit ID <KEYID> Glass Key ID <LOTJUDGE>Lot Judge <LSDITEM> LOT Specific Data Item defined by LG Display <LSDVALUE> LOT Specific Data Value defined by LG Display <DVNAME> APD Data Name defined by LG Display <DVVALUE> APD Data Value defined by LG Display |  |  |  |
| Exception |  |  |  |  |

| S6,F84 |  | LOT Actual Processing Data Report Acknowledge | ECSEQP |  |
| --- | --- | --- | --- | --- |
| Description | Acknowledge |  |  |  |
| Structure | L,2 1. U2[1]<UNITID> 2. B[1]<ACKC6> |  |  |  |
| Notice | <UNITID>Unit ID <ACKC6> 0 = Accepted 1 = Not Accepted 2 ~ 63 Reserved |  |  |  |
| Exception |  |  |  |  |

| S6,F85 |  | Dummy Lot Request | ECSEQP | reply |
| --- | --- | --- | --- | --- |
| Description | This message is used to request Dummy Lot. |  |  |  |
| Structure | U2[1]<UNITID> |  |  |  |
| Notice | <UNITID> Unit ID |  |  |  |
| Exception |  |  |  |  |

| S6,F86 |  | Dummy Lot Request Acknowledge | ECSEQP |  |
| --- | --- | --- | --- | --- |
| Description | Acknowledge |  |  |  |
| Structure | L,2 1. U2[1]<UNITID> 2. B[1]<ACKC6> |  |  |  |
| Notice | <UNITID>Unit ID <ACKC6> 0 = Accepted 1 = Not Accepted 2 ~ 63 Reserved |  |  |  |
| Exception |  |  |  |  |

| S6,F89 |  | Rework Request | ECSEQP | reply |
| --- | --- | --- | --- | --- |
| Description | This message is used to request rework. |  |  |  |
| Structure | L,4 - U2[1] <UNITID> - A[16] <LOTID> - A[1] <RESERVECD> - L,n [n = The Number of Slot ] 1.L,2 - A[3] <SLOTID1> - A[1] <SELECTED1> . . n.L,2 - A[3] <SLOTIDn> - A[1] <SELECTEDn> |  |  |  |
| Notice | <UNITID> Unit ID <LOTID> Lot ID <RESERVECD> Rework reservation code R : Reservation of rework S : Rework Start <SELECTED> Glass Selected Information. 0 : Not Selected 1 : Selected |  |  |  |
| Exception |  |  |  |  |

| S6,F90 |  | Rework Request Acknowledge | ECSEQP |  |
| --- | --- | --- | --- | --- |
| Description | Acknowledge |  |  |  |
| Structure | L,2 1. U2[1]<UNITID> 2. B[1]<ACKC6> |  |  |  |
| Notice | <UNITID>Unit ID <ACKC6> 0 = Accepted 1 = Not Accepted 2 ~ 63 Reserved |  |  |  |
| Exception |  |  |  |  |

| S6,F91 |  | Equipment Status Change Report | ECSEQP | reply |
| --- | --- | --- | --- | --- |
| Description | When the Equipment status is changed, the Equipment reports this message |  |  |  |
| Structure | L,3 - U2[1] <UNITID> - A[1] <EQPSTATUS> 3. A[10] <ALID> 4. A[120] <ALTX> |  |  |  |
| Notice | <UNITID>Unit ID <EQPSTATUS> Equipment status R : RUN E : Experiment V : Development C : Model Change M : Monitoring I : IDLE D : DOWN P : Process Down <ALID> Alarm Identification If <EQPSTATUS> is ‘R’ or ‘I’, then <ALID> is fixed by ‘0’ <ALTX> Alarm Message |  |  |  |
| Exception |  |  |  |  |

| S6,F92 |  | Equipment Status Change Report Acknowledge | ECSEQP |  |
| --- | --- | --- | --- | --- |
| Description | Acknowledge |  |  |  |
| Structure | L,2 1. U2[1]<UNITID> 2. B[1]<ACKC6> |  |  |  |
| Notice | <UNITID> Unit ID <ACKC6> 0 = Accepted 1 = Not Accepted 2 ~ 63 Reserved |  |  |  |
| Exception |  |  |  |  |

| S6,F93 |  | Scrap Data Report | ECSEQP | reply |
| --- | --- | --- | --- | --- |
| Description | When a glass or a panel is scrapped, the Equipment reports this message |  |  |  |
| Structure | L,5 - A[16] <LOTID> - A[16] <GLSID> - U2[1] <UNITID> - A[1] <SCRPTYPE> - A[4] <SCRPCODE.> |  |  |  |
| Notice | <LOTID> LOT ID <GLSID> Glass ID <UNITID> Unit ID <SCRPTYPE> Scrap Type P = Panel Scarp G = Glass Scrap <SCRPCODE>Scrap Code defined by LG Display |  |  |  |
| Exception |  |  |  |  |

| S6,F94 |  | Scrap Data Report Acknowledge | ECSEQP |  |
| --- | --- | --- | --- | --- |
| Description | Acknowledge |  |  |  |
| Structure | L,2 1. U2[1]<UNITID> 2. B[1]<ACKC6> |  |  |  |
| Notice | <UNITID> Unit ID <ACKC6> 0 = Accepted 1 = Not Accepted 2 ~ 63 Reserved |  |  |  |
| Exception |  |  |  |  |

| S6,F95 |  | OCR( VCR ) Status Report | ECSEQP | reply |
| --- | --- | --- | --- | --- |
| Description | This message is used in OCR Status Report . |  |  |  |
| Structure | L,2 - U2[1]<UNITID> - B[1] <OCRSTATUS> |  |  |  |
| Notice | <UNITID> Unit ID <OCRSTATUS> Status for OCR 0 : OCR off 1 : OCR On |  |  |  |
| Exception |  |  |  |  |

| S6,F96 |  | OCR( VCR ) Status Report Acknowledge | ECSEQP |  |
| --- | --- | --- | --- | --- |
| Description | Acknowledge |  |  |  |
| Structure | L,2 1. U2[1]<UNITID> 2. B[1]<ACKC6> |  |  |  |
| Notice | <UNITID> Unit ID <ACKC6> 0 = Accepted 1 = Not Accepted 2 ~ 63 Reserved |  |  |  |
| Exception |  |  |  |  |

| S7,F19 |  | Current EPPD Request | ECSEQP | reply |
| --- | --- | --- | --- | --- |
| Description | This message is used to request the transmission of the current equipment process program directory. This is the list of all the RECIPE IDs of the process programs stored in the equipment |  |  |  |
| Structure | U2[1] <UNITID> |  |  |  |
| Notice | <UNITID> Unit ID |  |  |  |
| Exception |  |  |  |  |

| S7,F20 |  | Current EPPD Request Acknowledge | ECSEQP |  |
| --- | --- | --- | --- | --- |
| Description | This message is used to transmit the current EPPD. |  |  |  |
| Structure | L,2 - U2[1] <UNITID> - L,n (n = Recipe List Count) - A[24] <RECIPEID1> . . . n. A[24] <RECIPEIDn> |  |  |  |
| Notice | <UNITID> Unit ID <RECIPEID> Recipe ID |  |  |  |
| Exception |  |  |  |  |

| S7,F61 |  | Recipe ID Confirm Request | ECSEQP | reply |
| --- | --- | --- | --- | --- |
| Description | This message is used to ask the equipment whether the recipe ID is or not in the list, which the equipment has. |  |  |  |
| Structure | L,14 - A[16] <LOTID> - A[14] <CSTID> - A[24] <RECIPEID> - A[8] <PRCID> - B[1] <PORTID> - U2[1] <UNITID> - A[5] <GLSTHICK> - A[14] <PARTNUM> - A[12] <PRODTYPE> - A[12] <ATTRIBUTE> - A[1] <MODECODE> - A[1] <MULTIFLAG> - L, m [ m = the number of LSD] 1. L,2 - A[24] <LSDITEM1> - A[40] <LSDVALUE1> : m. L,2 - A[24] <LSDITEMm> - A[40] <LSDVALUEm> - L,n [ n = The Number of Slot] 1. L,10 - A[3] <SLOTID1> - A[16] <SLTLOTID1> 3. A[16] <GLSID1> - A[24] <SRECIPEID1> - B[1] <GLSEXIST1> - A[2] <GLSJUDGE1> - A[14] <PARTNUM1> 8. A[2000] <PNLIF1> - A[2000] <SUBMDLIF1> - L,p [ p = the number of GSD] 1. L, 2 - A[24] <GSDITEM11> - A[40] <GSDVALUE11> : p. L, 2 - A[24] <GSDITEM1p> - A[40] <GSDVALUE1p> : |  |  |  |

| Structure | : |
| --- | --- |
|  | n. L,10 |
|  | 1. A[3] <SLOTIDn> |
|  | 2. A[16] <SLTLOTIDn> |
|  | 3. A[16] <GLSIDn> |
|  | 4. A[24] <SRECIPEIDn> |
|  | 5. B[1]<GLSEXISTn> |
|  | 6. A[2] <GLSJUDGEn> |
|  | 7. A[14] <PARTNUMn> |
|  | 8. A[2000] <PNLIFn> |
|  | 9. A[2000] <SUBMDLIFn> |
|  | 10. L,q [ q = the number of GSD] |
|  | 1. L, 2 |
|  | 1. A[24] <GSDITEMn1> |
|  | 2. A[40] <GSDVALUEn1> |
|  | : |
|  | q. L, 2 |
|  | 1. A[24] <GSDITEMnq> |
|  | 2. A[40] <GSDVALUEnq> |
| Notice | <UNITID> Unit ID <LOTID> LOT ID <CSTID> Cassette ID <SLOTID> Slot ID <RECIPEID> Recipe ID <PRCID> Processing Code <SLTLOTID> Each Slot LOT ID <SRECIPEID> Each Slot Recipe ID <GLSEXIST>Glass Existence 0 = Not exist , 1 = Exist <GLSID> Glass ID <GLSJUDGE> Glass Grade Code <PNLIF> Panel Information <SUBMDLIF> Sub Panel Information <PORTID> Port ID <GLSTHICK> Glass’s Thickness <PARTNUM> Part Number <PRODTYPE> Type of Producing <ATTRIBUTE> LOT Attribute <MODECODE> Host Online / Offline Code N : Recipe has downloaded by Host (Host Online Mode) F : Recipe has inputted by Operator (Host Offline Mode) <MULTIFLAG> Multi Flag T : True(Multi Lot or Multi Recipe) F : False(Single Lot and Single Recipe) <KEYID>Glass Key ID <LSDITEM> LOT Specific Data Item defined by LG Display <LSDVALUE> LOT Specific Data Value defined by LG Display <GSDITEM> Glass Specific Data Item defined by LG Display <GSDVALUE> Glass Specific Data Value defined by LG Display |
| Exception |  |

| S7,F62 |  | Recipe ID Confirm and Sampling Data Request Acknowledge | ECSEQP |  |
| --- | --- | --- | --- | --- |
| Description | This message is used to transmit the confirm result of the recipe id and the sampling data of the equipment. |  |  |  |
| Structure | L,7 1.U2[1] <UNITID> 2.B[1] <PORTID> 3.A[24] <RECIPEID> 4.A[16] <LOTID> 5.A[1] <SAMPLINGFLAG> 6.B[1] <ACKC7M> - L,n [n = The Number of Slot ] 1.L,3 - A[3] <SLOTID1> - A[24] <SRECIPEID1> 3. B[1] <ACKC7S1> . . n.L,3 - A[3] <SLOTIDn> - A[24] <SRECIPEIDn> - B[1] <ACKC7Sn> |  |  |  |
| Notice | <UNITID> Unit ID <PORTID> Port ID <RECIPEID> Recipe ID ( This is recipe for LOT ) <LOTID> Lot ID < SAMPLINGFLAG> Sampling Flag W : Wait equipment’s Sampling Data S : Lot start without equipment’s Sampling Data <ACKC7M> Acknowledge code, 1 byte 0 = Accept 1 = Not accept 2 = Process Condition is not ready 3 – 63 Reserved <SLOTID> Slot ID <SRECIPEID> Each Slot Recipe ID <ACKC7S> 0 = Accept 1 = Not accept 2 – 63 Reserved |  |  |  |
| Exception |  |  |  |  |

| S7,F63 |  | Sampling Data Report | ECSEQP | reply |
| --- | --- | --- | --- | --- |
| Description | This message is used to report the sampling data of the equipment |  |  |  |
| Structure | L,5 - U2[1] <UNITID> - B[1] <PORTID> - A[24] <RECIPEID> - A[16] <LOTID> - L,n [n = The Number of Slot ] 1.L,4 - A[3] <SLOTID1> - A[24] <SRECIPEID1> 3. B[1] <ACKC7S1> 4. A[1] <SELECTED1> . . n.L,4 - A[3] <SLOTIDn> - A[24] <SRECIPEIDn> - B[1] <ACKC7Sn> - A[1] <SELECTEDn> |  |  |  |
| Notice | <UNITID> Unit ID <PORTID> Port ID <RECIPEID> Recipe ID ( This is recipe for LOT ) <LOTID> Lot ID <SELECTED> 0 : Not Selected 1 : Selected |  |  |  |
| S7,F64 |  | Sampling Data Report Acknowledge | ECSEQP |  |
| Description | Acknowledge |  |  |  |
| Structure | L,4 - U2[1] <UNITID> - B[1] <PORTID> - A[16] <LOTID> 4. B[1] <ACKC7> |  |  |  |
| Notice | <UNITID> Unit ID <PORTID> Port ID <LOTID> Lot ID <ACKC2> 0 = Accepted >0 = Error, not accepted 1 – 63 Reserved |  |  |  |
| Exception |  |  |  |  |

| S7,F67 |  | Current Recipe Group List Request | ECSEQP | reply |
| --- | --- | --- | --- | --- |
| Description | This message is used to request the transmission of the current equipment process program group. This is the list of all the Groups of the process programs stored in the equipment |  |  |  |
| Structure | U2[1] <UNITID> |  |  |  |
| Notice | <UNITID> Unit ID |  |  |  |
| Exception |  |  |  |  |

| S7,F68 |  | Current Recipe Group List | ECSEQP |  |
| --- | --- | --- | --- | --- |
| Description | This message is used to transmit the current EPPG. |  |  |  |
| Structure | L,2 - U2[1] <UNITID> - L,n (n = Group List Count) - A[12] <GROUPID1> . . . n. A[12] < GROUPID n> |  |  |  |
| Notice | <UNITID> Unit ID <GROUPID> Recipe Group ID |  |  |  |
| Exception |  |  |  |  |

| S7,F69 |  | Current Recipe ID List Request by Group | ECSEQP | reply |
| --- | --- | --- | --- | --- |
| Description | This message is used to request the transmission of the current equipment process program directory. This is the list of all RECIPE IDs of the specified Recipe Group IDs in the equipment |  |  |  |
| Structure | L,2 - U2[1] <UNITID> - L,n (n = Group ID Count) - A[12] <GROUPID1> . . n. A[12] <GROUPIDn> |  |  |  |
| Notice | <UNITID> Unit ID <GROUPID> Recipe Group ID |  |  |  |
| Exception | Equipment using Recipe Group needs This Message. ‘n=0’ means all Recipe Group. |  |  |  |

| S7,F70 |  | Current Recipe ID List Request by Group Acknowledge | ECSEQP |  |
| --- | --- | --- | --- | --- |
| Description | This message is used to transmit all the RECIPE IDs of the specified Recipe Group IDs in the equipment |  |  |  |
| Structure | L,2 - U2[1] <UNITID> - L,m (m = Group ID Count) 1. L,2 - A[12] <GROUPID> - L,n (n = Recipe List Count) - A[24] <RECIPE ID1> . . n. A[24] <RECIPE IDn> . . m. L,2 - A[12] <GROUPID> - L,n (n = Recipe List Count) - A[24] <RECIPE ID1> . . n. A[24] <RECIPE IDn> |  |  |  |
| Notice | <UNITID> Unit ID <GROUPID> Recipe Group ID <RECIPEID> Recipe ID |  |  |  |
| Exception |  |  |  |  |

| S7,F77 |  | Recipe Body Request | ECSEQP | reply |
| --- | --- | --- | --- | --- |
| Description | Recipe Body List Request |  |  |  |
| Structure | L,6 1.A[16]<LOTID> 2.A[14]<CSTID > 3.A[24]<RECIPEID > 4.A[8]<PRCID > 5.B[1]<PORTID> 6.U2[1]<UNITID> |  |  |  |
| Notice | <LOTID> LOT ID <CSTID> Cassette ID <RECIPEID> Recipe ID <PRCID> Processing Code <PORTID> Port ID <UNITID> Unit ID |  |  |  |
| Exception |  |  |  |  |

| S7,F78 |  | Recipe Body | ECSEQP |  |
| --- | --- | --- | --- | --- |
| Description | This message reports the recipe parameter of the equipment |  |  |  |
| Structure | L,8 - A[16] <LOTID> - A[14]<CSTID> - A[24]<RECIPEID> - A[12]<RECIPEIDVER> - A[8]<PRCID> - B[1]<PORTID> - U2[1]<UNITID> - L,m [ m = the number of Unit] 1.L.6 - U2[1] <EUNITID1> - A[24] <LOWERRECIPEID1> - A[12] <LOWERRECIPEIDVER1> - A[1]<LEVEL1> - A[1]<RECIPEIDTYPE1> - L,p [ p = Number of Step ] 1. L,2 1. A[14] <STEP11> - L, n [n = Number of Parameter] 1. L,2 - A[24] <RPARMNAME111> - A[24] <RPARMVAL111> : n.L,2 - A[24] <RPARMNAME11n> - A[24] <RPARMVAL11n> : p. L,2 1. A[14] <STEP1p> - L, q [q = Number of Parameter] 1. L,2 - A[24] <RPARMNAME1p1> - A[24] <RPARMVAL1p1> : q.L,2 - A[24] <RPARMNAME1pq> - A[24] <RPARMVAL1pq> : |  |  |  |

|  | : m.L,6 - U2[1] <EUNITIDm> - A[24] <LOWERRECIPEIDm> - A[12] <LOWERRECIPEIDVERm> - A[1]<LEVELm> - A[1]<RECIPEIDTYPEm> - L,r [ r = Number of Step ] 1. L,2 1. A[14] <STEPm1> - L, s [s = Number of Parameter] 1. L,2 - A[24] <RPARMNAMEm11> - A[24] <RPARMVALm11> : s.L,2 - A[24] <RPARMNAMEm1s> - A[24] <RPARMVALm1s> : r. L,2 - A[14] <STEPmr> - L, t [t = Number of Parameter] 1. L,2 - A[24] <RPARMNAMEmr1> - A[24] <RPARMVALmr1> : t.L,2 - A[24] <RPARMNAMEmrt> - A[24] <RPARMVALmrt> |
| --- | --- |
| Notice | <LOTID> LOT ID <CSTID> Cassette ID <PRCID> Processing Code <RECIPEID> Recipe ID <PORTID> Port ID <UNITID> Unit ID <EUNITID> Each Unit ID <LOWERRECIPEID> Lower Recipe ID <LOWERRECIPEIDVER> Lower Recipe ID Version <LEVEL> Level of RECIPEID 1 : Level 1 RECIPEID 2 : Level 2 RECIPEID 3 : Level 3 RECIPEID < RPARMNAME> Recipe Parameter Name < RPARMVAL> Recipe Parameter Value <RECIPEIDTYPE>Type of Recipe E : Equipment Recipe / M : Machine Recipe / U : Unit Recipe 1,2,3,.. : Special Recipe (Defined with LG Display) <STEP> Recipe Step 0 = Step Not Used, Another Number is Recipe Step |
| Exception |  |

| S9,F1 |  | Unrecognized Device ID | ECSEQP |  |
| --- | --- | --- | --- | --- |
| Description | The device ID in the message block header did not correspond to any known device ID in the node detecting the error. |  |  |  |
| Structure | <MHEAD> |  |  |  |
| Notice |  |  |  |  |
| Exception |  |  |  |  |

| S9,F3 |  | Unrecognized Stream Type | ECSEQP |  |
| --- | --- | --- | --- | --- |
| Description | The equipment does not recognize the stream type in the message block header. |  |  |  |
| Structure | <MHEAD> |  |  |  |
| Notice |  |  |  |  |
| Exception |  |  |  |  |

| S9,F5 |  | Unrecognized Function Type | ECSEQP |  |
| --- | --- | --- | --- | --- |
| Description | This message indicates that the function in the message ID is not recognized by the receiver. |  |  |  |
| Structure | <MHEAD> |  |  |  |
| Notice |  |  |  |  |
| Exception |  |  |  |  |

| S9,F7 |  | Illegal Data | ECSEQP |  |
| --- | --- | --- | --- | --- |
| Description | This message indicates that the stream and function were recognized, but the associated data format could not be interpreted. |  |  |  |
| Structure | <MHEAD> |  |  |  |
| Notice |  |  |  |  |
| Exception |  |  |  |  |

| S9,F9 |  | Transaction Timer Timeout | ECSEQP |  |
| --- | --- | --- | --- | --- |
| Description | This message indicates that a transaction(receive) timer has timed out and that the corresponding transaction has been aborted. It is up to the host to respond to this error in an appropriate manner to keep the system operational. |  |  |  |
| Structure | <SHEAD> |  |  |  |
| Notice |  |  |  |  |
| Exception |  |  |  |  |

| S9,F11 |  | Data Too Long | ECSEQP |  |
| --- | --- | --- | --- | --- |
| Description | This message to the host indicates that the equipment has been sent more data than it can handle. |  |  |  |
| Structure | <MHEAD> |  |  |  |
| Notice |  |  |  |  |
| Exception |  |  |  |  |

| S9,F13 |  | Conversation Timeout | ECSEQP |  |
| --- | --- | --- | --- | --- |
| Description | Data were expected but none was received within a reasonable length of time. Resources have been cleared. |  |  |  |
| Structure | L,2 1. A[6] <MEXP> 2. A[14] <EDID> |  |  |  |
| Notice | <MEXP> Message Expected The format is “SxxFyy” Ex) S07F83, S02F41 etc. <EDID> Expected Data ID MAX 14 Byte Case S7F83 : <EDID>= ‘<CSTID>’ Case S2F41 : <EDID>= ‘<CSTID>’ |  |  |  |
| Exception |  |  |  |  |

| S10,F3 |  | Terminal Display, Single | ECSEQP |  |
| --- | --- | --- | --- | --- |
| Description | Data to be displayed |  |  |  |
| Structure | L,2 1. B[1] <TID> 2. A[80] <TEXT> |  |  |  |
| Notice | <TID> Terminal Number Fixed as ‘0’ <TEXT> A single line of characters. |  |  |  |
| Exception |  |  |  |  |

| S10,F4 |  | Terminal Display, Single Acknowledge | ECSEQP |  |
| --- | --- | --- | --- | --- |
| Description | Acknowledge or Error |  |  |  |
| Structure | B[1] <ACKC10> |  |  |  |
| Notice | <ACKC10> Acknowledge Code, 1 byte 0 = Accepted for display 1 = Message will not be displayed 2 = Terminal not available 3 – 63 Reserved |  |  |  |
| Exception |  |  |  |  |

ChapterⅡ. ECS-Equipment HSMS Exception Specification

For LG Display

# 1. Scenario

### Inform processing glass count change scenario

장비로 투입될 Glass 수량이 변경 되었을 경우 ECS는 해당 메시지를 장비로 송신한다.

    - Cassette Loading 시

    - Offline에서 Online 전환 시

    - Cassette가 port에 로딩되고 작업준비가 된 후, 해당 Cassette가 cancel 되었을 시

S1F69 S1F70

ECS	EQP

Change Glass Count

### Monitoring Lot Status Send

    - 모니터링 Lot의 시작과 종료를 장비로 전달한다.

    - 상세 사용은 LG CNS와 LG Display와 협의한다.

S1F73 S1F74

ECS	EQP

Monitoring Lot Status Data Send

### Equipment’s Part Data Report

    - 온라인 전환 시 사용

-Glass 또는 장비의 Part가 Unit 또는 Sub Unit간에 이동이 있을 경우 장비는 보고한다.

S1F77

S1F78

ECS	EQP

Equipment’s Part Data Report

### Special Case of Glass Judge Report

S6F63

S6F64

S6F63

S6F64

S2F93

S2F94

S6F63

S6F64

S6F63 S6F64

S2F93 S2F94

All Glass Judge is reported

S6F65

S6F66

S6F83

S6F84

ECS	EQP

Unload Glass Data Report

#### <GLSJUDGE> = ‘**’

…

Unload Glass Data Report

#### <GLSJUDGE> = ‘**’

Various Data Report

Unload Glass Data Report

…

<GLSJUDGE> = ‘**’

Unload Glass Data Report

#### <GLSJUDGE> = ‘**’

Various Data Report

LOT End Data Send

LOT Actual Processing Data Report

### Chamber Status Change Report

    - Chamber Status Change Report when chamber status is changed.

      - 장비 Chamber의 Processing 상태가 변경 되었을 경우 장비는 ECS로 해당 메시지를 보고한다.

-만약 응답이 없거나 통신에러가 발생하면 기존 상태를 유지한다.

      - 상세 사용은 LG CNS와 LG Display와 협의한다.

S6F97 S6F98

ECS	EQP

Chamber Status Change Report

    - Chamber Status Change Report ( In Online Sequence or In ECS Restart Sequence)

-장비는 장비 동작 모드 변경 보고 메시지(equipment operation mode change report message, S2F63)을 보낸 후 에 S6F97을 보고해야 한다.

-이 경우 모든 Chamber 상태를 보고해야 한다.

|  | S2F61 |  |
| --- | --- | --- |
|  | S2F62 |  |
|  | S2F63 | E |
|  | S2F64 | <O |
|  | S6F97 S6F98 | C |
|  | Refer to Ch 7.1 ( Control State and Equipment Operation Mode Change )after this point. |  |

ECS	EQP

quipment Operation Mode Report PMODE> = defined number

hamber Status Change Report

### Recipe Management Sequence

    - Equipment Recipe List Request

-ECS가 장비의 recipe list를 조회 시 사용한다.

S15F71

S15F72

ECS	EQP

Recipe List Report Request

    - Equipment Lower Recipe List Request

-ECS가 장비의 하위 recipe list를 조회 시 사용한다.

S15F73 S15F74

ECS	EQP

Lower Recipe List Report Request

    - Equipment Recipe Parameter Request

-장비의 Recipe Parameter를 조회 시 사용한다.

S15F75 S15F76

ECS	EQP

Recipe Parameter Report Request

    - Equipment Recipe Parameter Change Send

      - 장비의 Recipe Parameter를 변경할 경우 사용한다.

S15F77 S15F78

ECS	EQP

Recipe Parameter Change Send

    - Equipment Recipe Change Report

      - 장비의 Recipe Parameter를 변경할 경우 사용한다.

S15F79 S15F80

ECS	EQP

Recipe Parameter Change Report

    - Equipment Operation Reference Data Send

      - 장비로 Operation Reference Data를 전송할 경우 사용한다.

S15F81 S15F82

ECS	EQP

Operation Reference Data Send

    - Equipment Operation Reference Data Apply Report

-장비가 operation reference data를 적용하면 장비는 이 메시지를 전송한다.

S15F83 S15F84

ECS	EQP

Operation Reference Data Apply

Report

    - Current Recipe ID List Request

      - 현재 장비 process program directory의 전송을 요청 시

      - 장비에 등록된 process program의 모든 Recipe ID 와 Recipe ID Version 목록이다.

S15F65 S15F66

ECS	EQP

Current Recipe ID List Request

    - Equipment Constant Recipe Request

S7F79 S7F80

ECS	EQP

Equipment Constant Recipe

Request

    - Equipment Constant Recipe Parameter Request

S15F95 S15F96

ECS	EQP

Equipment Constant Recipe

Parameter Request

    - Equipment Constant Recipe Parameter Change Command

S15F97 S15F98

ECS	EQP

Equipment Constant recipe parameter change request

    - Equipment Constant Recipe Parameter Change Report

S15F99 S15F100

ECS	EQP

Equipment Constant recipe parameter change report

### Buffer Lot Status Send

    - Buffer Lot 상태를 전달할 경우 사용한다.

    - 상세 사용은 LG CNS와 LG Display와 협의한다.

S1F79 S1F80

ECS	EQP

Buffer Lot Status Data Send

### Remained Glass Count Request

    - 장비가 해당 Glass가 속한 Lot의 투입 대기 Glass의 수량을 요청할 경우 사용한다.

    - 상세 사용은 LG CNS와 LG Display와 협의한다.

S1F71 S1F72

ECS	EQP

Remained Glass Count Request

### Glass Call Request

-수동으로 장비에 glass를 투입할 때 사용한다.

-만약 장비에서 이를 사용하려면 추후 ECS와 상세 협의가 필요하다.

S2F77

S2F78

S6F61

S6F62

S2F87

S2F88

이후의 Sequence는 장비의 현재 상태에 따라 결정된다.

- 장비내의 Unloading Glass가 존재하지 않으면 ‘LR’ sequence (7.3.1)를

- 장비내의 Unloading Glass가 존재하는 경우 ‘ER’ sequence(7.3.3 )를 따른다.

ECS	EQP

Glass Call Request

Load Glass Data Request

Load Glass Data Send

<GLSSTATUS> = ‘1’(Exist)

<ACK> = ‘0’(Data Accepted)

### Carrier Transfer Mode Change Report

    - 장비의 Carrier 반송 모드가 변경 보고 용도로 사용한다.

      - 본 메시지는 장비의 Carrier 반송 모드가 변경이 되었을 경우 장비에서 보고한다.

      - 장비의 보고에 대한 ECS 응답이 없는 경우 현재 상태를 유지한다.

      - 상세 사용 및 용도는 LG CNS 및 LG Display 와 협의한다.

S2F71 S2F72

ECS	EQP

Carrier Transfer Mode Change Report

    - 장비의 온라인 상태로 변경 ( ECS Restart 포함 )중에 Carrier 반송 모드 변경 보고

      - 장비는 Online 변경 중 Operation Mode 변경 보고 이후에 Carrier 반송 모드 상태를 보고한다.

|  | S2F61 |  |
| --- | --- | --- |
|  | S2F62 |  |
|  | S2F63 | E |
|  | S2F64 | <O |
|  | S2F71 | C R |
|  | S2F72 |  |
|  | Refer to Ch 7.1 ( Control State and Equipment Operation Mode Change )after this point. |  |

ECS	EQP

quipment Operation Mode Report PMODE> = defined number

arrier Transfer Mode Change eport

### PD Down Request

-장비에 Pause를 요청할 때 사용한다.

-PD Request 시나리오는 Machine/Unit 단에서 Glass Hold 가 가능한 장비에 한해서 적용한다.

## ECS	EQP

PD Down Request (RCMD : Pause)

S2F99 S2F100

If (Not Exist)

Glass Exist ?

If (Exist)

Processing Glass

S6F63	Unload Glass Data Report S6F64

#### S6F67 S6F68

S6F67 S6F68

Glass Move Request

#### <GLSMVSTATUS> = ‘UW’

Glass Move Request

#### <GLSMVSTATUS> = ‘UR’

Unloading Glass

장비는 Unloading Glass Count 만큼 각 Glass 별 S6F73 보고 실시함. 단, A/B Stick 보고 순서는 무관함.

S6F73 S6F74

S6F73 S6F74

#### S6F91 S6F92

Glass Actual Processing Data Report

Glass Actual Processing Data Report

Equipment Status Change Report

#### < EQPSTATUS > = ‘P’

PD Down Release Button Click by Operator (ECS or EQP)

if ECS Release PD Down Request (RCMD : Resume)

Load Glass Data Send

S2F99 S2F100

S6F91

S6F92

S6F61 S6F62 S2F87 S2F88

Equipment Status Change Report

#### < EQPSTATUS > =  ‘R/I/D’

Load Glass Data Request

<ACK> = ‘0’(Data Accepted)

### LOT End Glass Injection Send

    - This Message is used when the last glass of LOT was injected in Equipment

1.12.1 In case the normal sequence

## ECS	EQP

First Glass of Next LOT Injection

S2F71 S2F72

Last Glass of Current LOT Injection

LOT End Glass Injection Send

10.2 In case the abnormal sequence

## ECS	EQP

First Glass of Next LOT Injection

S2F71 S2F72

Lot Abort by operator

LOT End Glass Injection Send

### Multi Recipe Information Send

    - This message is used when equipment requires recipe and sampling codes for each slots

    - If equipment want to use this sequence, it needs detailed conference later with ECS.

## ECS	EQP

S7F81

S7F82

Multi Recipe Information Send

### Port State and Cassette Map Data Send Scenario

    - This message is used when equipment requires the map data for specific port.

    - ECS informs the map data to equipment when Port state is changed

## ECS	EQP

Port State Changing

S1F81 S1F82

Port State Data Send

### Batch Job (Glass Injection) End Data Send

    - This message is used when the last glass of Batch Job is injected in Equipment.

    - If equipment want to use this sequence, it needs detailed conference later with ECS.

|  | The last glass of Batch Job is injected in Equipment. |
| --- | --- |
|  | S2F91 S2F92 |

ECS	EQP

Batch Job(Glass Injection) End Data Send

### Injecting Glass Data Request

    - This message is used when the glass is injecting in Equipment.

    - If equipment want to use this sequence, it needs detailed conference later with ECS.

|  | ‘Load Complete’ signal on from Loader |
| --- | --- |
|  | S2F83 S2F84 |

ECS	EQP

Injecting Glass Data Request

### Port State Data Report

    - This message is used when the State (LR, LC, UR, UC) of Port in Equipment is changed.

    - If equipment want to use this sequence, it needs detailed conference later with ECS.

## ECS	EQP

The State of Port

in Equipment is changed

S1F83 S1F84

Port State Data Report

### Port Status Change Data Report

    - This message is used when the Status (RUN, IDLE, DOWN) of Port in Equipment is changed.

    - If equipment want to use this sequence, it needs detailed conference later with ECS.

## ECS	EQP

The Status of Port

in Equipment is Changed

S1F85 S1F86

Port Status Change Data Report

### Lot State Change Report

    - This message is used when the Lot State (R, W, P, E) of port in Equipment is changed.

    - If equipment want to use this sequence, it needs detailed conference later with ECS.

## ECS	EQP

The Lot State of Equipment is changed

S6F81 S6F82

Lot State Change Report

### Mask Data Report

    - This message is used after the ID of Mask reading in EQP .

    - If equipment want to use this sequence, it needs detailed conference later with ECS.

## ECS	EQP

The ID of Mask reads

S1F89 S1F90

Mask Data Report

### All Mask Position Data Report

    - In case of changing to Online

    - In case of mask moving between each sub unit.

S1F91 S1F92

ECS	EQP

All Mask Position Data Report

### Unload Process Report

- Equipment performs this scenario when unit’s status is changed.

1. 22.1 Unload Start & End

## ECS	EQP

Unload Only Case Occur

Unload Process Report (Start)

#### S2F85 S2F86

S6F63 S6F64

Unload Process Report

#### <UNLOADSTATUS>

= ‘S’ ( Start )

#### <UNLOADTYPE>

= ‘A’ (Auto)

Unload Glass Data Report

#### S6F67

S6F68

Glass Move Request

#### <GLSMVSTATUS> = ‘UW’

S6F67 S6F68

Glass Move Request

#### <GLSMVSTATUS> = ‘UR’

Unloading…

S6F63 S6F64

Unload Glass Data Report

#### S6F67 S6F68

Glass Move Request

#### <GLSMVSTATUS> = ‘UW’

S6F67 S6F68

Glass Move Request

<GLSMVSTATUS> = ‘UR’

Unload Only Case End

S2F85 S2F86

S6F63 S6F64

S6F61

S6F62

Unload Process Report

Unload Process Report (End)

<UNLOADSTATUS>

= ‘E’ ( End )

#### <UNLOADTYPE>

= ‘A’ (Auto)

Unload Glass Data Report

Load Glass Data Request

Copyright  2011 LG CNS Co., Ltd., All rights reserved	157/219

1. 22.2 Unload Start & End (EQP Restart After Unload Start) 1/2

## ECS	EQP

Unload Only Case Occur

Unload Process Report (Start)

#### S2F85 S2F86

S6F63 S6F64

Unload Process Report

#### <UNLOADSTATUS>

= ‘S’ ( Start )

#### <UNLOADTYPE>

= ‘A’ (Auto)

Unload Glass Data Report

#### S6F67 S6F68

Glass Move Request

#### <GLSMVSTATUS> = ‘UW’

S6F67 S6F68

Glass Move Request

#### <GLSMVSTATUS> = ‘UR’

Unloading…

EQP Restart

S1F1 S1F2

Are You There Request

If RCS = 2 (Offline) then Equipment maintain Offline state.

#### S2F61 S2F62

RCS = 2

Control State Change Request (Report current control state )

<RCS> = ‘0’ ( Online – Remote )

<RCS> = ‘1’ ( Online – Local )

<RCS> = ‘2’ (Offline) Current Control State ?

#### S2F63 S2F64

RCS = 0 or 1

Equipment Operation Mode Report

<OPMODE> = defined number

Copyright  2011 LG CNS Co., Ltd., All rights reserved	158/219

1. 22.2 Unload Start & End (EQP Restart After Unload Start) 2/2

## ECS	EQP

#### S2F17 S2F18

S1F61 S1F62

Date and Time Request

All Equipment Status Report

If OCR( VCR ) is equipped in the Equipment

S1F67 S1F68

S6F95

S6F96

S2F85

S2F86

S6F63 S6F64

All Glass Position Data Report

OCR( VCR ) Status Report

Unload Process Report

#### <UNLOADSTATUS>

= ‘S’ ( Start )

#### <UNLOADTYPE>

= ‘A’ (Auto)

Unload Glass Data Report

#### S6F67 S6F68

Glass Move Request

#### <GLSMVSTATUS> = ‘UW’

S6F67 S6F68

Glass Move Request

<GLSMVSTATUS> = ‘UR’

Unloading…

Unload Only Case End

S2F85 S2F86

S6F63 S6F64

S6F61 S6F62

Unload Process Report

Unload Process Report (End)

<UNLOADSTATUS>

= ‘E’ ( End )

#### <UNLOADTYPE>

= ‘A’ (Auto)

Unload Glass Data Report

Load Glass Data Request

Copyright  2011 LG CNS Co., Ltd., All rights reserved	159/219

1. 22.3 Unload Start & End (ECS Restart After Unload Start) 1/2

## ECS	EQP

Unload Only Case Occur

Unload Process Report (Start)

#### S2F85 S2F86

S6F63 S6F64

Unload Process Report

#### <UNLOADSTATUS>

= ‘S’ ( Start )

#### <UNLOADTYPE>

= ‘A’ (Auto)

Unload Glass Data Report

#### S6F67 S6F68

Glass Move Request

#### <GLSMVSTATUS> = ‘UW’

S6F67	Glass Move Request

#### S6F68

<GLSMVSTATUS> = ‘UR’

Unloading…

ECS Restart

#### S1F1 S1F2

Are You There Request

If RCS = 2 (Offline) then Equipment maintain Offline state.

#### S2F61 S2F62

RCS = 2

Control State Change Request (Report current control state )

<RCS> = ‘0’ ( Online – Remote )

<RCS> = ‘1’ ( Online – Local )

<RCS> = ‘2’ (Offline) Current Control State ?

RCS = 0 or 1

#### S2F63 S2F64

Equipment Operation Mode Report

<OPMODE> = defined number

Copyright  2011 LG CNS Co., Ltd., All rights reserved	160/219

- 22.3 Unload Start & End (ECS Restart After Unload Start) 2/2

## ECS	EQP

#### S2F17 S2F18

S1F61 S1F62

Date and Time Request

All Equipment Status Report

If OCR( VCR ) is equipped in the Equipment

S1F67 S1F68

S6F95

S6F96

S2F85 S2F86

S6F63 S6F64

All Glass Position Data Report

OCR( VCR ) Status Report Unload Process Report

#### <UNLOADSTATUS>

= ‘S’ ( Start )

#### <UNLOADTYPE>

= ‘A’ (Auto)

Unload Glass Data Report

#### S6F67 S6F68

Glass Move Request

#### <GLSMVSTATUS> = ‘UW’

S6F67 S6F68

Glass Move Request

<GLSMVSTATUS> = ‘UR’

Unloading…

Unload Only Case End

S2F85 S2F86

S6F63 S6F64

S6F61 S6F62

Unload Process Report

Unload Process Report (End)

<UNLOADSTATUS>

= ‘E’ ( End )

#### <UNLOADTYPE>

= ‘A’ (Auto)

Unload Glass Data Report

Load Glass Data Request

Copyright  2011 LG CNS Co., Ltd., All rights reserved	161/219

# Exception Message Structure

| S1,F69 |  | Change Glass Count Send | ECSEQP | reply |
| --- | --- | --- | --- | --- |
| Description | This message is used when ECS informs engaged glass’ quantity data to the Equipment. |  |  |  |
| Structure | L,2 - U2[1] <UNITID> - B[1] <GLSCNT> |  |  |  |
| Notice | <GLSCNT> Total glasses count for processing <UNITID> Unit ID |  |  |  |
| Exception |  |  |  |  |

| S1,F70 |  | Change Glass Count Send Acknowledge | ECSEQP |  |
| --- | --- | --- | --- | --- |
| Description | Acknowledge |  |  |  |
| Structure | L,2 1. U2[1]<UNITID> 2. B[1]<ACKC1> |  |  |  |
| Notice | <UNITID> Unit ID <ACKC1> 0 = Accepted 1 = Not Accepted |  |  |  |
| Exception |  |  |  |  |

| S1,F71 |  | Remained Glass Count Request | ECSEQP | reply |
| --- | --- | --- | --- | --- |
| Description | The Equipment requests remained glass count from the lot. |  |  |  |
| Structure | L,3 1.U2[1] <UNITID> 2.B[1] <PORTID> 3.A[16] <LOTID> |  |  |  |
| Notice | <UNITID> Unit ID <PORTID> Port ID <LOTID> Lot ID |  |  |  |
| Exception |  |  |  |  |

| S1,F72 |  | Remained Glass Count Send | ECSEQP |  |
| --- | --- | --- | --- | --- |
| Description | Acknowledge |  |  |  |
| Structure | L,4 1.U2[1] <UNITID> 2.B[1] <PORTID> 3.A[16] <LOTID> 4. B[1] <REMAINGLSCNT> |  |  |  |
| Notice | <UNITID> Unit ID <PORTID> Port ID <LOTID> Lot ID <REMAINGLSCNT> Remained Glass Count of Requested Lot |  |  |  |
| Exception |  |  |  |  |

| S1,F73 |  | Monitoring Lot Status Data Send | ECSEQP | reply |
| --- | --- | --- | --- | --- |
| Description | This message sends monitoring lot status data. |  |  |  |
| Structure | L,2 - U2[1] <UNITID> - A[1] <MLOTSTATUS> |  |  |  |
| Notice | <UNITID> Unit ID <MLOTSTATUS> Monitoring Lot Status S = Start / E = End |  |  |  |
| Exception |  |  |  |  |

| S1,F74 |  | Monitoring Lot Status Data Send Acknowledge | ECSEQP |  |
| --- | --- | --- | --- | --- |
| Description | Acknowledge |  |  |  |
| Structure | L,2 1. U2[1]<UNITID> 2. B[1]<ACKC1> |  |  |  |
| Notice | <UNITID> Unit ID <ACKC1> 0 = Accepted 1 = Not Accepted |  |  |  |
| Exception |  |  |  |  |

| S1,F77 |  | Equipment’s Part Data Report | ECSEQP | reply |
| --- | --- | --- | --- | --- |
| Description | This message is used for tracking of glasses which moved in among sub units |  |  |  |
| Structure | L,n (n=SubUnit count) 1.L,3 1.U2[1] <UNITID1> 2.B[1] <SUNITID1> 3.L,m ( m = glass count ) 1.L,2 1.A[6] <CRRID1> 2.U2[1] <GLSCODE1> . . m.L,2 1.A[6] <CRRIDm> 2.U2[1] <GLSCODEm> . . n.L,3 1.U2[1] <UNITIDn> 2.B[1] <SUNITIDn> 3.L,p ( p = glass count ) 1.L,2 1.A[6] <CRRIDn1> 2.U2[1] <GLSCODEn1> . . p.L,2 1.A[6] <CRRIDnp> 2.U2[1]<GLSCODEnp> |  |  |  |
| Notice | <UNITID> Unit ID <SUNITID> Sub Unit ID <CRRID> Carrier ID <GLSCODE> Unique Glass Code defined by ECS. Equipment must keep this code, while glass is in equipment. |  |  |  |
| Exception | A one-to-one correspondence of UNITID and SUNITID depends on Equipment unit configuration |  |  |  |

| S1,F78 |  | Equipment’s Part Data Report Acknowledge | ECSEQP |  |
| --- | --- | --- | --- | --- |
| Description | Acknowledge |  |  |  |
| Structure | L,2 1. U2[1]<UNITID> 2. B[1]<ACKC1> |  |  |  |
| Notice | <UNITID> Unit ID <ACKC1> 0 = Accepted 1 = Not Accepted |  |  |  |
| Exception |  |  |  |  |

| S1,F79 |  | Buffer Lot Status Data Send | ECSEQP | reply |
| --- | --- | --- | --- | --- |
| Description | This message sends Buffer lot status data. |  |  |  |
| Structure | L,3 - U2[1] <UNITID> - A[1] <BLOTTYPE> - A[1] <BLOTSTATUS> |  |  |  |
| Notice | <UNITID> Unit ID <BLOTSTATUS> Buffer Lot Status S = Start / E = End <BLOTTYPE> Buffer Lot Type L = Load to Buffer / U = Unload from Buffer |  |  |  |
| Exception |  |  |  |  |

| S1,F80 |  | Buffer Lot Status Data Send Acknowledge | ECSEQP |  |
| --- | --- | --- | --- | --- |
| Description | Acknowledge |  |  |  |
| Structure | L,2 1. U2[1]<UNITID> 2. B[1]<ACKC1> |  |  |  |
| Notice | <UNITID> Unit ID <ACKC1> 0 = Accepted 1 = Not Accepted |  |  |  |
| Exception |  |  |  |  |

| S1,F81 |  | Port State Data Send | ECSEQP | reply |
| --- | --- | --- | --- | --- |
| Description | This message is used to transfer Port State Data and Map Data |  |  |  |
| Structure | L,7 - U2[1]<UNITID> - A[14] <CSTID> - B[1] <PORTID> - A[2] <PORTSTATE> - A[1] <LOTSTATE> - A[40] <CSTMAP> - A[40] <SELECTEDMAP> |  |  |  |
| Notice | <UNITID> Controller ID <CSTID> CST ID <PORTID> Port ID <PORTSTATE> Port State LC : Cassette Load Complete UC : Cassette Unload Complete LR : Cassette Load Request UR : Cassette Unload Request <LOTSTATE> LOT State R : Ready W : Wait P : Processing E : End G : Going <CSTMAP> Cassette Mapping Information <SELECTEDMAP> Selected glass in the Cassette Map e.g. In case there are 2 glassed in Slot 1 and Slot 40. Then MAP = “1000000000000000000000000000000000000001” - : Glass Not Exist - : Glass Exist |  |  |  |
| Exception | In case of PORTSTATE is UC, MAP is space. |  |  |  |

| S1,F82 |  | Port State Data Send Acknowledge | ECSEQP |  |
| --- | --- | --- | --- | --- |
| Description | Acknowledge |  |  |  |
| Structure | L,2 1.U2[1]<UNITID> 2. B[1] <ACKC1> |  |  |  |
| Notice | <UNITID> Controller ID <ACKC1> 0 = Accepted 1 = Not Accepted |  |  |  |
| Exception |  |  |  |  |

| S1,F83 |  | Port State Data Report | ECSEQP | reply |
| --- | --- | --- | --- | --- |
| Description | This message is used when the State (LR, LC, UR, UC) of Port in Equipment is changed. |  |  |  |
| Structure | L,5 - U2[1] <UNITID> - A[14] <CSTID> - B[1] <PORTID> - A[2] <PORTSTATE> - A[1] <TRSMODE> |  |  |  |
| Notice | <UNITID> Controller ID <CSTID> CST ID <PORTID> Port ID <PORTSTATE> Port State LC : Cassette Load Complete UC : Cassette Unload Complete LR : Cassette Load Request UR : Cassette Unload Request <TRSMODE> Transfer Mode E : PIO Enable D : PIO Disable |  |  |  |
| Exception |  |  |  |  |

| S1,F84 |  | Port State Data Report Acknowledge | ECSEQP |  |
| --- | --- | --- | --- | --- |
| Description | Acknowledge |  |  |  |
| Structure | L,2 1.U2[1]<UNITID> 2. B[1]<ACKC1> |  |  |  |
| Notice | <UNITID> Controller ID <ACKC1> 0 = Accepted 1 = Not Accepted |  |  |  |
| Exception |  |  |  |  |

| S1,F85 |  | Port Status Change Data Report | ECSEQP | reply |
| --- | --- | --- | --- | --- |
| Description | This message is used when the Status (RUN, IDLE, DOWN) of Port in Equipment is changed. |  |  |  |
| Structure | L,2 - U2[1] <UNITID> - L,n [n = the number of Port] 1. L,2 - B[1] <PORTID1> - A[1] <PORTSTATUS1> . . n. L,2 - B[1] <PORTIDn> - A[1] <PORTSTATUSn> |  |  |  |
| Notice | <UNITID> Controller ID <PORTID> Port ID <PORTSTATUS> Port Status R : RUN I : IDLE D : DOWN |  |  |  |
| Exception |  |  |  |  |

| S1,F86 |  | Port Status Change Data Report Acknowledge | ECSEQP |  |
| --- | --- | --- | --- | --- |
| Description | Acknowledge |  |  |  |
| Structure | L,2 1.U2[1]<UNITID> 2. B[1]<ACKC1> |  |  |  |
| Notice | <UNITID> Controller ID <ACKC1> 0 = Accepted 1 = Not Accepted |  |  |  |
| Exception |  |  |  |  |

| S1,F89 |  | Mask Data Report | ECSEQP | reply |
| --- | --- | --- | --- | --- |
| Description | This message is used after the RFID of Mask reading in EQP |  |  |  |
| Structure | L,4 - U2[1] <UNITID> - A[14] <MASKID> - B[1] <PORTID> - L,n [n = the number of MaskSheet] - A[20] <MASKSHEETID1> . . n. A[20] <MASKSHEETIDn> |  |  |  |
| Notice | <UNITID> Controller ID <MASKID> MASK ID <PORTID> Port ID <MASKSHEETID> MaskSheet ID |  |  |  |
| Exception |  |  |  |  |

| S1,F90 |  | Mask Data Report Acknowledge | ECSEQP |  |
| --- | --- | --- | --- | --- |
| Description | Acknowledge |  |  |  |
| Structure | L,2 1.U2[1]<UNITID> 2. B[1]<ACKC1> |  |  |  |
| Notice | <UNITID> Controller ID <ACKC1> 0 = Accepted 1 = Not Accepted |  |  |  |
| Exception |  |  |  |  |

| S1,F91 |  | All Mask Position Data Report | ECSEQP | reply |
| --- | --- | --- | --- | --- |
| Description | This message is used for tracking of glasses which moved in among sub units |  |  |  |
| Structure | L,n [n=SubUnit count] 1.L,4 1.U2[1] <UNITID1> 2.B[1] <SUNITID1> 3.A[14]<MASKID1> 4.L,m [ m = masksheet count ] 1.A[20] <MASKSHEETID1> . . m.A[20]<MASKSHEETIDm> . . n.L,4 1.U2[1] <UNITID1> 2.B[1] <SUNITID1> 3.A[14]<MASKID1> 4.L,m [ m = masksheet count ] 1.A[20] <MASKSHEETID1> . . m.A[20]<MASKSHEETIDm> |  |  |  |
| Notice | <UNITID> Unit ID <SUNITID> Sub Unit ID <MASKID> Mask ID <MASKSHEETID>Masksheet ID |  |  |  |
| Exception | A one-to-one correspondence of UNITID and SUNITID depends on Equipment unit configuration |  |  |  |

| S1,F92 |  | All Mask Position Data Report Acknowledge | ECSEQP |  |
| --- | --- | --- | --- | --- |
| Description | Acknowledge |  |  |  |
| Structure | B[1]<ACKC1> |  |  |  |
| Notice | <ACKC1> 0 = Accepted 1 = Not Accepted |  |  |  |
| Exception |  |  |  |  |

| S2,F71 |  | Carrier Transfer Mode Change Report | ECSEQP | reply |
| --- | --- | --- | --- | --- |
| Description | This message is reported by equipment, when carrier transfer mode is changed |  |  |  |
| Structure | L,2 - U2[1] <UNITID> - A[1] <CTMODE> |  |  |  |
| Notice | <UNITID> Unit ID <CTMODE> Carrier Transfer Mode The following CTMODE values are defined. 1 = Normal Transferring 2 = Batch Transferring |  |  |  |
| Exception |  |  |  |  |

| S2,F72 |  | Carrier Transfer Mode Change Report Acknowledge | ECSEQP |  |
| --- | --- | --- | --- | --- |
| Description | Acknowledge |  |  |  |
| Structure | L,2 1. U2[1]<UNITID> 2. B[1]<ACKC2> |  |  |  |
| Notice | <UNITID>Unit ID <ACKC2> 0 = Accepted >0 = Error, not accepted 1 – 63 Reserved |  |  |  |
| Exception |  |  |  |  |

| S2,F77 |  | Glass Call Request | ECSEQP | reply |
| --- | --- | --- | --- | --- |
| Description | This message is used when operator wants to inject glass into the process. |  |  |  |
| Structure | L,3 - U2[1] <UNITID> - B[1] <STAGEID> - U2[1] <TARGETID> |  |  |  |
| Notice | <UNITID> Unit ID <STAGEID>Stage ID <TARGETID> Target Unit ID |  |  |  |
| Exception |  |  |  |  |

| S2,F78 |  | Glass Call Request Acknowledge | ECSEQP |  |
| --- | --- | --- | --- | --- |
| Description | Acknowledge |  |  |  |
| Structure | L,2 1. U2[1]<UNITID> 2. B[1]<ACKC2> |  |  |  |
| Notice | <UNITID>Unit ID <ACKC2> 0 = Accepted >0 = Error, not accepted 1 – 63 Reserved |  |  |  |
| Exception |  |  |  |  |

| S2,F81 |  | Lost Glass Data Request | ECSEQP | reply |
| --- | --- | --- | --- | --- |
| Description | The Equipment only can use this control sequence in its manual operation. It can be use when equipment lost glass data, but equipment still have Glass ID or Glass code. |  |  |  |
| Structure | L,2 - U2[1]<UNITID> - A[16] <GLSID> - U2[1] <GLSCODE> - A[1] <REQOPTION> |  |  |  |
| Notice | <UNITID> Unit ID <GLSID> Glass ID <REQOPTION> Request Option C : Glass Code I : Glass ID A : Glass Code and Glass ID |  |  |  |
| Exception |  |  |  |  |

| S2,F82 |  | Glass Data Send | ECSEQP |  |
| --- | --- | --- | --- | --- |
| Description | The Equipment (manual operation) sends S2F81 and then ECS sends ‘Glass Load Data’ as reply. ‘Glass Load Data’ is data about injected glass when glass inject to equipment. |  |  |  |
| Structure | L, 3 1. U2[1] <UNITID> 2. B[1]<ACKC2> - L,n [ n = 0 ( Glass Data Not Exist ) or 1 ( Glass Data Exist ) ] 1. L,21 - A[16] <LOTID> - A[14] <CSTID> - A[3] <SLOTID> - A[24] <RECIPEID> - A[8] <PRCID> - A[1] <GLSODR> - A[16] <GLSID> - A[2] <GLSJUDGE> 9. A[2000] <PNLIF> - A[2000] <SUBMDLIF> - B[1] <PORTID> - U2[1] <GLSCODE> - B[1] <REINPUT> - A[5] <GLSTHICK> - A[14] <PARTNUM> - A[12] <PRODTYPE> - A[12] <ATTRIBUTE> - A[1] <GLSTYPE> - A[32]<KEYID> - L, p [ p = the number of LSD] 1. L,2 - A[24] <LSDITEM1> - A[40] <LSDVALUE1> : p. L,2 - A[24] <LSDITEMp> - A[40] <LSDCALUEp> - L,q [ q = the number of GSD] 1.L,2 - A[24] <GSDITEM11> - A[40] <GSDVALUE11> : q. L,2 - A[24] <GSDITEM1q> - A[40] <GSDVALUE1q> |  |  |  |
| Exception |  |  |  |  |

| S2,F83 |  | Injecting Glass Data Request | ECSEQP | reply |
| --- | --- | --- | --- | --- |
| Description | This message is used |  |  |  |
| Structure | L,2 - U2[1] <UNITID> - B[1] <STAGEID> |  |  |  |
| Notice | <UNITID> Unit ID <STAGEID> Stage ID |  |  |  |
| Exception |  |  |  |  |

| S2,F84 |  | Injecting Glass Data Request Acknowledge | ECSEQP |  |
| --- | --- | --- | --- | --- |
| Description | Data of Injecting Glass |  |  |  |
| Structure | L, 4 - U2[1] <UNITID> - B[1] <STAGEID> - A[1]<GLSSTATUS> - L,n [ n = 0 ( GLSSTATUS = 0 ) or 1 (GLSSTATUS = 1 ) ] 1. L,23 - A[16] <LOTID> - U2[1] <LOTNUMBER> - A[14] <CSTID> - A[3] <SLOTID> - A[24] <RECIPEID> - A[8] <PRCID> - A[1] <GLSODR> - A[16] <GLSID> - A[2] <GLSJUDGE> 10. A[2000] <PNLIF> - A[2000] <SUBMDLIF> - B[1] <PORTID> - U2[1] <GLSCODE> - B[1] <REINPUT> - A[5] <GLSTHICK> - A[14] <PARTNUM> - A[16] <PRODTYPE> - A[12] <ATTRIBUTE> - A[1] <GLSTYPE> - A[32]<KEYID> - L,p [ p = the number of SUBGLS] 1. L, 3 - A[16] <SUBGLSID1> - A[2] <SUBGLSJUDGE1> - A[2000] <SUBPNLIF1> : p. L, 3 - A[16] <SUBGLSIDp> - A[2] <SUBGLSJUDGEp> - A[2000] <SUBPNLIFp> - L, q [ q = the number of LSD] 1. L,2 - A[24] <LSDITEM1> - A[40] <LSDVALUE1> : |  |  |  |

| Structure | : |
| --- | --- |
|  | q. L,2 |
|  | 1. A[24] <LSDITEMq> |
|  | 2. A[40] <LSDCALUEq> |
|  | 23. L,r [ r = the number of GSD] |
|  | 1.L,2 |
|  | 1. A[24] <GSDITEM1> |
|  | 2. A[40] <GSDVALUE1> |
|  | : |
|  | r. L,2 |
|  | 1. A[24] <GSDITEMr> |
|  | 2. A[40] <GSDVALUEr> |
| Notice | <UNITID> Unit ID <STAGEID> Stage ID <GLSSTATUS> Status of the glass that would be loaded 0 = Not Exist 1 = Exist <LOTID> LOT ID <LOTNUMBER> Lot Unique Number <CSTID> Cassette ID <SLOTID> Slot ID <RECIPEID> Recipe ID <PRCID> Processing Code <GLSODR> Glass Order 0 = The first glass 1 = All glasses except the first glass and the last glass 2 = The Last glass( In this case, this value may not be delivered for Scrap or reason of LOT Abort ) 3 = the only glass, when there is just one in a LOT. <GLSID> Glass ID <GLSJUDGE> Glass Grade Code <PNLIF> Panel Judge Information <SUBMDLIF> Sub Panel Information <SUBGLSID> Sub Glass ID <SUBGLSJUDGE> Sub Glass Grade Code <SUBPNLIF> Sub Panel Information <PORTID> Port ID <GLSCODE> Glass ID Code. <REINPUT> Glass Re-input Flag 0 = Not Re-input, 1 = Re-input <GLSTHICK> Glass’s Thickness <PARTNUM> Part Number <PRODTYPE> Type of Producing <ATTRIBUTE> LOT Attribute <GLSTYPE> Glass Type N = Normal Glass D = Dummy Glass <KEYID>Glass Key ID <LSDITEM> LOT Specific Data Item defined by LG Display <LSDVALUE> LOT Specific Data Value defined by LG Display <GSDITEM> Glass Specific Data Item defined by LG Display <GSDVALUE> Glass Specific Data Value defined by LG Display |
| Exception |  |

| S2,F85 |  | Unload Process Report | ECSEQP | reply |
| --- | --- | --- | --- | --- |
| Description | Change Unload Process status of the equipment |  |  |  |
| Structure | L,3 - U2[1] <UNITID> - A[1] <UNLOADSTATUS> - A[1] <UNLOADTYPE> |  |  |  |
| Notice | <UNITID>Controller ID <UNLOADSTATUS> Unload Status S : Start E : End <UNLOADTYPE> Unload Type A : EQP Auto Report M : Operator Manual Report |  |  |  |
| Exception |  |  |  |  |

| S2,F86 |  | Unload Process Report Acknowledge | ECSEQP |  |
| --- | --- | --- | --- | --- |
| Description | Acknowledge |  |  |  |
| Structure | L,2 1. U2[1]<UNITID> 2. B[1]<ACKC6> |  |  |  |
| Notice | <UNITID> Controller ID <ACKC2> 0 = Accepted 1 = Not Accepted 2 ~ 63 Reserved |  |  |  |
| Exception |  |  |  |  |

| S2,F91 |  | Batch Job(Glass Injection) End Data Send | ECSEQP | reply |
| --- | --- | --- | --- | --- |
| Description | This Message is used when the last glass of Batch Job was injected in Equipment. |  |  |  |
| Structure | L,6 - U2[1] <UNITID> - A[1] <BATCHFLAG> - A[16] <LOTID> - U2[1] <LOTNUMBER> - A[14] <CSTID> - B[1]<PORTID> |  |  |  |
| Notice | <UNITID> Unit ID <BATCHFLAG> Batch Flag I : Injection End A : Lot Aborted C : Lot Cancel <LOTID> LOT ID <LOTNUMBER> Lot Unique Number <PORTID> Port ID <CSTID> Cassette ID <RECIPEID> Recipe ID <SLOTID> Slot ID <GLSID> Glass ID <PORTID> Port ID |  |  |  |
| Exception |  |  |  |  |
| S2,F92 |  | Batch Job(Glass Injection) End Data Send Acknowledge | ECSEQP |  |
| Description | Acknowledge |  |  |  |
| Structure | L,2 1.U2[1]<UNITID> 2.B[1]<ACKC2> |  |  |  |
| Notice | <UNITID> Unit ID <ACKC2> 0 = Accepted 1 = Not Accepted 2 – 63 Reserved |  |  |  |
| Exception |  |  |  |  |

| S2,F93 |  | Various Data Report | ECSEQP | reply |
| --- | --- | --- | --- | --- |
| Description | This message is used in various data report to ECS. |  |  |  |
| Structure | L,3 1. U2[1] <UNITID> 2. U2[1] <CEID> - L,n [ n = Number of slot ] 1. L,7 - A[48] <SLOTID1> 2. A[48] <GLSID1> 3. A[48] <GLSJUDGE1> 4. A[2000] <PNLIF11> 5. A[2000] <PNLIF21> 6. A[2000] <SUBMDLIF11> 7. A[2000] <SUBMDLIF21> . . . n. L,7 - A[48] <SLOTIDn> - A[48] <GLSIDn> - A[48] <GLSJUDGEn> 4. A[2000] <PNLIF1n> 5. A[2000] <PNLIF2n> - A[2000] <SUBMDLIF1n> - A[2000] <SUBMDLIF2n> |  |  |  |
| Notice | <UNITID> Unit ID <CEID> Collected Event ID <SLOTID> Slot ID <GLSID> Glass ID <GLSJUDGE> Glass Judge <PNLIF1> Panel Judge of panle1 ~ panel2000 <PNLIF2> Panel Judge of panle2001 ~ panel4000 <SUBMDLIF1> Sub Panel Judge of sub panle1 ~ sub pane2000 <SUBMDLIF2> Sub Panel Judge of sub panle2001 ~ sub pane4000 |  |  |  |
| Exception |  |  |  |  |

| S2,F94 |  | Various Data Report Acknowledge | ECSEQP |  |
| --- | --- | --- | --- | --- |
| Description | Acknowledge |  |  |  |
| Structure | L,3 1. U2[1] <UNITID> 2. U2[1] <CEID> 3. B[1] <ACKC2> |  |  |  |
| Notice | <UNITID> Unit ID <CEID> Collected Event ID <ACKC2> Acknowledge code - : Accepted - : Not Accepted > 1 : Error |  |  |  |
| Exception |  |  |  |  |

| S2,F99 |  | Equipment PD Down Request | ECSEQP | reply |
| --- | --- | --- | --- | --- |
| Description | This message is used when requesting the PD Down state change. |  |  |  |
| Structure | L,3 - U2[1] <UNITID> - A[6] <RCMD> - A[120] <HOSTMSG> |  |  |  |
| Notice | <UNITID>Unit ID <RCMD>Remote command code or string – PAUSE / RESUME <HOSTMSG> Host message in Korean and English |  |  |  |
| Exception |  |  |  |  |

| S2,F100 |  | Equipment PD Down Request Acknowledge | ECSEQP |  |
| --- | --- | --- | --- | --- |
| Description | Acknowledge |  |  |  |
| Structure | L,2 1. U2[1]<UNITID> 2. B[1]<ACKC6> |  |  |  |
| Notice | <UNITID> Unit ID <ACKC6> 0 = Accepted 1 = Not Accepted 2 = Already Release 3~ 63 Reserved |  |  |  |
| Exception |  |  |  |  |

| S6,F81 |  | Lot State Change Report | ECSEQP | reply |
| --- | --- | --- | --- | --- |
| Description | This message is used when the Lot State (R, W, P, E, G) of port in Equipment is changed. |  |  |  |
| Structure | L,5 - U2[1] <UNITID> - A[14] <CSTID> - A[24] <RECIPEID> - B[1] <PORTID> - A[1] <LOTSTATE> |  |  |  |
| Notice | <UNITID> Controller ID <CSTID> CST ID <RECIPEID> Recipe ID <PORTID> Port ID <LOTSTATE> LOT State R : Ready W : Wait P : Processing E : End |  |  |  |
| Exception |  |  |  |  |

| S6,F82 |  | Lot State Change Report Acknowledge | ECSEQP |  |
| --- | --- | --- | --- | --- |
| Description | Acknowledge |  |  |  |
| Structure | L,2 1.U2[1]<UNITID> 2. B[1]<ACKC1> |  |  |  |
| Notice | <UNITID> Controller ID <ACKC1> 0 = Accepted 1 = Not Accepted |  |  |  |
| Exception |  |  |  |  |

| S6,F97 |  | Chamber Status Change Report | ECSEQP | reply |
| --- | --- | --- | --- | --- |
| Description | This message is used in Chamber Status Change Report. |  |  |  |
| Structure | L,n [n = the number of Chamber] 1. L,2 - U2[1] <UNITID1> - A[1] <CMSTATUS1> . . n. L,2 - U2[1] <UNITIDn> - A[1] <CMSTATUSn> |  |  |  |
| Notice | <UNITID> Unit ID <CMSTATUS> Chamber Status N : Normal / D : Dummy / S : Self Cleaning / E : Empty Discharge / X : No Use |  |  |  |
| Exception |  |  |  |  |

| S6,F98 |  | Chamber Status Change Report Acknowledge | ECSEQP |  |
| --- | --- | --- | --- | --- |
| Description | Acknowledge |  |  |  |
| Structure | L,n [n = the number of Chamber] 1. L,2 1. U2[1] <UNITID1> 2. B[1] < ACKC6 > . . n. L,2 1. U2[1] <UNITIDn> 2. B[1] < ACKC6 > |  |  |  |
| Notice | <UNITID>Unit ID <ACKC6> 0 = Accepted 1 = Not Accepted 2 ~ 63 Reserved |  |  |  |
| Exception |  |  |  |  |

| S7,F79 |  | Equipment Constant Recipe Request | ECSEQP | reply |
| --- | --- | --- | --- | --- |
| Description | Equipment Constant Recipe Request |  |  |  |
| Structure | L, 6 1.A[16]<LOTID> 2.A[14]<CSTID > 3.A [24]<RECIPEID> 4.A [8]<PRCID > 5.B [1]<PORTID> 6.U2[1]<UNITID> |  |  |  |
| Notice | <LOTID> LOT ID <CSTID> Cassette ID <RECIPEID> Recipe ID <PRCID> Processing Code <PORTID> Port ID <UNITID> Unit ID |  |  |  |
| Exception |  |  |  |  |

| S7,F80 |  | Equipment Constant Recipe | ECSEQP |  |
| --- | --- | --- | --- | --- |
| Description | This message reports the Equipment Constant Recipe parameter of the equipment |  |  |  |
| Structure | L,8 - A[16] <LOTID> - A[14]<CSTID> - A[24]<RECIPEID> - A[12]<RECIPEIDVER> - A[8]<PRCID> - B[1]<PORTID> - U2[1]<UNITID> - L,m [ m = the number of Unit] 1.L.6 - U2[1] <EUNITID1> - A[24] <LOWERRECIPEID1> - A[12] <LOWERRECIPEIDVER1> - A[1]<LEVEL1> - A[1]<RECIPEIDTYPE1> - L,p [ p = Number of Step ] 1. L,2 1. A[14] <STEP11> - L, n [n = Number of Parameter] 1. L,2 - A[24] <RPARMNAME111> - A[24] <RPARMVAL111> : n.L,2 - A[24] <RPARMNAME11n> - A[24] <RPARMVAL11n> : p. L,2 1. A[14] <STEP1p> - L, q [q = Number of Parameter] 1. L,2 - A[24] <RPARMNAME1p1> - A[24] <RPARMVAL1p1> : q.L,2 - A[24] <RPARMNAME1pq> - A[24] <RPARMVAL1pq> : |  |  |  |

|  | : m.L,6 - U2[1] <EUNITIDm> - A[24] <LOWERRECIPEIDm> - A[12] <LOWERRECIPEIDVERm> - A[1]<LEVELm> - A[1]<RECIPEIDTYPEm> - L,r [ r = Number of Step ] 1. L,2 1. A[14] <STEPm1> - L, s [s = Number of Parameter] 1. L,2 - A[24] <RPARMNAMEm11> - A[24] <RPARMVALm11> : s.L,2 - A[24] <RPARMNAMEm1s> - A[24] <RPARMVALm1s> : r. L,2 - A[14] <STEPmr> - L, t [t = Number of Parameter] 1. L,2 - A[24] <RPARMNAMEmr1> - A[24] <RPARMVALmr1> : t.L,2 - A[24] <RPARMNAMEmrt> - A[24] <RPARMVALmrt> |
| --- | --- |
| Notice | <LOTID> LOT ID <CSTID> Cassette ID <PRCID> Processing Code <RECIPEID> Recipe ID <PORTID> Port ID <UNITID> Unit ID <EUNITID> Each Unit ID <LOWERRECIPEID> Lower Recipe ID <LOWERRECIPEIDVER> Lower Recipe ID Version <LEVEL> Level of RECIPEID 1 : Level 1 RECIPEID 2 : Level 2 RECIPEID 3 : Level 3 RECIPEID < RPARMNAME> Recipe Parameter Name < RPARMVAL> Recipe Parameter Value <RECIPEIDTYPE>Type of Recipe E : Equipment Recipe / M : Machine Recipe / U : Unit Recipe 1,2,3,.. : Special Recipe (Defined with LG Display) <STEP> Recipe Step 0 = Step Not Used, Another Number is Recipe Step |
| Exception |  |

| S7,F81 |  | Multi Recipe Information Send | ECS  EQP | reply |
| --- | --- | --- | --- | --- |
| Description | This message is used to send each slot recipe ID to EQP. |  |  |  |
| Structure | L,5 - U2[1]<UNITID> - B[1]<PORTID> - A[16]<LOTID> - U2[1] <LOTNUMBER> 5.L,n ( n= The Number of slot ) 1.L,4 1.A[3]<SLOTID> 2.B[1]<SELECTED> 3.A[24]<RECIPEID> 4.B[1]<SAMPLECODE> : n.L,4 1.A[3]<SLOTID> 2.B[1]<SELECTED> 3.A[24]<RECIPEID> 4.B[1]<SAMPLECODE> |  |  |  |
| Notice | <UNITID> Controller ID <PORTID> Port ID <LOTID> Lot ID <LOTNUMBER> Lot Unique Number <SLOTID> Slot ID <SELECTED> Glass Injection Flag 0 : Not Injection 1 : Injection <RECIPEID> Recipe ID <SAMPLECODE> Glass Inspection flag 0 : Not Inspection 1 : Inspection |  |  |  |
| Exception |  |  |  |  |

| S7,F82 |  | Multi Recipe Information Send Acknowledge | ECS  EQP |  |
| --- | --- | --- | --- | --- |
| Description | Acknowledge |  |  |  |
| Structure | L,2 1.U2[1]<UNITID> 2.B[1]<ACKC7> |  |  |  |
| Notice | <UNITID> Controller ID <ACKC7> Acknowledge code, 1byte 0 = Accepted 1 = Not Accepted 2 ~ 63 Reserved |  |  |  |
| Exception |  |  |  |  |

| S15,F65 |  | Current Recipe ID List Request | ECSEQP | reply |
| --- | --- | --- | --- | --- |
| Description | This message is used to request the transmission of the current equipment process program directory. This is the list of all the RECIPE IDs & Recipe ID Version of the process programs stored in the equipment |  |  |  |
| Structure | U2[1] <UNITID> |  |  |  |
| Notice | <UNITID> Unit ID |  |  |  |
| Exception |  |  |  |  |

| S15,F66 |  | Current Recipe ID List Request Acknowledge | ECSEQP |  |
| --- | --- | --- | --- | --- |
| Description | This message is used to transmit the current Recipe ID List |  |  |  |
| Structure | L,2 - U2[1] <UNITID> - L,n [ n = Recipe List Count ] 1.L,3 - A[24] <RECIPEID1> - A[1] <RECIPEIDTYPE1> - L,p [p = Number of Recipe ID Version] - A[12]<RECIPEIDVER11> : n. A[12]<RECIPEIDVER1p> : n.L,3 - A[24] <RECIPEIDn> - A[1] <RECIPEIDTYPEn> - L,q [q = Number of Recipe ID Version] - A[12]<RECIPEIDVERn1> . . q. A[12]<RECIPEIDVERnq> |  |  |  |
| Notice | <UNITID> Unit ID <RECIPEID> Recipe ID <RECIPEIDTYPE>Type of Recipe E : Equipment Recipe / M : Machine Recipe / U : Unit Recipe 1,2,3,.. : Special Recipe (Defined with LG Display) <RECIPEIDVER> Recipe ID Version |  |  |  |
| Exception |  |  |  |  |

| S15,F71 |  | Recipe List Report Request | ECSEQP | reply |
| --- | --- | --- | --- | --- |
| Description | This message is used when ECS requests recipe list of the equipment |  |  |  |
| Structure | L,4 - U2[1] <UNITID> - A[1] <LEVEL> - A[1] <RECIPEIDTYPE> - L,m [ m = the number of LSD ] 1. L,2 - A[24] <ESDITEM1> - A[40] <ESDVALUE1> : m. L,2 1.A[24] <ESDITEMm> 2.A[40] <ESDVALUEm> |  |  |  |
| Notice | <UNITID> Unit ID <LEVEL> Level of RECIPEID 1 : Level 1 RECIPEID 2 : Level 2 RECIPEID 3 : Level 3 RECIPEID <RECIPEIDTYPE>Type of Recipe E : Equipment Recipe / M : Machine Recipe / U : Unit Recipe 1,2,3,.. : Special Recipe (Defined with LG Display) <ESDITEM> EQP Specific Data Item defined by LG Display <ESDVALUE> EQP Specific Data Value defined by LG Display |  |  |  |
| Exception |  |  |  |  |

| S15,F72 |  | Equipment Recipe List Report | ECS EQP |  |
| --- | --- | --- | --- | --- |
| Description | Acknowledge |  |  |  |
| Structure | L,2 - L, m [ m = Number of RECIPEID ] 1. L,5 - U2[1] <UNITID1> - A[24] <RECIPEID1> - A[1] <LEVEL1> - A[1] <RECIPEIDTYPE1> - L, p [ p = Number of version ] 1. L, 2 - A[12] <RECIPEIDVER11> - A[1] <RECIPEIDSTATUS11> : p. L, 2 - A[12] <RECIPEIDVER1p> - A[1] <RECIPEIDSTATUS1p> : m. L,5 - U2[1] <UNITIDm> - A[24] <RECIPEIDm> - A[1] <LEVELm> - A[1] <RECIPEIDTYPEm> - L, q [ q = Number of version ] 1. L, 2 - A[12] <RECIPEIDVERm1> - A[1] <RECIPEIDSTATUSm1> : q. L, 2 - A[12] <RECIPEIDVERmq> - A[1] <RECIPEIDSTATUSmq> - L, r [ r = Number of ESD ] 1. L,2 - A[24] <ESDITEM1> - A[40] <ESDVALUE1> : r. L,2 - A[24] <ESDITEMr> - A[40] <ESDVALUEr> |  |  |  |
| Notice | <UNITID> Unit ID <RECIPEIDTYPE>Type of Recipe E : Equipment Recipe / M : Machine Recipe / U : Unit Recipe 1,2,3,.. : Special Recipe (Defined with LG Display) <RECIPEID> Recipe ID <LEVEL> Level of Recipe ID <RECIPEIDVER> Recipe ID Version <RECIPEIDSTATUS> Recipe ID Status R : Running (using) I : Idle (Editable) D : Disable (Locked) |  |  |  |
| Exception |  |  |  |  |

| Notice | <ESDITEM> EQP Specific Data Item defined by LG Display <ESDVALUE> EQP Specific Data Value defined by LG Display |
| --- | --- |
| Exception |  |

| S15,F73 |  | Lower Recipe List Report Request | ECSEQP | reply |
| --- | --- | --- | --- | --- |
| Description | This message is used when ECS requests lower recipe list of the equipment |  |  |  |
| Structure | L,2 |  |  |  |
|  | 1. L, 5 |  |  |  |
|  | 1. U2[1] <UNITID> |  |  |  |
|  | 2. A[24] <RECIPEID> |  |  |  |
|  | 3. A[1] <LEVEL> |  |  |  |
|  | 4. A[1] <RECIPEIDTYPE> |  |  |  |
|  | 5. A[12] <RECIPEIDVER> |  |  |  |
|  | 2. L,m [ m = the number of ESD ] |  |  |  |
|  | 1. L,2 |  |  |  |
|  | 1. A[24] <ESDITEM1> |  |  |  |
|  | 2. A[40] <ESDVALUE1> |  |  |  |
|  | : |  |  |  |
|  | m. L,2 |  |  |  |
|  | 1.A[24] <ESDITEMm> |  |  |  |
|  | 2.A[40] <ESDVALUEm> |  |  |  |
| Notice | <UNITID> Unit ID <RECIPEID> Recipe ID <LEVEL> Level of Recipe ID <RECIPEIDVER> Recipe ID Version <RECIPEIDTYPE>Type of Recipe E : Equipment Recipe / M : Machine Recipe / U : Unit Recipe 1,2,3,.. : Special Recipe (Defined with LG Display) <ESDITEM> EQP Specific Data Item defined by LG. Display <ESDVALUE> EQP Specific Data Value defined by LG. Display |  |  |  |
| Exception |  |  |  |  |

| S15,F74 |  | Lower Recipe List Report | ECS EQP |  |
| --- | --- | --- | --- | --- |
| Description | Acknowledge |  |  |  |
| Structure | L,2 |  |  |  |
|  | 1. L, m [ m = Number of RECIPEID ] |  |  |  |
|  | 1. L, 5 |  |  |  |
|  | 1. U2[1] <UNITID1> |  |  |  |
|  | 2. A[24] <RECIPEID 1> |  |  |  |
|  | 3. A[1] <LEVEL1> |  |  |  |
|  | 4. A[1] <RECIPEIDTYPE> |  |  |  |
|  | 5. L, p [ p = Number of version ] |  |  |  |
|  | 1. A[12] <RECIPEIDVER11> |  |  |  |
|  | : |  |  |  |
|  | p. A[12] <RECIPEIDVER1p> |  |  |  |
|  | : |  |  |  |
|  | m. L, 5 |  |  |  |
|  | 1. U2[1] <UNITIDm> |  |  |  |
|  | 2. A[24] <RECIPEIDm> |  |  |  |
|  | 3. A[1] <LEVELm> |  |  |  |
|  | 4. A[1] <RECIPEIDTYPEm> |  |  |  |
|  | 5. L, q [ q = Number of version ] |  |  |  |
|  | 1. A[12] <RECIPEIDVERm1> |  |  |  |
|  | : |  |  |  |
|  | q. A[12] <RECIPEIDVER1mq> |  |  |  |
|  | 2. L, r [ r = Number of ESD ] |  |  |  |
|  | 1. L,2 |  |  |  |
|  | 1. A[24] <ESDITEM1> |  |  |  |
|  | 2. A[40] <ESDVALUE1> |  |  |  |
|  | : |  |  |  |
|  | r. L,2 |  |  |  |
|  | 1. A[24] <ESDITEMr> |  |  |  |
|  | 2. A[40] <ESDVALUEr> |  |  |  |
| Notice | <UNITID> Unit ID <RECIPEID> Recipe ID <LEVEL> Level of Recipe ID <RECIPEIDVER> Recipe ID Version <RECIPEIDTYPE>Type of Recipe E : Equipment Recipe / M : Machine Recipe / U : Unit Recipe 1,2,3,.. : Special Recipe (Defined with LG Display) <ESDITEM> LOT Specific Data Item defined by LG. Display <ESDVALUE> LOT Specific Data Value defined by LG. Display |  |  |  |
| Exception |  |  |  |  |

| S15,F75 |  | Recipe Parameter Report Request | ECSEQP | reply |
| --- | --- | --- | --- | --- |
| Description | -This message is used when ECS requests recipe parameter of the equipment |  |  |  |
| Structure | L,6 |  |  |  |
|  | 1. U2[1] <UNITID> |  |  |  |
|  | 2. A[24] <RECIPEID> |  |  |  |
|  | 3. A[1] <LEVEL> |  |  |  |
|  | 4. A[1] <RECIPEIDTYPE> |  |  |  |
|  | 5. A[12] <RECIPEIDVER> |  |  |  |
|  | 6. L,m [ m = the number of ESD ] |  |  |  |
|  | 1. L,2 |  |  |  |
|  | 1. A[24] <ESDITEM1> |  |  |  |
|  | 2. A[40] <ESDVALUE1> |  |  |  |
|  | : |  |  |  |
|  | m. L,2 |  |  |  |
|  | 1.A[24] <ESDITEMm> |  |  |  |
|  | 2.A[40] <ESDVALUEm> |  |  |  |
| Notice | <UNITID> Unit ID <RECIPEID> Recipe ID <LEVEL> Level of Recipe ID <RECIPEIDVER> Recipe ID Version <RECIPEIDTYPE>Type of Recipe E : Equipment Recipe / M : Machine Recipe / U : Unit Recipe 1,2,3,.. : Special Recipe (Defined with LG Display) <ESDITEM> EQP Specific Data Item defined by LG. Display <ESDVALUE> EQP Specific Data Value defined by LG. Display |  |  |  |
| Exception |  |  |  |  |

| S15,F76 |  | Recipe Parameter Report | ECS EQP |  |
| --- | --- | --- | --- | --- |
| Description | This message reports the Recipe Parameter of the equipment |  |  |  |
| Structure | L,2 |  |  |  |
|  | 1. L, 6 |  |  |  |
|  | 1. U2[1] <UNITID1> |  |  |  |
|  | 2. A[24] <RECIPEID1> |  |  |  |
|  | 3. A[1] <LEVEL1> |  |  |  |
|  | 4. A[1] <RECIPEIDTYPE1> |  |  |  |
|  | 5. A[12] <RECIPEIDVER1> |  |  |  |
|  | 6. L,p [ p = Number of Step ] |  |  |  |
|  | 1. L,2 |  |  |  |
|  | 1. A[14] <STEP1> |  |  |  |
|  | 2. L, n [n = Number of Parameter] |  |  |  |
|  | 1. L,2 |  |  |  |
|  | 1. A[24] <RPARMNAME11> |  |  |  |
|  | 2. A[24] <RPARMVAL11> |  |  |  |
|  | : |  |  |  |
|  | n.L,2 |  |  |  |
|  | 1. A[24] <RPARMNAME1n> |  |  |  |
|  | 2. A[24] <RPARMVAL1n> |  |  |  |
|  | : |  |  |  |
|  | p. L,2 |  |  |  |
|  | 1. A[14] <STEPp> |  |  |  |
|  | 2. L, q [q = Number of Parameter] |  |  |  |
|  | 1. L,2 |  |  |  |
|  | 1. A[24] <RPARMNAMEp1> |  |  |  |
|  | 2. A[24] <RPARMVALp1> |  |  |  |
|  | : |  |  |  |
|  | q.L,2 |  |  |  |
|  | 1. A[24] <RPARMNAMEpq> |  |  |  |
|  | 2. A[24] <RPARMVALpq> |  |  |  |
|  | 2. L, s [ s = Number of ESD ] |  |  |  |
|  | 1. L,2 |  |  |  |
|  | 1. A[24] <ESDITEM1> |  |  |  |
|  | 2. A[40] <ESDVALUE1> |  |  |  |
|  | : |  |  |  |
|  | s. L,2 |  |  |  |
|  | 1. A[24] <ESDITEMs> |  |  |  |
|  | 2. A[40] <ESDVALUEs> |  |  |  |
| Notice | <UNITID> Unit ID <RECIPEID> Recipe ID <LEVEL> Level of Recipe ID <RECIPEIDVER> Recipe ID Version <RECIPEIDTYPE>Type of Recipe E : Equipment Recipe / M : Machine Recipe / U : Unit Recipe 1,2,3,.. : Special Recipe (Defined with LG Display) <RPARMNAME> Recipe Parameter Name <RPARMVAL> Recipe Parameter Value <ESDITEM> EQP Specific Data Item defined by LG. Display <ESDVALUE> EQP Specific Data Value defined by LG. Display <STEP> Recipe Step 0 = Step Not Used, Another Number is Recipe Step |  |  |  |

| S15,F77 |  | Recipe Parameter Change Send | ECSEQP | reply |
| --- | --- | --- | --- | --- |
| Description | This message commands Recipe Parameter change to the equipment |  |  |  |
| Structure | L, 8 |  |  |  |
|  | 1. U2[1] <UNITID1> |  |  |  |
|  | 2. A[1] <RECIPEIDCMD1> |  |  |  |
|  | 3. A[24] <RECIPEID1> |  |  |  |
|  | 4. A[1] <LEVEL1> |  |  |  |
|  | 5. A[12] <RECIPEIDVER1> |  |  |  |
|  | 6. A[1] <RECIPEIDTYPE1> |  |  |  |
|  | 7. L,p [ p = Number of Step ] |  |  |  |
|  | 1. L,2 |  |  |  |
|  | 1. A[14] <STEP1> |  |  |  |
|  | 2. L, n [n = Number of Parameter] |  |  |  |
|  | 1. L,2 |  |  |  |
|  | 1. A[24] <RPARMNAME11> |  |  |  |
|  | 2. A[24] <RPARMVAL11> |  |  |  |
|  | : |  |  |  |
|  | n.L,2 |  |  |  |
|  | 1. A[24] <RPARMNAME1n> |  |  |  |
|  | 2. A[24] <RPARMVAL1n> |  |  |  |
|  | : |  |  |  |
|  | p. L,2 |  |  |  |
|  | 1. A[14] <STEPp> |  |  |  |
|  | 2. L, q [q = Number of Parameter] |  |  |  |
|  | 1. L,2 |  |  |  |
|  | 1. A[24] <RPARMNAMEp1 |  |  |  |
|  | 2. A[24] <RPARMVALp1> |  |  |  |
|  | : |  |  |  |
|  | q.L,2 |  |  |  |
|  | 1. A[24] <RPARMNAMEpq> |  |  |  |
|  | 2. A[24] <RPARMVALpq> |  |  |  |
|  | 8. L, r [ r = Number of ESD ] |  |  |  |
|  | 1. L,2 |  |  |  |
|  | 1. A[24] <ESDITEM11> |  |  |  |
|  | 2. A[40] <ESDVALUE11> |  |  |  |
|  | : |  |  |  |
|  | r. L,2 |  |  |  |
|  | 1. A[24] <ESDITEM1r> |  |  |  |
|  | 2. A[40] <ESDVALUE1r> |  |  |  |

| Notice | <UNITID> Unit ID <RECIPEIDCMD> RECIPEID Command B : Built C : change D : Delete R : Rename <RECIPEID> Recipe ID <LEVEL> Level of RECIPEID <RECIPEIDVER> RECIPEID Version <RECIPEIDTYPE>Type of Recipe E : Equipment Recipe / M : Machine Recipe / U : Unit Recipe 1,2,3,.. : Special Recipe (Defined with LG Display) < RPARMNAME> Recipe Parameter Name < RPARMVAL> Recipe Parameter Value <ESDITEM> EQP Specific Data Item defined by LG. Display <ESDVALUE> EQP Specific Data Value defined by LG. Display <STEP> Recipe Step 0 = Step Not Used, Another Number is Recipe Step |
| --- | --- |
| Exception |  |

| S15,F78 |  | Recipe Change Command Acknowledge | ECS EQP |  |
| --- | --- | --- | --- | --- |
| Description | Recipe Parameter change command acknowledge |  |  |  |
| Structure | L, 6 |  |  |  |
|  | 1. U2[1] <UNITID> |  |  |  |
|  | 2. A[24] <RECIPEID> |  |  |  |
|  | 3. A[1] <LEVEL> |  |  |  |
|  | 4. A[12] <RECIPEIDVER> |  |  |  |
|  | 5. A[1] <RECIPEIDTYPE> |  |  |  |
|  | 6. B[1] <REACK> |  |  |  |
| Notice | <UNITID> Unit ID <LEVEL> Level of RECIPEID <RECIPEIDVER> RECIPEID Version <RECIPEIDTYPE>Type of Recipe E : Equipment Recipe / M : Machine Recipe / U : Unit Recipe 1,2,3,.. : Special Recipe (Defined with LG Display) <REACK> - : Accepted - : Permission not granted 2 : RECIPEID using - : Equipment not responding - : Version not found (Roll back) 5 : RECIPEID not found - : Length error - : Parameter range error - : Same name of recipes exist |  |  |  |
| Exception |  |  |  |  |

| S15,F79 |  | Recipe Parameter Change Report | ECS EQP | Reply |
| --- | --- | --- | --- | --- |
| Description | This message reports Recipe Parameter change from equipment |  |  |  |
| Structure | L, 8 - U2[1] <UNITID1> - A[1] <RECIPEIDCMD1> - A[24] <RECIPEID1> - A[1] <LEVEL1> - A[12] <RECIPEIDVER1> - A[1] <RECIPEIDTYPE1> - L,p [ p = Number of Step ] 1. L,2 1. A[14] <STEP1> - L, n [n = Number of Parameter] 1. L,2 - A[24] <RPARMNAME11> - A[24] <RPARMVAL11> : n.L,2 - A[24] <RPARMNAME1n> - A[24] <RPARMVAL1n> : p. L,2 1. A[14] <STEP1p> - L, q [q = Number of Parameter] 1. L,2 - A[24] <RPARMNAMEp1> - A[24] <RPARMVALp1> : q.L,2 - A[24] <RPARMNAMEpq> - A[24] <RPARMVALpq> - L, r [ r = Number of ESD ] 1. L,2 - A[24] <ESDITEM11> - A[40] <ESDVALUE11> : r. L,2 - A[24] <ESDITEM1r> - A[40] <ESDVALUE1r> |  |  |  |

| Notice | <UNITID> Unit ID <RECIPEIDCMD> RECIPEID Command B : Built C : change D : Delete R : Rename <RECIPEID> Recipe ID <LEVEL> Level of RECIPEID <RECIPEIDVER> RECIPEID Version <RECIPEIDTYPE>Type of Recipe E : Equipment Recipe / M : Machine Recipe / U : Unit Recipe 1,2,3,.. : Special Recipe (Defined with LG Display) < RPARMNAME> Recipe Parameter Name < RPARMVAL> Recipe Parameter Value <ESDITEM> EQP Specific Data Item defined by LG. Display <ESDVALUE> EQP Specific Data Value defined by LG. Display <STEP> Recipe Step 0 = Step Not Used, Another Number is Recipe Step |
| --- | --- |
| Exception |  |

| S15,F80 |  | Recipe Change Report Acknowledge | ECS  EQP |  |
| --- | --- | --- | --- | --- |
| Description | Recipe Parameter change acknowledge |  |  |  |
| Structure | L, 6 |  |  |  |
|  | 1. U2[1] <UNITID1> |  |  |  |
|  | 2. A[24] <RECIPEID1> |  |  |  |
|  | 3. A[1] <LEVEL1> |  |  |  |
|  | 4. A[12] <RECIPEIDVER1> |  |  |  |
|  | 5. A[1] <RECIPEIDTYPE1> |  |  |  |
|  | 6. B[1] <PHACK1> |  |  |  |
| Notice | <UNITID> Unit ID <LEVEL> Level of RECIPEID <RECIPEIDVER> RECIPEID Version <RECIPEIDTYPE>Type of Recipe E : Equipment Recipe / M : Machine Recipe / U : Unit Recipe 1,2,3,.. : Special Recipe (Defined with LG Display) <PHACK> - : Accepted - : Permission not granted 2 : Length error 3 : RECIPEID not found 4 : Parameter range error 5 : Saved for additional use |  |  |  |
| Exception |  |  |  |  |

| S15,F81 |  | Equipment Operation Reference Data Send | ECS  EQP | Reply |
| --- | --- | --- | --- | --- |
| Description | This message sends equipment operation reference data |  |  |  |
| Structure | L,6 - U2[1] <UNITID> - A[8] <USEMODE> - A[16] <LOTID> - L,n [ n = Number of Panel Data ] 1. L, 2 - A[288] <LOTPRITEMm1> - A[450] <LOTPRDATAm1> : n. L, 2 - A[288] <LOTPRITEMmn> - A[450] <LOTPRDATAmn> - L, v [ v = Number of LOT DV Item] 1. L, 2 - A[14] <LOTDVNAME1> - A[14] <LOTDVVAL1> : - L, 2 - A[14] <LOTDVNAMEv> - A[14] <LOTDVVALv> - L,m [ m = Number of Glass ] 1. L,5 1. A[16] <GLSID1> - A[24] <RECIPEID1> - A[40] <EQPINFO1> - L, n [ n = Number of Panel Data] 1. L, 2 - A[288] <GLSPRITEM1> - A[450] <GLSPRDATA1> : n. L, 2 - A[288] <GLSPPITEMn> - A[450] <GLSPRDATAn> - L, p [ p = Number of Item ] 1. L, 2 - A[14] <GLSDVNAME11> - A[14] <GLSDVVAL11> : n. L, 2 - A[14] <DVNAME1n> - A[14] <DVVAL1n> : m. L,5 - A[16] <GLSIDm> - A[24] <RECIPEIDm> - A[40] <EQPINFOm> - L, n [ n = Number of Panel Data] 1. L, 2 - A[288] <GLSPRITEMm1> - A[450] <GLSPRDATAm1> : n. L, 2 - A[288] <GLSPRITEMmn> - A[450] <GLSPRDATAmn> - L, p [ p = Number of Item ] 1. L, 2 - A[14] <GLSDVNAMEm1> - A[14] <GLSDVVALm1> : p. L, 2 - A[14] <GLSDVNAMEmp> - A[14] <GLSDVVALmp> |  |  |  |

| Notice | <UNITID> Unit ID <USEMODE> Message Using Mode <RECIPEID> Recipe ID <EQPINFO> Equipment Information <LOTPRITEM> Lot Process Reference Item <LOTPRDATA> Lot Process Reference Data <LOTDVNAME> LOT Data Name <LOTDVVAL> LOT Data Value <GLSPRITEM> Glass Process Reference Item <GLSPRDATA> Glass Process Reference Data <GLSDVNAME> Data Name <GLSDVVAL> Data Value |
| --- | --- |
| Exception |  |

| S15,F82 |  | Equipment Operation Reference Data Send Acknowledge | ECS  EQP |  |
| --- | --- | --- | --- | --- |
| Description | Operation Reference Data Send Acknowledge |  |  |  |
| Structure | L, m [ m = Number of Unit ] |  |  |  |
|  | 1. L, 2 |  |  |  |
|  | 1. U2[1] <UNITID> |  |  |  |
|  | 2. B[1] <DACK> |  |  |  |
|  | : |  |  |  |
|  | m. L, 2 |  |  |  |
|  | 1. U2[1] <UNITID> |  |  |  |
|  | 2. B[1] <DACK> |  |  |  |
| Notice | <UNITID> Unit ID <DACK> - : Accepted - : Accepted and changed parameter 2 : Invalid parameter 3 : Parameter range error 4 : Length error - : Saved for additional use - : Saved for additional use |  |  |  |
| Exception |  |  |  |  |

| S15,F83 |  | Equipment Operation Reference Data Apply Report | ECS  EQP | Reply |
| --- | --- | --- | --- | --- |
| Description | When equipment applied operation reference data, equipment sends this message. |  |  |  |
| Structure | L,4 - U2[1] <UNITID> - A[24] <RECIPEID> - A[16] <LOTID> - B[1] <AACK> |  |  |  |
| Notice | <UNITID> Unit ID <RCP> Recipe <LOTID> LOT ID <AACK> - : Updated - : Error occurred - : RCP not found - : The Recipe is Not Using Now (Request Recipe Unmatched) 4 : Disable Equipment Condition About R2R Request - : Overlapping R2R Request - : One Time Variation Range Over 7 : User Manual Cancel 8 : Parameter Range Error 9 : Invalid Parameter |  |  |  |
| Exception |  |  |  |  |

| S15,F84 |  | Operation Reference Data Send Acknowledge | ECS  EQP |  |
| --- | --- | --- | --- | --- |
| Description | Equipment Operation Reference Data Apply Acknowledge |  |  |  |
| Structure | Header Only |  |  |  |
| Notice |  |  |  |  |
| Exception |  |  |  |  |

| S15,F95 |  | Equipment Constant Recipe Parameter Request | ECSEQP | reply |
| --- | --- | --- | --- | --- |
| Description | -This message is used when ECS requests Equipment Constant Recipe Parameter recipe parameter of the equipment |  |  |  |
| Structure | L,6 - U2[1] <UNITID> - A[24] <RECIPEID> - A[1] <LEVEL> - A[1] <RECIPEIDTYPE> - A[12] <RECIPEIDVER> - L,m [ m = the number of ESD ] 1. L,2 - A[24] <ESDITEM1> - A[40] <ESDVALUE1> : m. L,2 1.A[24] <ESDITEMm> 2.A[40] <ESDVALUEm> |  |  |  |
| Notice | <UNITID> Unit ID <RECIPEID> Recipe ID <LEVEL> Level of Recipe ID <RECIPEIDVER> Recipe ID Version <RECIPEIDTYPE>Type of Recipe E : Equipment Recipe / M : Machine Recipe / U : Unit Recipe 1,2,3,.. : Special Recipe (Defined with LG Display) <ESDITEM> EQP Specific Data Item defined by LG. Display <ESDVALUE> EQP Specific Data Value defined by LG. Display |  |  |  |
| Exception |  |  |  |  |

| S15,F96 |  | Equipment Constant Recipe Parameter Send | ECS EQP |  |
| --- | --- | --- | --- | --- |
| Description | This message reports the Equipment Constant Recipe Parameter of the equipment |  |  |  |
| Structure | L,2 1. L, 6 - U2[1] <UNITID1> - A[24] <RECIPEID1> - A[1] <LEVEL1> - A[1] <RECIPEIDTYPE1> - A[12] <RECIPEIDVER1> - L,p [ p = Number of Step ] 1. L,2 1. A[14] <STEP1> - L, n [n = Number of Parameter] 1. L,2 - A[24] <RPARMNAME11> - A[24] <RPARMVAL11> : n.L,2 - A[24] <RPARMNAME1n> - A[24] <RPARMVAL1n> : p. L,2 - A[14] <STEPp> - L, q [q = Number of Parameter] 1. L,2 - A[24] <RPARMNAMEp1> - A[24] <RPARMVALp1> : q.L,2 - A[24] <RPARMNAMEpq> - A[24] <RPARMVALpq> - L, s [ s = Number of ESD ] 1. L,2 - A[24] <ESDITEM1> - A[40] <ESDVALUE1> : s. L,2 - A[24] <ESDITEMs> - A[40] <ESDVALUEs> |  |  |  |
| Notice | <UNITID> Unit ID <RECIPEID> Recipe ID <LEVEL> Level of Recipe ID <RECIPEIDVER> Recipe ID Version <RECIPEIDTYPE>Type of Recipe E : Equipment Recipe / M : Machine Recipe / U : Unit Recipe 1,2,3,.. : Special Recipe (Defined with LG Display) <ESDITEM> EQP Specific Data Item defined by LG. Display <ESDVALUE> EQP Specific Data Value defined by LG. Display <STEP> Recipe Step 0 = Step Not Used, Another Number is Recipe Step |  |  |  |

| S15,F97 |  | Equipment Constant Recipe Change Command | ECSEQP | reply |
| --- | --- | --- | --- | --- |
| Description | This message commands Equipment Constant Recipe Parameter change to the equipment |  |  |  |
| Structure | L, 8 - U2[1] <UNITID1> - A[1] <RECIPEIDCMD1> - A[24] <RECIPEID1> - A[1] <LEVEL1> - A[12] <RECIPEIDVER1> - A[1] <RECIPEIDTYPE1> - L,p [ p = Number of Step ] 1. L,2 1. A[14] <STEP1> - L, n [n = Number of Parameter] 1. L,2 - A[24] <RPARMNAME11> - A[24] <RPARMVAL11> : n.L,2 - A[24] <RPARMNAME1n> - A[24] <RPARMVAL1n> : p. L,2 - A[14] <STEPp> - L, q [q = Number of Parameter] 1. L,2 - A[24] <RPARMNAMEp1 - A[24] <RPARMVALp1> : q.L,2 - A[24] <RPARMNAMEpq> - A[24] <RPARMVALpq> - L, r [ r = Number of ESD ] 1. L,2 - A[24] <ESDITEM11> - A[40] <ESDVALUE11> : r. L,2 - A[24] <ESDITEM1r> - A[40] <ESDVALUE1r> |  |  |  |

| Notice | <UNITID> Unit ID <RECIPEIDCMD> RECIPEID Command B : Built C : change D : Delete R : Rename <RECIPEID> Recipe ID <LEVEL> Level of RECIPEID <RECIPEIDVER> RECIPEID Version <RECIPEIDTYPE>Type of Recipe E : Equipment Recipe / M : Machine Recipe / U : Unit Recipe 1,2,3,.. : Special Recipe (Defined with LG Display) < RPARMNAME> Recipe Parameter Name < RPARMVAL> Recipe Parameter Value <ESDITEM> EQP Specific Data Item defined by LG. Display <ESDVALUE> EQP Specific Data Value defined by LG. Display <STEP> Recipe Step 0 = Step Not Used, Another Number is Recipe Step |
| --- | --- |
| Exception |  |

| S15,F98 |  | Equipment Constant Recipe Change Command Acknowledge | ECS EQP |  |
| --- | --- | --- | --- | --- |
| Description | Equipment Constant Recipe Change Command Acknowledge |  |  |  |
| Structure | L, 7 - U2[1] <UNITID> - A[24] <RECIPEID> - A[1] <LEVEL> - A[12] <RECIPEIDVER> - A[1] <RECIPEIDTYPE> - B[1] <REACK> - A[180]<EQPMSG> |  |  |  |
| Notice | <UNITID> Unit ID <LEVEL> Level of RECIPEID <RECIPEIDVER> RECIPEID Version <RECIPEIDTYPE>Type of Recipe E : Equipment Recipe / M : Machine Recipe / U : Unit Recipe 1,2,3,.. : Special Recipe (Defined with LG Display) <REACK> - : Accepted - : Permission not granted 2 : RECIPEID using - : Equipment not responding - : Version not found (Roll back) 5 : RECIPEID not found - : Length error - : Parameter range error - : Same name of recipes exist 9 : Invalid parameter 10 : Equipment in Off-line Mode |  |  |  |
| Exception |  |  |  |  |

| S15,F99 |  | Equipment Constant Recipe Change Report | ECS EQP | Reply |
| --- | --- | --- | --- | --- |
| Description | This message reports Equipment Constant Recipe Parameter change from equipment |  |  |  |
| Structure | L, 8 - U2[1] <UNITID1> - A[1] <RECIPEIDCMD1> - A[24] <RECIPEID1> - A[1] <LEVEL1> - A[12] <RECIPEIDVER1> - A[1] <RECIPEIDTYPE1> - L,p [ p = Number of Step ] 1. L,2 1. A[14] <STEP1> - L, n [n = Number of Parameter] 1. L,2 - A[24] <RPARMNAME11> - A[24] <RPARMVAL11> : n.L,2 - A[24] <RPARMNAME1n> - A[24] <RPARMVAL1n> : p. L,2 1. A[14] <STEP1p> - L, q [q = Number of Parameter] 1. L,2 - A[24] <RPARMNAMEp1> - A[24] <RPARMVALp1> : q.L,2 - A[24] <RPARMNAMEpq> - A[24] <RPARMVALpq> - L, r [ r = Number of ESD ] 1. L,2 - A[24] <ESDITEM11> - A[40] <ESDVALUE11> : r. L,2 - A[24] <ESDITEM1r> - A[40] <ESDVALUE1r> |  |  |  |

| Notice | <UNITID> Unit ID <RECIPEIDCMD> RECIPEID Command B : Built C : change D : Delete R : Rename <RECIPEID> Recipe ID <LEVEL> Level of RECIPEID <RECIPEIDVER> RECIPEID Version <RECIPEIDTYPE>Type of Recipe E : Equipment Recipe / M : Machine Recipe / U : Unit Recipe 1,2,3,.. : Special Recipe (Defined with LG Display) < RPARMNAME> Recipe Parameter Name < RPARMVAL> Recipe Parameter Value <ESDITEM> EQP Specific Data Item defined by LG. Display <ESDVALUE> EQP Specific Data Value defined by LG. Display <STEP> Recipe Step 0 = Step Not Used, Another Number is Recipe Step |
| --- | --- |
| Exception |  |

| S15,F100 |  | Equipment Constant Recipe Change Report Acknowledge | ECS  EQP |  |
| --- | --- | --- | --- | --- |
| Description | Equipment Constant Recipe Change Report Acknowledge |  |  |  |
| Structure | L, 7 |  |  |  |
|  | 1. U2[1] <UNITID1> |  |  |  |
|  | 2. A[24] <RECIPEID1> |  |  |  |
|  | 3. A[1] <LEVEL1> |  |  |  |
|  | 4. A[12] <RECIPEIDVER1> |  |  |  |
|  | 5. A[1] <RECIPEIDTYPE1> |  |  |  |
|  | 6. B[1] <PHACK1> |  |  |  |
|  | 7. A[180]<HOSTMSG> |  |  |  |
| Notice | <UNITID> Unit ID <LEVEL> Level of RECIPEID <RECIPEIDVER> RECIPEID Version <RECIPEIDTYPE>Type of Recipe E : Equipment Recipe / M : Machine Recipe / U : Unit Recipe 1,2,3,.. : Special Recipe (Defined with LG Display) <PHACK> - : Accepted - : Permission not granted 2 : Length error 3 : RECIPEID not found 4 : Parameter range error 5 : Saved for additional use |  |  |  |
| Exception |  |  |  |  |

# Attached page

- The Confirming Screen Related to Recipe Command

이 장에서는 Recipe Command와 관련된 장비의 확인 화면에 대해 설명한다. 아래 예를 참고한다.

### * Recipe Creation/Change/Deletion confirmation window

  - 장비 recipe의 생성/변경/삭제를 표시한다. 이 window는 operation confirmation window와는 독립적으로 팝

② RECIPE INFORMATION

RECIPEID

③ OK

① Recipe Created/Changed/Deleted ( From Host/Equipment )

업된다. Recipe의 생성/변경/삭제 확인 window는 5초 후에 자동으로 닫힌다.

| Unit | Recipe | Description |
| --- | --- | --- |

### ① Recipe Creation / Change / Deletion Description

    - equipment message를 표시한다.

### ② Recipe Information

    - equipment recipe의 생성/변경/삭제의 상세

### ③ OK Button

    - 클릭하면 창 종료

### * Recipe Creation/Change/Deletion confirmation window ( denied from Host )

  - 장비 recipe의 생성/변경/삭제를 표시한다. 이 window는 operation confirmation window와는 독립적으로 팝 업된다. Recipe의 생성/변경/삭제 확인 window는 5초 후에 자동으로 닫힌다.

| Unit | Recipe | Description |
| --- | --- | --- |

② RECIPE INFORMATION

RECIPEID

③ OK

① Recipe Creation/Change/Deletion was denied From host ! (Recipe 생성/변경/삭제 실행이 Host로부터 거부되었습니다!)

① Recipe Creation / Change / Deletion Description

    - ‘Description’에는 denial reason / code가 표시된다.

### ② Recipe Information

    - equipment recipe의 생성/변경/삭제의 상세

### ③ OK Button

    - 클릭하면 창 종료

### * Recipe Creation/Change/Deletion confirmation window ( Equipment denied Recipe Command)

  - 장비 recipe의 생성/변경/삭제를 표시한다. 이 window는 operation confirmation window와는 독립적으로 팝 업된다. Recipe의 생성/변경/삭제 확인 window는 5초 후에 자동으로 닫힌다.

| Unit | Recipe | Description |
| --- | --- | --- |

② RECIPE INFORMATION

RECIPEID

③ OK

① Recipe Creation/Change/Deletion command From host was denied ! (Host로부터의 Recipe 생성/변경/삭제 명령을 장비가 거부하였습니다!)

① Recipe Creation / Change / Deletion Description

    - ‘Description’에는 denial reason / code가 표시된다.

### ② Recipe Information

      - equipment recipe의 생성/변경/삭제의 상세

### ③ OK Button

      - 클릭하면 창 종료