Loader Communication Specifications

for LG Display ToE

CC-Link Type Ver 0.8

- From ToE Ver 0.1 to Ver 0.2

| 국 | 신호 | Contents | Description |
| --- | --- | --- | --- |
| 34 |  | Port CST LIFT SENSOR | "Port CST 들림감지센서" Add |
| 63 |  | Robot Action Enable | "Equipment #4" Add |

- From ToE Ver 0.2 to Ver 0.3

| 국 | 신호 | Contents | Description |
| --- | --- | --- | --- |
| 11 |  | 11.Robot #3(Robostar) | "Robostar I/O Map" Add |

- From ToE Ver 0.3 to Ver 0.4

| 국 | 신호 | Contents | Description |
| --- | --- | --- | --- |
| 1 |  | 1.Robot #1(Hyundai) | "Hyundai I/O Map" Add |
| 6 |  | 6.Robot #2 (Hyundai_TRF) | "Hyundai (TRF) I/O Map" Add |
| 6 |  | 6.Robot #2(Yaskawa-XRC) | "Yaskawa-XRC I/O Map" Add |

- From ToE Ver 0.4 to Ver 0.5

| 국 | 신호 | Contents | Description |
| --- | --- | --- | --- |
| 6 |  | 6.Robot #2 (Hyundai_TRF) | "Hyundai (TRF) I/O Map" V1.0 Add |
| 39 |  | 39.IONIZER | "SIGNAL TOWER RED/YELLOW/GREEN" Add |

- From ToE Ver 0.5 to Ver 0.6

| 국 | 신호 | Contents | Description |
| --- | --- | --- | --- |
| 17 |  | 16.Ionizer Error | "Ionizer Error Port 7~8" Delete |
| 38 |  | 35.PORT | "Port I/O Port 7,8" Delete |
| 57~60 |  | 45.Mapping | "Mapping In Port 7~8" Delete |
| 61~62 |  | 61.ALIGN STAGE | "Align Stage In/Out" Delete |
| 57~62 |  | 57.Buffer In TRF(CLIC) | "Buffer Sensor Input(TRF(CLIC))" Add |

- From ToE Ver 0.6 to Ver 0.7

| 국 | 신호 | Contents | Description |
| --- | --- | --- | --- |
| 16 |  | 16.Ionizer Error | "Ionizer Error Port 6" Delete |
| 34 |  | 34.PORT CST LIFT | "Port #6 CST 들림 감지 센서" Delete |
| 37 |  | 35.PORT | "Port I/O Port 6" Delete |
| 39 |  | 39.IONIZER | "Port #6 Ionizer" Delete |
| 41 |  | 41.STK IF | "STK Interlock Input(Port #6)" Delete |
| 43 |  | 41.STK IF | "STK Interlock Output(Port #6)" Delete |
| 44 |  | 41.STK IF | "STK Interlock Input(Port #6)" Delete |
| 55~56 |  | 45.Mapping | "Mapping In Port 6" Delete |
| 55~62 |  | 55.Buffer In TRF(CLIC) | "Buffer Sensor Input(TRF(CLIC))" Add |

Normal Station

예약국

MASTER STATION ECS

Q80BD-J61BT11N

Intelligent Device

1국

Intelligent Device Robot 1 (Hyundai)

5국 예약국

6국

Intelligent Device Robot 2 (Yaskawa)

10국 예약국

11국

Intelligent Device Robot 3 (Robostar)

15국 예약국

16국

Ionizer Error Port 1~5

17국 예약국

18국 예약국

19국

Loader TPD

20국

Robot,공통 I/O Robot 1,2

21국

Robot,공통 I/O Robot 3

22국

Interlock In EQP 1,2

23국

Interlock Out EQP 1

24국

Interlock Out EQP 2

25국

Interlock In EQP 3,4

26국

Interlock Out EQP 3

27국

Interlock Out EQP 4

28국

Interlock In EQP 5,6

29국

Interlock Out EQP 5

30국

Interlock Out EQP 6

31국

Interlock In EQP 7,8

32국

Interlock Out EQP 7

33국

Interlock Out EQP 8

34국

Port

CST LIFT SENSOR

35국

Port I/O Port 1,2

36국

Port I/O Port 3,4

37국

Port I/O Port 5,6

38국 예약국

39국

Ionizer I/O Port 1~5

40국 예약국

41국

STK Interlock In Port 1~5

42국

STK Interlock Out Port 1~4

43국

STK Interlock Out Port 5

44국

STK Interlock In (ULD) Port 1~5

45국

Mapping In Port 1

46국

Mapping In Port 1

47국

Mapping In Port 2

48국

Mapping In Port 2

49국

Mapping In Port 3

50국

Mapping In Port 3

51국

Mapping In Port 4

52국

Mapping In Port 4

53국

Mapping In Port 5

54국

Mapping In Port 5

55국

Buffer In TRF(CLIC)

56국

Buffer In TRF(CLIC)

57국

Buffer In TRF(CLIC)

58국

Buffer In TRF(CLIC)

59국

Buffer In TRF(CLIC)

60국

Buffer In TRF(CLIC)

61국

Buffer In TRF(CLIC)

62국

Buffer In TRF(CLIC)

63국

RAE&Buffer In Loader

64국

RAE&Buffer In Unloader

CC-LINK2.0 Double, 224 Points (14칸) / 32 Words

| 국번 | 마스터영역 |  |  |  |  |  | Remote Station Points |  | 리모트국 I/O(TB) |  |  |  | 리모트국 레지스터 |  | 리모트국 형식 | 용도 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | X |  | Y |  | Ww | Wr |  |  | 입력 |  | 출력 |  | Ww | Wr |  |  |
|  | Start | End | Start | End |  |  |  |  | Start | End | Start | End |  |  |  |  |
| 1 | X000 | X00F | Y000 | Y00F | Ww000 | Wr000 | 224 |  | X000 | X0DF | Y000 | Y0DF | Ww000 | Wr000 | Intelligent Device | Robot 1 (Hyundai) |
|  | X0D0 | X0DF | Y0D0 | Y0DF | Ww01F | Wr01F |  |  |  |  |  |  | Ww01F | Wr01F |  |  |
| 5 | X0E0 | X0EF | Y0E0 | Y0EF |  |  | 32 |  |  |  |  |  |  |  |  | 예약국 |
|  | X0F0 | X0FF | Y0F0 | Y0FF |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 | X100 | X10F | Y100 | Y10F | Ww020 | Wr020 | 224 |  | X000 | X07F | Y000 | Y07F | Ww000 | Wr000 | Intelligent Device | Robot 2 (Hyundai_TRF) |
|  | X1D0 | X1DF | Y1D0 | Y1DF | Ww03F | Wr03F |  |  |  |  |  |  | Ww00F | Wr00F |  |  |
| 10 | X1E0 | X1EF | Y1E0 | Y1EF | Ww040 | Wr040 | 32 |  |  |  |  |  |  |  |  | 예약국 |
|  | X1F0 | X1FF | Y1F0 | Y1FF | Ww047 | Wr047 |  |  |  |  |  |  |  |  |  |  |
| 6 | X100 | X10F | Y100 | Y10F | Ww020 | Wr020 | 128 |  | X000 | X07F | Y000 | Y07F | Ww000 | Wr000 | Intelligent Device | Robot 2 (Hyundai_TRF) Ver.1 Single Type |
|  | X170 | X17F | Y170 | Y17F | Ww02F | Wr02F |  |  |  |  |  |  | Ww00F | Wr00F |  |  |
| 10 | X180 | X18F | Y180 | Y18F | Ww030 | Wr034 | 128 |  |  |  |  |  |  |  |  | 예약국 |
|  | X1F0 | X1FF | Y1F0 | Y1FF | Ww047 | Wr047 |  |  |  |  |  |  |  |  |  |  |
| 6 | X100 | X10F | Y100 | Y10F | Ww020 | Wr020 | 128 |  | X000 | X07F | Y000 | Y07F | Ww000 | Wr000 | Intelligent Device | Robot 2 (Yaskawa-XRC) Ver.1 Single Type |
|  | X170 | X17F | Y170 | Y17F | Ww02F | Wr02F |  |  |  |  |  |  | Ww00F | Wr00F |  |  |
| 10 | X180 | X18F | Y180 | Y18F | Ww030 | Wr034 | 128 |  |  |  |  |  |  |  |  | 예약국 |
|  | X1F0 | X1FF | Y1F0 | Y1FF | Ww047 | Wr047 |  |  |  |  |  |  |  |  |  |  |
| 11 | X200 | X20F | Y200 | Y20F | Ww048 | Wr048 | 224 |  | X000 | X0DF | Y000 | Y0DF | Ww000 | Wr000 | Intelligent Device | Robot 3 (Robostar) |
|  | X2D0 | X2DF | Y2D0 | Y2DF | Ww067 | Wr067 |  |  |  |  |  |  | Ww01F | Wr01F |  |  |
| 15 | X2E0 | X2EF | Y2E0 | Y2EF | Ww068 | Wr068 | 32 |  |  |  |  |  |  |  |  | 예약국 |
|  | X2F0 | X2FF | Y2F0 | Y2FF | Ww06B | Wr06B |  |  |  |  |  |  |  |  |  |  |
| 16 | X300 | X30F | Y300 | Y30F | Ww06C | Wr06C | 32 |  | TB08 |  |  |  |  |  | Remote I/O | Ionier Error Port 1 ~ 5 |
|  | X310 | X31F | Y310 | Y31F | Ww06F | Wr06F |  |  |  | TB39 |  |  |  |  |  |  |
| 17 | X320 | X32F | Y320 | Y32F | Ww070 | Wr070 | 32 |  |  |  |  |  |  |  |  | 예약국 |
|  | X330 | X33F | Y330 | Y33F | Ww073 | Wr073 |  |  |  |  |  |  |  |  |  |  |
| 18 | X340 | X34F | Y340 | Y34F | Ww074 | Wr074 | 32 |  |  |  |  |  |  |  |  | 예약국 |
|  | X350 | X35F | Y350 | Y35F | Ww077 | Wr077 |  |  |  |  |  |  |  |  |  |  |
| 19 | X360 | X36F | Y360 | Y36F | Ww078 | Wr078 | 32 |  | TB08 | TB23 | TB24 | TB39 | Ww000 | Wr000 | Remote I/O | Loader TPD |
|  | X370 | X37F | Y370 | Y37F | Ww07B | Wr07B |  |  |  |  |  |  | Ww003 | Wr003 |  |  |
| 20 | X380 | X38F | Y380 | Y38F | Ww07C | Wr07C | 32 |  | TB08 | TB23 | TB24 | TB39 |  |  | Remote I/O | Robot,공통 I/O Robot 1,2 |
|  | X390 | X39F | Y390 | Y39F | Ww07F | Wr07F |  |  |  |  |  |  |  |  |  |  |
| 21 | X3A0 | X3AF | Y3A0 | Y3AF | Ww080 | Wr080 | 32 |  | TB08 | TB23 | TB24 | TB39 |  |  | Remote I/O | Robot,공통 I/O Robot 3 |
|  | X3B0 | X3BF | Y3B0 | Y3BF | Ww083 | Wr083 |  |  |  |  |  |  |  |  |  |  |
| 22 | X3C0 | X3CF | Y3C0 | Y3CF | Ww084 | Wr084 | 32 |  | TB08 | TB39 |  |  |  |  | Remote I/O | Interlock In EQP 1,2 |
|  | X3D0 | X3DF | Y3D0 | Y3DF | Ww087 | Wr087 |  |  |  |  |  |  |  |  |  |  |
| 23 | X3E0 | X3EF | Y3E0 | Y3EF | Ww088 | Wr088 | 32 |  |  |  | TB08 | TB23 |  |  | Remote I/O | Interlock Out EQP 1 |
|  | X3F0 | X3FF | Y3F0 | Y3FF | Ww08B | Wr08B |  |  |  |  |  |  |  |  |  |  |
| 24 | X400 | X40F | Y400 | Y40F | Ww08C | Wr08C | 32 |  |  |  | TB08 | TB23 |  |  | Remote I/O | Interlock Out EQP 2 |
|  | X410 | X41F | Y410 | Y41F | Ww08F | Wr08F |  |  |  |  |  |  |  |  |  |  |
| 25 | X420 | X42F | Y420 | Y42F | Ww090 | Wr090 | 32 |  | TB08 | TB39 |  |  |  |  | Remote I/O | Interlock In EQP 3,4 |
|  | X430 | X43F | Y430 | Y43F | Ww093 | Wr093 |  |  |  |  |  |  |  |  |  |  |
| 26 | X440 | X44F | Y440 | Y44F | Ww094 | Wr094 | 32 |  |  |  | TB08 | TB23 |  |  | Remote I/O | Interlock Out EQP 3 |
|  | X450 | X45F | Y450 | Y45F | Ww097 | Wr097 |  |  |  |  |  |  |  |  |  |  |
| 27 | X460 | X46F | Y460 | Y46F | Ww098 | Wr098 | 32 |  |  |  | TB08 | TB23 |  |  | Remote I/O | Interlock Out EQP 4 |
|  | X470 | X47F | Y470 | Y47F | Ww09B | Wr09B |  |  |  |  |  |  |  |  |  |  |
| 28 | X480 | X48F | Y480 | Y48F | Ww09C | Wr09C | 32 |  | TB08 | TB39 |  |  |  |  | Remote I/O | Interlock In EQP 5,6 |
|  | X490 | X49F | Y490 | Y49F | Ww09F | Wr09F |  |  |  |  |  |  |  |  |  |  |
| 29 | X4A0 | X4AF | Y4A0 | Y4AF | Ww0A0 | Wr0A0 | 32 |  |  |  | TB08 | TB23 |  |  | Remote I/O | Interlock Out EQP 5 |
|  | X4B0 | X4BF | Y4B0 | Y4BF | Ww0A3 | Wr0A3 |  |  |  |  |  |  |  |  |  |  |
| 30 | X4C0 | X4CF | Y4C0 | Y4CF | Ww0A4 | Wr0A4 | 32 |  |  |  | TB08 | TB23 |  |  | Remote I/O | Interlock Out EQP 6 |
|  | X4D0 | X4DF | Y4D0 | Y4DF | Ww0A7 | Wr0A7 |  |  |  |  |  |  |  |  |  |  |
| 31 | X4E0 | X4EF | Y4E0 | Y4EF | Ww0A8 | Wr0A8 | 32 |  | TB08 | TB39 |  |  |  |  | Remote I/O | Interlock In EQP 7,8 |
|  | X4F0 | X4FF | Y4F0 | Y4FF | Ww0AB | Wr0AB |  |  |  |  |  |  |  |  |  |  |
| 32 | X500 | X50F | Y500 | Y50F | Ww0AC | Wr0AC | 32 |  |  |  | TB08 | TB23 |  |  | Remote I/O | Interlock Out EQP 7 |
|  | X510 | X51F | Y510 | Y51F | Ww0AF | Wr0AF |  |  |  |  |  |  |  |  |  |  |
| 33 | X520 | X52F | Y520 | Y52F | Ww0B0 | Wr0B0 | 32 |  |  |  | TB08 | TB23 |  |  | Remote I/O | Interlock Out EQP 8 |
|  | X530 | X53F | Y530 | Y53F | Ww0B3 | Wr0B3 |  |  |  |  |  |  |  |  |  |  |
| 34 | X540 | X54F | Y540 | Y54F | Ww0B4 | Wr0B4 | 32 |  | TB08 | TB39 |  |  |  |  | Remote I/O | PORT CST LIFT |
|  | X550 | X55F | Y550 | Y55F | Ww0B7 | Wr0B7 |  |  |  |  |  |  |  |  |  |  |

| 국번 | 마스터영역 |  |  |  |  |  | Remote Station Points |  | 리모트국 I/O(TB) |  |  |  | 리모트국 레지스터 |  | 리모트국 형식 | 용도 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | X |  | Y |  | Ww | Wr |  |  | 입력 |  | 출력 |  | Ww | Wr |  |  |
|  | Start | End | Start | End |  |  |  |  | Start | End | Start | End |  |  |  |  |
| 35 | X560 | X56F | Y560 | Y56F | Ww0B8 | Wr0B8 | 32 |  | TB08 | TB23 | TB24 | TB39 |  |  | Remote I/O | Port I/O |
|  | X570 | X57F | Y570 | Y57F | Ww0BB | Wr0BB |  |  |  |  |  |  |  |  |  | Port 1,2 |
| 36 | X580 | X58F | Y580 | Y58F | Ww0BC | Wr0BC | 32 |  | TB08 | TB23 | TB24 | TB39 |  |  | Remote I/O | Port I/O |
|  | X590 | X59F | Y590 | Y59F | Ww0BF | Wr0BF |  |  |  |  |  |  |  |  |  | Port 3,4 |
| 37 | X5A0 | X5AF | Y5A0 | Y5AF | Ww0C0 | Wr0C0 | 32 |  | TB08 | TB23 | TB24 | TB39 |  |  | Remote I/O | Port I/O |
|  | X5B0 | X5BF | Y5B0 | Y5BF | Ww0C3 | Wr0C3 |  |  |  |  |  |  |  |  |  | Port 5 |
| 38 | X5C0 | X5CF | Y5C0 | Y5CF | Ww0C4 | Wr0C4 | 32 |  |  |  |  |  |  |  |  | 예약국 |
|  | X5D0 | X5DF | Y5D0 | Y5DF | Ww0C7 | Wr0C7 |  |  |  |  |  |  |  |  |  |  |
| 39 | X5E0 | X5EF | Y5E0 | Y5EF | Ww0C8 | Wr0C8 | 32 |  | TB08 | TB23 |  |  |  |  | Remote I/O | Ionizer I/O |
|  | X5F0 | X5FF | Y5F0 | Y5FF | Ww0CB | Wr0CB |  |  |  |  | TB24 | TB39 |  |  |  | Port 1~5 |
| 40 | X600 | X60F | Y600 | Y60F | Ww0CC | Wr0CC | 32 |  |  |  |  |  |  |  |  | 예약국 |
|  | X610 | X61F | Y610 | Y61F | Ww0CF | Wr0CF |  |  |  |  |  |  |  |  |  |  |
| 41 | X620 | X62F | Y620 | Y62F | Ww0D0 | Wr0D0 | 32 |  | TB08 | TB39 |  |  |  |  | Remote I/O | STK Interlock In |
|  | X630 | X63F | Y630 | Y63F | Ww0D3 | Wr0D3 |  |  |  |  |  |  |  |  |  | Port 1~5 |
| 42 | X640 | X64F | Y640 | Y64F | Ww0D4 | Wr0D4 | 32 |  |  |  | TB08 | TB23 |  |  | Remote I/O | STK Interlock Out |
|  | X650 | X65F | Y650 | Y65F | Ww0D7 | Wr0D7 |  |  |  |  |  |  |  |  |  | Port 1~4 |
| 43 | X660 | X66F | Y660 | Y66F | Ww0D8 | Wr0D8 | 32 |  |  |  | TB08 | TB23 |  |  | Remote I/O | STK Interlock Out |
|  | X670 | X67F | Y670 | Y67F | Ww0DB | Wr0DB |  |  |  |  |  |  |  |  |  | Port 5 |
| 44 | X680 | X68F | Y680 | Y68F | Ww0DC | Wr0DC | 32 |  | TB08 | TB39 |  |  |  |  | Remote I/O | STK Interlock In |
|  | X690 | X69F | Y690 | Y69F | Ww0DF | Wr0DF |  |  |  |  |  |  |  |  |  | (ULD) Port 1~5 |
| 45 | X6A0 | X6AF | Y6A0 | Y6AF | Ww0E0 | Wr0E0 | 32 |  | X000 | X01F |  |  |  |  | Remote I/O | Mapping In |
|  | X6B0 | X6BF | Y6B0 | Y6BF | Ww0E3 | Wr0E3 |  |  |  |  |  |  |  |  |  | Port 1 |
| 46 | X6C0 | X6CF | Y6C0 | Y6CF | Ww0E4 | Wr0E4 | 32 |  | X000 | X01F |  |  |  |  | Remote I/O | Mapping In |
|  | X6D0 | X6DF | Y6D0 | Y6DF | Ww0E7 | Wr0E7 |  |  |  |  |  |  |  |  |  | Port 1 |
| 47 | X6E0 | X6EF | Y6E0 | Y6EF | Ww0E8 | Wr0E8 | 32 |  | X000 | X01F |  |  |  |  | Remote I/O | Mapping In |
|  | X6F0 | X6FF | Y6F0 | Y6FF | Ww0EB | Wr0EB |  |  |  |  |  |  |  |  |  | Port 2 |
| 48 | X700 | X70F | Y700 | Y70F | Ww0EC | Wr0EC | 32 |  | X000 | X01F |  |  |  |  | Remote I/O | Mapping In |
|  | X710 | X71F | Y710 | Y71F | Ww0EF | Wr0EF |  |  |  |  |  |  |  |  |  | Port 2 |
| 49 | X720 | X72F | Y720 | Y72F | Ww0F0 | Wr0F0 | 32 |  | X000 | X01F |  |  |  |  | Remote I/O | Mapping In |
|  | X730 | X73F | Y730 | Y73F | Ww0F3 | Wr0F3 |  |  |  |  |  |  |  |  |  | Port 3 |
| 50 | X740 | X74F | Y740 | Y74F | Ww0F4 | Wr0F4 | 32 |  | X000 | X01F |  |  |  |  | Remote I/O | Mapping In |
|  | X750 | X75F | Y750 | Y75F | Ww0F7 | Wr0F7 |  |  |  |  |  |  |  |  |  | Port 3 |
| 51 | X760 | X76F | Y760 | Y76F | Ww0F8 | Wr0F8 | 32 |  | X000 | X01F |  |  |  |  | Remote I/O | Mapping In |
|  | X770 | X77F | Y770 | Y77F | Ww0FB | Wr0FB |  |  |  |  |  |  |  |  |  | Port 4 |
| 52 | X780 | X78F | Y780 | Y78F | Ww0FC | Wr0FC | 32 |  | X000 | X01F |  |  |  |  | Remote I/O | Mapping In |
|  | X790 | X79F | Y790 | Y79F | Ww0FF | Wr0FF |  |  |  |  |  |  |  |  |  | Port 4 |
| 53 | X7A0 | X7AF | Y7A0 | Y7AF | Ww100 | Wr100 | 32 |  | X000 | X01F |  |  |  |  | Remote I/O | Mapping In |
|  | X7B0 | X7BF | Y7B0 | Y7BF | Ww103 | Wr103 |  |  |  |  |  |  |  |  |  | Port 5 |
| 54 | X7C0 | X7CF | Y7C0 | Y7CF | Ww104 | Wr104 | 32 |  | X000 | X01F |  |  |  |  | Remote I/O | Mapping In |
|  | X7D0 | X7DF | Y7D0 | Y7DF | Ww107 | Wr107 |  |  |  |  |  |  |  |  |  | Port 5 |
| 55 | X7E0 | X7EF | Y7E0 | Y7EF | Ww108 | Wr108 | 32 |  | X000 | X01F |  |  |  |  | Remote I/O | Buffer In |
|  | X7F0 | X7FF | Y7F0 | Y7FF | Ww10B | Wr10B |  |  |  |  |  |  |  |  |  | TRF(CLIC) |
| 56 | X800 | X80F | Y800 | Y80F | Ww10C | Wr10C | 32 |  | X000 | X01F |  |  |  |  | Remote I/O | Buffer In |
|  | X810 | X81F | Y810 | Y81F | Ww10F | Wr10F |  |  |  |  |  |  |  |  |  | TRF(CLIC) |
| 57 | X820 | X82F | Y820 | Y82F | Ww110 | Wr110 | 32 |  | TB08 | TB39 |  |  |  |  | Remote I/O | Buffer In |
|  | X830 | X83F | Y830 | Y83F | Ww113 | Wr113 |  |  |  |  |  |  |  |  |  | TRF(CLIC) |
| 58 | X840 | X84F | Y840 | Y84F | Ww114 | Wr114 | 32 |  | TB08 | TB39 |  |  |  |  | Remote I/O | Buffer In |
|  | X850 | X85F | Y850 | Y85F | Ww117 | Wr117 |  |  |  |  |  |  |  |  |  | TRF(CLIC) |
| 59 | X860 | X86F | Y860 | Y86F | Ww118 | Wr118 | 32 |  | TB08 | TB39 |  |  |  |  | Remote I/O | Buffer In |
|  | X870 | X87F | Y870 | Y87F | Ww11B | Wr11B |  |  |  |  |  |  |  |  |  | TRF(CLIC) |
| 60 | X880 | X88F | Y880 | Y88F | Ww11C | Wr11C | 32 |  | TB08 | TB39 |  |  |  |  | Remote I/O | Buffer In |
|  | X890 | X89F | Y890 | Y89F | Ww11F | Wr11F |  |  |  |  |  |  |  |  |  | TRF(CLIC) |
| 61 | X8A0 | X8AF | Y8A0 | Y8AF | Ww120 | Wr120 | 32 |  | TB08 | TB39 |  |  |  |  | Remote I/O | Buffer In |
|  | X8B0 | X8BF | Y8B0 | Y8BF | Ww123 | Wr123 |  |  |  |  |  |  |  |  |  | TRF(CLIC) |
| 62 | X8C0 | X8CF | Y8C0 | Y8CF | Ww124 | Wr124 | 32 |  | TB08 | TB39 |  |  |  |  | Remote I/O | Buffer In |
|  | X8D0 | X8DF | Y8D0 | Y8DF | Ww127 | Wr127 |  |  |  |  |  |  |  |  |  | TRF(CLIC) |
| 63 | X8E0 | X8EF | Y8E0 | Y8EF | Ww128 | Wr128 | 32 |  | TB08 | TB39 |  |  |  |  | Remote I/O | EQP RAE |
|  | X8F0 | X8FF | Y8F0 | Y8FF | Ww12B | Wr12B |  |  |  |  |  |  |  |  |  | Buffer In(LD) |
| 64 | X900 | X90F | Y900 | Y90F | Ww12C | Wr12C | 32 |  | TB08 | TB39 |  |  |  |  | Remote I/O | EQP RAE |
|  | X910 | X91F | Y910 | Y91F | Ww12F | Wr12F |  |  |  |  |  |  |  |  |  | Buffer In(ULD) |

| Station No. |  | 1 |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Station Type |  | Intelligent Device |  |  |  |
| 용 도 |  | Robot #1 (Hyundai 기준) |  |  |  |
| 입력 |  | Name | 출력 |  | Name |
| Master | Remote |  | Master | Remote |  |
| X000 | X000 | RUNNING | Y000 | Y000 |  |
| X001 | X001 | SERVO ON | Y001 | Y001 | EXTERNAL RESET |
| X002 | X002 | ROBOT 에러 | Y002 | Y002 | ALARM/ERROR RESET |
| X003 | X003 | ROBOT READY OK | Y003 | Y003 | EXTERNAL START |
| X004 | X004 | REMOTE MODE | Y004 | Y004 | EXTERNAL HOLD (B접점) |
| X005 | X005 | TEACH MODE | Y005 | Y005 |  |
| X006 | X006 | PLAYBACK MODE | Y006 | Y006 |  |
| X007 | X007 | COMMAND READY OK | Y007 | Y007 | MAIN PROGRAM CALL |
| X008 | X008 | 작업원점 | Y008 | Y008 |  |
| X009 | X009 | ARM 원점(L/R) | Y009 | Y009 | REMOTE MODE |
| X00A | X00A |  | Y00A | Y00A | TEACH MODE |
| X00B | X00B |  | Y00B | Y00B | PLAYBACK MODE |
| X00C | X00C |  | Y00C | Y00C |  |
| X00D | X00D |  | Y00D | Y00D |  |
| X00E | X00E | 비상정지 | Y00E | Y00E | SIMULATION MODE |
| X00F | X00F | HOLD 중 | Y00F | Y00F | 흡착무효(무효:1) |
| X010 | X010 | COMMAND 실행완료 | Y010 | Y010 | COMMAND START |
| X011 | X011 | LOADING COMPLETE | Y011 | Y011 | COMMAND 일치 |
| X012 | X012 | UNLOADING COMPLETE | Y012 | Y012 | 저온/고온 구분(1:고온) |
| X013 | X013 | 상핸드 ALIGNMENT COMPLETE | Y013 | Y013 | COMMAND 불일치 |
| X014 | X014 | COMMAND 일치요구 | Y014 | Y014 |  |
| X015 | X015 | COMMAND 실행 중 | Y015 | Y015 | GLASS 검출무효(무효:1) |
| X016 | X016 | COMMAND 이상 CODE STROBE | Y016 | Y016 | COMMAND 이상코드 ACK |
| X017 | X017 | ROBOT 이상 CODE STROBE | Y017 | Y017 |  |
| X018 | X018 | 실행중 COMMAND 1 | Y018 | Y018 | COMMNAD MOTION 1 |
| X019 | X019 | 실행중 COMMAND 2 | Y019 | Y019 | COMMNAD MOTION 2 |
| X01A | X01A | 실행중 COMMAND 4 | Y01A | Y01A | COMMNAD MOTION 4 |
| X01B | X01B | 실행중 COMMAND 8 | Y01B | Y01B | COMMNAD MOTION 8 |
| X01C | X01C | 실행중 COMMAND 16 | Y01C | Y01C | COMMNAD MOTION 16 |
| X01D | X01D | 실행중 COMMAND 32 | Y01D | Y01D | COMMNAD MOTION 32 |
| X01E | X01E | 실행중 COMMAND 64 | Y01E | Y01E | COMMNAD MOTION 64 |
| X01F | X01F | 실행중 COMMAND 128 | Y01F | Y01F | COMMNAD MOTION 128 |
| X020 | X020 | ROBOT HAND(상:0,하:1) | Y020 | Y020 | ROBOT HAND(상:0,하:1) |
| X021 | X021 | 1st TARGET(단:0,다:1) | Y021 | Y021 | 1st TARGET(단:0,다:1) |
| X022 | X022 | 1st CST CODE(30 단) | Y022 | Y022 | 1st CST CODE(30 단) |
| X023 | X023 | 1st CST CODE(RESERVE) | Y023 | Y023 | 1st CST CODE(RESERVE) |
| X024 | X024 | 2nd TARGET(단:0,다:1) | Y024 | Y024 | 2nd TARGET(단:0,다:1) |
| X025 | X025 | 2nd CST CODE(30 단) | Y025 | Y025 | 2nd CST CODE(30 단) |
| X026 | X026 | 2nd CST CODE(RESERVE) | Y026 | Y026 | 2nd CST CODE(RESERVE) |
| X027 | X027 | 3rd TARGET(단:0,다:1) | Y027 | Y027 | 3rd TARGET(단:0,다:1) |
| X028 | X028 | 3rd CST CODE(30 단) | Y028 | Y028 | 3rd CST CODE(30 단) |
| X029 | X029 | 3rd CST CODE(RESERVE) | Y029 | Y029 | 3rd CST CODE(RESERVE) |
| X02A | X02A | FORWARD POSITION 복귀 완료 | Y02A | Y02A |  |
| X02B | X02B | Extended Hand Down Start | Y02B | Y02B |  |
| X02C | X02C | WAIT POSITION COMPLETE ON | Y02C | Y02C | WAIT POSITION COMPLETE ACK |
| X02D | X02D | ROBOT 경고발생 | Y02D | Y02D | 더블핸드 |
| X02E | X02E | Unloading/Loading 1 차완료 (0.5”) | Y02E | Y02E | ROBOT SPEED INPUT |
| X02F | X02F | 장비측 핸드 진입 요청 | Y02F | Y02F | 장비측 핸드 진입 OK |
| X030 | X030 | GLASS 두께확인 1 | Y030 | Y030 | Glass 두께지정 1 |
| X031 | X031 | GLASS 두께확인 2 | Y031 | Y031 | Glass 두께지정 2 |
| X032 | X032 | GLASS 두께확인 4 | Y032 | Y032 | Glass 두께지정 4 |
| X033 | X033 |  | Y033 | Y033 |  |
| X034 | X034 |  | Y034 | Y034 |  |
| X035 | X035 |  | Y035 | Y035 |  |
| X036 | X036 |  | Y036 | Y036 |  |
| X037 | X037 |  | Y037 | Y037 |  |
| Wr000 | Wr000 | 1st POSITION MOTION (STAGE, SLOT NO) | Ww000 | Ww000 | 1st POSITION MOTION (STAGE, SLOT NO) |
| Wr001 | Wr001 | 2nd POSITION MOTION (STAGE, SLOT NO) | Ww001 | Ww001 | 2nd POSITION MOTION (STAGE, SLOT NO) |
| Wr002 | Wr002 | 3rd POSITION MOTION (STAGE, SLOT NO) | Ww002 | Ww002 | 3rd POSITION MOTION (STAGE, SLOT NO) |
| Wr003 | Wr003 | 로봇이상코드 | Ww003 | Ww003 |  |
| Wr004 | Wr004 |  | Ww004 | Ww004 |  |
| Wr005 | Wr005 |  | Ww005 | Ww005 |  |
| Wr006 | Wr006 |  | Ww006 | Ww006 |  |
| Wr007 | Wr007 |  | Ww007 | Ww007 |  |

| 입력 |  | Name | 출력 |  | Name |
| --- | --- | --- | --- | --- | --- |
| Master | Remote |  | Master | Remote |  |
| X038 | X038 | GLASS SIZE 1 | Y038 | Y038 | GLASS SIZE 1 |
| X039 | X039 | GLASS SIZE 2 | Y039 | Y039 | GLASS SIZE 2 |
| X03A | X03A | GLASS SIZE 4 | Y03A | Y03A | GLASS SIZE 4 |
| X03B | X03B | GLASS SIZE 8 | Y03B | Y03B | GLASS SIZE 8 |
| X03C | X03C | GLASS SIZE 16 | Y03C | Y03C | GLASS SIZE 16 |
| X03D | X03D | GLASS SIZE 32 | Y03D | Y03D | GLASS SIZE 32 |
| X03E | X03E | GLASS SIZE 64 | Y03E | Y03E | GLASS SIZE 64 |
| X03F | X03F | GLASS SIZE 128 | Y03F | Y03F | GLASS SIZE 128 |
| X040 | X040 | COMMAND 이상코드 1 | Y040 | Y040 | VACCUM 흡착면(상면:0) |
| X041 | X041 | COMMAND 이상코드 2 | Y041 | Y041 |  |
| X042 | X042 | COMMAND 이상코드 4 | Y042 | Y042 |  |
| X043 | X043 | COMMAND 이상코드 8 | Y043 | Y043 |  |
| X044 | X044 | COMMAND 이상코드 16 | Y044 | Y044 |  |
| X045 | X045 | COMMAND 이상코드 32 | Y045 | Y045 |  |
| X046 | X046 | COMMAND 이상코드 64 | Y046 | Y046 |  |
| X047 | X047 | COMMAND 이상코드 128 | Y047 | Y047 |  |
| X048 | X048 | ROBOT MOVING 중 | Y048 | Y048 | Command Complete ACK |
| X049 | X049 | ROBOT LOCK 유효 | Y049 | Y049 | Loading Complete ACK |
| X04A | X04A | 필드버스 이상 | Y04A | Y04A | Unloading Complete ACK |
| X04B | X04B | 배터리 전압 저하 | Y04B | Y04B |  |
| X04C | X04C | 로봇 조작 에러 | Y04C | Y04C |  |
| X04D | X04D | 그리스부족 사전 경고 | Y04D | Y04D |  |
| X04E | X04E | 부하율 초과 경고 | Y04E | Y04E |  |
| X04F | X04F | 라이트 커튼 동작 에러 ( B 접점 ) | Y04F | Y04F |  |
| X050 | X050 | 하핸드 ALIGNMENT COMPLETE | Y050 | Y050 |  |
| X051 | X051 | Upper Hand Home Position | Y051 | Y051 |  |
| X052 | X052 | Lower Hand Home Position | Y052 | Y052 |  |
| X053 | X053 |  | Y053 | Y053 |  |
| X054 | X054 |  | Y054 | Y054 |  |
| X055 | X055 | 1st CST Code(2^2) | Y055 | Y055 | 1st CST Code(2^2) |
| X056 | X056 | 2nd CST Code(2^2) | Y056 | Y056 | 2nd CST Code(2^2) |
| X057 | X057 | 3rd CST Code(2^2) | Y057 | Y057 | 3rd CST Code(2^2) |
| X058 | X058 |  | Y058 | Y058 |  |
| X059 | X059 |  | Y059 | Y059 |  |
| X05A | X05A | 동작 실행 중 Upper Sub Position Bit 1 | Y05A | Y05A | Upper Sub Position Bit 1 |
| X05B | X05B | 동작 실행 중 Upper Sub Position Bit 2 | Y05B | Y05B | Upper Sub Position Bit 2 |
| X05C | X05C | 동작 실행 중 Upper Sub Position Bit 3 | Y05C | Y05C | Upper Sub Position Bit 3 |
| X05D | X05D | 동작 실행 중 Lower Sub Position Bit 1 | Y05D | Y05D | Lower Sub Position Bit 1 |
| X05E | X05E | 동작 실행 중 Lower Sub Position Bit 2 | Y05E | Y05E | Lower Sub Position Bit 2 |
| X05F | X05F | 동작 실행 중 Lower Sub Position Bit 3 | Y05F | Y05F | Lower Sub Position Bit 3 |
| X060 | X060 | PLAYBACK 속도(%) | Y060 | Y060 | PLAYBACK 속도설정(OPTION) |
| X061 | X061 |  | Y061 | Y061 |  |
| X062 | X062 |  | Y062 | Y062 |  |
| X063 | X063 |  | Y063 | Y063 |  |
| X064 | X064 |  | Y064 | Y064 |  |
| X065 | X065 |  | Y065 | Y065 |  |
| X066 | X066 |  | Y066 | Y066 |  |
| X067 | X067 |  | Y067 | Y067 |  |
| X068 | X068 | Upper Hand Front Glass OK | Y068 | Y068 |  |
| X069 | X069 | Upper Hand Front VAC ON | Y069 | Y069 |  |
| X06A | X06A | Upper Hand Rear Glass OK | Y06A | Y06A |  |
| X06B | X06B | Upper Hand Rear VAC ON | Y06B | Y06B |  |
| X06C | X06C | Lower Hand Front Glass OK | Y06C | Y06C |  |
| X06D | X06D | Lower Hand Front VAC ON | Y06D | Y06D |  |
| X06E | X06E | Lower Hand Rear Glass OK | Y06E | Y06E |  |
| X06F | X06F | Lower Hand Rear VAC ON | Y06F | Y06F |  |
| Wr008 | Wr008 | 하핸드 좌(8bit)-1/우(8bit)-2 흡착 압력(0~100kpa) | Ww008 | Ww008 |  |
| Wr009 | Wr009 | 상핸드 좌(8bit)-3/우(8bit)-4 흡착 압력(0~100kpa) | Ww009 | Ww009 |  |
| Wr00A | Wr00A | 하핸드(8bit)-1/상핸드(8bit) 흡착 ON 소요시간 | Ww00A | Ww00A |  |
| Wr00B | Wr00B | 1-Axis ~ 6-Axis 이동중 (0~5-bit) | Ww00B | Ww00B |  |
| Wr00C | Wr00C | 제어기 내부 온도(8bit)-1 | Ww00C | Ww00C |  |
| Wr00D | Wr00D | 1(X), 2(TH) 축 부하율 (x200%) | Ww00D | Ww00D |  |
| Wr00E | Wr00E | 3(Z1), 4(Z2) 축 부하율 (x200%) | Ww00E | Ww00E |  |
| Wr00F | Wr00F | 5(L), 6(R) 축 부하율 (x200%) | Ww00F | Ww00F |  |

| 입력 |  | Name | 출력 |  | Name |
| --- | --- | --- | --- | --- | --- |
| Master | Remote |  | Master | Remote |  |
| X070 | X070 |  | Y070 | Y070 |  |
| X071 | X071 |  | Y071 | Y071 |  |
| X072 | X072 |  | Y072 | Y072 |  |
| X073 | X073 |  | Y073 | Y073 |  |
| X074 | X074 |  | Y074 | Y074 |  |
| X075 | X075 |  | Y075 | Y075 |  |
| X076 | X076 |  | Y076 | Y076 |  |
| X077 | X077 |  | Y077 | Y077 |  |
| X078 | X078 |  | Y078 | Y078 |  |
| X079 | X079 |  | Y079 | Y079 |  |
| X07A | X07A |  | Y07A | Y07A |  |
| X07B | X07B |  | Y07B | Y07B |  |
| X07C | X07C |  | Y07C | Y07C |  |
| X07D | X07D |  | Y07D | Y07D |  |
| X07E | X07E |  | Y07E | Y07E |  |
| X07F | X07F |  | Y07F | Y07F |  |
| X080 | X080 |  | Y080 | Y080 |  |
| X081 | X081 |  | Y081 | Y081 |  |
| X082 | X082 |  | Y082 | Y082 |  |
| X083 | X083 |  | Y083 | Y083 |  |
| X084 | X084 |  | Y084 | Y084 |  |
| X085 | X085 |  | Y085 | Y085 |  |
| X086 | X086 |  | Y086 | Y086 |  |
| X087 | X087 |  | Y087 | Y087 |  |
| X088 | X088 |  | Y088 | Y088 |  |
| X089 | X089 |  | Y089 | Y089 |  |
| X08A | X08A |  | Y08A | Y08A |  |
| X08B | X08B |  | Y08B | Y08B |  |
| X08C | X08C |  | Y08C | Y08C |  |
| X08D | X08D |  | Y08D | Y08D |  |
| X08E | X08E |  | Y08E | Y08E |  |
| X08F | X08F |  | Y08F | Y08F |  |
| X090 | X090 |  | Y090 | Y090 |  |
| X091 | X091 |  | Y091 | Y091 |  |
| X092 | X092 |  | Y092 | Y092 |  |
| X093 | X093 |  | Y093 | Y093 |  |
| X094 | X094 |  | Y094 | Y094 |  |
| X095 | X095 |  | Y095 | Y095 |  |
| X096 | X096 |  | Y096 | Y096 |  |
| X097 | X097 |  | Y097 | Y097 |  |
| X098 | X098 |  | Y098 | Y098 |  |
| X099 | X099 |  | Y099 | Y099 |  |
| X09A | X09A |  | Y09A | Y09A |  |
| X09B | X09B |  | Y09B | Y09B |  |
| X09C | X09C |  | Y09C | Y09C |  |
| X09D | X09D |  | Y09D | Y09D |  |
| X09E | X09E |  | Y09E | Y09E |  |
| X09F | X09F |  | Y09F | Y09F |  |
| X0A0 | X0A0 |  | Y0A0 | Y0A0 |  |
| X0A1 | X0A1 |  | Y0A1 | Y0A1 |  |
| X0A2 | X0A2 |  | Y0A2 | Y0A2 |  |
| X0A3 | X0A3 |  | Y0A3 | Y0A3 |  |
| X0A4 | X0A4 |  | Y0A4 | Y0A4 |  |
| X0A5 | X0A5 |  | Y0A5 | Y0A5 |  |
| X0A6 | X0A6 |  | Y0A6 | Y0A6 |  |
| X0A7 | X0A7 |  | Y0A7 | Y0A7 |  |
| Wr010 | Wr010 |  | Ww010 | Ww010 |  |
| Wr011 | Wr011 |  | Ww011 | Ww011 |  |
| Wr012 | Wr012 |  | Ww012 | Ww012 |  |
| Wr013 | Wr013 |  | Ww013 | Ww013 |  |
| Wr014 | Wr014 |  | Ww014 | Ww014 | Single Stage #1~#15 Hand Extend I/L Setting (0~15-bit) |
| Wr015 | Wr015 |  | Ww015 | Ww015 | Multi Stage #1~#15 Hand Extend I/L Setting (0~15-bit) |
| Wr016 | Wr016 |  | Ww016 | Ww016 |  |
| Wr017 | Wr017 |  | Ww017 | Ww017 |  |

| 입력 |  | Name | 출력 |  | Name |
| --- | --- | --- | --- | --- | --- |
| Master | Remote |  | Master | Remote |  |
| X0A8 | X0A8 |  | Y0A8 | Y070 |  |
| X0A9 | X0A9 |  | Y0A9 | Y0A9 |  |
| X0AA | X0AA |  | Y0AA | Y0AA |  |
| X0AB | X0AB |  | Y0AB | Y0AB |  |
| X0AC | X0AC |  | Y0AC | Y0AC |  |
| X0AD | X0AD |  | Y0AD | Y0AD |  |
| X0AE | X0AE |  | Y0AE | Y0AE |  |
| X0AF | X0AF |  | Y0AF | Y0AF |  |
| X0B0 | X0B0 |  | Y0B0 | Y0B0 |  |
| X0B1 | X0B1 |  | Y0B1 | Y0B1 |  |
| X0B2 | X0B2 |  | Y0B2 | Y0B2 |  |
| X0B3 | X0B3 |  | Y0B3 | Y0B3 |  |
| X0B4 | X0B4 |  | Y0B4 | Y0B4 |  |
| X0B5 | X0B5 |  | Y0B5 | Y0B5 |  |
| X0B6 | X0B6 |  | Y0B6 | Y0B6 |  |
| X0B7 | X0B7 |  | Y0B7 | Y0B7 |  |
| X0B8 | X0B8 |  | Y0B8 | Y0B8 |  |
| X0B9 | X0B9 |  | Y0B9 | Y0B9 |  |
| X0BA | X0BA |  | Y0BA | Y0BA |  |
| X0BB | X0BB |  | Y0BB | Y0BB |  |
| X0BC | X0BC |  | Y0BC | Y0BC |  |
| X0BD | X0BD |  | Y0BD | Y0BD |  |
| X0BE | X0BE |  | Y0BE | Y0BE |  |
| X0BF | X0BF |  | Y0BF | Y0BF |  |
| X0C0 | X0C0 |  | Y0C0 | Y0C0 |  |
| X0C1 | X0C1 |  | Y0C1 | Y0C1 |  |
| X0C2 | X0C2 |  | Y0C2 | Y0C2 |  |
| X0C3 | X0C3 |  | Y0C3 | Y0C3 |  |
| X0C4 | X0C4 |  | Y0C4 | Y0C4 |  |
| X0C5 | X0C5 |  | Y0C5 | Y0C5 |  |
| X0C6 | X0C6 |  | Y0C6 | Y0C6 |  |
| X0C7 | X0C7 |  | Y0C7 | Y0C7 |  |
| X0C8 | X0C8 |  | Y0C8 | Y0C8 |  |
| X0C9 | X0C9 |  | Y0C9 | Y0C9 |  |
| X0CA | X0CA |  | Y0CA | Y0CA |  |
| X0CB | X0CB |  | Y0CB | Y0CB |  |
| X0CC | X0CC |  | Y0CC | Y0CC |  |
| X0CD | X0CD |  | Y0CD | Y0CD |  |
| X0CE | X0CE |  | Y0CE | Y0CE |  |
| X0CF | X0CF |  | Y0CF | Y0CF |  |
| X0D0 | X0D0 |  | Y0D0 | Y0D0 |  |
| X0D1 | X0D1 |  | Y0D1 | Y0D1 |  |
| X0D2 | X0D2 |  | Y0D2 | Y0D2 |  |
| X0D3 | X0D3 |  | Y0D3 | Y0D3 |  |
| X0D4 | X0D4 |  | Y0D4 | Y0D4 |  |
| X0D5 | X0D5 |  | Y0D5 | Y0D5 |  |
| X0D6 | X0D6 |  | Y0D6 | Y0D6 |  |
| X0D7 | X0D7 |  | Y0D7 | Y0D7 |  |
| X0D8 | X0D8 |  | Y0D8 | Y0D8 |  |
| X0D9 | X0D9 |  | Y0D9 | Y0D9 |  |
| X0DA | X0DA |  | Y0DA | Y0DA |  |
| X0DB | X0DB |  | Y0DB | Y0DB |  |
| X0DC | X0DC |  | Y0DC | Y0DC |  |
| X0DD | X0DD |  | Y0DD | Y0DD |  |
| X0DE | X0DE |  | Y0DE | Y0DE |  |
| X0DF | X0DF |  | Y0DF | Y0DF |  |
| Wr018 | Wr018 |  | Ww018 | Ww018 |  |
| Wr019 | Wr019 |  | Ww019 | Ww019 |  |
| Wr01A | Wr01A |  | Ww01A | Ww01A |  |
| Wr01B | Wr01B |  | Ww01B | Ww01B |  |
| Wr01C | Wr01C |  | Ww01C | Ww01C |  |
| Wr01D | Wr01D |  | Ww01D | Ww01D |  |
| Wr01E | Wr01E |  | Ww01E | Ww01E |  |
| Wr01F | Wr01F |  | Ww01F | Ww01F |  |

CC-LINK2.0 Double, 224 Points (14칸) / 32 Words

| Station No. |  | 6 |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Station Type |  | Intelligent Device |  |  |  |
| 용 도 |  | Robot #2 (Hyundai 기준) |  |  |  |
| 입력 |  | Name | 출력 |  | Name |
| Master | Remote |  | Master | Remote |  |
| X100 | X000 | RUNNING | Y100 | Y000 |  |
| X101 | X001 | SERVO ON | Y101 | Y001 | EXTERNAL RESET |
| X102 | X002 | ROBOT 에러 | Y102 | Y002 | ALARM/ERROR RESET |
| X103 | X003 | ROBOT READY OK | Y103 | Y003 | EXTERNAL START |
| X104 | X004 | REMOTE MODE | Y104 | Y004 | EXTERNAL HOLD (B접점) |
| X105 | X005 | TEACH MODE | Y105 | Y005 |  |
| X106 | X006 | PLAYBACK MODE | Y106 | Y006 |  |
| X107 | X007 | COMMAND READY OK | Y107 | Y007 | MAIN PROGRAM CALL |
| X108 | X008 | 작업원점 | Y108 | Y008 |  |
| X109 | X009 | ARM 원점(L/R) | Y109 | Y009 | REMOTE MODE |
| X10A | X00A | R HAND GLASS ok | Y10A | Y00A | TEACH MODE |
| X10B | X00B | L HAND GLASS ok | Y10B | Y00B | PLAYBACK MODE |
| X10C | X00C | R HAND 흡착 ON | Y10C | Y00C |  |
| X10D | X00D | L HAND 흡착 ON | Y10D | Y00D |  |
| X10E | X00E | 비상정지 | Y10E | Y00E | SIMULATION MODE |
| X10F | X00F | HOLD 중 | Y10F | Y00F | 흡착무효(무효:1) |
| X110 | X010 | COMMAND 실행완료 | Y110 | Y010 | COMMAND START |
| X111 | X011 | LOADING COMPLETE | Y111 | Y011 | COMMAND 일치 |
| X112 | X012 | UNLOADING COMPLETE | Y112 | Y012 | 저온/고온 구분(1:고온) |
| X113 | X013 | 상핸드 ALIGNMENT COMPLETE | Y113 | Y013 | COMMAND 불일치 |
| X114 | X014 | COMMAND 일치요구 | Y114 | Y014 |  |
| X115 | X015 | COMMAND 실행 중 | Y115 | Y015 | GLASS 검출무효(무효:1) |
| X116 | X016 | COMMAND 이상 CODE STROBE | Y116 | Y016 | COMMAND 이상코드 ACK |
| X117 | X017 | ROBOT 이상 CODE STROBE | Y117 | Y017 |  |
| X118 | X018 | 실행중 COMMAND 1 | Y118 | Y018 | COMMNAD MOTION 1 |
| X119 | X019 | 실행중 COMMAND 2 | Y119 | Y019 | COMMNAD MOTION 2 |
| X11A | X01A | 실행중 COMMAND 4 | Y11A | Y01A | COMMNAD MOTION 4 |
| X11B | X01B | 실행중 COMMAND 8 | Y11B | Y01B | COMMNAD MOTION 8 |
| X11C | X01C | 실행중 COMMAND 16 | Y11C | Y01C | COMMNAD MOTION 16 |
| X11D | X01D | 실행중 COMMAND 32 | Y11D | Y01D | COMMNAD MOTION 32 |
| X11E | X01E | 실행중 COMMAND 64 | Y11E | Y01E | COMMNAD MOTION 64 |
| X11F | X01F | 실행중 COMMAND 128 | Y11F | Y01F | COMMNAD MOTION 128 |
| X120 | X020 | ROBOT HAND(상:0,하:1) | Y120 | Y020 | ROBOT HAND(상:0,하:1) |
| X121 | X021 | 1st TARGET(단:0,다:1) | Y121 | Y021 | 1st TARGET(단:0,다:1) |
| X122 | X022 | 1st CST CODE(30 단) | Y122 | Y022 | 1st CST CODE(30 단) |
| X123 | X023 | 1st CST CODE(RESERVE) | Y123 | Y023 | 1st CST CODE(RESERVE) |
| X124 | X024 | 2nd TARGET(단:0,다:1) | Y124 | Y024 | 2nd TARGET(단:0,다:1) |
| X125 | X025 | 2nd CST CODE(30 단) | Y125 | Y025 | 2nd CST CODE(30 단) |
| X126 | X026 | 2nd CST CODE(RESERVE) | Y126 | Y026 | 2nd CST CODE(RESERVE) |
| X127 | X027 | 3rd TARGET(단:0,다:1) | Y127 | Y027 | 3rd TARGET(단:0,다:1) |
| X128 | X028 | 3rd CST CODE(30 단) | Y128 | Y028 | 3rd CST CODE(30 단) |
| X129 | X029 | 3rd CST CODE(RESERVE) | Y129 | Y029 | 3rd CST CODE(RESERVE) |
| X12A | X02A | FORWARD POSITION 복귀 완료 | Y12A | Y02A |  |
| X12B | X02B | Extended Hand Down Start | Y12B | Y02B |  |
| X12C | X02C | WAIT POSITION COMPLETE ON | Y12C | Y02C | WAIT POSITION COMPLETE ACK |
| X12D | X02D | ROBOT 경고발생 | Y12D | Y02D | 더블핸드 |
| X12E | X02E | Unloading/Loading 1 차완료 (0.5”) | Y12E | Y02E | ROBOT SPEED INPUT |
| X12F | X02F | 장비측 핸드 진입 요청 | Y12F | Y02F | 장비측 핸드 진입 OK |
| X130 | X030 | GLASS 두께확인 1 | Y130 | Y030 | Glass 두께지정 1 |
| X131 | X031 | GLASS 두께확인 2 | Y131 | Y031 | Glass 두께지정 2 |
| X132 | X032 | GLASS 두께확인 4 | Y132 | Y032 | Glass 두께지정 4 |
| X133 | X033 |  | Y133 | Y033 |  |
| X134 | X034 |  | Y134 | Y034 |  |
| X135 | X035 |  | Y135 | Y035 |  |
| X136 | X036 |  | Y136 | Y036 |  |
| X137 | X037 |  | Y137 | Y037 |  |
| Wr020 | Wr000 | 1st POSITION MOTION (STAGE, SLOT NO) | Ww020 | Ww000 | 1st POSITION MOTION (STAGE, SLOT NO) |
| Wr021 | Wr001 | 2nd POSITION MOTION (STAGE, SLOT NO) | Ww021 | Ww001 | 2nd POSITION MOTION (STAGE, SLOT NO) |
| Wr022 | Wr002 | 3rd POSITION MOTION (STAGE, SLOT NO) | Ww022 | Ww002 | 3rd POSITION MOTION (STAGE, SLOT NO) |
| Wr023 | Wr003 | 로봇이상코드 | Ww023 | Ww003 |  |
| Wr024 | Wr004 |  | Ww024 | Ww004 |  |
| Wr025 | Wr005 |  | Ww025 | Ww005 |  |
| Wr026 | Wr006 |  | Ww026 | Ww006 |  |
| Wr027 | Wr007 |  | Ww027 | Ww007 |  |

| 입력 |  | Name | 출력 |  | Name |
| --- | --- | --- | --- | --- | --- |
| Master | Remote |  | Master | Remote |  |
| X138 | X038 | GLASS SIZE 1 | Y138 | Y038 | GLASS SIZE 1 |
| X139 | X039 | GLASS SIZE 2 | Y139 | Y039 | GLASS SIZE 2 |
| X13A | X03A | GLASS SIZE 4 | Y13A | Y03A | GLASS SIZE 4 |
| X13B | X03B | GLASS SIZE 8 | Y13B | Y03B | GLASS SIZE 8 |
| X13C | X03C | GLASS SIZE 16 | Y13C | Y03C | GLASS SIZE 16 |
| X13D | X03D | GLASS SIZE 32 | Y13D | Y03D | GLASS SIZE 32 |
| X13E | X03E | GLASS SIZE 64 | Y13E | Y03E | GLASS SIZE 64 |
| X13F | X03F | GLASS SIZE 128 | Y13F | Y03F | GLASS SIZE 128 |
| X140 | X040 | COMMAND 이상코드 1 | Y140 | Y040 | VACCUM 흡착면(상면:0) |
| X141 | X041 | COMMAND 이상코드 2 | Y141 | Y041 |  |
| X142 | X042 | COMMAND 이상코드 4 | Y142 | Y042 |  |
| X143 | X043 | COMMAND 이상코드 8 | Y143 | Y043 |  |
| X144 | X044 | COMMAND 이상코드 16 | Y144 | Y044 |  |
| X145 | X045 | COMMAND 이상코드 32 | Y145 | Y045 |  |
| X146 | X046 | COMMAND 이상코드 64 | Y146 | Y046 |  |
| X147 | X047 | COMMAND 이상코드 128 | Y147 | Y047 |  |
| X148 | X048 | ROBOT MOVING 중 | Y148 | Y048 | Command Complete ACK |
| X149 | X049 | ROBOT LOCK 유효 | Y149 | Y049 | Loading Complete ACK |
| X14A | X04A | 필드버스 이상 | Y14A | Y04A | Unloading Complete ACK |
| X14B | X04B | 배터리 전압 저하 | Y14B | Y04B |  |
| X14C | X04C | 로봇 조작 에러 | Y14C | Y04C |  |
| X14D | X04D | 그리스부족 사전 경고 | Y14D | Y04D |  |
| X14E | X04E | 부하율 초과 경고 | Y14E | Y04E |  |
| X14F | X04F | 라이트 커튼 동작 에러 ( B 접점 ) | Y14F | Y04F |  |
| X150 | X050 | 하핸드 ALIGNMENT COMPLETE | Y150 | Y050 |  |
| X151 | X051 | Upper Hand Home Position | Y151 | Y051 |  |
| X152 | X052 | Lower Hand Home Position | Y152 | Y052 |  |
| X153 | X053 |  | Y153 | Y053 |  |
| X154 | X054 |  | Y154 | Y054 |  |
| X155 | X055 | 1st CST Code(2^2) | Y155 | Y055 | 1st CST Code(2^2) |
| X156 | X056 | 2nd CST Code(2^2) | Y156 | Y056 | 2nd CST Code(2^2) |
| X157 | X057 | 3rd CST Code(2^2) | Y157 | Y057 | 3rd CST Code(2^2) |
| X158 | X058 |  | Y158 | Y058 |  |
| X159 | X059 |  | Y159 | Y059 |  |
| X15A | X05A | 동작 실행 중 Upper Sub Position Bit 1 | Y15A | Y05A | Upper Sub Position Bit 1 |
| X15B | X05B | 동작 실행 중 Upper Sub Position Bit 2 | Y15B | Y05B | Upper Sub Position Bit 2 |
| X15C | X05C | 동작 실행 중 Upper Sub Position Bit 3 | Y15C | Y05C | Upper Sub Position Bit 3 |
| X15D | X05D | 동작 실행 중 Lower Sub Position Bit 1 | Y15D | Y05D | Lower Sub Position Bit 1 |
| X15E | X05E | 동작 실행 중 Lower Sub Position Bit 2 | Y15E | Y05E | Lower Sub Position Bit 2 |
| X15F | X05F | 동작 실행 중 Lower Sub Position Bit 3 | Y15F | Y05F | Lower Sub Position Bit 3 |
| X160 | X060 | PLAYBACK 속도(%) | Y160 | Y060 | PLAYBACK 속도설정(OPTION) |
| X161 | X061 |  | Y161 | Y061 |  |
| X162 | X062 |  | Y162 | Y062 |  |
| X163 | X063 |  | Y163 | Y063 |  |
| X164 | X064 |  | Y164 | Y064 |  |
| X165 | X065 |  | Y165 | Y065 |  |
| X166 | X066 |  | Y166 | Y066 |  |
| X167 | X067 |  | Y167 | Y067 |  |
| X168 | X068 | Upper Hand Front Glass OK | Y168 | Y068 |  |
| X169 | X069 | Upper Hand Front VAC ON | Y169 | Y069 |  |
| X16A | X06A | Upper Hand Rear Glass OK | Y16A | Y06A |  |
| X16B | X06B | Upper Hand Rear VAC ON | Y16B | Y06B |  |
| X16C | X06C | Lower Hand Front Glass OK | Y16C | Y06C |  |
| X16D | X06D | Lower Hand Front VAC ON | Y16D | Y06D |  |
| X16E | X06E | Lower Hand Rear Glass OK | Y16E | Y06E |  |
| X16F | X06F | Lower Hand Rear VAC ON | Y16F | Y06F |  |
| Wr028 | Wr008 | 하핸드 좌(8bit)-1/우(8bit)-2 흡착 압력(0~100kpa) | Ww028 | Ww008 |  |
| Wr029 | Wr009 | 상핸드 좌(8bit)-3/우(8bit)-4 흡착 압력(0~100kpa) | Ww029 | Ww009 |  |
| Wr02A | Wr00A | 하핸드(8bit)-1/상핸드(8bit) 흡착 ON 소요시간 | Ww02A | Ww00A |  |
| Wr02B | Wr00B | 1-Axis ~ 6-Axis 이동중 (0~5-bit) | Ww02B | Ww00B |  |
| Wr02C | Wr00C | 제어기 내부 온도(8bit)-1 | Ww02C | Ww00C |  |
| Wr02D | Wr00D | 1(X), 2(TH) 축 부하율 (x200%) | Ww02D | Ww00D |  |
| Wr02E | Wr00E | 3(Z1), 4(Z2) 축 부하율 (x200%) | Ww02E | Ww00E |  |
| Wr02F | Wr00F | 5(L), 6(R) 축 부하율 (x200%) | Ww02F | Ww00F |  |

| 입력 |  | Name | 출력 |  | Name |
| --- | --- | --- | --- | --- | --- |
| Master | Remote |  | Master | Remote |  |
| X170 | X070 |  | Y170 | Y070 |  |
| X171 | X071 |  | Y171 | Y071 |  |
| X172 | X072 |  | Y172 | Y072 |  |
| X173 | X073 |  | Y173 | Y073 |  |
| X174 | X074 |  | Y174 | Y074 |  |
| X175 | X075 |  | Y175 | Y075 |  |
| X176 | X076 |  | Y176 | Y076 |  |
| X177 | X077 |  | Y177 | Y077 |  |
| X178 | X078 |  | Y178 | Y078 |  |
| X179 | X079 |  | Y179 | Y079 |  |
| X17A | X07A |  | Y17A | Y07A |  |
| X17B | X07B |  | Y17B | Y07B |  |
| X17C | X07C |  | Y17C | Y07C |  |
| X17D | X07D |  | Y17D | Y07D |  |
| X17E | X07E |  | Y17E | Y07E |  |
| X17F | X07F |  | Y17F | Y07F |  |
| X180 | X080 |  | Y180 | Y080 |  |
| X181 | X081 |  | Y181 | Y081 |  |
| X182 | X082 |  | Y182 | Y082 |  |
| X183 | X083 |  | Y183 | Y083 |  |
| X184 | X084 |  | Y184 | Y084 |  |
| X185 | X085 |  | Y185 | Y085 |  |
| X186 | X086 |  | Y186 | Y086 |  |
| X187 | X087 |  | Y187 | Y087 |  |
| X188 | X088 |  | Y188 | Y088 |  |
| X189 | X089 |  | Y189 | Y089 |  |
| X18A | X08A |  | Y18A | Y08A |  |
| X18B | X08B |  | Y18B | Y08B |  |
| X18C | X08C |  | Y18C | Y08C |  |
| X18D | X08D |  | Y18D | Y08D |  |
| X18E | X08E |  | Y18E | Y08E |  |
| X18F | X08F |  | Y18F | Y08F |  |
| X190 | X090 |  | Y190 | Y090 |  |
| X191 | X091 |  | Y191 | Y091 |  |
| X192 | X092 |  | Y192 | Y092 |  |
| X193 | X093 |  | Y193 | Y093 |  |
| X194 | X094 |  | Y194 | Y094 |  |
| X195 | X095 |  | Y195 | Y095 |  |
| X196 | X096 |  | Y196 | Y096 |  |
| X197 | X097 |  | Y197 | Y097 |  |
| X198 | X098 |  | Y198 | Y098 |  |
| X199 | X099 |  | Y199 | Y099 |  |
| X19A | X09A |  | Y19A | Y09A |  |
| X19B | X09B |  | Y19B | Y09B |  |
| X19C | X09C |  | Y19C | Y09C |  |
| X19D | X09D |  | Y19D | Y09D |  |
| X19E | X09E |  | Y19E | Y09E |  |
| X19F | X09F |  | Y19F | Y09F |  |
| X1A0 | X0A0 |  | Y1A0 | Y0A0 |  |
| X1A1 | X0A1 |  | Y1A1 | Y0A1 |  |
| X1A2 | X0A2 |  | Y1A2 | Y0A2 |  |
| X1A3 | X0A3 |  | Y1A3 | Y0A3 |  |
| X1A4 | X0A4 |  | Y1A4 | Y0A4 |  |
| X1A5 | X0A5 |  | Y1A5 | Y0A5 |  |
| X1A6 | X0A6 |  | Y1A6 | Y0A6 |  |
| X1A7 | X0A7 |  | Y1A7 | Y0A7 |  |
| Wr030 | Wr010 |  | Ww030 | Ww010 |  |
| Wr031 | Wr011 |  | Ww031 | Ww011 |  |
| Wr032 | Wr012 |  | Ww032 | Ww012 |  |
| Wr033 | Wr013 |  | Ww033 | Ww013 |  |
| Wr034 | Wr014 |  | Ww034 | Ww014 | Single Stage #1~#15 Hand Extend I/L Setting (0~15-bit) |
| Wr035 | Wr015 |  | Ww035 | Ww015 | Multi Stage #1~#15 Hand Extend I/L Setting (0~15-bit) |
| Wr036 | Wr016 |  | Ww036 | Ww016 |  |
| Wr037 | Wr017 |  | Ww037 | Ww017 |  |

| 입력 |  | Name | 출력 |  | Name |
| --- | --- | --- | --- | --- | --- |
| Master | Remote |  | Master | Remote |  |
| X1A8 | X0A8 |  | Y1A8 | Y070 |  |
| X1A9 | X0A9 |  | Y1A9 | Y0A9 |  |
| X1AA | X0AA |  | Y1AA | Y0AA |  |
| X1AB | X0AB |  | Y1AB | Y0AB |  |
| X1AC | X0AC |  | Y1AC | Y0AC |  |
| X1AD | X0AD |  | Y1AD | Y0AD |  |
| X1AE | X0AE |  | Y1AE | Y0AE |  |
| X1AF | X0AF |  | Y1AF | Y0AF |  |
| X1B0 | X0B0 |  | Y1B0 | Y0B0 |  |
| X1B1 | X0B1 |  | Y1B1 | Y0B1 |  |
| X1B2 | X0B2 |  | Y1B2 | Y0B2 |  |
| X1B3 | X0B3 |  | Y1B3 | Y0B3 |  |
| X1B4 | X0B4 |  | Y1B4 | Y0B4 |  |
| X1B5 | X0B5 |  | Y1B5 | Y0B5 |  |
| X1B6 | X0B6 |  | Y1B6 | Y0B6 |  |
| X1B7 | X0B7 |  | Y1B7 | Y0B7 |  |
| X1B8 | X0B8 |  | Y1B8 | Y0B8 |  |
| X1B9 | X0B9 |  | Y1B9 | Y0B9 |  |
| X1BA | X0BA |  | Y1BA | Y0BA |  |
| X1BB | X0BB |  | Y1BB | Y0BB |  |
| X1BC | X0BC |  | Y1BC | Y0BC |  |
| X1BD | X0BD |  | Y1BD | Y0BD |  |
| X1BE | X0BE |  | Y1BE | Y0BE |  |
| X1BF | X0BF |  | Y1BF | Y0BF |  |
| X1C0 | X0C0 |  | Y1C0 | Y0C0 |  |
| X1C1 | X0C1 |  | Y1C1 | Y0C1 |  |
| X1C2 | X0C2 |  | Y1C2 | Y0C2 |  |
| X1C3 | X0C3 |  | Y1C3 | Y0C3 |  |
| X1C4 | X0C4 |  | Y1C4 | Y0C4 |  |
| X1C5 | X0C5 |  | Y1C5 | Y0C5 |  |
| X1C6 | X0C6 |  | Y1C6 | Y0C6 |  |
| X1C7 | X0C7 |  | Y1C7 | Y0C7 |  |
| X1C8 | X0C8 |  | Y1C8 | Y0C8 |  |
| X1C9 | X0C9 |  | Y1C9 | Y0C9 |  |
| X1CA | X0CA |  | Y1CA | Y0CA |  |
| X1CB | X0CB |  | Y1CB | Y0CB |  |
| X1CC | X0CC |  | Y1CC | Y0CC |  |
| X1CD | X0CD |  | Y1CD | Y0CD |  |
| X1CE | X0CE |  | Y1CE | Y0CE |  |
| X1CF | X0CF |  | Y1CF | Y0CF |  |
| X1D0 | X0D0 |  | Y1D0 | Y0D0 |  |
| X1D1 | X0D1 |  | Y1D1 | Y0D1 |  |
| X1D2 | X0D2 |  | Y1D2 | Y0D2 |  |
| X1D3 | X0D3 |  | Y1D3 | Y0D3 |  |
| X1D4 | X0D4 |  | Y1D4 | Y0D4 |  |
| X1D5 | X0D5 |  | Y1D5 | Y0D5 |  |
| X1D6 | X0D6 |  | Y1D6 | Y0D6 |  |
| X1D7 | X0D7 |  | Y1D7 | Y0D7 |  |
| X1D8 | X0D8 |  | Y1D8 | Y0D8 |  |
| X1D9 | X0D9 |  | Y1D9 | Y0D9 |  |
| X1DA | X0DA |  | Y1DA | Y0DA |  |
| X1DB | X0DB |  | Y1DB | Y0DB |  |
| X1DC | X0DC |  | Y1DC | Y0DC |  |
| X1DD | X0DD |  | Y1DD | Y0DD |  |
| X1DE | X0DE |  | Y1DE | Y0DE |  |
| X1DF | X0DF |  | Y1DF | Y0DF |  |
| Wr038 | Wr018 |  | Ww038 | Ww018 |  |
| Wr039 | Wr019 |  | Ww039 | Ww019 |  |
| Wr03A | Wr01A |  | Ww03A | Ww01A |  |
| Wr03B | Wr01B |  | Ww03B | Ww01B |  |
| Wr03C | Wr01C |  | Ww03C | Ww01C |  |
| Wr03D | Wr01D |  | Ww03D | Ww01D |  |
| Wr03E | Wr01E |  | Ww03E | Ww01E |  |
| Wr03F | Wr01F |  | Ww03F | Ww01F |  |

CC-LINK2.0 Single, 128 Points (14칸) / 16 Words

| Station No. |  | 6 |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Station Type |  | Intelligent Device |  |  |  |
| 용 도 |  | Robot #2 (Hyundai 재활용 기준) |  |  |  |
| 입력 |  | Name | 출력 |  | Name |
| Master | Remote |  | Master | Remote |  |
| X100 | X000 | Running | Y100 | Y000 |  |
| X101 | X001 | Servo ON | Y101 | Y001 | External Reset |
| X102 | X002 | Alarm/Error | Y102 | Y002 | Alarm/Error Reset |
| X103 | X003 | Robot Ready OK | Y103 | Y003 | External Start |
| X104 | X004 | Remote Mode | Y104 | Y004 | External Hold |
| X105 | X005 | Teach Mode | Y105 | Y005 | EXTERNAL SERVO ON |
| X106 | X006 | Playback Mode | Y106 | Y006 |  |
| X107 | X007 | Main Prog. Ready OK (Command Ready) | Y107 | Y007 | Main Prog. Call |
| X108 | X008 | 작업원점 | Y108 | Y008 |  |
| X109 | X009 | Arm 원점(L/R) | Y109 | Y009 | Remote Mode |
| X10A | X00A | R HAND GLASS ok | Y10A | Y00A | Teach Mode |
| X10B | X00B | L HAND GLASS ok | Y10B | Y00B | Playback Mode |
| X10C | X00C | R HAND 흡착 ON | Y10C | Y00C |  |
| X10D | X00D | L HAND 흡착 ON | Y10D | Y00D |  |
| X10E | X00E | 비상정지 | Y10E | Y00E |  |
| X10F | X00F | Holding | Y10F | Y00F | 흡착확인무효 |
| X110 | X010 | COMMAND 실행완료 | Y110 | Y010 | 동작 Command START |
| X111 | X011 | LOADING COMPLETE | Y111 | Y011 | Command 일치 |
| X112 | X012 | UNLOADING COMPLETE | Y112 | Y012 | Command 완료 Ack |
| X113 | X013 | ALIGNMENT COMPLETE | Y113 | Y013 | Command 불일치 |
| X114 | X014 | COMMAND 일치요구 | Y114 | Y014 |  |
| X115 | X015 | COMMAND 실행중 | Y115 | Y015 | GLASS 검출무효 |
| X116 | X016 | 이상 CODE STROBE | Y116 | Y016 | 이상코드 ACK |
| X117 | X017 | Robot 이상 CODE STROBE | Y117 | Y017 |  |
| X118 | X018 | 실행중 COMMAND 1 | Y118 | Y018 | COMMNAD MOTION 1 |
| X119 | X019 | 실행중 COMMAND 2 | Y119 | Y019 | COMMNAD MOTION 2 |
| X11A | X01A | 실행중 COMMAND 4 | Y11A | Y01A | COMMNAD MOTION 4 |
| X11B | X01B | 실행중 COMMAND 8 | Y11B | Y01B | COMMNAD MOTION 8 |
| X11C | X01C | 실행중 COMMAND 16 | Y11C | Y01C | COMMNAD MOTION 16 |
| X11D | X01D | 실행중 COMMAND 32 | Y11D | Y01D | COMMNAD MOTION 32 |
| X11E | X01E | 실행중 COMMAND 64 | Y11E | Y01E | COMMNAD MOTION 64 |
| X11F | X01F |  | Y11F | Y01F |  |
| Wr020 | Wr000 | 1st POS MOTION | Ww020 | Ww000 | 1st POS MOTION |
| Wr021 | Wr001 | 2nd POS MOTION | Ww021 | Ww001 | 2nd POS MOTION |
| Wr022 | Wr002 | 3rd POS MOTION | Ww022 | Ww002 | 3rd POS MOTION |
| Wr023 | Wr003 | Robot 이상 Code | Ww023 | Ww003 | Robot 속도(1~100) |

| 입력 |  | Name | 출력 |  | Name |
| --- | --- | --- | --- | --- | --- |
| Master | Remote |  | Master | Remote |  |
| X120 | X020 | ROBOT ARM(상:0,하:1) | Y120 | Y020 | ROBOT ARM(상:0,하:1) |
| X121 | X021 | 1st TARGET(단:0,다:1) | Y121 | Y021 | 1st TARGET(단:0,다:1) |
| X122 | X022 | 1st CST CODE(20 단) | Y122 | Y022 | 1st CST CODE(20 단) |
| X123 | X023 | 1st CST CODE(RESERVE) | Y123 | Y023 | 1st CST CODE(RESERVE) |
| X124 | X024 | 2nd TARGET(단:0,다:1) | Y124 | Y024 | 2nd TARGET(단:0,다:1) |
| X125 | X025 | 2nd CST CODE(20 단) | Y125 | Y025 | 2nd CST CODE(20 단) |
| X126 | X026 | 2nd CST CODE(RESERVE) | Y126 | Y026 | 2nd CST CODE(RESERVE) |
| X127 | X027 | 3rd TARGET(단:0,다:1) | Y127 | Y027 | 3rd TARGET(단:0,다:1) |
| X128 | X028 | 3rd CST CODE(20 단) | Y128 | Y028 | 3rd CST CODE(20 단) |
| X129 | X029 | 3rd CST CODE(RESERVE) | Y129 | Y029 | 3rd CST CODE(RESERVE) |
| X12A | X02A | Forward Wait Position 복귀 완료 | Y12A | Y02A |  |
| X12B | X02B |  | Y12B | Y02B |  |
| X12C | X02C |  | Y12C | Y02C |  |
| X12D | X02D |  | Y12D | Y02D |  |
| X12E | X02E |  | Y12E | Y02E |  |
| X12F | X02F |  | Y12F | Y02F |  |
| X130 | X030 | Glass 두께확인 1 | Y130 | Y030 | Glass 두께지정 1 |
| X131 | X031 | Glass 두께확인 2 | Y131 | Y031 | Glass 두께지정 2 |
| X132 | X032 | Glass 두께확인 4 | Y132 | Y032 | Glass 두께지정 4 |
| X133 | X033 |  | Y133 | Y033 |  |
| X134 | X034 |  | Y134 | Y034 |  |
| X135 | X035 |  | Y135 | Y035 |  |
| X136 | X036 |  | Y136 | Y036 |  |
| X137 | X037 |  | Y137 | Y037 |  |
| X138 | X038 | GLASS SIZE 1(1/3 size) | Y138 | Y038 | GLASS SIZE 1(1/3 size) |
| X139 | X039 | GLASS SIZE 2(1/2 size) | Y139 | Y039 | GLASS SIZE 2(1/2 size) |
| X13A | X03A | GLASS SIZE 4(2/3 size) | Y13A | Y03A | GLASS SIZE 4(2/3 size) |
| X13B | X03B | GLASS SIZE 8(1/1 size) | Y13B | Y03B | GLASS SIZE 8(1/1 size) |
| X13C | X03C |  | Y13C | Y03C |  |
| X13D | X03D |  | Y13D | Y03D |  |
| X13E | X03E |  | Y13E | Y03E |  |
| X13F | X03F |  | Y13F | Y03F |  |
| Wr024 | Wr004 |  | Ww024 | Ww004 |  |
| Wr025 | Wr005 |  | Ww025 | Ww005 |  |
| Wr026 | Wr006 |  | Ww026 | Ww006 |  |
| Wr027 | Wr007 |  | Ww027 | Ww007 |  |

| 입력 |  | Name | 출력 |  | Name |
| --- | --- | --- | --- | --- | --- |
| Master | Remote |  | Master | Remote |  |
| X140 | X040 | 이상코드 1 | Y140 | Y040 |  |
| X141 | X041 | 이상코드 2 | Y141 | Y041 |  |
| X142 | X042 | 이상코드 4 | Y142 | Y042 |  |
| X143 | X043 | 이상코드 8 | Y143 | Y043 |  |
| X144 | X044 | 이상코드 16 | Y144 | Y044 |  |
| X145 | X045 | 이상코드 32 | Y145 | Y045 |  |
| X146 | X046 | 이상코드 64 | Y146 | Y046 |  |
| X147 | X047 |  | Y147 | Y047 |  |
| X148 | X048 | 원점복귀동작중 | Y148 | Y048 |  |
| X149 | X049 |  | Y149 | Y049 |  |
| X14A | X04A |  | Y14A | Y04A |  |
| X14B | X04B |  | Y14B | Y04B |  |
| X14C | X04C |  | Y14C | Y04C |  |
| X14D | X04D |  | Y14D | Y04D |  |
| X14E | X04E |  | Y14E | Y04E |  |
| X14F | X04F |  | Y14F | Y04F |  |
| X150 | X050 | 로봇이상 1 | Y150 | Y050 |  |
| X151 | X051 | 로봇이상 2 | Y151 | Y051 |  |
| X152 | X052 | 로봇이상 4 | Y152 | Y052 |  |
| X153 | X053 | 로봇이상 8 | Y153 | Y053 |  |
| X154 | X054 | 로봇이상 16 | Y154 | Y054 |  |
| X155 | X055 | 로봇이상 32 | Y155 | Y055 |  |
| X156 | X056 | 로봇이상 64 | Y156 | Y056 |  |
| X157 | X057 | 로보이상 128 | Y157 | Y057 |  |
| X158 | X058 |  | Y158 | Y058 |  |
| X159 | X059 |  | Y159 | Y059 |  |
| X15A | X05A |  | Y15A | Y05A |  |
| X15B | X05B |  | Y15B | Y05B |  |
| X15C | X05C |  | Y15C | Y05C |  |
| X15D | X05D |  | Y15D | Y05D |  |
| X15E | X05E |  | Y15E | Y05E |  |
| X15F | X05F |  | Y15F | Y05F |  |
| Wr028 | Wr008 |  | Ww028 | Ww008 |  |
| Wr029 | Wr009 |  | Ww029 | Ww009 |  |
| Wr02A | Wr00A |  | Ww02A | Ww00A |  |
| Wr02B | Wr00B |  | Ww02B | Ww00B |  |

| 입력 |  | Name | 출력 |  | Name |
| --- | --- | --- | --- | --- | --- |
| Master | Remote |  | Master | Remote |  |
| X160 | X060 |  | Y160 | Y060 |  |
| X161 | X061 |  | Y161 | Y061 |  |
| X162 | X062 |  | Y162 | Y062 |  |
| X163 | X063 |  | Y163 | Y063 |  |
| X164 | X064 |  | Y164 | Y064 |  |
| X165 | X065 |  | Y165 | Y065 |  |
| X166 | X066 |  | Y166 | Y066 |  |
| X167 | X067 |  | Y167 | Y067 |  |
| X168 | X068 |  | Y168 | Y068 |  |
| X169 | X069 |  | Y169 | Y069 |  |
| X16A | X06A |  | Y16A | Y06A |  |
| X16B | X06B |  | Y16B | Y06B |  |
| X16C | X06C |  | Y16C | Y06C |  |
| X16D | X06D |  | Y16D | Y06D |  |
| X16E | X06E |  | Y16E | Y06E |  |
| X16F | X06F |  | Y16F | Y06F |  |
| X170 | X070 |  | Y170 | Y070 |  |
| X171 | X071 |  | Y171 | Y071 |  |
| X172 | X072 |  | Y172 | Y072 |  |
| X173 | X073 |  | Y173 | Y073 |  |
| X174 | X074 |  | Y174 | Y074 |  |
| X175 | X075 |  | Y175 | Y075 |  |
| X176 | X076 |  | Y176 | Y076 |  |
| X177 | X077 |  | Y177 | Y077 |  |
| X178 | X078 |  | Y178 | Y078 |  |
| X179 | X079 |  | Y179 | Y079 |  |
| X17A | X07A |  | Y17A | Y07A |  |
| X17B | X07B |  | Y17B | Y07B |  |
| X17C | X07C |  | Y17C | Y07C |  |
| X17D | X07D |  | Y17D | Y07D |  |
| X17E | X07E |  | Y17E | Y07E |  |
| X17F | X07F |  | Y17F | Y07F |  |
| Wr02C | Wr00C |  | Ww02C | Ww00C |  |
| Wr02D | Wr00D |  | Ww02D | Ww00D |  |
| Wr02E | Wr00E |  | Ww02E | Ww00E |  |
| Wr02F | Wr00F |  | Ww02F | Ww00F |  |

| Station No. |  | 6 |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Station Type |  | Intelligent Device |  |  |  |
| 용 도 |  | Robot #2 (XRC Yaskawa 기준) |  |  |  |
| 입력 |  | Name | 출력 |  | Name |
| Master | Remote |  | Master | Remote |  |
| X100 | X000 | Running | Y100 | Y000 |  |
| X101 | X001 | Servo ON | Y101 | Y001 |  |
| X102 | X002 | Alarm/Error 발생 中 | Y102 | Y002 | Alarm/Error Reset |
| X103 | X003 | Remote Mode | Y103 | Y003 | External Start |
| X104 | X004 | Teach Mode | Y104 | Y004 |  |
| X105 | X005 | Play Mode | Y105 | Y005 | Remote Mode Select |
| X106 | X006 | Holding | Y106 | Y006 | Teach Mode Select |
| X107 | X007 | Top Master Job | Y107 | Y007 | Master Job Call |
| X108 | X008 | 작업 원점 | Y108 | Y008 | Play Mode Select |
| X109 | X009 | Arm 원점 (L/R) | Y109 | Y009 |  |
| X10A | X00A | R Hand Glass OK | Y10A | Y00A |  |
| X10B | X00B | L Hand Glass OK | Y10B | Y00B |  |
| X10C | X00C | R Hand 흡착 ON | Y10C | Y00C |  |
| X10D | X00D | L Hand 흡착 ON | Y10D | Y00D |  |
| X10E | X00E | 비상 정지 | Y10E | Y00E |  |
| X10F | X00F | Teach Lock on | Y10F | Y00F |  |
| X110 | X010 | 작업 완료 출력 | Y110 | Y010 | 동작 Command Start |
| X111 | X011 | Loading Complete | Y111 | Y011 | 단(#1,#2) Shutter 인터락 |
| X112 | X012 | Unloading Complete | Y112 | Y012 | 단(#3,#4) Shutter 인터락 |
| X113 | X013 |  | Y113 | Y013 |  |
| X114 | X014 | 단/다 Stage 확인 | Y114 | Y014 | 단/다 Stage 선택 |
| X115 | X015 | Command 실행 中 | Y115 | Y015 | Glass 검출 무효 |
| X116 | X016 | 이상코드 Strobe | Y116 | Y016 | 이상코드 ACK |
| X117 | X017 | 상/하 Hand 확인 | Y117 | Y017 | 상/하 Hand 지정 |
| X118 | X018 | 실행 中 Command 1 | Y118 | Y018 | Command 지정 1 |
| X119 | X019 | 실행 中 Command 2 | Y119 | Y019 | Command 지정 2 |
| X11A | X01A | 실행 中 Command 4 | Y11A | Y01A | Command 지정 4 |
| X11B | X01B | 실행 中 Command 8 | Y11B | Y01B | Command 지정 8 |
| X11C | X01C | 실행 中 Command 16 | Y11C | Y01C | Command 지정 16 |
| X11D | X01D |  | Y11D | Y01D |  |
| X11E | X01E |  | Y11E | Y01E |  |
| X11F | X01F |  | Y11F | Y01F |  |
| Wr020 | Wr000 |  | Ww020 | Ww000 |  |
| Wr021 | Wr001 |  | Ww021 | Ww001 |  |
| Wr022 | Wr002 |  | Ww022 | Ww002 |  |
| Wr023 | Wr003 |  | Ww023 | Ww003 |  |

| 입력 |  | Name | 출력 |  | Name |
| --- | --- | --- | --- | --- | --- |
| Master | Remote |  | Master | Remote |  |
| X120 | X020 | 실행 中 Address 1 | Y120 | Y020 | Address 지정 1 |
| X121 | X021 | 실행 中 Address 2 | Y121 | Y021 | Address 지정 2 |
| X122 | X022 | 실행 中 Address 4 | Y122 | Y022 | Address 지정 4 |
| X123 | X023 | 실행 中 Address 8 | Y123 | Y023 | Address 지정 8 |
| X124 | X024 | 실행 中 Address 16 | Y124 | Y024 | Address 지정 16 |
| X125 | X025 |  | Y125 | Y025 |  |
| X126 | X026 |  | Y126 | Y026 |  |
| X127 | X027 |  | Y127 | Y027 |  |
| X128 | X028 | 실행 中 위치 코드 1 | Y128 | Y028 | 위치 코드 1 |
| X129 | X029 | 실행 中 위치 코드 2 | Y129 | Y029 | 위치 코드 2 |
| X12A | X02A | 실행 中 위치 코드 4 | Y12A | Y02A | 위치 코드 4 |
| X12B | X02B |  | Y12B | Y02B |  |
| X12C | X02C |  | Y12C | Y02C |  |
| X12D | X02D |  | Y12D | Y02D |  |
| X12E | X02E |  | Y12E | Y02E |  |
| X12F | X02F |  | Y12F | Y02F |  |
| X130 | X030 | Glass 두께 확인 1 | Y130 | Y030 | Glass 두께 지정 1 |
| X131 | X031 | Glass 두께 확인 2 | Y131 | Y031 | Glass 두께 지정 2 |
| X132 | X032 | Glass 두께 확인 4 | Y132 | Y032 | Glass 두께 지정 4 |
| X133 | X033 |  | Y133 | Y033 |  |
| X134 | X034 |  | Y134 | Y034 |  |
| X135 | X035 |  | Y135 | Y035 |  |
| X136 | X036 |  | Y136 | Y036 |  |
| X137 | X037 |  | Y137 | Y037 |  |
| X138 | X038 |  | Y138 | Y038 |  |
| X139 | X039 |  | Y139 | Y039 |  |
| X13A | X03A |  | Y13A | Y03A |  |
| X13B | X03B |  | Y13B | Y03B |  |
| X13C | X03C |  | Y13C | Y03C |  |
| X13D | X03D |  | Y13D | Y03D |  |
| X13E | X03E |  | Y13E | Y03E |  |
| X13F | X03F |  | Y13F | Y03F |  |
| Wr024 | Wr004 |  | Ww024 | Ww004 |  |
| Wr025 | Wr005 |  | Ww025 | Ww005 |  |
| Wr026 | Wr006 |  | Ww026 | Ww006 |  |
| Wr027 | Wr007 |  | Ww027 | Ww007 |  |

| 입력 |  | Name | 출력 |  | Name |
| --- | --- | --- | --- | --- | --- |
| Master | Remote |  | Master | Remote |  |
| X140 | X040 | 이상 코드 1 | Y140 | Y040 |  |
| X141 | X041 | 이상 코드 2 | Y141 | Y041 |  |
| X142 | X042 | 이상 코드 4 | Y142 | Y042 |  |
| X143 | X043 | 이상 코드 8 | Y143 | Y043 |  |
| X144 | X044 | 이상 코드 16 | Y144 | Y044 |  |
| X145 | X045 |  | Y145 | Y045 |  |
| X146 | X046 |  | Y146 | Y046 |  |
| X147 | X047 |  | Y147 | Y047 |  |
| X148 | X048 |  | Y148 | Y048 |  |
| X149 | X049 |  | Y149 | Y049 |  |
| X14A | X04A |  | Y14A | Y04A |  |
| X14B | X04B |  | Y14B | Y04B |  |
| X14C | X04C |  | Y14C | Y04C |  |
| X14D | X04D |  | Y14D | Y04D |  |
| X14E | X04E |  | Y14E | Y04E |  |
| X14F | X04F |  | Y14F | Y04F |  |
| X150 | X050 |  | Y150 | Y050 |  |
| X151 | X051 |  | Y151 | Y051 |  |
| X152 | X052 |  | Y152 | Y052 |  |
| X153 | X053 |  | Y153 | Y053 |  |
| X154 | X054 |  | Y154 | Y054 |  |
| X155 | X055 |  | Y155 | Y055 |  |
| X156 | X056 |  | Y156 | Y056 |  |
| X157 | X057 |  | Y157 | Y057 |  |
| X158 | X058 |  | Y158 | Y058 |  |
| X159 | X059 |  | Y159 | Y059 |  |
| X15A | X05A |  | Y15A | Y05A |  |
| X15B | X05B |  | Y15B | Y05B |  |
| X15C | X05C |  | Y15C | Y05C |  |
| X15D | X05D |  | Y15D | Y05D |  |
| X15E | X05E |  | Y15E | Y05E |  |
| X15F | X05F |  | Y15F | Y05F |  |
| Wr028 | Wr008 |  | Ww028 | Ww008 |  |
| Wr029 | Wr009 |  | Ww029 | Ww009 |  |
| Wr02A | Wr00A |  | Ww02A | Ww00A |  |
| Wr02B | Wr00B |  | Ww02B | Ww00B |  |

| 입력 |  | Name | 출력 |  | Name |
| --- | --- | --- | --- | --- | --- |
| Master | Remote |  | Master | Remote |  |
| X160 | X060 |  | Y160 | Y060 |  |
| X161 | X061 |  | Y161 | Y061 |  |
| X162 | X062 |  | Y162 | Y062 |  |
| X163 | X063 |  | Y163 | Y063 |  |
| X164 | X064 |  | Y164 | Y064 |  |
| X165 | X065 |  | Y165 | Y065 |  |
| X166 | X066 |  | Y166 | Y066 |  |
| X167 | X067 |  | Y167 | Y067 |  |
| X168 | X068 |  | Y168 | Y068 |  |
| X169 | X069 |  | Y169 | Y069 |  |
| X16A | X06A |  | Y16A | Y06A |  |
| X16B | X06B |  | Y16B | Y06B |  |
| X16C | X06C |  | Y16C | Y06C |  |
| X16D | X06D |  | Y16D | Y06D |  |
| X16E | X06E |  | Y16E | Y06E |  |
| X16F | X06F |  | Y16F | Y06F |  |
| X170 | X070 |  | Y170 | Y070 |  |
| X171 | X071 |  | Y171 | Y071 |  |
| X172 | X072 |  | Y172 | Y072 |  |
| X173 | X073 |  | Y173 | Y073 |  |
| X174 | X074 |  | Y174 | Y074 |  |
| X175 | X075 |  | Y175 | Y075 |  |
| X176 | X076 |  | Y176 | Y076 |  |
| X177 | X077 |  | Y177 | Y077 |  |
| X178 | X078 |  | Y178 | Y078 |  |
| X179 | X079 |  | Y179 | Y079 |  |
| X17A | X07A |  | Y17A | Y07A |  |
| X17B | X07B |  | Y17B | Y07B |  |
| X17C | X07C |  | Y17C | Y07C |  |
| X17D | X07D |  | Y17D | Y07D |  |
| X17E | X07E |  | Y17E | Y07E |  |
| X17F | X07F |  | Y17F | Y07F |  |
| Wr02C | Wr00C |  | Ww02C | Ww00C |  |
| Wr02D | Wr00D |  | Ww02D | Ww00D |  |
| Wr02E | Wr00E |  | Ww02E | Ww00E |  |
| Wr02F | Wr00F |  | Ww02F | Ww00F |  |

| Station No. |  | 11 |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Station Type |  | Intelligent Device |  |  |  |
| 용 도 |  | Robot #3 (Robostar) |  |  |  |
| 입력 |  | Name | 출력 |  | Name |
| Master | Remote |  | Master | Remote |  |
| X200 | X000 | 상 Hand 상면 (Upper Position) Front 흡착 On | Y200 | Y000 |  |
| X201 | X001 | 상 Hand 상면 (Upper Position) Back 흡착 On | Y201 | Y001 |  |
| X202 | X002 | 하 Hand 상면 (Upper Position) Front 흡착 On | Y202 | Y002 |  |
| X203 | X003 | 하 Hand 상면 (Upper Position) Back 흡착 On | Y203 | Y003 |  |
| X204 | X004 | 상 Hand 상면 (Upper Position) Front Glass 검출 | Y204 | Y004 |  |
| X205 | X005 | 상 Hand 상면 (Upper Position) Back Glass 검출 | Y205 | Y005 |  |
| X206 | X006 | 하 Hand 상면 (Upper Position) Front Glass 검출 | Y206 | Y006 |  |
| X207 | X007 | 하 Hand 상면 (Upper Position) Back Glass 검출 | Y207 | Y007 |  |
| X208 | X008 | 상 Hand 상면 (Upper Position) 흡착 On | Y208 | Y008 |  |
| X209 | X009 | 하 Hand 상면 (Upper Position) 흡착 On | Y209 | Y009 |  |
| X20A | X00A | Spare | Y20A | Y00A |  |
| X20B | X00B | Spare | Y20B | Y00B |  |
| X20C | X00C | 상 Hand 상면 (Upper Position) Glass 검출 | Y20C | Y00C |  |
| X20D | X00D | 하 Hand 상면 (Upper Position) Glass 검출 | Y20D | Y00D |  |
| X20E | X00E | Spare | Y20E | Y00E |  |
| X20F | X00F | Spare | Y20F | Y00F |  |
| X210 | X010 | Spare | Y210 | Y010 |  |
| X211 | X011 | Spare | Y211 | Y011 |  |
| X212 | X012 | Spare | Y212 | Y012 |  |
| X213 | X013 | Spare | Y213 | Y013 |  |
| X214 | X014 | Spare | Y214 | Y014 |  |
| X215 | X015 | Spare | Y215 | Y015 |  |
| X216 | X016 | Spare | Y216 | Y016 |  |
| X217 | X017 | GLASS 전달 요청 | Y217 | Y017 | GLASS 전달 |
| X218 | X018 | RETURN Override 1 | Y218 | Y018 | Override 1 |
| X219 | X019 | RETURN Override 2 | Y219 | Y019 | Override 2 |
| X21A | X01A | RETURN Override 4 | Y21A | Y01A | Override 4 |
| X21B | X01B | RETURN Override 8 | Y21B | Y01B | Override 8 |
| X21C | X01C | RETURN Override 16 | Y21C | Y01C | Override 16 |
| X21D | X01D | RETURN Override 32 | Y21D | Y01D | Override 32 |
| X21E | X01E | RETURN Override 64 | Y21E | Y01E | Override 64 |
| X21F | X01F |  | Y21F | Y01F |  |
| X220 | X020 | ROBOT 1축 MOVING 중 | Y220 | Y020 |  |
| X221 | X021 | ROBOT 2축 MOVING 중 | Y221 | Y021 |  |
| X222 | X022 | ROBOT 3축 MOVING 중 | Y222 | Y022 |  |
| X223 | X023 | ROBOT 4축 MOVING 중 | Y223 | Y023 |  |
| X224 | X024 | ROBOT 5축 MOVING 중 | Y224 | Y024 |  |
| X225 | X025 | ROBOT 6축 MOVING 중 | Y225 | Y025 |  |
| X226 | X026 | 하 HAND ALIGNMENT 완료 | Y226 | Y026 |  |
| X227 | X027 | 상 HAND ALIGNMENT 완료 | Y227 | Y027 |  |
| X228 | X028 | RETURN Upper Sub Position 1 | Y228 | Y028 | Upper Sub Position 1 |
| X229 | X029 | RETURN Upper Sub Position 2 | Y229 | Y029 | Upper Sub Position 2 |
| X22A | X02A | RETURN Upper Sub Position 4 | Y22A | Y02A | Upper Sub Position 4 |
| X22B | X02B | RETURN Lower Sub Position 1 | Y22B | Y02B | Lower Sub Position 1 |
| X22C | X02C | RETURN Lower Sub Position 2 | Y22C | Y02C | Lower Sub Position 2 |
| X22D | X02D | RETURN Lower Sub Position 4 | Y22D | Y02D | Lower Sub Position 4 |
| X22E | X02E | 배터리 전압 저하 | Y22E | Y02E | Spare |
| X22F | X02F | 핸드 진입요청 | Y22F | Y02F | 핸드 진입요청 응답 |
| X230 | X030 | RETURN COMMAND 1 | Y230 | Y030 | COMMAND 1 |
| X231 | X031 | RETURN COMMAND 2 | Y231 | Y031 | COMMAND 2 |
| X232 | X032 | RETURN COMMAND 4 | Y232 | Y032 | COMMAND 4 |
| X233 | X033 | RETURN COMMAND 8 | Y233 | Y033 | COMMAND 8 |
| X234 | X034 | RETURN COMMAND 16 | Y234 | Y034 | COMMAND 16 |
| X235 | X035 | RETURN COMMAND 32 | Y235 | Y035 | COMMAND 32 |
| X236 | X036 | RETURN COMMAND 64 | Y236 | Y036 | COMMAND 64 |
| X237 | X037 | Spare | Y237 | Y037 |  |
| Wr048 | Wr000 | Return 1st POSITION MOTION | Ww048 | Ww000 | 1st POSITION MOTION (STAGE, SLOT NO) |
| Wr049 | Wr001 | Return 2nd POSITION MOTION | Ww049 | Ww001 | 2nd POSITION MOTION (STAGE, SLOT NO) |
| Wr04A | Wr002 | Return 3rd POSITION MOTION | Ww04A | Ww002 | 3rd POSITION MOTION (STAGE, SLOT NO) |
| Wr04B | Wr003 | 로봇이상코드 (SYSTEM) | Ww04B | Ww003 |  |
| Wr04C | Wr004 | 1(X), 2(TH) 축 부하율 (x200%) | Ww04C | Ww004 |  |
| Wr04D | Wr005 | 3(Z1), 4(Z2) 축 부하율 (x200%) | Ww04D | Ww005 |  |
| Wr04E | Wr006 | 5(L), 6(R) 축 부하율 (x200%) | Ww04E | Ww006 |  |
| Wr04F | Wr007 | TH ALIGN, 주행 ALIGN 보정값 | Ww04F | Ww007 |  |

| 입력 |  | Name | 출력 |  | Name |
| --- | --- | --- | --- | --- | --- |
| Master | Remote |  | Master | Remote |  |
| X238 | X038 | RETURN GLSS size data 1 | Y238 | Y038 | GLSS size data 1 |
| X239 | X039 | RETURN GLSS size data 2 | Y239 | Y039 | GLSS size data 2 |
| X23A | X03A | RETURN GLSS size data 4 | Y23A | Y03A | GLSS size data 4 |
| X23B | X03B | RETURN GLSS size data 8 | Y23B | Y03B | GLSS size data 8 |
| X23C | X03C | RETURN GLSS size data 16 | Y23C | Y03C | GLSS size data 16 |
| X23D | X03D | RETURN GLSS size data 32 | Y23D | Y03D | GLSS size data 32 |
| X23E | X03E | Spare | Y23E | Y03E | Spare |
| X23F | X03F | Spare | Y23F | Y03F | Spare |
| X240 | X040 | RETURN GLSS Think data 1 | Y240 | Y040 | GLSS Think data 1 |
| X241 | X041 | RETURN GLSS Think data 2 | Y241 | Y041 | GLSS Think data 2 |
| X242 | X042 | RETURN GLSS Think data 4 | Y242 | Y042 | GLSS Think data 4 |
| X243 | X043 | Spare | Y243 | Y043 | Spare |
| X244 | X044 | RETURN 1st 반송 구분 (단:0,다:1) | Y244 | Y044 | 1st 반송 구분 (단:0,다:1) |
| X245 | X045 | RETURN 1st CST code 1 (30 단) | Y245 | Y045 | 1st CST code 1 (30 단) |
| X246 | X046 | RETURN 1st CST code 2 (RESERVE) | Y246 | Y046 | 1st CST code 2 (RESERVE) |
| X247 | X047 | Spare | Y247 | Y047 | Spare |
| X248 | X048 | RETURN 2nd 반송구분 (단:0,다:1) | Y248 | Y048 | 2nd 반송구분 (단:0,다:1) |
| X249 | X049 | RETURN 2nd CST code 1 (30 단) | Y249 | Y049 | 2nd CST code 1 (30 단) |
| X24A | X04A | RETURN 2nd CST code 2 (RESERVE) | Y24A | Y04A | 2nd CST code 2 (RESERVE) |
| X24B | X04B | Spare | Y24B | Y04B | Spare |
| X24C | X04C | RETURN 3rd 반송 구분 (단:0,다:1) | Y24C | Y04C | 3rd 반송 구분 (단:0,다:1) |
| X24D | X04D | RETURN 3rd CST code 1 (30 단) | Y24D | Y04D | 3rd CST code 1 (30 단) |
| X24E | X04E | RETURN 3rd CST code 2 (RESERVE) | Y24E | Y04E | 3rd CST code 2 (RESERVE) |
| X24F | X04F | Spare | Y24F | Y04F | Spare |
| X250 | X050 | COMMAND ALARM CODE 1 | Y250 | Y050 | SERVO ON |
| X251 | X051 | COMMAND ALARM CODE 2 | Y251 | Y051 | EXTERNAL START |
| X252 | X052 | COMMAND ALARM CODE 4 | Y252 | Y052 | COMMAND 기동 |
| X253 | X053 | COMMAND ALARM CODE 8 | Y253 | Y053 | ROBOT 원점 복귀 |
| X254 | X054 | COMMAND ALARM CODE 16 | Y254 | Y054 | VACUUM Position |
| X255 | X055 | COMMAND ALARM CODE 32 | Y255 | Y055 | ROBOT HAND (상:0,하:1) |
| X256 | X056 | COMMAND ALARM CODE 64 | Y256 | Y056 | ROBOT HOLD |
| X257 | X057 | ALARM STROB | Y257 | Y057 | ROBOT HOLD 해제 |
| X258 | X058 | SERVO ON 中 | Y258 | Y058 | ALARM RESET |
| X259 | X059 | RETURN 반송 핸드 (상:0,하:1) | Y259 | Y059 | 비상정지 |
| X25A | X05A | RETURN DUAL HAND | Y25A | Y05A | 이상코드 ACK |
| X25B | X05B | Command 정상 완료 | Y25B | Y05B | Spare |
| X25C | X05C | HAND 원점 | Y25C | Y05C | DUAL HAND |
| X25D | X05D | ROBOT HOME 위치 | Y25D | Y05D | REMOTE MODE |
| X25E | X05E | ROBOT HOLD 中 | Y25E | Y05E | TEACH MODE |
| X25F | X05F | 비상정지 (EMO 줄력 中) | Y25F | Y05F | PROGRAM JOB SELECT |
| X260 | X060 | RETURN REMOTE MODE | Y260 | Y060 | COMMAND 일치 응답 |
| X261 | X061 | RETURN TEACH MODE | Y261 | Y061 | COMMAND 불일치 응답 |
| X262 | X062 | RETURN VACUUM POSITION | Y262 | Y062 | COMMAND 완료 ACK |
| X263 | X063 | COMMAND 일치 확인요청 | Y263 | Y063 | Spare |
| X264 | X064 | RETURN SIMULATION MODE | Y264 | Y064 | SIMULATION MODE |
| X265 | X065 | COMMAND READY | Y265 | Y065 | GLASS 검출 무효 |
| X266 | X066 | ROBOT 동작 中 | Y266 | Y066 |  |
| X267 | X067 | DRAIN 동작 실행 중 | Y267 | Y067 |  |
| X268 | X068 | Forward Position 복귀 완료 | Y268 | Y068 |  |
| X269 | X069 | 수납완료 | Y269 | Y069 |  |
| X26A | X06A | 취출완료 | Y26A | Y06A |  |
| X26B | X06B | COMMAND ALARM Strobe (JOB ALARM) | Y26B | Y06B |  |
| X26C | X06C | ROBOT ALARM Strobe (SYSTEM ALARM) | Y26C | Y06C |  |
| X26D | X06D | PSEL Complete (MASTER JOB Complete) | Y26D | Y06D |  |
| X26E | X06E | PROGRAM RUN (MASTER JOB 구동중) | Y26E | Y06E |  |
| X26F | X06F | PROGRAM READY (MASTER JOB READY) | Y26F | Y06F |  |
| Wr050 | Wr008 | 1축 누적거리(rev) | Ww050 | Ww008 |  |
| Wr051 | Wr009 | 2축 누적거리(Km) | Ww051 | Ww009 |  |
| Wr052 | Wr00A | 3축 누적거리(Km) | Ww052 | Ww00A |  |
| Wr053 | Wr00B | 4축 누적거리(Km) | Ww053 | Ww00B |  |
| Wr054 | Wr00C | 5축 누적거리(Km) | Ww054 | Ww00C |  |
| Wr055 | Wr00D | 6축 누적거리(Km) | Ww055 | Ww00D |  |
| Wr056 | Wr00E | 하핸드 좌(8bit)-1/우(8bit)-2 흡착 압력(0~100kpa) | Ww056 | Ww00E |  |
| Wr057 | Wr00F | 상핸드 좌(8bit)-3/우(8bit)-4 흡착 압력(0~100kpa) | Ww057 | Ww00F |  |

| 입력 |  | Name | 출력 |  | Name |
| --- | --- | --- | --- | --- | --- |
| Master | Remote |  | Master | Remote |  |
| X270 | X070 |  | Y270 | Y070 |  |
| X271 | X071 |  | Y271 | Y071 |  |
| X272 | X072 |  | Y272 | Y072 |  |
| X273 | X073 |  | Y273 | Y073 |  |
| X274 | X074 |  | Y274 | Y074 |  |
| X275 | X075 |  | Y275 | Y075 |  |
| X276 | X076 |  | Y276 | Y076 |  |
| X277 | X077 |  | Y277 | Y077 |  |
| X278 | X078 |  | Y278 | Y078 |  |
| X279 | X079 |  | Y279 | Y079 |  |
| X27A | X07A |  | Y27A | Y07A |  |
| X27B | X07B |  | Y27B | Y07B |  |
| X27C | X07C |  | Y27C | Y07C |  |
| X27D | X07D |  | Y27D | Y07D |  |
| X27E | X07E |  | Y27E | Y07E |  |
| X27F | X07F |  | Y27F | Y07F |  |
| X280 | X080 |  | Y280 | Y080 |  |
| X281 | X081 |  | Y281 | Y081 |  |
| X282 | X082 |  | Y282 | Y082 |  |
| X283 | X083 |  | Y283 | Y083 |  |
| X284 | X084 |  | Y284 | Y084 |  |
| X285 | X085 |  | Y285 | Y085 |  |
| X286 | X086 |  | Y286 | Y086 |  |
| X287 | X087 |  | Y287 | Y087 |  |
| X288 | X088 |  | Y288 | Y088 |  |
| X289 | X089 |  | Y289 | Y089 |  |
| X28A | X08A |  | Y28A | Y08A |  |
| X28B | X08B |  | Y28B | Y08B |  |
| X28C | X08C |  | Y28C | Y08C |  |
| X28D | X08D |  | Y28D | Y08D |  |
| X28E | X08E |  | Y28E | Y08E |  |
| X28F | X08F |  | Y28F | Y08F |  |
| X290 | X090 |  | Y290 | Y090 |  |
| X291 | X091 |  | Y291 | Y091 |  |
| X292 | X092 |  | Y292 | Y092 |  |
| X293 | X093 |  | Y293 | Y093 |  |
| X294 | X094 |  | Y294 | Y094 |  |
| X295 | X095 |  | Y295 | Y095 |  |
| X296 | X096 |  | Y296 | Y096 |  |
| X297 | X097 |  | Y297 | Y097 |  |
| X298 | X098 |  | Y298 | Y098 |  |
| X299 | X099 |  | Y299 | Y099 |  |
| X29A | X09A |  | Y29A | Y09A |  |
| X29B | X09B |  | Y29B | Y09B |  |
| X29C | X09C |  | Y29C | Y09C |  |
| X29D | X09D |  | Y29D | Y09D |  |
| X29E | X09E |  | Y29E | Y09E |  |
| X29F | X09F |  | Y29F | Y09F |  |
| X2A0 | X0A0 |  | Y2A0 | Y0A0 |  |
| X2A1 | X0A1 |  | Y2A1 | Y0A1 |  |
| X2A2 | X0A2 |  | Y2A2 | Y0A2 |  |
| X2A3 | X0A3 |  | Y2A3 | Y0A3 |  |
| X2A4 | X0A4 |  | Y2A4 | Y0A4 |  |
| X2A5 | X0A5 |  | Y2A5 | Y0A5 |  |
| X2A6 | X0A6 |  | Y2A6 | Y0A6 |  |
| X2A7 | X0A7 |  | Y2A7 | Y0A7 |  |
| Wr058 | Wr010 |  | Ww058 | Ww010 |  |
| Wr059 | Wr011 |  | Ww059 | Ww011 |  |
| Wr05A | Wr012 |  | Ww05A | Ww012 |  |
| Wr05B | Wr013 |  | Ww05B | Ww013 |  |
| Wr05C | Wr014 |  | Ww05C | Ww014 |  |
| Wr05D | Wr015 |  | Ww05D | Ww015 |  |
| Wr05E | Wr016 |  | Ww05E | Ww016 |  |
| Wr05F | Wr017 |  | Ww05F | Ww017 |  |

| 입력 |  | Name | 출력 |  | Name |
| --- | --- | --- | --- | --- | --- |
| Master | Remote |  | Master | Remote |  |
| X2A8 | X0A8 |  | Y2A8 | Y070 |  |
| X2A9 | X0A9 |  | Y2A9 | Y0A9 |  |
| X2AA | X0AA |  | Y2AA | Y0AA |  |
| X2AB | X0AB |  | Y2AB | Y0AB |  |
| X2AC | X0AC |  | Y2AC | Y0AC |  |
| X2AD | X0AD |  | Y2AD | Y0AD |  |
| X2AE | X0AE |  | Y2AE | Y0AE |  |
| X2AF | X0AF |  | Y2AF | Y0AF |  |
| X2B0 | X0B0 |  | Y2B0 | Y0B0 |  |
| X2B1 | X0B1 |  | Y2B1 | Y0B1 |  |
| X2B2 | X0B2 |  | Y2B2 | Y0B2 |  |
| X2B3 | X0B3 |  | Y2B3 | Y0B3 |  |
| X2B4 | X0B4 |  | Y2B4 | Y0B4 |  |
| X2B5 | X0B5 |  | Y2B5 | Y0B5 |  |
| X2B6 | X0B6 |  | Y2B6 | Y0B6 |  |
| X2B7 | X0B7 |  | Y2B7 | Y0B7 |  |
| X2B8 | X0B8 |  | Y2B8 | Y0B8 |  |
| X2B9 | X0B9 |  | Y2B9 | Y0B9 |  |
| X2BA | X0BA |  | Y2BA | Y0BA |  |
| X2BB | X0BB |  | Y2BB | Y0BB |  |
| X2BC | X0BC |  | Y2BC | Y0BC |  |
| X2BD | X0BD |  | Y2BD | Y0BD |  |
| X2BE | X0BE |  | Y2BE | Y0BE |  |
| X2BF | X0BF |  | Y2BF | Y0BF |  |
| X2C0 | X0C0 |  | Y2C0 | Y0C0 |  |
| X2C1 | X0C1 |  | Y2C1 | Y0C1 |  |
| X2C2 | X0C2 |  | Y2C2 | Y0C2 |  |
| X2C3 | X0C3 |  | Y2C3 | Y0C3 |  |
| X2C4 | X0C4 |  | Y2C4 | Y0C4 |  |
| X2C5 | X0C5 |  | Y2C5 | Y0C5 |  |
| X2C6 | X0C6 |  | Y2C6 | Y0C6 |  |
| X2C7 | X0C7 |  | Y2C7 | Y0C7 |  |
| X2C8 | X0C8 |  | Y2C8 | Y0C8 |  |
| X2C9 | X0C9 |  | Y2C9 | Y0C9 |  |
| X2CA | X0CA |  | Y2CA | Y0CA |  |
| X2CB | X0CB |  | Y2CB | Y0CB |  |
| X2CC | X0CC |  | Y2CC | Y0CC |  |
| X2CD | X0CD |  | Y2CD | Y0CD |  |
| X2CE | X0CE |  | Y2CE | Y0CE |  |
| X2CF | X0CF |  | Y2CF | Y0CF |  |
| X2D0 | X0D0 |  | Y2D0 | Y0D0 |  |
| X2D1 | X0D1 |  | Y2D1 | Y0D1 |  |
| X2D2 | X0D2 |  | Y2D2 | Y0D2 |  |
| X2D3 | X0D3 |  | Y2D3 | Y0D3 |  |
| X2D4 | X0D4 |  | Y2D4 | Y0D4 |  |
| X2D5 | X0D5 |  | Y2D5 | Y0D5 |  |
| X2D6 | X0D6 |  | Y2D6 | Y0D6 |  |
| X2D7 | X0D7 |  | Y2D7 | Y0D7 |  |
| X2D8 | X0D8 |  | Y2D8 | Y0D8 |  |
| X2D9 | X0D9 |  | Y2D9 | Y0D9 |  |
| X2DA | X0DA |  | Y2DA | Y0DA |  |
| X2DB | X0DB |  | Y2DB | Y0DB |  |
| X2DC | X0DC |  | Y2DC | Y0DC |  |
| X2DD | X0DD |  | Y2DD | Y0DD |  |
| X2DE | X0DE |  | Y2DE | Y0DE |  |
| X2DF | X0DF |  | Y2DF | Y0DF |  |
| Wr060 | Wr018 |  | Ww060 | Ww018 |  |
| Wr061 | Wr019 |  | Ww061 | Ww019 |  |
| Wr062 | Wr01A |  | Ww062 | Ww01A |  |
| Wr063 | Wr01B |  | Ww063 | Ww01B |  |
| Wr064 | Wr01C |  | Ww064 | Ww01C |  |
| Wr065 | Wr01D |  | Ww065 | Ww01D |  |
| Wr066 | Wr01E |  | Ww066 | Ww01E |  |
| Wr067 | Wr01F |  | Ww067 | Ww01F |  |

| Station No. |  |  | 16 |  |  |
| --- | --- | --- | --- | --- | --- |
| Station Type |  |  | Remote I/O(AJ65SBTB1-32DT1) |  |  |
| 용 도 |  |  | Port #1 ~ #5 Ionizer Error |  |  |
| 구분 | TB No. | Address | Name | 접점종류 | 비고 |
| CC-Link | TB01 |  | DA |  |  |
|  | TB02 |  | DB |  |  |
|  | TB03 |  | DG |  |  |
|  | TB04 |  | SLD |  |  |
|  | TB05 |  | DC 24V |  |  |
|  | TB06 |  | FG |  |  |
|  | TB07 |  | DC 0V |  |  |
| 입력 | TB08 | X300 | Port 1-1 Ionizer Error Report |  | Ionier 고전압 발생 저하 Error Report |
|  | TB09 | X301 | Port 1-2 Ionizer Error Report |  |  |
|  | TB10 | X302 | Port 2-1 Ionizer Error Report |  |  |
|  | TB11 | X303 | Port 2-2 Ionizer Error Report |  |  |
|  | TB12 | X304 | Port 3-1 Ionizer Error Report |  |  |
|  | TB13 | X305 | Port 3-2 Ionizer Error Report |  |  |
|  | TB14 | X306 | Port 4-1 Ionizer Error Report |  |  |
|  | TB15 | X307 | Port 4-2 Ionizer Error Report |  |  |
|  | TB16 | X308 | Port 5-1 Ionizer Error Report |  |  |
|  | TB17 | X309 | Port 5-2 Ionizer Error Report |  |  |
|  | TB18 | X30A |  |  |  |
|  | TB19 | X30B |  |  |  |
|  | TB20 | X30C | Spare |  |  |
|  | TB21 | X30D | EQP #1 L/L X-Ray Ionizer State On |  | EQP #1 X-Ray Ionizer State |
|  | TB22 | X30E | EQP #1 L/L X-Ray Ionizer Plasma State On |  |  |
|  | TB23 | X30F | EQP #1 L/L X-Ray Ionizer Voltage State On |  |  |
|  | TB24 | X310 | Port 1-1 Ionizer Error Report |  | Ionizer 방전 Tip Clean 주기 도달 Report |
|  | TB25 | X311 | Port 1-2 Ionizer Error Report |  |  |
|  | TB26 | X312 | Port 2-1 Ionizer Error Report |  |  |
|  | TB27 | X313 | Port 2-2 Ionizer Error Report |  |  |
|  | TB28 | X314 | Port 3-1 Ionizer Error Report |  |  |
|  | TB29 | X315 | Port 3-2 Ionizer Error Report |  |  |
|  | TB30 | X316 | Port 4-1 Ionizer Error Report |  |  |
|  | TB31 | X317 | Port 4-2 Ionizer Error Report |  |  |
|  | TB32 | X318 | Port 5-1 Ionizer Error Report |  |  |
|  | TB33 | X319 | Port 5-2 Ionizer Error Report |  |  |
|  | TB34 | X31A |  |  |  |
|  | TB35 | X31B |  |  |  |
|  | TB36 | X31C | Spare |  |  |
|  | TB37 | X31D | EQP #2 L/L X-Ray Ionizer State On |  | EQP #2 X-Ray Ionizer State |
|  | TB38 | X31E | EQP #2 L/L X-Ray Ionizer Plasma State On |  |  |
|  | TB39 | X31F | EQP #2 L/L X-Ray Ionizer Voltage State On |  |  |
| COM | TB40 | COM+ | DC 24V(P24) |  |  |
|  | TB41 | COM- | DC 0V(N24) |  |  |

| Station No. |  |  | 19 |  |  |
| --- | --- | --- | --- | --- | --- |
| Station Type |  |  | Remote I/O(AJ65SBTB1-32D1) |  |  |
| 용 도 |  |  | Loader TPD Input |  |  |
| 구분 | TB No. | Address | Name | 접점종류 | 비고 |
| CC-Link | TB01 |  | DA |  |  |
|  | TB02 |  | DB |  |  |
|  | TB03 |  | DG |  |  |
|  | TB04 |  | SLD |  |  |
|  | TB05 |  | DC 24V |  |  |
|  | TB06 |  | FG |  |  |
|  | TB07 |  | DC 0V |  |  |
| 입력 | TB08 | X360 |  |  | Loader |
|  | TB09 | X361 |  |  |  |
|  | TB10 | X362 |  |  |  |
|  | TB11 | X363 |  |  |  |
|  | TB12 | X364 |  |  |  |
|  | TB13 | X365 |  |  |  |
|  | TB14 | X366 |  |  |  |
|  | TB15 | X367 |  |  |  |
|  | TB16 | X368 |  |  |  |
|  | TB17 | X369 |  |  |  |
|  | TB18 | X36A |  |  |  |
|  | TB19 | X36B |  |  |  |
|  | TB20 | X36C |  |  |  |
|  | TB21 | X36D |  |  |  |
|  | TB22 | X36E |  |  |  |
|  | TB23 | X36F |  |  |  |
|  | TB24 | X370 |  |  |  |
|  | TB25 | X371 |  |  |  |
|  | TB26 | X372 |  |  |  |
|  | TB27 | X373 |  |  |  |
|  | TB28 | X374 |  |  |  |
|  | TB29 | X375 |  |  |  |
|  | TB30 | X376 |  |  |  |
|  | TB31 | X377 |  |  |  |
|  | TB32 | X378 |  |  |  |
|  | TB33 | X379 |  |  |  |
|  | TB34 | X37A |  |  |  |
|  | TB35 | X37B |  |  |  |
|  | TB36 | X37C |  |  |  |
|  | TB37 | X37D |  |  |  |
|  | TB38 | X37E |  |  |  |
|  | TB39 | X37F |  |  |  |
|  | Wr000 | Wr078 | 유량 압력값 CDA |  |  |
|  | Wr001 | Wr079 | 유량 압력값 IONIZER 1 ~ 4 |  |  |
|  | Wr002 | Wr07A | 유량 압력값 IONIZER 5 ~ 8 |  |  |
|  | Wr003 | Wr07B |  |  |  |
| COM | TB40 | COM+ | DC 24V(P24) |  |  |
|  | TB41 | COM- | DC 0V(N24) |  |  |

| Station No. |  |  | 20 |  |  |
| --- | --- | --- | --- | --- | --- |
| Station Type |  |  | Remote I/O(AJ65SBTB1-32DT1) |  |  |
| 용 도 |  |  | Robot, 공통 In/Out |  |  |
| 구분 | TB No. | Address | Name | 접점종류 | 비고 |
| CC-Link | TB01 |  | DA |  |  |
|  | TB02 |  | DB |  |  |
|  | TB03 |  | DG |  |  |
|  | TB04 |  | SLD |  |  |
|  | TB05 |  | DC 24V |  |  |
|  | TB06 |  | FG |  |  |
|  | TB07 |  | DC 0V |  |  |
| 입력 | TB08 | X380 | 비상정지 | b | Robot 1 |
|  | TB09 | X381 | EFU Down |  |  |
|  | TB10 | X382 | Maint Interlock |  |  |
|  | TB11 | X383 | DOOR-SENSOR ( Safety Relay ) | b |  |
|  | TB12 | X384 | EQP #1 비상정지 | b |  |
|  | TB13 | X385 | EQP #2 비상정지 | b |  |
|  | TB14 | X386 | EQP #3 비상정지 | b |  |
|  | TB15 | X387 | EQP #4 비상정지 | b |  |
|  | TB16 | X388 | 비상정지 | b | Robot 2 |
|  | TB17 | X389 |  |  |  |
|  | TB18 | X38A |  |  |  |
|  | TB19 | X38B | DOOR-SENSOR ( Safety Relay ) | b |  |
|  | TB20 | X38C | EQP #1 비상정지 | b |  |
|  | TB21 | X38D | EQP #2 비상정지 | b |  |
|  | TB22 | X38E | EQP #3 비상정지 | b |  |
|  | TB23 | X38F | EQP #4 비상정지 | b |  |
| 출력 | TB24 | Y390 | Robot 외부 Servo On |  | Robot 1 |
|  | TB25 | Y391 | Robot 외부 비상정지 (EMS 케이블) | b |  |
|  | TB26 | Y392 | Robot 외부 Hold (사용안함) | b |  |
|  | TB27 | Y393 | Robot 외부 Safety Plug (EMS 케이블) | b |  |
|  | TB28 | Y394 | Door Lock 해제 |  |  |
|  | TB29 | Y395 | EMO Safety Relay Reset(Inline Loader) |  |  |
|  | TB30 | Y396 | Door Lock Safety Relay Reset(Inline Loader) |  |  |
|  | TB31 | Y397 | Robot 외부 Servo On |  | Robot 2 |
|  | TB32 | Y398 | Robot 외부 비상정지 (EMS 케이블) | b |  |
|  | TB33 | Y399 | Robot 외부 Hold(사용안함) | b |  |
|  | TB34 | Y39A | Robot 외부 Safety Plug (EMS 케이블) | b |  |
|  | TB35 | Y39B | Door Lock 해제 |  |  |
|  | TB36 | Y39C | EMO Safety Relay Reset(Inline Loader) |  |  |
|  | TB37 | Y39D | Door Lock Safety Relay Reset(Inline Loader) |  |  |
|  | TB38 | Y39E | Buzzer On |  |  |
|  | TB39 | Y39F | Melody On |  |  |
| COM | TB40 | COM+ | DC 24V(P24) |  |  |
|  | TB41 | COM- | DC 0V(N24) |  |  |

| Station No. |  |  | 21 |  |  |
| --- | --- | --- | --- | --- | --- |
| Station Type |  |  | Remote I/O(AJ65SBTB1-32DT1) |  |  |
| 용 도 |  |  | Robot, 공통 In/Out |  |  |
| 구분 | TB No. | Address | Name | 접점종류 | 비고 |
| CC-Link | TB01 |  | DA |  |  |
|  | TB02 |  | DB |  |  |
|  | TB03 |  | DG |  |  |
|  | TB04 |  | SLD |  |  |
|  | TB05 |  | DC 24V |  |  |
|  | TB06 |  | FG |  |  |
|  | TB07 |  | DC 0V |  |  |
| 입력 | TB08 | X3A0 | 비상정지 | b | Robot 3 |
|  | TB09 | X3A1 |  |  |  |
|  | TB10 | X3A2 |  |  |  |
|  | TB11 | X3A3 | DOOR-SENSOR ( Safety Relay ) | b |  |
|  | TB12 | X3A4 | EQP #1 비상정지 | b |  |
|  | TB13 | X3A5 | EQP #2 비상정지 | b |  |
|  | TB14 | X3A6 | EQP #3 비상정지 | b |  |
|  | TB15 | X3A7 | EQP #4 비상정지 | b |  |
|  | TB16 | X3A8 | EQP #1 X-Ray Ionizer Down Report | b | X-Ray Ionizer Down Report |
|  | TB17 | X3A9 | EQP #2 X-Ray Ionizer Down Report | b |  |
|  | TB18 | X3AA | EQP #3 X-Ray Ionizer Down Report | b |  |
|  | TB19 | X3AB | EQP #4 X-Ray Ionizer Down Report | b |  |
|  | TB20 | X3AC | EQP #5 X-Ray Ionizer Down Report | b |  |
|  | TB21 | X3AD | EQP #6 X-Ray Ionizer Down Report | b |  |
|  | TB22 | X3AE |  |  |  |
|  | TB23 | X3AF |  |  |  |
| 출력 | TB24 | Y3B0 | Robot 외부 Servo On |  | Robot 3 |
|  | TB25 | Y3B1 | Robot 외부 비상정지 (EMS 케이블) | b |  |
|  | TB26 | Y3B2 | Robot 외부 Hold (사용안함) | b |  |
|  | TB27 | Y3B3 | Robot 외부 Safety Plug (EMS 케이블) | b |  |
|  | TB28 | Y3B4 | Door Lock 해제 |  |  |
|  | TB29 | Y3B5 | EMO Safety Relay Reset(Inline Loader) |  |  |
|  | TB30 | Y3B6 | Door Lock Safety Relay Reset(Inline Loader) |  |  |
|  | TB31 | Y3B7 |  |  |  |
|  | TB32 | Y3B8 |  |  |  |
|  | TB33 | Y3B9 |  |  |  |
|  | TB34 | Y3BA |  |  |  |
|  | TB35 | Y3BB |  |  |  |
|  | TB36 | Y3BC |  |  |  |
|  | TB37 | Y3BD |  |  |  |
|  | TB38 | Y3BE |  |  |  |
|  | TB39 | Y3BF |  |  |  |
| COM | TB40 | COM+ | DC 24V(P24) |  |  |
|  | TB41 | COM- | DC 0V(N24) |  |  |

| Station No. |  |  | 22 |  |  |
| --- | --- | --- | --- | --- | --- |
| Station Type |  |  | Remote I/O(AJ65SBTB1-32D1) |  |  |
| 용 도 |  |  | Equipment #1,2 Interlock Input |  |  |
| 구분 | TB No. | Address | Name | 접점종류 | 비고 |
| CC-Link | TB01 |  | DA |  |  |
|  | TB02 |  | DB |  |  |
|  | TB03 |  | DG |  |  |
|  | TB04 |  | SLD |  |  |
|  | TB05 |  | DC 24V |  |  |
|  | TB06 |  | FG |  |  |
|  | TB07 |  | DC 0V |  |  |
| 입력 | TB08 | X3C0 | Normal Status_SIG OUT #1(Pin #19) |  | Equipment #1 |
|  | TB09 | X3C1 | Unload Enable_SIG OUT #2(Pin #20) |  |  |
|  | TB10 | X3C2 | Load Enable_SIG OUT #3(Pin #21) |  |  |
|  | TB11 | X3C3 | Exchange Enable_SIG OUT #4(Pin #22) |  |  |
|  | TB12 | X3C4 | Pin Action Busy_SIG OUT #5(Pin #23) |  |  |
|  | TB13 | X3C5 | Stage Description #1_SIG OUT #6(Pin #24) |  |  |
|  | TB14 | X3C6 | Stage Descriotion #2_SIG OUT #7(Pin #25) |  |  |
|  | TB15 | X3C7 | Stage Description #3_SIG OUT #8(Pin #26) |  |  |
|  | TB16 | X3C8 | Stage Description #4_SIG OUT #9(Pin #28) |  |  |
|  | TB17 | X3C9 | Unload Confirm(Pin #29) |  |  |
|  | TB18 | X3CA | Spare_SIG OUT #11(Pin #30) |  |  |
|  | TB19 | X3CB | Spare_SIG OUT #12(Pin #31) |  |  |
|  | TB20 | X3CC | Spare_SIG OUT #13(Pin #32) |  |  |
|  | TB21 | X3CD | Robot Action Enable #1(Pin #34) |  |  |
|  | TB22 | X3CE | Robot Action Enable #2(Pin #35) |  |  |
|  | TB23 | X3CF | Robot Action Enable #3(Pin #36) |  |  |
|  | TB24 | X3D0 | Normal Status_SIG OUT #1(Pin #19) |  | Equipment #2 |
|  | TB25 | X3D1 | Unload Enable_SIG OUT #2(Pin #20) |  |  |
|  | TB26 | X3D2 | Load Enable_SIG OUT #3(Pin #21) |  |  |
|  | TB27 | X3D3 | Exchange Enable_SIG OUT #4(Pin #22) |  |  |
|  | TB28 | X3D4 | Pin Action Busy_SIG OUT #5(Pin #23) |  |  |
|  | TB29 | X3D5 | Stage Description #1_SIG OUT #6(Pin #24) |  |  |
|  | TB30 | X3D6 | Stage Descriotion #2_SIG OUT #7(Pin #25) |  |  |
|  | TB31 | X3D7 | Stage Description #3_SIG OUT #8(Pin #26) |  |  |
|  | TB32 | X3D8 | Stage Description #4_SIG OUT #9(Pin #28) |  |  |
|  | TB33 | X3D9 | Unload Confirm(Pin #29) |  |  |
|  | TB34 | X3DA | Spare_SIG OUT #11(Pin #30) |  |  |
|  | TB35 | X3DB | Spare_SIG OUT #12(Pin #31) |  |  |
|  | TB36 | X3DC | Spare_SIG OUT #13(Pin #32) |  |  |
|  | TB37 | X3DD | Robot Action Enable #1(Pin #34) |  |  |
|  | TB38 | X3DE | Robot Action Enable #2(Pin #35) |  |  |
|  | TB39 | X3DF | Robot Action Enable #3(Pin #36) |  |  |
| COM | TB40 | COM+ | DC 24V(P24) |  |  |
|  | TB41 | COM- | DC 0V(N24) |  |  |

| Station No. |  |  | 23 |  |  |
| --- | --- | --- | --- | --- | --- |
| Station Type |  |  | Remote I/O(AJ65SBTB2N-16R) |  |  |
| 용 도 |  |  | Equipment #1 Interlock Output |  |  |
| 구분 | TB No. | Address | Name | 접점종류 | 비고 |
| CC-Link | TB01 |  | DA |  |  |
|  | TB02 |  | DB |  |  |
|  | TB03 |  | DG |  |  |
|  | TB04 |  | SLD |  |  |
|  | TB05 |  | DC 24V |  |  |
|  | TB06 |  | FG |  |  |
|  | TB07 |  | DC 0V |  |  |
| 출력 | TB08 | Y3E0 | Normal Status_SIG IN #1(Pin #1) |  | Equipment #1 |
|  | TB09 | COM B | Common SIG IN #1 ~ #8(Pin #9) |  |  |
|  | TB10 | Y3E1 | Unload Ready_SIG IN #2(Pin #2) |  |  |
|  | TB11 | COM B | Common SIG IN #1 ~ #8(Pin #9) |  |  |
|  | TB12 | Y3E2 | Load Ready_SIG IN #3(Pin #3) |  |  |
|  | TB13 | COM B | Common SIG IN #1 ~ #8(Pin #9) |  |  |
|  | TB14 | Y3E3 | Exchange Ready_SIG IN #4(Pin #4) |  |  |
|  | TB15 | COM B | Common SIG IN #1 ~ #8(Pin #9) |  |  |
|  | TB16 | Y3E4 | Robot Busy_SIG IN #5(Pin #5) |  |  |
|  | TB17 | COM B | Common SIG IN #1 ~ #8(Pin #9) |  |  |
|  | TB18 | Y3E5 | Robot End_SIG IN #6(Pin #6) |  |  |
|  | TB19 | COM B | Common SIG IN #1 ~ #8(Pin #9) |  |  |
|  | TB20 | Y3E6 | Next Glass Exist_SIG IN #7(Pin #7) |  |  |
|  | TB21 | COM B | Common SIG IN #1 ~ #8(Pin #9) |  |  |
|  | TB22 | Y3E7 | Pin Action Request_SIG IN #8(Pin #8) |  |  |
|  | TB23 | COM B | Common SIG IN #1 ~ #8(Pin #9) |  |  |
|  | TB24 | Y3E8 | Stage Description #1_SIG IN #9(Pin #10) |  |  |
|  | TB25 | COM B | Common SIG IN #9 ~ #16(Pin #18) |  |  |
|  | TB26 | Y3E9 | Stage Description #2_SIG IN #10(Pin #11) |  |  |
|  | TB27 | COM B | Common SIG IN #9 ~ #16(Pin #18) |  |  |
|  | TB28 | Y3EA | Stage Description #3_SIG IN #11(Pin #12) |  |  |
|  | TB29 | COM B | Common SIG IN #9 ~ #16(Pin #18) |  |  |
|  | TB30 | Y3EB | Stage Description #4_SIG IN #12(Pin #13) |  |  |
|  | TB31 | COM B | Common SIG IN #9 ~ #16(Pin #18) |  |  |
|  | TB32 | Y3EC | Unload Complete_SIG IN #13(Pin #14) |  |  |
|  | TB33 | COM B | Common SIG IN #9 ~ #16(Pin #18) |  |  |
|  | TB34 | Y3ED | Load Complete_SIG IN #14(Pin #15) |  |  |
|  | TB35 | COM B | Common SIG IN #9 ~ #16(Pin #18) |  |  |
|  | TB36 | Y3EE | Spare_SIG IN #15(Pin #16) |  |  |
|  | TB37 | COM B | Common SIG IN #9 ~ #16(Pin #18) |  |  |
|  | TB38 | Y3EF | Spare_SIG IN #16(Pin #17) |  |  |
|  | TB39 | COM B | Common SIG IN #9 ~ #16(Pin #18) |  |  |
| COM | TB40 | COM A | DC 24V |  |  |
|  | TB41 | COM B | DC 0V |  |  |

| Station No. |  |  | 24 |  |  |
| --- | --- | --- | --- | --- | --- |
| Station Type |  |  | Remote I/O(AJ65SBTB2N-16R) |  |  |
| 용 도 |  |  | Equipment #2 Interlock Output |  |  |
| 구분 | TB No. | Address | Name | 접점종류 | 비고 |
| CC-Link | TB01 |  | DA |  |  |
|  | TB02 |  | DB |  |  |
|  | TB03 |  | DG |  |  |
|  | TB04 |  | SLD |  |  |
|  | TB05 |  | DC 24V |  |  |
|  | TB06 |  | FG |  |  |
|  | TB07 |  | DC 0V |  |  |
| 출력 | TB08 | Y400 | Normal Status_SIG IN #1(Pin #1) |  | Equipment #2 |
|  | TB09 | COM B | Common SIG IN #1 ~ #8(Pin #9) |  |  |
|  | TB10 | Y401 | Unload Ready_SIG IN #2(Pin #2) |  |  |
|  | TB11 | COM B | Common SIG IN #1 ~ #8(Pin #9) |  |  |
|  | TB12 | Y402 | Load Ready_SIG IN #3(Pin #3) |  |  |
|  | TB13 | COM B | Common SIG IN #1 ~ #8(Pin #9) |  |  |
|  | TB14 | Y403 | Exchange Ready_SIG IN #4(Pin #4) |  |  |
|  | TB15 | COM B | Common SIG IN #1 ~ #8(Pin #9) |  |  |
|  | TB16 | Y404 | Robot Busy_SIG IN #5(Pin #5) |  |  |
|  | TB17 | COM B | Common SIG IN #1 ~ #8(Pin #9) |  |  |
|  | TB18 | Y405 | Robot End_SIG IN #6(Pin #6) |  |  |
|  | TB19 | COM B | Common SIG IN #1 ~ #8(Pin #9) |  |  |
|  | TB20 | Y406 | Next Glass Exist_SIG IN #7(Pin #7) |  |  |
|  | TB21 | COM B | Common SIG IN #1 ~ #8(Pin #9) |  |  |
|  | TB22 | Y407 | Pin Action Request_SIG IN #8(Pin #8) |  |  |
|  | TB23 | COM B | Common SIG IN #1 ~ #8(Pin #9) |  |  |
|  | TB24 | Y408 | Stage Description #1_SIG IN #9(Pin #10) |  |  |
|  | TB25 | COM B | Common SIG IN #9 ~ #16(Pin #18) |  |  |
|  | TB26 | Y409 | Stage Description #2_SIG IN #10(Pin #11) |  |  |
|  | TB27 | COM B | Common SIG IN #9 ~ #16(Pin #18) |  |  |
|  | TB28 | Y40A | Stage Description #3_SIG IN #11(Pin #12) |  |  |
|  | TB29 | COM B | Common SIG IN #9 ~ #16(Pin #18) |  |  |
|  | TB30 | Y40B | Stage Description #4_SIG IN #12(Pin #13) |  |  |
|  | TB31 | COM B | Common SIG IN #9 ~ #16(Pin #18) |  |  |
|  | TB32 | Y40C | Unload Complete_SIG IN #13(Pin #14) |  |  |
|  | TB33 | COM B | Common SIG IN #9 ~ #16(Pin #18) |  |  |
|  | TB34 | Y40D | Load Complete_SIG IN #14(Pin #15) |  |  |
|  | TB35 | COM B | Common SIG IN #9 ~ #16(Pin #18) |  |  |
|  | TB36 | Y40E | Spare_SIG IN #15(Pin #16) |  |  |
|  | TB37 | COM B | Common SIG IN #9 ~ #16(Pin #18) |  |  |
|  | TB38 | Y40F | Spare_SIG IN #16(Pin #17) |  |  |
|  | TB39 | COM B | Common SIG IN #9 ~ #16(Pin #18) |  |  |
| COM | TB40 | COM A | DC 24V |  |  |
|  | TB41 | COM B | DC 0V |  |  |

| Station No. |  |  | 25 |  |  |
| --- | --- | --- | --- | --- | --- |
| Station Type |  |  | Remote I/O(AJ65SBTB1-32D1) |  |  |
| 용 도 |  |  | Equipment #3,4 Interlock Input |  |  |
| 구분 | TB No. | Address | Name | 접점종류 | 비고 |
| CC-Link | TB01 |  | DA |  |  |
|  | TB02 |  | DB |  |  |
|  | TB03 |  | DG |  |  |
|  | TB04 |  | SLD |  |  |
|  | TB05 |  | DC 24V |  |  |
|  | TB06 |  | FG |  |  |
|  | TB07 |  | DC 0V |  |  |
| 입력 | TB08 | X420 | Normal Status_SIG OUT #1(Pin #19) |  | Equipment #3 |
|  | TB09 | X421 | Unload Enable_SIG OUT #2(Pin #20) |  |  |
|  | TB10 | X422 | Load Enable_SIG OUT #3(Pin #21) |  |  |
|  | TB11 | X423 | Exchange Enable_SIG OUT #4(Pin #22) |  |  |
|  | TB12 | X424 | Pin Action Busy_SIG OUT #5(Pin #23) |  |  |
|  | TB13 | X425 | Stage Description #1_SIG OUT #6(Pin #24) |  |  |
|  | TB14 | X426 | Stage Descriotion #2_SIG OUT #7(Pin #25) |  |  |
|  | TB15 | X427 | Stage Description #3_SIG OUT #8(Pin #26) |  |  |
|  | TB16 | X428 | Stage Description #4_SIG OUT #9(Pin #28) |  |  |
|  | TB17 | X429 | Unload Confirm(Pin #29) |  |  |
|  | TB18 | X42A | Spare_SIG OUT #11(Pin #30) |  |  |
|  | TB19 | X42B | Spare_SIG OUT #12(Pin #31) |  |  |
|  | TB20 | X42C | Spare_SIG OUT #13(Pin #32) |  |  |
|  | TB21 | X42D | Robot Action Enable #1(Pin #34) |  |  |
|  | TB22 | X42E | Robot Action Enable #2(Pin #35) |  |  |
|  | TB23 | X42F | Robot Action Enable #3(Pin #36) |  |  |
|  | TB24 | X430 | Normal Status_SIG OUT #1(Pin #19) |  | Equipment #4 |
|  | TB25 | X431 | Unload Enable_SIG OUT #2(Pin #20) |  |  |
|  | TB26 | X432 | Load Enable_SIG OUT #3(Pin #21) |  |  |
|  | TB27 | X433 | Exchange Enable_SIG OUT #4(Pin #22) |  |  |
|  | TB28 | X434 | Pin Action Busy_SIG OUT #5(Pin #23) |  |  |
|  | TB29 | X435 | Stage Description #1_SIG OUT #6(Pin #24) |  |  |
|  | TB30 | X436 | Stage Descriotion #2_SIG OUT #7(Pin #25) |  |  |
|  | TB31 | X437 | Stage Description #3_SIG OUT #8(Pin #26) |  |  |
|  | TB32 | X438 | Stage Description #4_SIG OUT #9(Pin #28) |  |  |
|  | TB33 | X439 | Unload Confirm(Pin #29) |  |  |
|  | TB34 | X43A | Spare_SIG OUT #11(Pin #30) |  |  |
|  | TB35 | X43B | Spare_SIG OUT #12(Pin #31) |  |  |
|  | TB36 | X43C | Spare_SIG OUT #13(Pin #32) |  |  |
|  | TB37 | X43D | Robot Action Enable #1(Pin #34) |  |  |
|  | TB38 | X43E | Robot Action Enable #2(Pin #35) |  |  |
|  | TB39 | X43F | Robot Action Enable #3(Pin #36) |  |  |
| COM | TB40 | COM+ | DC 24V(P24) |  |  |
|  | TB41 | COM- | DC 0V(N24) |  |  |

| Station No. |  |  | 26 |  |  |
| --- | --- | --- | --- | --- | --- |
| Station Type |  |  | Remote I/O(AJ65SBTB2N-16R) |  |  |
| 용 도 |  |  | Equipment #3 Interlock Output |  |  |
| 구분 | TB No. | Address | Name | 접점종류 | 비고 |
| CC-Link | TB01 |  | DA |  |  |
|  | TB02 |  | DB |  |  |
|  | TB03 |  | DG |  |  |
|  | TB04 |  | SLD |  |  |
|  | TB05 |  | DC 24V |  |  |
|  | TB06 |  | FG |  |  |
|  | TB07 |  | DC 0V |  |  |
| 출력 | TB08 | Y440 | Normal Status_SIG IN #1(Pin #1) |  | Equipment #3 |
|  | TB09 | COM B | Common SIG IN #1 ~ #8(Pin #9) |  |  |
|  | TB10 | Y441 | Unload Ready_SIG IN #2(Pin #2) |  |  |
|  | TB11 | COM B | Common SIG IN #1 ~ #8(Pin #9) |  |  |
|  | TB12 | Y442 | Load Ready_SIG IN #3(Pin #3) |  |  |
|  | TB13 | COM B | Common SIG IN #1 ~ #8(Pin #9) |  |  |
|  | TB14 | Y443 | Exchange Ready_SIG IN #4(Pin #4) |  |  |
|  | TB15 | COM B | Common SIG IN #1 ~ #8(Pin #9) |  |  |
|  | TB16 | Y444 | Robot Busy_SIG IN #5(Pin #5) |  |  |
|  | TB17 | COM B | Common SIG IN #1 ~ #8(Pin #9) |  |  |
|  | TB18 | Y445 | Robot End_SIG IN #6(Pin #6) |  |  |
|  | TB19 | COM B | Common SIG IN #1 ~ #8(Pin #9) |  |  |
|  | TB20 | Y446 | Next Glass Exist_SIG IN #7(Pin #7) |  |  |
|  | TB21 | COM B | Common SIG IN #1 ~ #8(Pin #9) |  |  |
|  | TB22 | Y447 | Pin Action Request_SIG IN #8(Pin #8) |  |  |
|  | TB23 | COM B | Common SIG IN #1 ~ #8(Pin #9) |  |  |
|  | TB24 | Y448 | Stage Description #1_SIG IN #9(Pin #10) |  |  |
|  | TB25 | COM B | Common SIG IN #9 ~ #16(Pin #18) |  |  |
|  | TB26 | Y449 | Stage Description #2_SIG IN #10(Pin #11) |  |  |
|  | TB27 | COM B | Common SIG IN #9 ~ #16(Pin #18) |  |  |
|  | TB28 | Y44A | Stage Description #3_SIG IN #11(Pin #12) |  |  |
|  | TB29 | COM B | Common SIG IN #9 ~ #16(Pin #18) |  |  |
|  | TB30 | Y44B | Stage Description #4_SIG IN #12(Pin #13) |  |  |
|  | TB31 | COM B | Common SIG IN #9 ~ #16(Pin #18) |  |  |
|  | TB32 | Y44C | Unload Complete_SIG IN #13(Pin #14) |  |  |
|  | TB33 | COM B | Common SIG IN #9 ~ #16(Pin #18) |  |  |
|  | TB34 | Y44D | Load Complete_SIG IN #14(Pin #15) |  |  |
|  | TB35 | COM B | Common SIG IN #9 ~ #16(Pin #18) |  |  |
|  | TB36 | Y44E | Spare_SIG IN #15(Pin #16) |  |  |
|  | TB37 | COM B | Common SIG IN #9 ~ #16(Pin #18) |  |  |
|  | TB38 | Y44F | Spare_SIG IN #16(Pin #17) |  |  |
|  | TB39 | COM B | Common SIG IN #9 ~ #16(Pin #18) |  |  |
| COM | TB40 | COM A | DC 24V |  |  |
|  | TB41 | COM B | DC 0V |  |  |

| Station No. |  |  | 27 |  |  |
| --- | --- | --- | --- | --- | --- |
| Station Type |  |  | Remote I/O(AJ65SBTB2N-16R) |  |  |
| 용 도 |  |  | Equipment #4 Interlock Output |  |  |
| 구분 | TB No. | Address | Name | 접점종류 | 비고 |
| CC-Link | TB01 |  | DA |  |  |
|  | TB02 |  | DB |  |  |
|  | TB03 |  | DG |  |  |
|  | TB04 |  | SLD |  |  |
|  | TB05 |  | DC 24V |  |  |
|  | TB06 |  | FG |  |  |
|  | TB07 |  | DC 0V |  |  |
| 출력 | TB08 | Y460 | Normal Status_SIG IN #1(Pin #1) |  | Equipment #4 |
|  | TB09 | COM B | Common SIG IN #1 ~ #8(Pin #9) |  |  |
|  | TB10 | Y461 | Unload Ready_SIG IN #2(Pin #2) |  |  |
|  | TB11 | COM B | Common SIG IN #1 ~ #8(Pin #9) |  |  |
|  | TB12 | Y462 | Load Ready_SIG IN #3(Pin #3) |  |  |
|  | TB13 | COM B | Common SIG IN #1 ~ #8(Pin #9) |  |  |
|  | TB14 | Y463 | Exchange Ready_SIG IN #4(Pin #4) |  |  |
|  | TB15 | COM B | Common SIG IN #1 ~ #8(Pin #9) |  |  |
|  | TB16 | Y464 | Robot Busy_SIG IN #5(Pin #5) |  |  |
|  | TB17 | COM B | Common SIG IN #1 ~ #8(Pin #9) |  |  |
|  | TB18 | Y465 | Robot End_SIG IN #6(Pin #6) |  |  |
|  | TB19 | COM B | Common SIG IN #1 ~ #8(Pin #9) |  |  |
|  | TB20 | Y466 | Next Glass Exist_SIG IN #7(Pin #7) |  |  |
|  | TB21 | COM B | Common SIG IN #1 ~ #8(Pin #9) |  |  |
|  | TB22 | Y467 | Pin Action Request_SIG IN #8(Pin #8) |  |  |
|  | TB23 | COM B | Common SIG IN #1 ~ #8(Pin #9) |  |  |
|  | TB24 | Y468 | Stage Description #1_SIG IN #9(Pin #10) |  |  |
|  | TB25 | COM B | Common SIG IN #9 ~ #16(Pin #18) |  |  |
|  | TB26 | Y469 | Stage Description #2_SIG IN #10(Pin #11) |  |  |
|  | TB27 | COM B | Common SIG IN #9 ~ #16(Pin #18) |  |  |
|  | TB28 | Y46A | Stage Description #3_SIG IN #11(Pin #12) |  |  |
|  | TB29 | COM B | Common SIG IN #9 ~ #16(Pin #18) |  |  |
|  | TB30 | Y46B | Stage Description #4_SIG IN #12(Pin #13) |  |  |
|  | TB31 | COM B | Common SIG IN #9 ~ #16(Pin #18) |  |  |
|  | TB32 | Y46C | Unload Complete_SIG IN #13(Pin #14) |  |  |
|  | TB33 | COM B | Common SIG IN #9 ~ #16(Pin #18) |  |  |
|  | TB34 | Y46D | Load Complete_SIG IN #14(Pin #15) |  |  |
|  | TB35 | COM B | Common SIG IN #9 ~ #16(Pin #18) |  |  |
|  | TB36 | Y46E | Spare_SIG IN #15(Pin #16) |  |  |
|  | TB37 | COM B | Common SIG IN #9 ~ #16(Pin #18) |  |  |
|  | TB38 | Y46F | Spare_SIG IN #16(Pin #17) |  |  |
|  | TB39 | COM B | Common SIG IN #9 ~ #16(Pin #18) |  |  |
| COM | TB40 | COM A | DC 24V |  |  |
|  | TB41 | COM B | DC 0V |  |  |

| Station No. |  |  | 28 |  |  |
| --- | --- | --- | --- | --- | --- |
| Station Type |  |  | Remote I/O(AJ65SBTB1-32D1) |  |  |
| 용 도 |  |  | Equipment #5,6 Interlock Input |  |  |
| 구분 | TB No. | Address | Name | 접점종류 | 비고 |
| CC-Link | TB01 |  | DA |  |  |
|  | TB02 |  | DB |  |  |
|  | TB03 |  | DG |  |  |
|  | TB04 |  | SLD |  |  |
|  | TB05 |  | DC 24V |  |  |
|  | TB06 |  | FG |  |  |
|  | TB07 |  | DC 0V |  |  |
| 입력 | TB08 | X480 | Normal Status_SIG OUT #1(Pin #19) |  | Equipment #5 |
|  | TB09 | X481 | Unload Enable_SIG OUT #2(Pin #20) |  |  |
|  | TB10 | X482 | Load Enable_SIG OUT #3(Pin #21) |  |  |
|  | TB11 | X483 | Exchange Enable_SIG OUT #4(Pin #22) |  |  |
|  | TB12 | X484 | Pin Action Busy_SIG OUT #5(Pin #23) |  |  |
|  | TB13 | X485 | Stage Description #1_SIG OUT #6(Pin #24) |  |  |
|  | TB14 | X486 | Stage Descriotion #2_SIG OUT #7(Pin #25) |  |  |
|  | TB15 | X487 | Stage Description #3_SIG OUT #8(Pin #26) |  |  |
|  | TB16 | X488 | Stage Description #4_SIG OUT #9(Pin #28) |  |  |
|  | TB17 | X489 | Unload Confirm(Pin #29) |  |  |
|  | TB18 | X48A | Spare_SIG OUT #11(Pin #30) |  |  |
|  | TB19 | X48B | Spare_SIG OUT #12(Pin #31) |  |  |
|  | TB20 | X48C | Spare_SIG OUT #13(Pin #32) |  |  |
|  | TB21 | X48D | Robot Action Enable #1(Pin #34) |  |  |
|  | TB22 | X48E | Robot Action Enable #2(Pin #35) |  |  |
|  | TB23 | X48F | Robot Action Enable #3(Pin #36) |  |  |
|  | TB24 | X490 | Normal Status_SIG OUT #1(Pin #19) |  | Equipment #6 |
|  | TB25 | X491 | Unload Enable_SIG OUT #2(Pin #20) |  |  |
|  | TB26 | X492 | Load Enable_SIG OUT #3(Pin #21) |  |  |
|  | TB27 | X493 | Exchange Enable_SIG OUT #4(Pin #22) |  |  |
|  | TB28 | X494 | Pin Action Busy_SIG OUT #5(Pin #23) |  |  |
|  | TB29 | X495 | Stage Description #1_SIG OUT #6(Pin #24) |  |  |
|  | TB30 | X496 | Stage Descriotion #2_SIG OUT #7(Pin #25) |  |  |
|  | TB31 | X497 | Stage Description #3_SIG OUT #8(Pin #26) |  |  |
|  | TB32 | X498 | Stage Description #4_SIG OUT #9(Pin #28) |  |  |
|  | TB33 | X499 | Unload Confirm(Pin #29) |  |  |
|  | TB34 | X49A | Spare_SIG OUT #11(Pin #30) |  |  |
|  | TB35 | X49B | Spare_SIG OUT #12(Pin #31) |  |  |
|  | TB36 | X49C | Spare_SIG OUT #13(Pin #32) |  |  |
|  | TB37 | X49D | Robot Action Enable #1(Pin #34) |  |  |
|  | TB38 | X49E | Robot Action Enable #2(Pin #35) |  |  |
|  | TB39 | X49F | Robot Action Enable #3(Pin #36) |  |  |
| COM | TB40 | COM+ | DC 24V(P24) |  |  |
|  | TB41 | COM- | DC 0V(N24) |  |  |

| Station No. |  |  | 29 |  |  |
| --- | --- | --- | --- | --- | --- |
| Station Type |  |  | Remote I/O(AJ65SBTB2N-16R) |  |  |
| 용 도 |  |  | Equipment #5 Interlock Output |  |  |
| 구분 | TB No. | Address | Name | 접점종류 | 비고 |
| CC-Link | TB01 |  | DA |  |  |
|  | TB02 |  | DB |  |  |
|  | TB03 |  | DG |  |  |
|  | TB04 |  | SLD |  |  |
|  | TB05 |  | DC 24V |  |  |
|  | TB06 |  | FG |  |  |
|  | TB07 |  | DC 0V |  |  |
| 출력 | TB08 | Y4A0 | Normal Status_SIG IN #1(Pin #1) |  | Equipment #5 |
|  | TB09 | COM B | Common SIG IN #1 ~ #8(Pin #9) |  |  |
|  | TB10 | Y4A1 | Unload Ready_SIG IN #2(Pin #2) |  |  |
|  | TB11 | COM B | Common SIG IN #1 ~ #8(Pin #9) |  |  |
|  | TB12 | Y4A2 | Load Ready_SIG IN #3(Pin #3) |  |  |
|  | TB13 | COM B | Common SIG IN #1 ~ #8(Pin #9) |  |  |
|  | TB14 | Y4A3 | Exchange Ready_SIG IN #4(Pin #4) |  |  |
|  | TB15 | COM B | Common SIG IN #1 ~ #8(Pin #9) |  |  |
|  | TB16 | Y4A4 | Robot Busy_SIG IN #5(Pin #5) |  |  |
|  | TB17 | COM B | Common SIG IN #1 ~ #8(Pin #9) |  |  |
|  | TB18 | Y4A5 | Robot End_SIG IN #6(Pin #6) |  |  |
|  | TB19 | COM B | Common SIG IN #1 ~ #8(Pin #9) |  |  |
|  | TB20 | Y4A6 | Next Glass Exist_SIG IN #7(Pin #7) |  |  |
|  | TB21 | COM B | Common SIG IN #1 ~ #8(Pin #9) |  |  |
|  | TB22 | Y4A7 | Pin Action Request_SIG IN #8(Pin #8) |  |  |
|  | TB23 | COM B | Common SIG IN #1 ~ #8(Pin #9) |  |  |
|  | TB24 | Y4A8 | Stage Description #1_SIG IN #9(Pin #10) |  |  |
|  | TB25 | COM B | Common SIG IN #9 ~ #16(Pin #18) |  |  |
|  | TB26 | Y4A9 | Stage Description #2_SIG IN #10(Pin #11) |  |  |
|  | TB27 | COM B | Common SIG IN #9 ~ #16(Pin #18) |  |  |
|  | TB28 | Y4AA | Stage Description #3_SIG IN #11(Pin #12) |  |  |
|  | TB29 | COM B | Common SIG IN #9 ~ #16(Pin #18) |  |  |
|  | TB30 | Y4AB | Stage Description #4_SIG IN #12(Pin #13) |  |  |
|  | TB31 | COM B | Common SIG IN #9 ~ #16(Pin #18) |  |  |
|  | TB32 | Y4AC | Unload Complete_SIG IN #13(Pin #14) |  |  |
|  | TB33 | COM B | Common SIG IN #9 ~ #16(Pin #18) |  |  |
|  | TB34 | Y4AD | Load Complete_SIG IN #14(Pin #15) |  |  |
|  | TB35 | COM B | Common SIG IN #9 ~ #16(Pin #18) |  |  |
|  | TB36 | Y4AE | Spare_SIG IN #15(Pin #16) |  |  |
|  | TB37 | COM B | Common SIG IN #9 ~ #16(Pin #18) |  |  |
|  | TB38 | Y4AF | Spare_SIG IN #16(Pin #17) |  |  |
|  | TB39 | COM B | Common SIG IN #9 ~ #16(Pin #18) |  |  |
| COM | TB40 | COM A | DC 24V |  |  |
|  | TB41 | COM B | DC 0V |  |  |

| Station No. |  |  | 30 |  |  |
| --- | --- | --- | --- | --- | --- |
| Station Type |  |  | Remote I/O(AJ65SBTB2N-16R) |  |  |
| 용 도 |  |  | Equipment #6 Interlock Output |  |  |
| 구분 | TB No. | Address | Name | 접점종류 | 비고 |
| CC-Link | TB01 |  | DA |  |  |
|  | TB02 |  | DB |  |  |
|  | TB03 |  | DG |  |  |
|  | TB04 |  | SLD |  |  |
|  | TB05 |  | DC 24V |  |  |
|  | TB06 |  | FG |  |  |
|  | TB07 |  | DC 0V |  |  |
| 출력 | TB08 | Y4C0 | Normal Status_SIG IN #1(Pin #1) |  | Equipment #6 |
|  | TB09 | COM B | Common SIG IN #1 ~ #8(Pin #9) |  |  |
|  | TB10 | Y4C1 | Unload Ready_SIG IN #2(Pin #2) |  |  |
|  | TB11 | COM B | Common SIG IN #1 ~ #8(Pin #9) |  |  |
|  | TB12 | Y4C2 | Load Ready_SIG IN #3(Pin #3) |  |  |
|  | TB13 | COM B | Common SIG IN #1 ~ #8(Pin #9) |  |  |
|  | TB14 | Y4C3 | Exchange Ready_SIG IN #4(Pin #4) |  |  |
|  | TB15 | COM B | Common SIG IN #1 ~ #8(Pin #9) |  |  |
|  | TB16 | Y4C4 | Robot Busy_SIG IN #5(Pin #5) |  |  |
|  | TB17 | COM B | Common SIG IN #1 ~ #8(Pin #9) |  |  |
|  | TB18 | Y4C5 | Robot End_SIG IN #6(Pin #6) |  |  |
|  | TB19 | COM B | Common SIG IN #1 ~ #8(Pin #9) |  |  |
|  | TB20 | Y4C6 | Next Glass Exist_SIG IN #7(Pin #7) |  |  |
|  | TB21 | COM B | Common SIG IN #1 ~ #8(Pin #9) |  |  |
|  | TB22 | Y4C7 | Pin Action Request_SIG IN #8(Pin #8) |  |  |
|  | TB23 | COM B | Common SIG IN #1 ~ #8(Pin #9) |  |  |
|  | TB24 | Y4C8 | Stage Description #1_SIG IN #9(Pin #10) |  |  |
|  | TB25 | COM B | Common SIG IN #9 ~ #16(Pin #18) |  |  |
|  | TB26 | Y4C9 | Stage Description #2_SIG IN #10(Pin #11) |  |  |
|  | TB27 | COM B | Common SIG IN #9 ~ #16(Pin #18) |  |  |
|  | TB28 | Y4CA | Stage Description #3_SIG IN #11(Pin #12) |  |  |
|  | TB29 | COM B | Common SIG IN #9 ~ #16(Pin #18) |  |  |
|  | TB30 | Y4CB | Stage Description #4_SIG IN #12(Pin #13) |  |  |
|  | TB31 | COM B | Common SIG IN #9 ~ #16(Pin #18) |  |  |
|  | TB32 | Y4CC | Unload Complete_SIG IN #13(Pin #14) |  |  |
|  | TB33 | COM B | Common SIG IN #9 ~ #16(Pin #18) |  |  |
|  | TB34 | Y4CD | Load Complete_SIG IN #14(Pin #15) |  |  |
|  | TB35 | COM B | Common SIG IN #9 ~ #16(Pin #18) |  |  |
|  | TB36 | Y4CE | Spare_SIG IN #15(Pin #16) |  |  |
|  | TB37 | COM B | Common SIG IN #9 ~ #16(Pin #18) |  |  |
|  | TB38 | Y4CF | Spare_SIG IN #16(Pin #17) |  |  |
|  | TB39 | COM B | Common SIG IN #9 ~ #16(Pin #18) |  |  |
| COM | TB40 | COM A | DC 24V |  |  |
|  | TB41 | COM B | DC 0V |  |  |

| Station No. |  |  | 31 |  |  |
| --- | --- | --- | --- | --- | --- |
| Station Type |  |  | Remote I/O(AJ65SBTB1-32D1) |  |  |
| 용 도 |  |  | Equipment #7,8 Interlock Input |  |  |
| 구분 | TB No. | Address | Name | 접점종류 | 비고 |
| CC-Link | TB01 |  | DA |  |  |
|  | TB02 |  | DB |  |  |
|  | TB03 |  | DG |  |  |
|  | TB04 |  | SLD |  |  |
|  | TB05 |  | DC 24V |  |  |
|  | TB06 |  | FG |  |  |
|  | TB07 |  | DC 0V |  |  |
| 입력 | TB08 | X4E0 | Normal Status_SIG OUT #1(Pin #19) |  | Equipment #7 |
|  | TB09 | X4E1 | Unload Enable_SIG OUT #2(Pin #20) |  |  |
|  | TB10 | X4E2 | Load Enable_SIG OUT #3(Pin #21) |  |  |
|  | TB11 | X4E3 | Exchange Enable_SIG OUT #4(Pin #22) |  |  |
|  | TB12 | X4E4 | Pin Action Busy_SIG OUT #5(Pin #23) |  |  |
|  | TB13 | X4E5 | Stage Description #1_SIG OUT #6(Pin #24) |  |  |
|  | TB14 | X4E6 | Stage Descriotion #2_SIG OUT #7(Pin #25) |  |  |
|  | TB15 | X4E7 | Stage Description #3_SIG OUT #8(Pin #26) |  |  |
|  | TB16 | X4E8 | Stage Description #4_SIG OUT #9(Pin #28) |  |  |
|  | TB17 | X4E9 | Unload Confirm(Pin #29) |  |  |
|  | TB18 | X4EA | Spare_SIG OUT #11(Pin #30) |  |  |
|  | TB19 | X4EB | Spare_SIG OUT #12(Pin #31) |  |  |
|  | TB20 | X4EC | Spare_SIG OUT #13(Pin #32) |  |  |
|  | TB21 | X4ED | Robot Action Enable #1(Pin #34) |  |  |
|  | TB22 | X4EE | Robot Action Enable #2(Pin #35) |  |  |
|  | TB23 | X4EF | Robot Action Enable #3(Pin #36) |  |  |
|  | TB24 | X4F0 | Normal Status_SIG OUT #1(Pin #19) |  | Equipment #8 |
|  | TB25 | X4F1 | Unload Enable_SIG OUT #2(Pin #20) |  |  |
|  | TB26 | X4F2 | Load Enable_SIG OUT #3(Pin #21) |  |  |
|  | TB27 | X4F3 | Exchange Enable_SIG OUT #4(Pin #22) |  |  |
|  | TB28 | X4F4 | Pin Action Busy_SIG OUT #5(Pin #23) |  |  |
|  | TB29 | X4F5 | Stage Description #1_SIG OUT #6(Pin #24) |  |  |
|  | TB30 | X4F6 | Stage Descriotion #2_SIG OUT #7(Pin #25) |  |  |
|  | TB31 | X4F7 | Stage Description #3_SIG OUT #8(Pin #26) |  |  |
|  | TB32 | X4F8 | Stage Description #4_SIG OUT #9(Pin #28) |  |  |
|  | TB33 | X4F9 | Unload Confirm(Pin #29) |  |  |
|  | TB34 | X4FA | Spare_SIG OUT #11(Pin #30) |  |  |
|  | TB35 | X4FB | Spare_SIG OUT #12(Pin #31) |  |  |
|  | TB36 | X4FC | Spare_SIG OUT #13(Pin #32) |  |  |
|  | TB37 | X4FD | Robot Action Enable #1(Pin #34) |  |  |
|  | TB38 | X4FE | Robot Action Enable #2(Pin #35) |  |  |
|  | TB39 | X4FF | Robot Action Enable #3(Pin #36) |  |  |
| COM | TB40 | COM+ | DC 24V(P24) |  |  |
|  | TB41 | COM- | DC 0V(N24) |  |  |

| Station No. |  |  | 32 |  |  |
| --- | --- | --- | --- | --- | --- |
| Station Type |  |  | Remote I/O(AJ65SBTB2N-16R) |  |  |
| 용 도 |  |  | Equipment #7 Interlock Output |  |  |
| 구분 | TB No. | Address | Name | 접점종류 | 비고 |
| CC-Link | TB01 |  | DA |  |  |
|  | TB02 |  | DB |  |  |
|  | TB03 |  | DG |  |  |
|  | TB04 |  | SLD |  |  |
|  | TB05 |  | DC 24V |  |  |
|  | TB06 |  | FG |  |  |
|  | TB07 |  | DC 0V |  |  |
| 출력 | TB08 | Y500 | Normal Status_SIG IN #1(Pin #1) |  | Equipment #7 |
|  | TB09 | COM B | Common SIG IN #1 ~ #8(Pin #9) |  |  |
|  | TB10 | Y501 | Unload Ready_SIG IN #2(Pin #2) |  |  |
|  | TB11 | COM B | Common SIG IN #1 ~ #8(Pin #9) |  |  |
|  | TB12 | Y502 | Load Ready_SIG IN #3(Pin #3) |  |  |
|  | TB13 | COM B | Common SIG IN #1 ~ #8(Pin #9) |  |  |
|  | TB14 | Y503 | Exchange Ready_SIG IN #4(Pin #4) |  |  |
|  | TB15 | COM B | Common SIG IN #1 ~ #8(Pin #9) |  |  |
|  | TB16 | Y504 | Robot Busy_SIG IN #5(Pin #5) |  |  |
|  | TB17 | COM B | Common SIG IN #1 ~ #8(Pin #9) |  |  |
|  | TB18 | Y505 | Robot End_SIG IN #6(Pin #6) |  |  |
|  | TB19 | COM B | Common SIG IN #1 ~ #8(Pin #9) |  |  |
|  | TB20 | Y506 | Next Glass Exist_SIG IN #7(Pin #7) |  |  |
|  | TB21 | COM B | Common SIG IN #1 ~ #8(Pin #9) |  |  |
|  | TB22 | Y507 | Pin Action Request_SIG IN #8(Pin #8) |  |  |
|  | TB23 | COM B | Common SIG IN #1 ~ #8(Pin #9) |  |  |
|  | TB24 | Y508 | Stage Description #1_SIG IN #9(Pin #10) |  |  |
|  | TB25 | COM B | Common SIG IN #9 ~ #16(Pin #18) |  |  |
|  | TB26 | Y509 | Stage Description #2_SIG IN #10(Pin #11) |  |  |
|  | TB27 | COM B | Common SIG IN #9 ~ #16(Pin #18) |  |  |
|  | TB28 | Y50A | Stage Description #3_SIG IN #11(Pin #12) |  |  |
|  | TB29 | COM B | Common SIG IN #9 ~ #16(Pin #18) |  |  |
|  | TB30 | Y50B | Stage Description #4_SIG IN #12(Pin #13) |  |  |
|  | TB31 | COM B | Common SIG IN #9 ~ #16(Pin #18) |  |  |
|  | TB32 | Y50C | Unload Complete_SIG IN #13(Pin #14) |  |  |
|  | TB33 | COM B | Common SIG IN #9 ~ #16(Pin #18) |  |  |
|  | TB34 | Y50D | Load Complete_SIG IN #14(Pin #15) |  |  |
|  | TB35 | COM B | Common SIG IN #9 ~ #16(Pin #18) |  |  |
|  | TB36 | Y50E | Spare_SIG IN #15(Pin #16) |  |  |
|  | TB37 | COM B | Common SIG IN #9 ~ #16(Pin #18) |  |  |
|  | TB38 | Y50F | Spare_SIG IN #16(Pin #17) |  |  |
|  | TB39 | COM B | Common SIG IN #9 ~ #16(Pin #18) |  |  |
| COM | TB40 | COM A | DC 24V |  |  |
|  | TB41 | COM B | DC 0V |  |  |

| Station No. |  |  | 33 |  |  |
| --- | --- | --- | --- | --- | --- |
| Station Type |  |  | Remote I/O(AJ65SBTB2N-16R) |  |  |
| 용 도 |  |  | Equipment #8 Interlock Output |  |  |
| 구분 | TB No. | Address | Name | 접점종류 | 비고 |
| CC-Link | TB01 |  | DA |  |  |
|  | TB02 |  | DB |  |  |
|  | TB03 |  | DG |  |  |
|  | TB04 |  | SLD |  |  |
|  | TB05 |  | DC 24V |  |  |
|  | TB06 |  | FG |  |  |
|  | TB07 |  | DC 0V |  |  |
| 출력 | TB08 | Y520 | Normal Status_SIG IN #1(Pin #1) |  | Equipment #8 |
|  | TB09 | COM B | Common SIG IN #1 ~ #8(Pin #9) |  |  |
|  | TB10 | Y521 | Unload Ready_SIG IN #2(Pin #2) |  |  |
|  | TB11 | COM B | Common SIG IN #1 ~ #8(Pin #9) |  |  |
|  | TB12 | Y522 | Load Ready_SIG IN #3(Pin #3) |  |  |
|  | TB13 | COM B | Common SIG IN #1 ~ #8(Pin #9) |  |  |
|  | TB14 | Y523 | Exchange Ready_SIG IN #4(Pin #4) |  |  |
|  | TB15 | COM B | Common SIG IN #1 ~ #8(Pin #9) |  |  |
|  | TB16 | Y524 | Robot Busy_SIG IN #5(Pin #5) |  |  |
|  | TB17 | COM B | Common SIG IN #1 ~ #8(Pin #9) |  |  |
|  | TB18 | Y525 | Robot End_SIG IN #6(Pin #6) |  |  |
|  | TB19 | COM B | Common SIG IN #1 ~ #8(Pin #9) |  |  |
|  | TB20 | Y526 | Next Glass Exist_SIG IN #7(Pin #7) |  |  |
|  | TB21 | COM B | Common SIG IN #1 ~ #8(Pin #9) |  |  |
|  | TB22 | Y527 | Pin Action Request_SIG IN #8(Pin #8) |  |  |
|  | TB23 | COM B | Common SIG IN #1 ~ #8(Pin #9) |  |  |
|  | TB24 | Y528 | Stage Description #1_SIG IN #9(Pin #10) |  |  |
|  | TB25 | COM B | Common SIG IN #9 ~ #16(Pin #18) |  |  |
|  | TB26 | Y529 | Stage Description #2_SIG IN #10(Pin #11) |  |  |
|  | TB27 | COM B | Common SIG IN #9 ~ #16(Pin #18) |  |  |
|  | TB28 | Y52A | Stage Description #3_SIG IN #11(Pin #12) |  |  |
|  | TB29 | COM B | Common SIG IN #9 ~ #16(Pin #18) |  |  |
|  | TB30 | Y52B | Stage Description #4_SIG IN #12(Pin #13) |  |  |
|  | TB31 | COM B | Common SIG IN #9 ~ #16(Pin #18) |  |  |
|  | TB32 | Y52C | Unload Complete_SIG IN #13(Pin #14) |  |  |
|  | TB33 | COM B | Common SIG IN #9 ~ #16(Pin #18) |  |  |
|  | TB34 | Y52D | Load Complete_SIG IN #14(Pin #15) |  |  |
|  | TB35 | COM B | Common SIG IN #9 ~ #16(Pin #18) |  |  |
|  | TB36 | Y52E | Spare_SIG IN #15(Pin #16) |  |  |
|  | TB37 | COM B | Common SIG IN #9 ~ #16(Pin #18) |  |  |
|  | TB38 | Y52F | Spare_SIG IN #16(Pin #17) |  |  |
|  | TB39 | COM B | Common SIG IN #9 ~ #16(Pin #18) |  |  |
| COM | TB40 | COM A | DC 24V |  |  |
|  | TB41 | COM B | DC 0V |  |  |

| Station No. |  |  | 34 |  |  |
| --- | --- | --- | --- | --- | --- |
| Station Type |  |  | Remote I/O(AJ65SBTB1-32D1) |  |  |
| 용 도 |  |  | Port CST LIFT SENSOR |  |  |
| 구분 | TB No. | Address | Name | 접점종류 | 비고 |
| CC-Link | TB01 |  | DA |  |  |
|  | TB02 |  | DB |  |  |
|  | TB03 |  | DG |  |  |
|  | TB04 |  | SLD |  |  |
|  | TB05 |  | DC 24V |  |  |
|  | TB06 |  | FG |  |  |
|  | TB07 |  | DC 0V |  |  |
| 입력 | TB08 | X540 | Port #1 CST 들림 감지 센서 | b |  |
|  | TB09 | X541 | Port #2 CST 들림 감지 센서 | b |  |
|  | TB10 | X542 | Port #3 CST 들림 감지 센서 | b |  |
|  | TB11 | X543 | Port #4 CST 들림 감지 센서 | b |  |
|  | TB12 | X544 | Port #5 CST 들림 감지 센서 | b |  |
|  | TB13 | X545 |  |  |  |
|  | TB14 | X546 |  |  |  |
|  | TB15 | X547 |  |  |  |
|  | TB16 | X548 |  |  |  |
|  | TB17 | X549 |  |  |  |
|  | TB18 | X54A |  |  |  |
|  | TB19 | X54B |  |  |  |
|  | TB20 | X54C |  |  |  |
|  | TB21 | X54D |  |  |  |
|  | TB22 | X54E |  |  |  |
|  | TB23 | X54F |  |  |  |
|  | TB24 | X550 |  |  |  |
|  | TB25 | X551 |  |  |  |
|  | TB26 | X552 |  |  |  |
|  | TB27 | X553 |  |  |  |
|  | TB28 | X554 |  |  |  |
|  | TB29 | X555 |  |  |  |
|  | TB30 | X556 |  |  |  |
|  | TB31 | X557 |  |  |  |
|  | TB32 | X558 |  |  |  |
|  | TB33 | X559 |  |  |  |
|  | TB34 | X55A |  |  |  |
|  | TB35 | X55B |  |  |  |
|  | TB36 | X55C |  |  |  |
|  | TB37 | X55D |  |  |  |
|  | TB38 | X55E |  |  |  |
|  | TB39 | X55F |  |  |  |
| COM | TB40 | COM+ | DC 24V(P24) |  |  |
|  | TB41 | COM- | DC 0V(N24) |  |  |

| Station No. |  |  | 35 |  |  |
| --- | --- | --- | --- | --- | --- |
| Station Type |  |  | Remote I/O(AJ65SBTB1-32DT1) |  |  |
| 용 도 |  |  | Port #1,2 In/Out |  |  |
| 구분 | TB No. | Address | Name | 접점종류 | 비고 |
| CC-Link | TB01 |  | DA |  |  |
|  | TB02 |  | DB |  |  |
|  | TB03 |  | DG |  |  |
|  | TB04 |  | SLD |  |  |
|  | TB05 |  | DC 24V |  |  |
|  | TB06 |  | FG |  |  |
|  | TB07 |  | DC 0V |  |  |
| 입력 | TB08 | X560 | CST 감지 센서(전) |  | Port #1 |
|  | TB09 | X561 | CST 감지 센서(후) |  |  |
|  | TB10 | X562 | Clapm 1-1 전진 |  |  |
|  | TB11 | X563 | Clapm 1-1 후진 |  |  |
|  | TB12 | X564 | Clapm 1-2 전진 |  |  |
|  | TB13 | X565 | Clapm 1-2 후진 |  |  |
|  | TB14 | X566 | Clapm 2-1 전진 |  |  |
|  | TB15 | X567 | Clapm 2-1 후진 |  |  |
|  | TB16 | X568 | CST 감지 센서(전) |  | Port #2 |
|  | TB17 | X569 | CST 감지 센서(후) |  |  |
|  | TB18 | X56A | Clapm 1-1 전진 |  |  |
|  | TB19 | X56B | Clapm 1-1 후진 |  |  |
|  | TB20 | X56C | Clapm 1-2 전진 |  |  |
|  | TB21 | X56D | Clapm 1-2 후진 |  |  |
|  | TB22 | X56E | Clapm 2-1 전진 |  |  |
|  | TB23 | X56F | Clapm 2-1 후진 |  |  |
| 출력 | TB24 | Y570 | Clamp 1 전진 |  | Port #1 |
|  | TB25 | Y571 | Clamp 1 후진 |  |  |
|  | TB26 | Y572 | Clamp 2 전진 |  |  |
|  | TB27 | Y573 | Clamp 2 후진 |  |  |
|  | TB28 | Y574 | Floating 풀림 (풀림 On , 잠김 Off) |  |  |
|  | TB29 | Y575 | Floating 잠김 (풀림 Off , 잠김 On) |  |  |
|  | TB30 | Y576 |  |  |  |
|  | TB31 | Y577 | Ionizer CDA 공급 On |  |  |
|  | TB32 | Y578 | Clamp 1 전진 |  | Port #2 |
|  | TB33 | Y579 | Clamp 1 후진 |  |  |
|  | TB34 | Y57A | Clamp 2 전진 |  |  |
|  | TB35 | Y57B | Clamp 2 후진 |  |  |
|  | TB36 | Y57C | Floating 풀림 (풀림 On , 잠김 Off) |  |  |
|  | TB37 | Y57D | Floating 잠김 (풀림 Off , 잠김 On) |  |  |
|  | TB38 | Y57E |  |  |  |
|  | TB39 | Y57F | Ionizer CDA 공급 On |  |  |
| COM | TB40 | COM+ | DC 24V(P24) |  |  |
|  | TB41 | COM- | DC 0V(N24) |  |  |

| Station No. |  |  | 36 |  |  |
| --- | --- | --- | --- | --- | --- |
| Station Type |  |  | Remote I/O(AJ65SBTB1-32DT1) |  |  |
| 용 도 |  |  | Port #3,4 In/Out |  |  |
| 구분 | TB No. | Address | Name | 접점종류 | 비고 |
| CC-Link | TB01 |  | DA |  |  |
|  | TB02 |  | DB |  |  |
|  | TB03 |  | DG |  |  |
|  | TB04 |  | SLD |  |  |
|  | TB05 |  | DC 24V |  |  |
|  | TB06 |  | FG |  |  |
|  | TB07 |  | DC 0V |  |  |
| 입력 | TB08 | X580 | CST 감지 센서(전) |  | Port #3 |
|  | TB09 | X581 | CST 감지 센서(후) |  |  |
|  | TB10 | X582 | Clapm 1-1 전진 |  |  |
|  | TB11 | X583 | Clapm 1-1 후진 |  |  |
|  | TB12 | X584 | Clapm 1-2 전진 |  |  |
|  | TB13 | X585 | Clapm 1-2 후진 |  |  |
|  | TB14 | X586 | Clapm 2-1 전진 |  |  |
|  | TB15 | X587 | Clapm 2-1 후진 |  |  |
|  | TB16 | X588 | CST 감지 센서(전) |  | Port #4 |
|  | TB17 | X589 | CST 감지 센서(후) |  |  |
|  | TB18 | X58A | Clapm 1-1 전진 |  |  |
|  | TB19 | X58B | Clapm 1-1 후진 |  |  |
|  | TB20 | X58C | Clapm 1-2 전진 |  |  |
|  | TB21 | X58D | Clapm 1-2 후진 |  |  |
|  | TB22 | X58E | Clapm 2-1 전진 |  |  |
|  | TB23 | X58F | Clapm 2-1 후진 |  |  |
| 출력 | TB24 | Y590 | Clamp 1 전진 |  | Port #3 |
|  | TB25 | Y591 | Clamp 1 후진 |  |  |
|  | TB26 | Y592 | Clamp 2 전진 |  |  |
|  | TB27 | Y593 | Clamp 2 후진 |  |  |
|  | TB28 | Y594 | Floating 풀림 (풀림 On , 잠김 Off) |  |  |
|  | TB29 | Y595 | Floating 잠김 (풀림 Off , 잠김 On) |  |  |
|  | TB30 | Y596 |  |  |  |
|  | TB31 | Y597 | Ionizer CDA 공급 On |  |  |
|  | TB32 | Y598 | Clamp 1 전진 |  | Port #4 |
|  | TB33 | Y599 | Clamp 1 후진 |  |  |
|  | TB34 | Y59A | Clamp 2 전진 |  |  |
|  | TB35 | Y59B | Clamp 2 후진 |  |  |
|  | TB36 | Y59C | Floating 풀림 (풀림 On , 잠김 Off) |  |  |
|  | TB37 | Y59D | Floating 잠김 (풀림 Off , 잠김 On) |  |  |
|  | TB38 | Y59E |  |  |  |
|  | TB39 | Y59F | Ionizer CDA 공급 On |  |  |
| COM | TB40 | COM+ | DC 24V(P24) |  |  |
|  | TB41 | COM- | DC 0V(N24) |  |  |

| Station No. |  |  | 37 |  |  |
| --- | --- | --- | --- | --- | --- |
| Station Type |  |  | Remote I/O(AJ65SBTB1-32DT1) |  |  |
| 용 도 |  |  | Port #5 In/Out |  |  |
| 구분 | TB No. | Address | Name | 접점종류 | 비고 |
| CC-Link | TB01 |  | DA |  |  |
|  | TB02 |  | DB |  |  |
|  | TB03 |  | DG |  |  |
|  | TB04 |  | SLD |  |  |
|  | TB05 |  | DC 24V |  |  |
|  | TB06 |  | FG |  |  |
|  | TB07 |  | DC 0V |  |  |
| 입력 | TB08 | X5A0 | CST 감지 센서(전) |  | Port #5 |
|  | TB09 | X5A1 | CST 감지 센서(후) |  |  |
|  | TB10 | X5A2 | Clapm 1-1 전진 |  |  |
|  | TB11 | X5A3 | Clapm 1-1 후진 |  |  |
|  | TB12 | X5A4 | Clapm 1-2 전진 |  |  |
|  | TB13 | X5A5 | Clapm 1-2 후진 |  |  |
|  | TB14 | X5A6 | Clapm 2-1 전진 |  |  |
|  | TB15 | X5A7 | Clapm 2-1 후진 |  |  |
|  | TB16 | X5A8 |  |  |  |
|  | TB17 | X5A9 |  |  |  |
|  | TB18 | X5AA |  |  |  |
|  | TB19 | X5AB |  |  |  |
|  | TB20 | X5AC |  |  |  |
|  | TB21 | X5AD |  |  |  |
|  | TB22 | X5AE |  |  |  |
|  | TB23 | X5AF |  |  |  |
| 출력 | TB24 | Y5B0 | Clamp 1 전진 |  | Port #5 |
|  | TB25 | Y5B1 | Clamp 1 후진 |  |  |
|  | TB26 | Y5B2 | Clamp 2 전진 |  |  |
|  | TB27 | Y5B3 | Clamp 2 후진 |  |  |
|  | TB28 | Y5B4 | Floating 풀림 (풀림 On , 잠김 Off) |  |  |
|  | TB29 | Y5B5 | Floating 잠김 (풀림 Off , 잠김 On) |  |  |
|  | TB30 | Y5B6 |  |  |  |
|  | TB31 | Y5B7 | Ionizer CDA 공급 On |  |  |
|  | TB32 | Y5B8 |  |  |  |
|  | TB33 | Y5B9 |  |  |  |
|  | TB34 | Y5BA |  |  |  |
|  | TB35 | Y5BB |  |  |  |
|  | TB36 | Y5BC |  |  |  |
|  | TB37 | Y5BD |  |  |  |
|  | TB38 | Y5BE |  |  |  |
|  | TB39 | Y5BF |  |  |  |
| COM | TB40 | COM+ | DC 24V(P24) |  |  |
|  | TB41 | COM- | DC 0V(N24) |  |  |

| Station No. |  |  | 39 |  |  |
| --- | --- | --- | --- | --- | --- |
| Station Type |  |  | Remote I/O(AJ65SBTB1-32DT1) |  |  |
| 용 도 |  |  | Port #1 ~ #5 Ionizer In/Out |  |  |
| 구분 | TB No. | Address | Name | 접점종류 | 비고 |
| CC-Link | TB01 |  | DA |  |  |
|  | TB02 |  | DB |  |  |
|  | TB03 |  | DG |  |  |
|  | TB04 |  | SLD |  |  |
|  | TB05 |  | DC 24V |  |  |
|  | TB06 |  | FG |  |  |
|  | TB07 |  | DC 0V |  |  |
| 입력 | TB08 | X5E0 | Port #1-1 Ionizer State On |  | Port #1 ~ #5 Ionizer State |
|  | TB09 | X5E1 | Port #1-2 Ionizer State On |  |  |
|  | TB10 | X5E2 | Port #2-1 Ionizer State On |  |  |
|  | TB11 | X5E3 | Port #2-2 Ionizer State On |  |  |
|  | TB12 | X5E4 | Port #3-1 Ionizer State On |  |  |
|  | TB13 | X5E5 | Port #3-2 Ionizer State On |  |  |
|  | TB14 | X5E6 | Port #4-1 Ionizer State On |  |  |
|  | TB15 | X5E7 | Port #4-2 Ionizer State On |  |  |
|  | TB16 | X5E8 | Port #5-1 Ionizer State On |  |  |
|  | TB17 | X5E9 | Port #5-2 Ionizer State On |  |  |
|  | TB18 | X5EA |  |  |  |
|  | TB19 | X5EB |  |  |  |
|  | TB20 | X5EC | X-Ray Ionizer Light Alarm |  | X-Ray Ionizer Alarm |
|  | TB21 | X5ED | X-Ray Ionizer Heavy Alarm |  |  |
|  | TB22 | X5EE | Special Ionizer #1 State On (EQP #1) |  | Special Ionizer State |
|  | TB23 | X5EF | Special Ionizer #2 State On (EQP #2) |  |  |
| 출력 | TB24 | Y5F0 | Port #1 Ionizer Remote Control On | B | Port #1 ~ #5 Ionizer Remote Control |
|  | TB25 | Y5F1 | Port #2 Ionizer Remote Control On | B |  |
|  | TB26 | Y5F2 | Port #3 Ionizer Remote Control On | B |  |
|  | TB27 | Y5F3 | Port #4 Ionizer Remote Control On | B |  |
|  | TB28 | Y5F4 | Port #5 Ionizer Remote Control On | B |  |
|  | TB29 | Y5F5 |  |  |  |
|  | TB30 | Y5F6 | EQP #1 L/L X-Ray Ionizer Remote Control On | B | X-Ray Ionizer Remote Control |
|  | TB31 | Y5F7 | EQP #2 L/L X-Ray Ionizer Remote Control On | B |  |
|  | TB32 | Y5F8 | SIGNAL TOWER RED |  | Signal Tower |
|  | TB33 | Y5F9 | SIGNAL TOWER YELLOW |  |  |
|  | TB34 | Y5FA | SIGNAL TOWER GREEN |  |  |
|  | TB35 | Y5FB | X-Ray Ionizer CDA On |  | X-Ray Ionizer CDA |
|  | TB36 | Y5FC |  |  |  |
|  | TB37 | Y5FD |  |  |  |
|  | TB38 | Y5FE | Special Ionizer #1 Remote Control On (EQP #1) | B | Special Output |
|  | TB39 | Y5FF | Special Ionizer #2 Remote Control On (EQP #2) | B |  |
| COM | TB40 | COM+ | DC 24V(P24) |  |  |
|  | TB41 | COM- | DC 0V(N24) |  |  |

| Station No. |  |  | 41 |  |  |
| --- | --- | --- | --- | --- | --- |
| Station Type |  |  | Remote I/O(AJ65SBTB1-32D1) |  |  |
| 용 도 |  |  | STK Interface(Port #1 ~ #5) Interlock Input (Loader) |  |  |
| 구분 | TB No. | Address | Name | 접점종류 | 비고 |
| CC-Link | TB01 |  | DA |  |  |
|  | TB02 |  | DB |  |  |
|  | TB03 |  | DG |  |  |
|  | TB04 |  | SLD |  |  |
|  | TB05 |  | DC 24V |  |  |
|  | TB06 |  | FG |  |  |
|  | TB07 |  | DC 0V |  |  |
| 입력 | TB08 | X620 | Port #1 STK TR_REQ(Pin #3) |  | STK Interlock Input (Port #1 ~ #4) |
|  | TB09 | X621 | Port #1 STK Complete(Pin #5) |  |  |
|  | TB10 | X622 | Port #2 STK TR_REQ(Pin #8) |  |  |
|  | TB11 | X623 | Port #2 STK Complete(Pin #10) |  |  |
|  | TB12 | X624 | Port #3 STK TR_REQ(Pin #13) |  |  |
|  | TB13 | X625 | Port #3 STK Complete(Pin #15) |  |  |
|  | TB14 | X626 | Port #4 STK TR_REQ(Pin #18) |  |  |
|  | TB15 | X627 | Port #4 STK Complete(Pin #20) |  |  |
|  | TB16 | X628 | Port #1 ~ #4 STK Busy(Pin #21) |  |  |
|  | TB17 | X629 |  |  |  |
|  | TB18 | X62A | Port #5 STK TR_REQ(Pin #3) |  | STK Interlock Input (Port #5) |
|  | TB19 | X62B | Port #5 STK Complete(Pin #5) |  |  |
|  | TB20 | X62C |  |  |  |
|  | TB21 | X62D |  |  |  |
|  | TB22 | X62E |  |  |  |
|  | TB23 | X62F |  |  |  |
|  | TB24 | X630 |  |  |  |
|  | TB25 | X631 |  |  |  |
|  | TB26 | X632 | Port #5 STK Busy(Pin #21) |  |  |
|  | TB27 | X633 |  |  |  |
|  | TB28 | X634 |  |  |  |
|  | TB29 | X635 |  |  |  |
|  | TB30 | X636 |  |  |  |
|  | TB31 | X637 |  |  |  |
|  | TB32 | X638 |  |  |  |
|  | TB33 | X639 |  |  |  |
|  | TB34 | X63A |  |  |  |
|  | TB35 | X63B |  |  |  |
|  | TB36 | X63C |  |  |  |
|  | TB37 | X63D |  |  |  |
|  | TB38 | X63E |  |  |  |
|  | TB39 | X63F |  |  |  |
| COM | TB40 | COM+ | DC 24V(P24) |  |  |
|  | TB41 | COM- | DC 0V(N24) |  |  |

| Station No. |  |  | 42 |  |  |
| --- | --- | --- | --- | --- | --- |
| Station Type |  |  | Remote I/O(AJ65SBTB2N-16R) |  |  |
| 용 도 |  |  | STK Interface(Port #1 ~ #4) Interlock Output |  |  |
| 구분 | TB No. | Address | Name | 접점종류 | 비고 |
| CC-Link | TB01 |  | DA |  |  |
|  | TB02 |  | DB |  |  |
|  | TB03 |  | DG |  |  |
|  | TB04 |  | SLD |  |  |
|  | TB05 |  | DC 24V |  |  |
|  | TB06 |  | FG |  |  |
|  | TB07 |  | DC 0V |  |  |
| 출력 | TB08 | Y640 | Port #1 STK LOAD_REQ(Pin #1) |  | STK Interlock Output (Port #1 ~ #4) |
|  | TB09 | COM B | Common Port #1 ~ #4(Pin #25) |  |  |
|  | TB10 | Y641 | Port #1 STK UNLOAD_REQ(Pin #2) |  |  |
|  | TB11 | COM B | Common Port #1 ~ #4(Pin #25) |  |  |
|  | TB12 | Y642 | Port #1 STK READY(Pin #4) |  |  |
|  | TB13 | COM B | Common Port #1 ~ #4(Pin #25) |  |  |
|  | TB14 | Y643 | Port #2 STK LOAD_REQ(Pin #6) |  |  |
|  | TB15 | COM B | Common Port #1 ~ #4(Pin #25) |  |  |
|  | TB16 | Y644 | Port #2 STK UNLOAD_REQ(Pin #7) |  |  |
|  | TB17 | COM B | Common Port #1 ~ #4(Pin #25) |  |  |
|  | TB18 | Y645 | Port #2 STK READY(Pin #9) |  |  |
|  | TB19 | COM B | Common Port #1 ~ #4(Pin #25) |  |  |
|  | TB20 | Y646 | Spare |  |  |
|  | TB21 | COM B |  |  |  |
|  | TB22 | Y647 | Spare |  |  |
|  | TB23 | COM B |  |  |  |
|  | TB24 | Y648 | Port #3 STK LOAD_REQ(Pin #11) |  |  |
|  | TB25 | COM B | Common Port #1 ~ #4(Pin #25) |  |  |
|  | TB26 | Y649 | Port #3 STK UNLOAD_REQ(Pin #112) |  |  |
|  | TB27 | COM B | Common Port #1 ~ #4(Pin #25) |  |  |
|  | TB28 | Y64A | Port #3 STK READY(Pin #14) |  |  |
|  | TB29 | COM B | Common Port #1 ~ #4(Pin #25) |  |  |
|  | TB30 | Y64B | Port #4 STK LOAD_REQ(Pin #16) |  |  |
|  | TB31 | COM B | Common Port #1 ~ #4(Pin #25) |  |  |
|  | TB32 | Y64C | Port #4 STK UNLOAD_REQ(Pin #17) |  |  |
|  | TB33 | COM B | Common Port #1 ~ #4(Pin #25) |  |  |
|  | TB34 | Y64D | Port #4 STK READY(Pin #19) |  |  |
|  | TB35 | COM B | Common Port #1 ~ #4(Pin #25) |  |  |
|  | TB36 | Y64E | Spare |  |  |
|  | TB37 | COM B |  |  |  |
|  | TB38 | Y64F | Port Occupy(Pin #22) |  |  |
|  | TB39 | COM B | Common Port #1 ~ #4(Pin #25) |  |  |
| COM | TB40 | COM A | DC 24V |  |  |
|  | TB41 | COM B | DC 0V |  |  |

| Station No. |  |  | 43 |  |  |
| --- | --- | --- | --- | --- | --- |
| Station Type |  |  | Remote I/O(AJ65SBTB2N-16R) |  |  |
| 용 도 |  |  | STK Interface(Port #5) Interlock Output |  |  |
| 구분 | TB No. | Address | Name | 접점종류 | 비고 |
| CC-Link | TB01 |  | DA |  |  |
|  | TB02 |  | DB |  |  |
|  | TB03 |  | DG |  |  |
|  | TB04 |  | SLD |  |  |
|  | TB05 |  | DC 24V |  |  |
|  | TB06 |  | FG |  |  |
|  | TB07 |  | DC 0V |  |  |
| 출력 | TB08 | Y660 | Port #5 STK LOAD_REQ(Pin #1) |  | STK Interlock Output (Port #5) |
|  | TB09 | COM B | Common Port #5 ~ #6(Pin #25) |  |  |
|  | TB10 | Y661 | Port #5 STK UNLOAD_REQ(Pin #2) |  |  |
|  | TB11 | COM B | Common Port #5 ~ #6(Pin #25) |  |  |
|  | TB12 | Y662 | Port #5 STK READY(Pin #4) |  |  |
|  | TB13 | COM B | Common Port #5 ~ #6(Pin #25) |  |  |
|  | TB14 | Y663 |  |  |  |
|  | TB15 | COM B |  |  |  |
|  | TB16 | Y664 |  |  |  |
|  | TB17 | COM B |  |  |  |
|  | TB18 | Y665 |  |  |  |
|  | TB19 | COM B |  |  |  |
|  | TB20 | Y666 | Spare |  |  |
|  | TB21 | COM B |  |  |  |
|  | TB22 | Y667 | Spare |  |  |
|  | TB23 | COM B |  |  |  |
|  | TB24 | Y668 |  |  |  |
|  | TB25 | COM B |  |  |  |
|  | TB26 | Y669 |  |  |  |
|  | TB27 | COM B |  |  |  |
|  | TB28 | Y66A |  |  |  |
|  | TB29 | COM B |  |  |  |
|  | TB30 | Y66B |  |  |  |
|  | TB31 | COM B |  |  |  |
|  | TB32 | Y66C |  |  |  |
|  | TB33 | COM B |  |  |  |
|  | TB34 | Y66D |  |  |  |
|  | TB35 | COM B |  |  |  |
|  | TB36 | Y66E |  |  |  |
|  | TB37 | COM B |  |  |  |
|  | TB38 | Y66F | Port Occupy(Pin #22) |  |  |
|  | TB39 | COM B | Common Port #5(Pin #25) |  |  |
| COM | TB40 | COM A | DC 24V |  |  |
|  | TB41 | COM B | DC 0V |  |  |

| Station No. |  |  | 44 |  |  |
| --- | --- | --- | --- | --- | --- |
| Station Type |  |  | Remote I/O(AJ65SBTB1-32D1) |  |  |
| 용 도 |  |  | STK Interface(Port #1 ~ #5) Interlock Input (Unloader) |  |  |
| 구분 | TB No. | Address | Name | 접점종류 | 비고 |
| CC-Link | TB01 |  | DA |  |  |
|  | TB02 |  | DB |  |  |
|  | TB03 |  | DG |  |  |
|  | TB04 |  | SLD |  |  |
|  | TB05 |  | DC 24V |  |  |
|  | TB06 |  | FG |  |  |
|  | TB07 |  | DC 0V |  |  |
| 입력 | TB08 | X680 | Port #1 STK TR_REQ(Pin #3) |  | STK Interlock Input (Port #1 ~ #4) |
|  | TB09 | X681 | Port #1 STK Complete(Pin #5) |  |  |
|  | TB10 | X682 | Port #2 STK TR_REQ(Pin #8) |  |  |
|  | TB11 | X683 | Port #2 STK Complete(Pin #10) |  |  |
|  | TB12 | X684 | Port #3 STK TR_REQ(Pin #13) |  |  |
|  | TB13 | X685 | Port #3 STK Complete(Pin #15) |  |  |
|  | TB14 | X686 | Port #4 STK TR_REQ(Pin #18) |  |  |
|  | TB15 | X687 | Port #4 STK Complete(Pin #20) |  |  |
|  | TB16 | X688 | Port #1 ~ #4 STK Busy(Pin #21) |  |  |
|  | TB17 | X689 |  |  |  |
|  | TB18 | X68A | Port #5 STK TR_REQ(Pin #3) |  | STK Interlock Input (Port #5) |
|  | TB19 | X68B | Port #5 STK Complete(Pin #5) |  |  |
|  | TB20 | X68C |  |  |  |
|  | TB21 | X68D |  |  |  |
|  | TB22 | X68E |  |  |  |
|  | TB23 | X68F |  |  |  |
|  | TB24 | X690 |  |  |  |
|  | TB25 | X691 |  |  |  |
|  | TB26 | X692 | Port #5 STK Busy(Pin #21) |  |  |
|  | TB27 | X693 |  |  |  |
|  | TB28 | X694 |  |  |  |
|  | TB29 | X695 |  |  |  |
|  | TB30 | X696 |  |  |  |
|  | TB31 | X697 |  |  |  |
|  | TB32 | X698 |  |  |  |
|  | TB33 | X699 |  |  |  |
|  | TB34 | X69A |  |  |  |
|  | TB35 | X69B |  |  |  |
|  | TB36 | X69C |  |  |  |
|  | TB37 | X69D |  |  |  |
|  | TB38 | X69E |  |  |  |
|  | TB39 | X69F |  |  |  |
| COM | TB40 | COM+ | DC 24V(P24) |  |  |
|  | TB41 | COM- | DC 0V(N24) |  |  |

| Station No. |  | 45 |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Station Type |  | Remote I/O |  |  |  |
| 용 도 |  | Port #1 Mapping Unit |  |  |  |
| 입력 |  | Name | 출력 |  | Name |
| Master | Remote |  | Master | Remote |  |
| X6A0 | X000 | Slot #1 Glass 감지 | Y6A0 | Y000 |  |
| X6A1 | X001 | Slot #2 Glass 감지 | Y6A1 | Y001 |  |
| X6A2 | X002 | Slot #3 Glass 감지 | Y6A2 | Y002 |  |
| X6A3 | X003 | Slot #4 Glass 감지 | Y6A3 | Y003 |  |
| X6A4 | X004 | Slot #5 Glass 감지 | Y6A4 | Y004 |  |
| X6A5 | X005 | Slot #6 Glass 감지 | Y6A5 | Y005 |  |
| X6A6 | X006 | Slot #7 Glass 감지 | Y6A6 | Y006 |  |
| X6A7 | X007 | Slot #8 Glass 감지 | Y6A7 | Y007 |  |
| X6A8 | X008 | Slot #9 Glass 감지 | Y6A8 | Y008 |  |
| X6A9 | X009 | Slot #10 Glass 감지 | Y6A9 | Y009 |  |
| X6AA | X00A | Slot #11 Glass 감지 | Y6AA | Y00A |  |
| X6AB | X00B | Slot #12 Glass 감지 | Y6AB | Y00B |  |
| X6AC | X00C | Slot #13 Glass 감지 | Y6AC | Y00C |  |
| X6AD | X00D | Slot #14 Glass 감지 | Y6AD | Y00D |  |
| X6AE | X00E | Slot #15 Glass 감지 | Y6AE | Y00E |  |
| X6AF | X00F | Slot #16 Glass 감지 | Y6AF | Y00F |  |
| X6B0 | X010 | Slot #17 Glass 감지 | Y6B0 | Y010 |  |
| X6B1 | X011 | Slot #18 Glass 감지 | Y6B1 | Y011 |  |
| X6B2 | X012 | Slot #19 Glass 감지 | Y6B2 | Y012 |  |
| X6B3 | X013 | Slot #20 Glass 감지 | Y6B3 | Y013 |  |
| X6B4 | X014 | Slot #21 Glass 감지 | Y6B4 | Y014 |  |
| X6B5 | X015 | Slot #22 Glass 감지 | Y6B5 | Y015 |  |
| X6B6 | X016 | Slot #23 Glass 감지 | Y6B6 | Y016 |  |
| X6B7 | X017 | Slot #24 Glass 감지 | Y6B7 | Y017 |  |
| X6B8 | X018 | Slot #25 Glass 감지 | Y6B8 | Y018 |  |
| X6B9 | X019 | Slot #26 Glass 감지 | Y6B9 | Y019 |  |
| X6BA | X01A | Slot #27 Glass 감지 | Y6BA | Y01A |  |
| X6BB | X01B | Slot #28 Glass 감지 | Y6BB | Y01B |  |
| X6BC | X01C | Slot #29 Glass 감지 | Y6BC | Y01C |  |
| X6BD | X01D | Slot #30 Glass 감지 | Y6BD | Y01D |  |
| X6BE | X01E |  | Y6BE | Y01E |  |
| X6BF | X01F |  | Y6BF | Y01F |  |

| Station No. |  | 46 |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Station Type |  | Remote I/O |  |  |  |
| 용 도 |  | Port #1 Mapping Unit |  |  |  |
| 입력 |  | Name | 출력 |  | Name |
| Master | Remote |  | Master | Remote |  |
| X6C0 | X000 | Slot #31 Glass 감지 | Y6C0 | Y000 |  |
| X6C1 | X001 | Slot #32 Glass 감지 | Y6C1 | Y001 |  |
| X6C2 | X002 | Slot #33 Glass 감지 | Y6C2 | Y002 |  |
| X6C3 | X003 | Slot #34 Glass 감지 | Y6C3 | Y003 |  |
| X6C4 | X004 | Slot #35 Glass 감지 | Y6C4 | Y004 |  |
| X6C5 | X005 | Slot #36 Glass 감지 | Y6C5 | Y005 |  |
| X6C6 | X006 | Slot #37 Glass 감지 | Y6C6 | Y006 |  |
| X6C7 | X007 | Slot #38 Glass 감지 | Y6C7 | Y007 |  |
| X6C8 | X008 | Slot #39 Glass 감지 | Y6C8 | Y008 |  |
| X6C9 | X009 | Slot #40 Glass 감지 | Y6C9 | Y009 |  |
| X6CA | X00A | Slot #41 Glass 감지 | Y6CA | Y00A |  |
| X6CB | X00B | Slot #42 Glass 감지 | Y6CB | Y00B |  |
| X6CC | X00C | Slot #43 Glass 감지 | Y6CC | Y00C |  |
| X6CD | X00D | Slot #44 Glass 감지 | Y6CD | Y00D |  |
| X6CE | X00E | Slot #45 Glass 감지 | Y6CE | Y00E |  |
| X6CF | X00F | Slot #46 Glass 감지 | Y6CF | Y00F |  |
| X6D0 | X010 | Slot #47 Glass 감지 | Y6D0 | Y010 |  |
| X6D1 | X011 | Slot #48 Glass 감지 | Y6D1 | Y011 |  |
| X6D2 | X012 | Slot #49 Glass 감지 | Y6D2 | Y012 |  |
| X6D3 | X013 | Slot #50 Glass 감지 | Y6D3 | Y013 |  |
| X6D4 | X014 | Slot #51 Glass 감지 | Y6D4 | Y014 |  |
| X6D5 | X015 | Slot #52 Glass 감지 | Y6D5 | Y015 |  |
| X6D6 | X016 | Slot #53 Glass 감지 | Y6D6 | Y016 |  |
| X6D7 | X017 | Slot #54 Glass 감지 | Y6D7 | Y017 |  |
| X6D8 | X018 | Slot #55 Glass 감지 | Y6D8 | Y018 |  |
| X6D9 | X019 | Slot #56 Glass 감지 | Y6D9 | Y019 |  |
| X6DA | X01A | Slot #57 Glass 감지 | Y6DA | Y01A |  |
| X6DB | X01B | Slot #58 Glass 감지 | Y6DB | Y01B |  |
| X6DC | X01C | Slot #59 Glass 감지 | Y6DC | Y01C |  |
| X6DD | X01D | Slot #60 Glass 감지 | Y6DD | Y01D |  |
| X6DE | X01E |  | Y6DE | Y01E |  |
| X6DF | X01F |  | Y6DF | Y01F |  |

| Station No. |  | 47 |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Station Type |  | Remote I/O |  |  |  |
| 용 도 |  | Port #2 Mapping Unit |  |  |  |
| 입력 |  | Name | 출력 |  | Name |
| Master | Remote |  | Master | Remote |  |
| X6E0 | X000 | Slot #1 Glass 감지 | Y6E0 | Y000 |  |
| X6E1 | X001 | Slot #2 Glass 감지 | Y6E1 | Y001 |  |
| X6E2 | X002 | Slot #3 Glass 감지 | Y6E2 | Y002 |  |
| X6E3 | X003 | Slot #4 Glass 감지 | Y6E3 | Y003 |  |
| X6E4 | X004 | Slot #5 Glass 감지 | Y6E4 | Y004 |  |
| X6E5 | X005 | Slot #6 Glass 감지 | Y6E5 | Y005 |  |
| X6E6 | X006 | Slot #7 Glass 감지 | Y6E6 | Y006 |  |
| X6E7 | X007 | Slot #8 Glass 감지 | Y6E7 | Y007 |  |
| X6E8 | X008 | Slot #9 Glass 감지 | Y6E8 | Y008 |  |
| X6E9 | X009 | Slot #10 Glass 감지 | Y6E9 | Y009 |  |
| X6EA | X00A | Slot #11 Glass 감지 | Y6EA | Y00A |  |
| X6EB | X00B | Slot #12 Glass 감지 | Y6EB | Y00B |  |
| X6EC | X00C | Slot #13 Glass 감지 | Y6EC | Y00C |  |
| X6ED | X00D | Slot #14 Glass 감지 | Y6ED | Y00D |  |
| X6EE | X00E | Slot #15 Glass 감지 | Y6EE | Y00E |  |
| X6EF | X00F | Slot #16 Glass 감지 | Y6EF | Y00F |  |
| X6F0 | X010 | Slot #17 Glass 감지 | Y6F0 | Y010 |  |
| X6F1 | X011 | Slot #18 Glass 감지 | Y6F1 | Y011 |  |
| X6F2 | X012 | Slot #19 Glass 감지 | Y6F2 | Y012 |  |
| X6F3 | X013 | Slot #20 Glass 감지 | Y6F3 | Y013 |  |
| X6F4 | X014 | Slot #21 Glass 감지 | Y6F4 | Y014 |  |
| X6F5 | X015 | Slot #22 Glass 감지 | Y6F5 | Y015 |  |
| X6F6 | X016 | Slot #23 Glass 감지 | Y6F6 | Y016 |  |
| X6F7 | X017 | Slot #24 Glass 감지 | Y6F7 | Y017 |  |
| X6F8 | X018 | Slot #25 Glass 감지 | Y6F8 | Y018 |  |
| X6F9 | X019 | Slot #26 Glass 감지 | Y6F9 | Y019 |  |
| X6FA | X01A | Slot #27 Glass 감지 | Y6FA | Y01A |  |
| X6FB | X01B | Slot #28 Glass 감지 | Y6FB | Y01B |  |
| X6FC | X01C | Slot #29 Glass 감지 | Y6FC | Y01C |  |
| X6FD | X01D | Slot #30 Glass 감지 | Y6FD | Y01D |  |
| X6FE | X01E |  | Y6FE | Y01E |  |
| X6FF | X01F |  | Y6FF | Y01F |  |

| Station No. |  | 48 |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Station Type |  | Remote I/O |  |  |  |
| 용 도 |  | Port #2 Mapping Unit |  |  |  |
| 입력 |  | Name | 출력 |  | Name |
| Master | Remote |  | Master | Remote |  |
| X700 | X000 | Slot #31 Glass 감지 | Y700 | Y000 |  |
| X701 | X001 | Slot #32 Glass 감지 | Y701 | Y001 |  |
| X702 | X002 | Slot #33 Glass 감지 | Y702 | Y002 |  |
| X703 | X003 | Slot #34 Glass 감지 | Y703 | Y003 |  |
| X704 | X004 | Slot #35 Glass 감지 | Y704 | Y004 |  |
| X705 | X005 | Slot #36 Glass 감지 | Y705 | Y005 |  |
| X706 | X006 | Slot #37 Glass 감지 | Y706 | Y006 |  |
| X707 | X007 | Slot #38 Glass 감지 | Y707 | Y007 |  |
| X708 | X008 | Slot #39 Glass 감지 | Y708 | Y008 |  |
| X709 | X009 | Slot #40 Glass 감지 | Y709 | Y009 |  |
| X70A | X00A | Slot #41 Glass 감지 | Y70A | Y00A |  |
| X70B | X00B | Slot #42 Glass 감지 | Y70B | Y00B |  |
| X70C | X00C | Slot #43 Glass 감지 | Y70C | Y00C |  |
| X70D | X00D | Slot #44 Glass 감지 | Y70D | Y00D |  |
| X70E | X00E | Slot #45 Glass 감지 | Y70E | Y00E |  |
| X70F | X00F | Slot #46 Glass 감지 | Y70F | Y00F |  |
| X710 | X010 | Slot #47 Glass 감지 | Y710 | Y010 |  |
| X711 | X011 | Slot #48 Glass 감지 | Y711 | Y011 |  |
| X712 | X012 | Slot #49 Glass 감지 | Y712 | Y012 |  |
| X713 | X013 | Slot #50 Glass 감지 | Y713 | Y013 |  |
| X714 | X014 | Slot #51 Glass 감지 | Y714 | Y014 |  |
| X715 | X015 | Slot #52 Glass 감지 | Y715 | Y015 |  |
| X716 | X016 | Slot #53 Glass 감지 | Y716 | Y016 |  |
| X717 | X017 | Slot #54 Glass 감지 | Y717 | Y017 |  |
| X718 | X018 | Slot #55 Glass 감지 | Y718 | Y018 |  |
| X719 | X019 | Slot #56 Glass 감지 | Y719 | Y019 |  |
| X71A | X01A | Slot #57 Glass 감지 | Y71A | Y01A |  |
| X71B | X01B | Slot #58 Glass 감지 | Y71B | Y01B |  |
| X71C | X01C | Slot #59 Glass 감지 | Y71C | Y01C |  |
| X71D | X01D | Slot #60 Glass 감지 | Y71D | Y01D |  |
| X71E | X01E |  | Y71E | Y01E |  |
| X71F | X01F |  | Y71F | Y01F |  |

| Station No. |  | 49 |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Station Type |  | Remote I/O |  |  |  |
| 용 도 |  | Port #3 Mapping Unit |  |  |  |
| 입력 |  | Name | 출력 |  | Name |
| Master | Remote |  | Master | Remote |  |
| X720 | X000 | Slot #1 Glass 감지 | Y720 | Y000 |  |
| X721 | X001 | Slot #2 Glass 감지 | Y721 | Y001 |  |
| X722 | X002 | Slot #3 Glass 감지 | Y722 | Y002 |  |
| X723 | X003 | Slot #4 Glass 감지 | Y723 | Y003 |  |
| X724 | X004 | Slot #5 Glass 감지 | Y724 | Y004 |  |
| X725 | X005 | Slot #6 Glass 감지 | Y725 | Y005 |  |
| X726 | X006 | Slot #7 Glass 감지 | Y726 | Y006 |  |
| X727 | X007 | Slot #8 Glass 감지 | Y727 | Y007 |  |
| X728 | X008 | Slot #9 Glass 감지 | Y728 | Y008 |  |
| X729 | X009 | Slot #10 Glass 감지 | Y729 | Y009 |  |
| X72A | X00A | Slot #11 Glass 감지 | Y72A | Y00A |  |
| X72B | X00B | Slot #12 Glass 감지 | Y72B | Y00B |  |
| X72C | X00C | Slot #13 Glass 감지 | Y72C | Y00C |  |
| X72D | X00D | Slot #14 Glass 감지 | Y72D | Y00D |  |
| X72E | X00E | Slot #15 Glass 감지 | Y72E | Y00E |  |
| X72F | X00F | Slot #16 Glass 감지 | Y72F | Y00F |  |
| X730 | X010 | Slot #17 Glass 감지 | Y730 | Y010 |  |
| X731 | X011 | Slot #18 Glass 감지 | Y731 | Y011 |  |
| X732 | X012 | Slot #19 Glass 감지 | Y732 | Y012 |  |
| X733 | X013 | Slot #20 Glass 감지 | Y733 | Y013 |  |
| X734 | X014 | Slot #21 Glass 감지 | Y734 | Y014 |  |
| X735 | X015 | Slot #22 Glass 감지 | Y735 | Y015 |  |
| X736 | X016 | Slot #23 Glass 감지 | Y736 | Y016 |  |
| X737 | X017 | Slot #24 Glass 감지 | Y737 | Y017 |  |
| X738 | X018 | Slot #25 Glass 감지 | Y738 | Y018 |  |
| X739 | X019 | Slot #26 Glass 감지 | Y739 | Y019 |  |
| X73A | X01A | Slot #27 Glass 감지 | Y73A | Y01A |  |
| X73B | X01B | Slot #28 Glass 감지 | Y73B | Y01B |  |
| X73C | X01C | Slot #29 Glass 감지 | Y73C | Y01C |  |
| X73D | X01D | Slot #30 Glass 감지 | Y73D | Y01D |  |
| X73E | X01E |  | Y73E | Y01E |  |
| X73F | X01F |  | Y73F | Y01F |  |

| Station No. |  | 50 |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Station Type |  | Remote I/O |  |  |  |
| 용 도 |  | Port #3 Mapping Unit |  |  |  |
| 입력 |  | Name | 출력 |  | Name |
| Master | Remote |  | Master | Remote |  |
| X740 | X000 | Slot #31 Glass 감지 | Y740 | Y000 |  |
| X741 | X001 | Slot #32 Glass 감지 | Y741 | Y001 |  |
| X742 | X002 | Slot #33 Glass 감지 | Y742 | Y002 |  |
| X743 | X003 | Slot #34 Glass 감지 | Y743 | Y003 |  |
| X744 | X004 | Slot #35 Glass 감지 | Y744 | Y004 |  |
| X745 | X005 | Slot #36 Glass 감지 | Y745 | Y005 |  |
| X746 | X006 | Slot #37 Glass 감지 | Y746 | Y006 |  |
| X747 | X007 | Slot #38 Glass 감지 | Y747 | Y007 |  |
| X748 | X008 | Slot #39 Glass 감지 | Y748 | Y008 |  |
| X749 | X009 | Slot #40 Glass 감지 | Y749 | Y009 |  |
| X74A | X00A | Slot #41 Glass 감지 | Y74A | Y00A |  |
| X74B | X00B | Slot #42 Glass 감지 | Y74B | Y00B |  |
| X74C | X00C | Slot #43 Glass 감지 | Y74C | Y00C |  |
| X74D | X00D | Slot #44 Glass 감지 | Y74D | Y00D |  |
| X74E | X00E | Slot #45 Glass 감지 | Y74E | Y00E |  |
| X74F | X00F | Slot #46 Glass 감지 | Y74F | Y00F |  |
| X750 | X010 | Slot #47 Glass 감지 | Y750 | Y010 |  |
| X751 | X011 | Slot #48 Glass 감지 | Y751 | Y011 |  |
| X752 | X012 | Slot #49 Glass 감지 | Y752 | Y012 |  |
| X753 | X013 | Slot #50 Glass 감지 | Y753 | Y013 |  |
| X754 | X014 | Slot #51 Glass 감지 | Y754 | Y014 |  |
| X755 | X015 | Slot #52 Glass 감지 | Y755 | Y015 |  |
| X756 | X016 | Slot #53 Glass 감지 | Y756 | Y016 |  |
| X757 | X017 | Slot #54 Glass 감지 | Y757 | Y017 |  |
| X758 | X018 | Slot #55 Glass 감지 | Y758 | Y018 |  |
| X759 | X019 | Slot #56 Glass 감지 | Y759 | Y019 |  |
| X75A | X01A | Slot #57 Glass 감지 | Y75A | Y01A |  |
| X75B | X01B | Slot #58 Glass 감지 | Y75B | Y01B |  |
| X75C | X01C | Slot #59 Glass 감지 | Y75C | Y01C |  |
| X75D | X01D | Slot #60 Glass 감지 | Y75D | Y01D |  |
| X75E | X01E |  | Y75E | Y01E |  |
| X75F | X01F |  | Y75F | Y01F |  |

| Station No. |  | 51 |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Station Type |  | Remote I/O |  |  |  |
| 용 도 |  | Port #4 Mapping Unit |  |  |  |
| 입력 |  | Name | 출력 |  | Name |
| Master | Remote |  | Master | Remote |  |
| X760 | X000 | Slot #1 Glass 감지 | Y760 | Y000 |  |
| X761 | X001 | Slot #2 Glass 감지 | Y761 | Y001 |  |
| X762 | X002 | Slot #3 Glass 감지 | Y762 | Y002 |  |
| X763 | X003 | Slot #4 Glass 감지 | Y763 | Y003 |  |
| X764 | X004 | Slot #5 Glass 감지 | Y764 | Y004 |  |
| X765 | X005 | Slot #6 Glass 감지 | Y765 | Y005 |  |
| X766 | X006 | Slot #7 Glass 감지 | Y766 | Y006 |  |
| X767 | X007 | Slot #8 Glass 감지 | Y767 | Y007 |  |
| X768 | X008 | Slot #9 Glass 감지 | Y768 | Y008 |  |
| X769 | X009 | Slot #10 Glass 감지 | Y769 | Y009 |  |
| X76A | X00A | Slot #11 Glass 감지 | Y76A | Y00A |  |
| X76B | X00B | Slot #12 Glass 감지 | Y76B | Y00B |  |
| X76C | X00C | Slot #13 Glass 감지 | Y76C | Y00C |  |
| X76D | X00D | Slot #14 Glass 감지 | Y76D | Y00D |  |
| X76E | X00E | Slot #15 Glass 감지 | Y76E | Y00E |  |
| X76F | X00F | Slot #16 Glass 감지 | Y76F | Y00F |  |
| X770 | X010 | Slot #17 Glass 감지 | Y770 | Y010 |  |
| X771 | X011 | Slot #18 Glass 감지 | Y771 | Y011 |  |
| X772 | X012 | Slot #19 Glass 감지 | Y772 | Y012 |  |
| X773 | X013 | Slot #20 Glass 감지 | Y773 | Y013 |  |
| X774 | X014 | Slot #21 Glass 감지 | Y774 | Y014 |  |
| X775 | X015 | Slot #22 Glass 감지 | Y775 | Y015 |  |
| X776 | X016 | Slot #23 Glass 감지 | Y776 | Y016 |  |
| X777 | X017 | Slot #24 Glass 감지 | Y777 | Y017 |  |
| X778 | X018 | Slot #25 Glass 감지 | Y778 | Y018 |  |
| X779 | X019 | Slot #26 Glass 감지 | Y779 | Y019 |  |
| X77A | X01A | Slot #27 Glass 감지 | Y77A | Y01A |  |
| X77B | X01B | Slot #28 Glass 감지 | Y77B | Y01B |  |
| X77C | X01C | Slot #29 Glass 감지 | Y77C | Y01C |  |
| X77D | X01D | Slot #30 Glass 감지 | Y77D | Y01D |  |
| X77E | X01E |  | Y77E | Y01E |  |
| X77F | X01F |  | Y77F | Y01F |  |

| Station No. |  | 52 |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Station Type |  | Remote I/O |  |  |  |
| 용 도 |  | Port #4 Mapping Unit |  |  |  |
| 입력 |  | Name | 출력 |  | Name |
| Master | Remote |  | Master | Remote |  |
| X780 | X000 | Slot #31 Glass 감지 | Y780 | Y000 |  |
| X781 | X001 | Slot #32 Glass 감지 | Y781 | Y001 |  |
| X782 | X002 | Slot #33 Glass 감지 | Y782 | Y002 |  |
| X783 | X003 | Slot #34 Glass 감지 | Y783 | Y003 |  |
| X784 | X004 | Slot #35 Glass 감지 | Y784 | Y004 |  |
| X785 | X005 | Slot #36 Glass 감지 | Y785 | Y005 |  |
| X786 | X006 | Slot #37 Glass 감지 | Y786 | Y006 |  |
| X787 | X007 | Slot #38 Glass 감지 | Y787 | Y007 |  |
| X788 | X008 | Slot #39 Glass 감지 | Y788 | Y008 |  |
| X789 | X009 | Slot #40 Glass 감지 | Y789 | Y009 |  |
| X78A | X00A | Slot #41 Glass 감지 | Y78A | Y00A |  |
| X78B | X00B | Slot #42 Glass 감지 | Y78B | Y00B |  |
| X78C | X00C | Slot #43 Glass 감지 | Y78C | Y00C |  |
| X78D | X00D | Slot #44 Glass 감지 | Y78D | Y00D |  |
| X78E | X00E | Slot #45 Glass 감지 | Y78E | Y00E |  |
| X78F | X00F | Slot #46 Glass 감지 | Y78F | Y00F |  |
| X790 | X010 | Slot #47 Glass 감지 | Y790 | Y010 |  |
| X791 | X011 | Slot #48 Glass 감지 | Y791 | Y011 |  |
| X792 | X012 | Slot #49 Glass 감지 | Y792 | Y012 |  |
| X793 | X013 | Slot #50 Glass 감지 | Y793 | Y013 |  |
| X794 | X014 | Slot #51 Glass 감지 | Y794 | Y014 |  |
| X795 | X015 | Slot #52 Glass 감지 | Y795 | Y015 |  |
| X796 | X016 | Slot #53 Glass 감지 | Y796 | Y016 |  |
| X797 | X017 | Slot #54 Glass 감지 | Y797 | Y017 |  |
| X798 | X018 | Slot #55 Glass 감지 | Y798 | Y018 |  |
| X799 | X019 | Slot #56 Glass 감지 | Y799 | Y019 |  |
| X79A | X01A | Slot #57 Glass 감지 | Y79A | Y01A |  |
| X79B | X01B | Slot #58 Glass 감지 | Y79B | Y01B |  |
| X79C | X01C | Slot #59 Glass 감지 | Y79C | Y01C |  |
| X79D | X01D | Slot #60 Glass 감지 | Y79D | Y01D |  |
| X79E | X01E |  | Y79E | Y01E |  |
| X79F | X01F |  | Y79F | Y01F |  |

| Station No. |  | 53 |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Station Type |  | Remote I/O |  |  |  |
| 용 도 |  | Port #5 Mapping Unit |  |  |  |
| 입력 |  | Name | 출력 |  | Name |
| Master | Remote |  | Master | Remote |  |
| X7A0 | X000 | Slot #1 Glass 감지 | Y7A0 | Y000 |  |
| X7A1 | X001 | Slot #2 Glass 감지 | Y7A1 | Y001 |  |
| X7A2 | X002 | Slot #3 Glass 감지 | Y7A2 | Y002 |  |
| X7A3 | X003 | Slot #4 Glass 감지 | Y7A3 | Y003 |  |
| X7A4 | X004 | Slot #5 Glass 감지 | Y7A4 | Y004 |  |
| X7A5 | X005 | Slot #6 Glass 감지 | Y7A5 | Y005 |  |
| X7A6 | X006 | Slot #7 Glass 감지 | Y7A6 | Y006 |  |
| X7A7 | X007 | Slot #8 Glass 감지 | Y7A7 | Y007 |  |
| X7A8 | X008 | Slot #9 Glass 감지 | Y7A8 | Y008 |  |
| X7A9 | X009 | Slot #10 Glass 감지 | Y7A9 | Y009 |  |
| X7AA | X00A | Slot #11 Glass 감지 | Y7AA | Y00A |  |
| X7AB | X00B | Slot #12 Glass 감지 | Y7AB | Y00B |  |
| X7AC | X00C | Slot #13 Glass 감지 | Y7AC | Y00C |  |
| X7AD | X00D | Slot #14 Glass 감지 | Y7AD | Y00D |  |
| X7AE | X00E | Slot #15 Glass 감지 | Y7AE | Y00E |  |
| X7AF | X00F | Slot #16 Glass 감지 | Y7AF | Y00F |  |
| X7B0 | X010 | Slot #17 Glass 감지 | Y7B0 | Y010 |  |
| X7B1 | X011 | Slot #18 Glass 감지 | Y7B1 | Y011 |  |
| X7B2 | X012 | Slot #19 Glass 감지 | Y7B2 | Y012 |  |
| X7B3 | X013 | Slot #20 Glass 감지 | Y7B3 | Y013 |  |
| X7B4 | X014 | Slot #21 Glass 감지 | Y7B4 | Y014 |  |
| X7B5 | X015 | Slot #22 Glass 감지 | Y7B5 | Y015 |  |
| X7B6 | X016 | Slot #23 Glass 감지 | Y7B6 | Y016 |  |
| X7B7 | X017 | Slot #24 Glass 감지 | Y7B7 | Y017 |  |
| X7B8 | X018 | Slot #25 Glass 감지 | Y7B8 | Y018 |  |
| X7B9 | X019 | Slot #26 Glass 감지 | Y7B9 | Y019 |  |
| X7BA | X01A | Slot #27 Glass 감지 | Y7BA | Y01A |  |
| X7BB | X01B | Slot #28 Glass 감지 | Y7BB | Y01B |  |
| X7BC | X01C | Slot #29 Glass 감지 | Y7BC | Y01C |  |
| X7BD | X01D | Slot #30 Glass 감지 | Y7BD | Y01D |  |
| X7BE | X01E |  | Y7BE | Y01E |  |
| X7BF | X01F |  | Y7BF | Y01F |  |

| Station No. |  | 54 |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Station Type |  | Remote I/O |  |  |  |
| 용 도 |  | Port #5 Mapping Unit |  |  |  |
| 입력 |  | Name | 출력 |  | Name |
| Master | Remote |  | Master | Remote |  |
| X7C0 | X000 | Slot #31 Glass 감지 | Y7C0 | Y000 |  |
| X7C1 | X001 | Slot #32 Glass 감지 | Y7C1 | Y001 |  |
| X7C2 | X002 | Slot #33 Glass 감지 | Y7C2 | Y002 |  |
| X7C3 | X003 | Slot #34 Glass 감지 | Y7C3 | Y003 |  |
| X7C4 | X004 | Slot #35 Glass 감지 | Y7C4 | Y004 |  |
| X7C5 | X005 | Slot #36 Glass 감지 | Y7C5 | Y005 |  |
| X7C6 | X006 | Slot #37 Glass 감지 | Y7C6 | Y006 |  |
| X7C7 | X007 | Slot #38 Glass 감지 | Y7C7 | Y007 |  |
| X7C8 | X008 | Slot #39 Glass 감지 | Y7C8 | Y008 |  |
| X7C9 | X009 | Slot #40 Glass 감지 | Y7C9 | Y009 |  |
| X7CA | X00A | Slot #41 Glass 감지 | Y7CA | Y00A |  |
| X7CB | X00B | Slot #42 Glass 감지 | Y7CB | Y00B |  |
| X7CC | X00C | Slot #43 Glass 감지 | Y7CC | Y00C |  |
| X7CD | X00D | Slot #44 Glass 감지 | Y7CD | Y00D |  |
| X7CE | X00E | Slot #45 Glass 감지 | Y7CE | Y00E |  |
| X7CF | X00F | Slot #46 Glass 감지 | Y7CF | Y00F |  |
| X7D0 | X010 | Slot #47 Glass 감지 | Y7D0 | Y010 |  |
| X7D1 | X011 | Slot #48 Glass 감지 | Y7D1 | Y011 |  |
| X7D2 | X012 | Slot #49 Glass 감지 | Y7D2 | Y012 |  |
| X7D3 | X013 | Slot #50 Glass 감지 | Y7D3 | Y013 |  |
| X7D4 | X014 | Slot #51 Glass 감지 | Y7D4 | Y014 |  |
| X7D5 | X015 | Slot #52 Glass 감지 | Y7D5 | Y015 |  |
| X7D6 | X016 | Slot #53 Glass 감지 | Y7D6 | Y016 |  |
| X7D7 | X017 | Slot #54 Glass 감지 | Y7D7 | Y017 |  |
| X7D8 | X018 | Slot #55 Glass 감지 | Y7D8 | Y018 |  |
| X7D9 | X019 | Slot #56 Glass 감지 | Y7D9 | Y019 |  |
| X7DA | X01A | Slot #57 Glass 감지 | Y7DA | Y01A |  |
| X7DB | X01B | Slot #58 Glass 감지 | Y7DB | Y01B |  |
| X7DC | X01C | Slot #59 Glass 감지 | Y7DC | Y01C |  |
| X7DD | X01D | Slot #60 Glass 감지 | Y7DD | Y01D |  |
| X7DE | X01E |  | Y7DE | Y01E |  |
| X7DF | X01F |  | Y7DF | Y01F |  |

| Station No. |  | 55 |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Station Type |  | Remote I/O |  |  |  |
| 용 도 |  | Buffer #1 Mapping Unit |  |  |  |
| 입력 |  | Name | 출력 |  | Name |
| Master | Remote |  | Master | Remote |  |
| X7E0 | X000 | Slot #1 Glass 감지 | Y7E0 | Y000 |  |
| X7E1 | X001 | Slot #2 Glass 감지 | Y7E1 | Y001 |  |
| X7E2 | X002 | Slot #3 Glass 감지 | Y7E2 | Y002 |  |
| X7E3 | X003 | Slot #4 Glass 감지 | Y7E3 | Y003 |  |
| X7E4 | X004 | Slot #5 Glass 감지 | Y7E4 | Y004 |  |
| X7E5 | X005 | Slot #6 Glass 감지 | Y7E5 | Y005 |  |
| X7E6 | X006 | Slot #7 Glass 감지 | Y7E6 | Y006 |  |
| X7E7 | X007 | Slot #8 Glass 감지 | Y7E7 | Y007 |  |
| X7E8 | X008 | Slot #9 Glass 감지 | Y7E8 | Y008 |  |
| X7E9 | X009 | Slot #10 Glass 감지 | Y7E9 | Y009 |  |
| X7EA | X00A | Slot #11 Glass 감지 | Y7EA | Y00A |  |
| X7EB | X00B | Slot #12 Glass 감지 | Y7EB | Y00B |  |
| X7EC | X00C | Slot #13 Glass 감지 | Y7EC | Y00C |  |
| X7ED | X00D | Slot #14 Glass 감지 | Y7ED | Y00D |  |
| X7EE | X00E | Slot #15 Glass 감지 | Y7EE | Y00E |  |
| X7EF | X00F | Slot #16 Glass 감지 | Y7EF | Y00F |  |
| X7F0 | X010 | Slot #17 Glass 감지 | Y7F0 | Y010 |  |
| X7F1 | X011 | Slot #18 Glass 감지 | Y7F1 | Y011 |  |
| X7F2 | X012 | Slot #19 Glass 감지 | Y7F2 | Y012 |  |
| X7F3 | X013 | Slot #20 Glass 감지 | Y7F3 | Y013 |  |
| X7F4 | X014 | Slot #21 Glass 감지 | Y7F4 | Y014 |  |
| X7F5 | X015 | Slot #22 Glass 감지 | Y7F5 | Y015 |  |
| X7F6 | X016 | Slot #23 Glass 감지 | Y7F6 | Y016 |  |
| X7F7 | X017 | Slot #24 Glass 감지 | Y7F7 | Y017 |  |
| X7F8 | X018 |  | Y7F8 | Y018 |  |
| X7F9 | X019 |  | Y7F9 | Y019 |  |
| X7FA | X01A |  | Y7FA | Y01A |  |
| X7FB | X01B |  | Y7FB | Y01B |  |
| X7FC | X01C |  | Y7FC | Y01C |  |
| X7FD | X01D |  | Y7FD | Y01D |  |
| X7FE | X01E |  | Y7FE | Y01E |  |
| X7FF | X01F |  | Y7FF | Y01F |  |

| Station No. |  | 56 |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Station Type |  | Remote I/O |  |  |  |
| 용 도 |  | Buffer #2 Mapping Unit |  |  |  |
| 입력 |  | Name | 출력 |  | Name |
| Master | Remote |  | Master | Remote |  |
| X800 | X000 | Slot #1 Glass 감지 | Y800 | Y000 |  |
| X801 | X001 | Slot #2 Glass 감지 | Y801 | Y001 |  |
| X802 | X002 | Slot #3 Glass 감지 | Y802 | Y002 |  |
| X803 | X003 | Slot #4 Glass 감지 | Y803 | Y003 |  |
| X804 | X004 | Slot #5 Glass 감지 | Y804 | Y004 |  |
| X805 | X005 | Slot #6 Glass 감지 | Y805 | Y005 |  |
| X806 | X006 | Slot #7 Glass 감지 | Y806 | Y006 |  |
| X807 | X007 | Slot #8 Glass 감지 | Y807 | Y007 |  |
| X808 | X008 | Slot #9 Glass 감지 | Y808 | Y008 |  |
| X809 | X009 | Slot #10 Glass 감지 | Y809 | Y009 |  |
| X80A | X00A | Slot #11 Glass 감지 | Y80A | Y00A |  |
| X80B | X00B | Slot #12 Glass 감지 | Y80B | Y00B |  |
| X80C | X00C | Slot #13 Glass 감지 | Y80C | Y00C |  |
| X80D | X00D | Slot #14 Glass 감지 | Y80D | Y00D |  |
| X80E | X00E | Slot #15 Glass 감지 | Y80E | Y00E |  |
| X80F | X00F | Slot #16 Glass 감지 | Y80F | Y00F |  |
| X810 | X010 | Slot #17 Glass 감지 | Y810 | Y010 |  |
| X811 | X011 | Slot #18 Glass 감지 | Y811 | Y011 |  |
| X812 | X012 | Slot #19 Glass 감지 | Y812 | Y012 |  |
| X813 | X013 | Slot #20 Glass 감지 | Y813 | Y013 |  |
| X814 | X014 | Slot #21 Glass 감지 | Y814 | Y014 |  |
| X815 | X015 | Slot #22 Glass 감지 | Y815 | Y015 |  |
| X816 | X016 | Slot #23 Glass 감지 | Y816 | Y016 |  |
| X817 | X017 | Slot #24 Glass 감지 | Y817 | Y017 |  |
| X818 | X018 |  | Y818 | Y018 |  |
| X819 | X019 |  | Y819 | Y019 |  |
| X81A | X01A |  | Y81A | Y01A |  |
| X81B | X01B |  | Y81B | Y01B |  |
| X81C | X01C |  | Y81C | Y01C |  |
| X81D | X01D |  | Y81D | Y01D |  |
| X81E | X01E |  | Y81E | Y01E |  |
| X81F | X01F |  | Y81F | Y01F |  |

| Station No. |  | 57 |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Station Type |  | Remote I/O |  |  |  |
| 용 도 |  | Buffer #3 Mapping Unit |  |  |  |
| 입력 |  | Name | 출력 |  | Name |
| Master | Remote |  | Master | Remote |  |
| X820 | X000 | Slot #1 Glass 감지 | Y820 | Y000 |  |
| X821 | X001 | Slot #2 Glass 감지 | Y821 | Y001 |  |
| X822 | X002 | Slot #3 Glass 감지 | Y822 | Y002 |  |
| X823 | X003 | Slot #4 Glass 감지 | Y823 | Y003 |  |
| X824 | X004 | Slot #5 Glass 감지 | Y824 | Y004 |  |
| X825 | X005 | Slot #6 Glass 감지 | Y825 | Y005 |  |
| X826 | X006 | Slot #7 Glass 감지 | Y826 | Y006 |  |
| X827 | X007 | Slot #8 Glass 감지 | Y827 | Y007 |  |
| X828 | X008 | Slot #9 Glass 감지 | Y828 | Y008 |  |
| X829 | X009 | Slot #10 Glass 감지 | Y829 | Y009 |  |
| X82A | X00A | Slot #11 Glass 감지 | Y82A | Y00A |  |
| X82B | X00B | Slot #12 Glass 감지 | Y82B | Y00B |  |
| X82C | X00C | Slot #13 Glass 감지 | Y82C | Y00C |  |
| X82D | X00D | Slot #14 Glass 감지 | Y82D | Y00D |  |
| X82E | X00E | Slot #15 Glass 감지 | Y82E | Y00E |  |
| X82F | X00F | Slot #16 Glass 감지 | Y82F | Y00F |  |
| X830 | X010 | Slot #17 Glass 감지 | Y830 | Y010 |  |
| X831 | X011 | Slot #18 Glass 감지 | Y831 | Y011 |  |
| X832 | X012 | Slot #19 Glass 감지 | Y832 | Y012 |  |
| X833 | X013 | Slot #20 Glass 감지 | Y833 | Y013 |  |
| X834 | X014 | Slot #21 Glass 감지 | Y834 | Y014 |  |
| X835 | X015 | Slot #22 Glass 감지 | Y835 | Y015 |  |
| X836 | X016 | Slot #23 Glass 감지 | Y836 | Y016 |  |
| X837 | X017 | Slot #24 Glass 감지 | Y837 | Y017 |  |
| X838 | X018 |  | Y838 | Y018 |  |
| X839 | X019 |  | Y839 | Y019 |  |
| X83A | X01A |  | Y83A | Y01A |  |
| X83B | X01B |  | Y83B | Y01B |  |
| X83C | X01C |  | Y83C | Y01C |  |
| X83D | X01D |  | Y83D | Y01D |  |
| X83E | X01E |  | Y83E | Y01E |  |
| X83F | X01F |  | Y83F | Y01F |  |

| Station No. |  | 58 |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Station Type |  | Remote I/O |  |  |  |
| 용 도 |  | Buffer #4 Mapping Unit |  |  |  |
| 입력 |  | Name | 출력 |  | Name |
| Master | Remote |  | Master | Remote |  |
| X840 | X000 | Slot #1 Glass 감지 | Y840 | Y000 |  |
| X841 | X001 | Slot #2 Glass 감지 | Y841 | Y001 |  |
| X842 | X002 | Slot #3 Glass 감지 | Y842 | Y002 |  |
| X843 | X003 | Slot #4 Glass 감지 | Y843 | Y003 |  |
| X844 | X004 | Slot #5 Glass 감지 | Y844 | Y004 |  |
| X845 | X005 | Slot #6 Glass 감지 | Y845 | Y005 |  |
| X846 | X006 | Slot #7 Glass 감지 | Y846 | Y006 |  |
| X847 | X007 | Slot #8 Glass 감지 | Y847 | Y007 |  |
| X848 | X008 | Slot #9 Glass 감지 | Y848 | Y008 |  |
| X849 | X009 | Slot #10 Glass 감지 | Y849 | Y009 |  |
| X84A | X00A | Slot #11 Glass 감지 | Y84A | Y00A |  |
| X84B | X00B | Slot #12 Glass 감지 | Y84B | Y00B |  |
| X84C | X00C | Slot #13 Glass 감지 | Y84C | Y00C |  |
| X84D | X00D | Slot #14 Glass 감지 | Y84D | Y00D |  |
| X84E | X00E | Slot #15 Glass 감지 | Y84E | Y00E |  |
| X84F | X00F | Slot #16 Glass 감지 | Y84F | Y00F |  |
| X850 | X010 | Slot #17 Glass 감지 | Y850 | Y010 |  |
| X851 | X011 | Slot #18 Glass 감지 | Y851 | Y011 |  |
| X852 | X012 | Slot #19 Glass 감지 | Y852 | Y012 |  |
| X853 | X013 | Slot #20 Glass 감지 | Y853 | Y013 |  |
| X854 | X014 | Slot #21 Glass 감지 | Y854 | Y014 |  |
| X855 | X015 | Slot #22 Glass 감지 | Y855 | Y015 |  |
| X856 | X016 | Slot #23 Glass 감지 | Y856 | Y016 |  |
| X857 | X017 | Slot #24 Glass 감지 | Y857 | Y017 |  |
| X858 | X018 |  | Y858 | Y018 |  |
| X859 | X019 |  | Y859 | Y019 |  |
| X85A | X01A |  | Y85A | Y01A |  |
| X85B | X01B |  | Y85B | Y01B |  |
| X85C | X01C |  | Y85C | Y01C |  |
| X85D | X01D |  | Y85D | Y01D |  |
| X85E | X01E |  | Y85E | Y01E |  |
| X85F | X01F |  | Y85F | Y01F |  |

| Station No. |  | 59 |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Station Type |  | Remote I/O |  |  |  |
| 용 도 |  | Buffer #5 Mapping Unit |  |  |  |
| 입력 |  | Name | 출력 |  | Name |
| Master | Remote |  | Master | Remote |  |
| X860 | X000 | Slot #1 Glass 감지 | Y860 | Y000 |  |
| X861 | X001 | Slot #2 Glass 감지 | Y861 | Y001 |  |
| X862 | X002 | Slot #3 Glass 감지 | Y862 | Y002 |  |
| X863 | X003 | Slot #4 Glass 감지 | Y863 | Y003 |  |
| X864 | X004 | Slot #5 Glass 감지 | Y864 | Y004 |  |
| X865 | X005 | Slot #6 Glass 감지 | Y865 | Y005 |  |
| X866 | X006 | Slot #7 Glass 감지 | Y866 | Y006 |  |
| X867 | X007 | Slot #8 Glass 감지 | Y867 | Y007 |  |
| X868 | X008 | Slot #9 Glass 감지 | Y868 | Y008 |  |
| X869 | X009 | Slot #10 Glass 감지 | Y869 | Y009 |  |
| X86A | X00A | Slot #11 Glass 감지 | Y86A | Y00A |  |
| X86B | X00B | Slot #12 Glass 감지 | Y86B | Y00B |  |
| X86C | X00C | Slot #13 Glass 감지 | Y86C | Y00C |  |
| X86D | X00D | Slot #14 Glass 감지 | Y86D | Y00D |  |
| X86E | X00E | Slot #15 Glass 감지 | Y86E | Y00E |  |
| X86F | X00F | Slot #16 Glass 감지 | Y86F | Y00F |  |
| X870 | X010 | Slot #17 Glass 감지 | Y870 | Y010 |  |
| X871 | X011 | Slot #18 Glass 감지 | Y871 | Y011 |  |
| X872 | X012 | Slot #19 Glass 감지 | Y872 | Y012 |  |
| X873 | X013 | Slot #20 Glass 감지 | Y873 | Y013 |  |
| X874 | X014 | Slot #21 Glass 감지 | Y874 | Y014 |  |
| X875 | X015 | Slot #22 Glass 감지 | Y875 | Y015 |  |
| X876 | X016 | Slot #23 Glass 감지 | Y876 | Y016 |  |
| X877 | X017 | Slot #24 Glass 감지 | Y877 | Y017 |  |
| X878 | X018 |  | Y878 | Y018 |  |
| X879 | X019 |  | Y879 | Y019 |  |
| X87A | X01A |  | Y87A | Y01A |  |
| X87B | X01B |  | Y87B | Y01B |  |
| X87C | X01C |  | Y87C | Y01C |  |
| X87D | X01D |  | Y87D | Y01D |  |
| X87E | X01E |  | Y87E | Y01E |  |
| X87F | X01F |  | Y87F | Y01F |  |

| Station No. |  | 60 |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Station Type |  | Remote I/O |  |  |  |
| 용 도 |  | Buffer #6 Mapping Unit |  |  |  |
| 입력 |  | Name | 출력 |  | Name |
| Master | Remote |  | Master | Remote |  |
| X880 | X000 | Slot #1 Glass 감지 | Y880 | Y000 |  |
| X881 | X001 | Slot #2 Glass 감지 | Y881 | Y001 |  |
| X882 | X002 | Slot #3 Glass 감지 | Y882 | Y002 |  |
| X883 | X003 | Slot #4 Glass 감지 | Y883 | Y003 |  |
| X884 | X004 | Slot #5 Glass 감지 | Y884 | Y004 |  |
| X885 | X005 | Slot #6 Glass 감지 | Y885 | Y005 |  |
| X886 | X006 | Slot #7 Glass 감지 | Y886 | Y006 |  |
| X887 | X007 | Slot #8 Glass 감지 | Y887 | Y007 |  |
| X888 | X008 | Slot #9 Glass 감지 | Y888 | Y008 |  |
| X889 | X009 | Slot #10 Glass 감지 | Y889 | Y009 |  |
| X88A | X00A | Slot #11 Glass 감지 | Y88A | Y00A |  |
| X88B | X00B | Slot #12 Glass 감지 | Y88B | Y00B |  |
| X88C | X00C | Slot #13 Glass 감지 | Y88C | Y00C |  |
| X88D | X00D | Slot #14 Glass 감지 | Y88D | Y00D |  |
| X88E | X00E | Slot #15 Glass 감지 | Y88E | Y00E |  |
| X88F | X00F | Slot #16 Glass 감지 | Y88F | Y00F |  |
| X890 | X010 | Slot #17 Glass 감지 | Y890 | Y010 |  |
| X891 | X011 | Slot #18 Glass 감지 | Y891 | Y011 |  |
| X892 | X012 | Slot #19 Glass 감지 | Y892 | Y012 |  |
| X893 | X013 | Slot #20 Glass 감지 | Y893 | Y013 |  |
| X894 | X014 | Slot #21 Glass 감지 | Y894 | Y014 |  |
| X895 | X015 | Slot #22 Glass 감지 | Y895 | Y015 |  |
| X896 | X016 | Slot #23 Glass 감지 | Y896 | Y016 |  |
| X897 | X017 | Slot #24 Glass 감지 | Y897 | Y017 |  |
| X898 | X018 |  | Y898 | Y018 |  |
| X899 | X019 |  | Y899 | Y019 |  |
| X89A | X01A |  | Y89A | Y01A |  |
| X89B | X01B |  | Y89B | Y01B |  |
| X89C | X01C |  | Y89C | Y01C |  |
| X89D | X01D |  | Y89D | Y01D |  |
| X89E | X01E |  | Y89E | Y01E |  |
| X89F | X01F |  | Y89F | Y01F |  |

| Station No. |  | 61 |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Station Type |  | Remote I/O |  |  |  |
| 용 도 |  | Buffer #7 Mapping Unit |  |  |  |
| 입력 |  | Name | 출력 |  | Name |
| Master | Remote |  | Master | Remote |  |
| X8A0 | X000 | Slot #1 Glass 감지 | Y8A0 | Y000 |  |
| X8A1 | X001 | Slot #2 Glass 감지 | Y8A1 | Y001 |  |
| X8A2 | X002 | Slot #3 Glass 감지 | Y8A2 | Y002 |  |
| X8A3 | X003 | Slot #4 Glass 감지 | Y8A3 | Y003 |  |
| X8A4 | X004 | Slot #5 Glass 감지 | Y8A4 | Y004 |  |
| X8A5 | X005 | Slot #6 Glass 감지 | Y8A5 | Y005 |  |
| X8A6 | X006 | Slot #7 Glass 감지 | Y8A6 | Y006 |  |
| X8A7 | X007 | Slot #8 Glass 감지 | Y8A7 | Y007 |  |
| X8A8 | X008 | Slot #9 Glass 감지 | Y8A8 | Y008 |  |
| X8A9 | X009 | Slot #10 Glass 감지 | Y8A9 | Y009 |  |
| X8AA | X00A | Slot #11 Glass 감지 | Y8AA | Y00A |  |
| X8AB | X00B | Slot #12 Glass 감지 | Y8AB | Y00B |  |
| X8AC | X00C | Slot #13 Glass 감지 | Y8AC | Y00C |  |
| X8AD | X00D | Slot #14 Glass 감지 | Y8AD | Y00D |  |
| X8AE | X00E | Slot #15 Glass 감지 | Y8AE | Y00E |  |
| X8AF | X00F | Slot #16 Glass 감지 | Y8AF | Y00F |  |
| X8B0 | X010 | Slot #17 Glass 감지 | Y8B0 | Y010 |  |
| X8B1 | X011 | Slot #18 Glass 감지 | Y8B1 | Y011 |  |
| X8B2 | X012 | Slot #19 Glass 감지 | Y8B2 | Y012 |  |
| X8B3 | X013 | Slot #20 Glass 감지 | Y8B3 | Y013 |  |
| X8B4 | X014 | Slot #21 Glass 감지 | Y8B4 | Y014 |  |
| X8B5 | X015 | Slot #22 Glass 감지 | Y8B5 | Y015 |  |
| X8B6 | X016 | Slot #23 Glass 감지 | Y8B6 | Y016 |  |
| X8B7 | X017 | Slot #24 Glass 감지 | Y8B7 | Y017 |  |
| X8B8 | X018 |  | Y8B8 | Y018 |  |
| X8B9 | X019 |  | Y8B9 | Y019 |  |
| X8BA | X01A |  | Y8BA | Y01A |  |
| X8BB | X01B |  | Y8BB | Y01B |  |
| X8BC | X01C |  | Y8BC | Y01C |  |
| X8BD | X01D |  | Y8BD | Y01D |  |
| X8BE | X01E |  | Y8BE | Y01E |  |
| X8BF | X01F |  | Y8BF | Y01F |  |

| Station No. |  | 62 |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Station Type |  | Remote I/O |  |  |  |
| 용 도 |  | Buffer #8 Mapping Unit |  |  |  |
| 입력 |  | Name | 출력 |  | Name |
| Master | Remote |  | Master | Remote |  |
| X8C0 | X000 | Slot #1 Glass 감지 | Y8C0 | Y000 |  |
| X8C1 | X001 | Slot #2 Glass 감지 | Y8C1 | Y001 |  |
| X8C2 | X002 | Slot #3 Glass 감지 | Y8C2 | Y002 |  |
| X8C3 | X003 | Slot #4 Glass 감지 | Y8C3 | Y003 |  |
| X8C4 | X004 | Slot #5 Glass 감지 | Y8C4 | Y004 |  |
| X8C5 | X005 | Slot #6 Glass 감지 | Y8C5 | Y005 |  |
| X8C6 | X006 | Slot #7 Glass 감지 | Y8C6 | Y006 |  |
| X8C7 | X007 | Slot #8 Glass 감지 | Y8C7 | Y007 |  |
| X8C8 | X008 | Slot #9 Glass 감지 | Y8C8 | Y008 |  |
| X8C9 | X009 | Slot #10 Glass 감지 | Y8C9 | Y009 |  |
| X8CA | X00A | Slot #11 Glass 감지 | Y8CA | Y00A |  |
| X8CB | X00B | Slot #12 Glass 감지 | Y8CB | Y00B |  |
| X8CC | X00C | Slot #13 Glass 감지 | Y8CC | Y00C |  |
| X8CD | X00D | Slot #14 Glass 감지 | Y8CD | Y00D |  |
| X8CE | X00E | Slot #15 Glass 감지 | Y8CE | Y00E |  |
| X8CF | X00F | Slot #16 Glass 감지 | Y8CF | Y00F |  |
| X8D0 | X010 | Slot #17 Glass 감지 | Y8D0 | Y010 |  |
| X8D1 | X011 | Slot #18 Glass 감지 | Y8D1 | Y011 |  |
| X8D2 | X012 | Slot #19 Glass 감지 | Y8D2 | Y012 |  |
| X8D3 | X013 | Slot #20 Glass 감지 | Y8D3 | Y013 |  |
| X8D4 | X014 | Slot #21 Glass 감지 | Y8D4 | Y014 |  |
| X8D5 | X015 | Slot #22 Glass 감지 | Y8D5 | Y015 |  |
| X8D6 | X016 | Slot #23 Glass 감지 | Y8D6 | Y016 |  |
| X8D7 | X017 | Slot #24 Glass 감지 | Y8D7 | Y017 |  |
| X8D8 | X018 |  | Y8D8 | Y018 |  |
| X8D9 | X019 |  | Y8D9 | Y019 |  |
| X8DA | X01A |  | Y8DA | Y01A |  |
| X8DB | X01B |  | Y8DB | Y01B |  |
| X8DC | X01C |  | Y8DC | Y01C |  |
| X8DD | X01D |  | Y8DD | Y01D |  |
| X8DE | X01E |  | Y8DE | Y01E |  |
| X8DF | X01F |  | Y8DF | Y01F |  |

| Station No. |  |  | 63 |  |  |
| --- | --- | --- | --- | --- | --- |
| Station Type |  |  | Remote I/O(AJ65SBTB1-32D1) |  |  |
| 용 도 |  |  | Equipment #1~4 RAE & Buffer Sensor Input(Loader) |  |  |
| 구분 | TB No. | Address | Name | 접점종류 | 비고 |
| CC-Link | TB01 |  | DA |  |  |
|  | TB02 |  | DB |  |  |
|  | TB03 |  | DG |  |  |
|  | TB04 |  | SLD |  |  |
|  | TB05 |  | DC 24V |  |  |
|  | TB06 |  | FG |  |  |
|  | TB07 |  | DC 0V |  |  |
| 입력 | TB08 | X8E0 | Robot Action Enable #1 |  | Equipment #1 |
|  | TB09 | X8E1 | Robot Action Enable #2 |  |  |
|  | TB10 | X8E2 | Robot Action Enable #3 |  |  |
|  | TB11 | X8E3 | Robot Action Enable #4 |  |  |
|  | TB12 | X8E4 | Robot Action Enable #1 |  | Equipment #2 |
|  | TB13 | X8E5 | Robot Action Enable #2 |  |  |
|  | TB14 | X8E6 | Robot Action Enable #3 |  |  |
|  | TB15 | X8E7 | Robot Action Enable #4 |  |  |
|  | TB16 | X8E8 | Robot Action Enable #1 |  | Equipment #3 |
|  | TB17 | X8E9 | Robot Action Enable #2 |  |  |
|  | TB18 | X8EA | Robot Action Enable #3 |  |  |
|  | TB19 | X8EB | Robot Action Enable #4 |  |  |
|  | TB20 | X8EC | Robot Action Enable #1 |  | Equipment #4 |
|  | TB21 | X8ED | Robot Action Enable #2 |  |  |
|  | TB22 | X8EE | Robot Action Enable #3 |  |  |
|  | TB23 | X8EF | Robot Action Enable #4 |  |  |
|  | TB24 | X8F0 | Buffer Stage #1 Sensor On |  | Buffer Stage Sensor Input (Loader) |
|  | TB25 | X8F1 | Buffer Stage #2 Sensor On |  |  |
|  | TB26 | X8F2 | Buffer Stage #3 Sensor On |  |  |
|  | TB27 | X8F3 | Buffer Stage #4 Sensor On |  |  |
|  | TB28 | X8F4 | Buffer Stage #5 Sensor On |  |  |
|  | TB29 | X8F5 | Buffer Stage #6 Sensor On |  |  |
|  | TB30 | X8F6 | Buffer Stage #7 Sensor On |  |  |
|  | TB31 | X8F7 | Buffer Stage #8 Sensor On |  |  |
|  | TB32 | X8F8 | Buffer Stage #9 Sensor On |  |  |
|  | TB33 | X8F9 | Buffer Stage #10 Sensor On |  |  |
|  | TB34 | X8FA | Buffer Stage #11 Sensor On |  |  |
|  | TB35 | X8FB | Buffer Stage #12 Sensor On |  |  |
|  | TB36 | X8FC | Buffer Stage #13 Sensor On |  |  |
|  | TB37 | X8FD | Buffer Stage #14 Sensor On |  |  |
|  | TB38 | X8FE | Buffer Stage #15 Sensor On |  |  |
|  | TB39 | X8FF | Buffer Stage #16 Sensor On |  |  |
| COM | TB40 | COM+ | DC 24V(P24) |  |  |
|  | TB41 | COM- | DC 0V(N24) |  |  |

| Station No. |  |  | 64 |  |  |
| --- | --- | --- | --- | --- | --- |
| Station Type |  |  | Remote I/O(AJ65SBTB1-32D1) |  |  |
| 용 도 |  |  | Equipment #1~4 RAE & Buffer Sensor Input(Unloader) |  |  |
| 구분 | TB No. | Address | Name | 접점종류 | 비고 |
| CC-Link | TB01 |  | DA |  |  |
|  | TB02 |  | DB |  |  |
|  | TB03 |  | DG |  |  |
|  | TB04 |  | SLD |  |  |
|  | TB05 |  | DC 24V |  |  |
|  | TB06 |  | FG |  |  |
|  | TB07 |  | DC 0V |  |  |
| 입력 | TB08 | X900 | Robot Action Enable #1 |  | Equipment #1 |
|  | TB09 | X901 | Robot Action Enable #2 |  |  |
|  | TB10 | X902 | Robot Action Enable #3 |  |  |
|  | TB11 | X903 | Robot Action Enable #4 |  |  |
|  | TB12 | X904 | Robot Action Enable #1 |  | Equipment #2 |
|  | TB13 | X905 | Robot Action Enable #2 |  |  |
|  | TB14 | X906 | Robot Action Enable #3 |  |  |
|  | TB15 | X907 | Robot Action Enable #4 |  |  |
|  | TB16 | X908 | Robot Action Enable #1 |  | Equipment #3 |
|  | TB17 | X909 | Robot Action Enable #2 |  |  |
|  | TB18 | X90A | Robot Action Enable #3 |  |  |
|  | TB19 | X90B | Robot Action Enable #4 |  |  |
|  | TB20 | X90C | Robot Action Enable #1 |  | Equipment #4 |
|  | TB21 | X90D | Robot Action Enable #2 |  |  |
|  | TB22 | X90E | Robot Action Enable #3 |  |  |
|  | TB23 | X90F | Robot Action Enable #4 |  |  |
|  | TB24 | X910 | Buffer Stage #1 Sensor On |  | Buffer Stage Sensor Input (Unloader) |
|  | TB25 | X911 | Buffer Stage #2 Sensor On |  |  |
|  | TB26 | X912 | Buffer Stage #3 Sensor On |  |  |
|  | TB27 | X913 | Buffer Stage #4 Sensor On |  |  |
|  | TB28 | X914 | Buffer Stage #5 Sensor On |  |  |
|  | TB29 | X915 | Buffer Stage #6 Sensor On |  |  |
|  | TB30 | X916 | Buffer Stage #7 Sensor On |  |  |
|  | TB31 | X917 | Buffer Stage #8 Sensor On |  |  |
|  | TB32 | X918 | Buffer Stage #9 Sensor On |  |  |
|  | TB33 | X919 | Buffer Stage #10 Sensor On |  |  |
|  | TB34 | X91A | Buffer Stage #11 Sensor On |  |  |
|  | TB35 | X91B | Buffer Stage #12 Sensor On |  |  |
|  | TB36 | X91C | Buffer Stage #13 Sensor On |  |  |
|  | TB37 | X91D | Buffer Stage #14 Sensor On |  |  |
|  | TB38 | X91E | Buffer Stage #15 Sensor On |  |  |
|  | TB39 | X91F | Buffer Stage #16 Sensor On |  |  |
| COM | TB40 | COM+ | DC 24V(P24) |  |  |
|  | TB41 | COM- | DC 0V(N24) |  |  |

# ※ Port Stage 구성

STK 측

Clamp2-1전진

Clamp2-1후진

Clamp1-2전진

Cassette

Clamp1-1후진

Clamp1-1전진

CST 감지(후)

Clamp1-2후진

CST 감지(전)

Robot 측

Cylinder 전진 (Clamp)

Sensor

|  | Port #1 | Port #2 | Port #3 | Port #4 | Port #5 | Port #6 |
| --- | --- | --- | --- | --- | --- | --- |
| CST 감지 전 | X560 | X568 | X580 | X588 | X5A0 | X5A8 |
| CST 감지 후 | X561 | X569 | X581 | X589 | X5A1 | X5A9 |
| Clamp 1-1 전진 | X562 | X56A | X582 | X58A | X5A2 | X5AA |
| Clamp 1-1 후진 | X563 | X56B | X583 | X58B | X5A3 | X5AB |
| Clamp 1-2 전진 | X564 | X56C | X584 | X58C | X5A4 | X5AC |
| Clamp 1-2 후진 | X565 | X56D | X585 | X58D | X5A5 | X5AD |
| Clamp 2-1 전진 | X566 | X56E | X586 | X58E | X5A6 | X5AE |
| Clamp 2-1 후진 | X567 | X56F | X587 | X58F | X5A7 | X5AF |